3GPP TSG RAN WG4 Meeting #104bis-e	R4-2216642

Electronic Meeting, October 10th – 19th, 2022

Source:			Rohde & Schwarz

Title:			Discussion on FR2 methods for UEs with multi-panel reception

Agenda Item:	6.5.2

Document for:	Endorsement

# Introduction

The study on NR FR2 OTA testing enhancements for UEs with multi-panel reception and 4DL layer [1] was revised during RAN #97-e meeting with the following objectives:

The objectives for FR2-1 OTA testing for UEs with multi-panel reception and 4DL layer are as follows.

- Define a test methodology for RF/RRM/Demodulation requirements testing for devices that can receive simultaneously from multiple Angle of Arrival (AoA)

  - The multiple AoA test setup should enable testing of up to 2 DL Layers with dual polarization for each angle

  - For RRM, the target should be to allow testing of 4 AoAs with 2 simultaneously active AoAs.

  - Define a test methodology for up to 4 DL MIMO layer demodulation testing

  - Note: Revisit whether or not to include the case of transmitting simultaneously in RAN#98

- Smartphone form factor should be the first priority, other UE types should also be discussed as 2nd priority

- Develop the related preliminary uncertainty assessments for the test methodologies

- FR2 test methods defined in TR 38.810 and TR 38.884 should be used as the baseline.

- The tests shall take the test system reuse, test system complexity and test time into account to keep the whole test costs within a reasonable level.

The WF in [2] listed a number of topics for further discussion, among which the feasibility analysis of supporting full rotational degrees of freedom for simultaneously two active AoA is the most critical one to make progress. We present in this contribution our views regarding this feasibility analysis, and other topics that would help simplify and/or clarify the approach.

# Feasibility analysis for full degrees of freedom

During RAN4#104-e meeting, several companies requested to study the feasibility of an FR2 test system enabling full degrees of freedom for a 2 active AoA environment for all three cases: RF, RRM and Demodulation. Although this topic was discussed already during the original FR2 Testability SI related to single active AoA to separate link from measurement antenna, and/or interferer antenna for blocking test cases (e.g. ACS, blocking, etc.), we present hereby a description of a potential implementation in order to clarify the concerns raised during last meeting.

In the following figure, a high-level sketch of such a system enabling full degrees of freedom for a 2 active AoA is presented. DFF is used as methodology for both AoA only for illustration purposes.

Figure 2-1: Example of 2AoA DFF test system with full degrees of freedom

The first AoA (AoA1) is presented with a typical distributed axes system where the elevation (θ1) is implemented with an antenna mast and the azimuth (ϕ1) with a turntable where the DUT is placed on.

The second AoA (AoA2) would require an independent positioning system combining elevation and azimuth, where the positioner for the elevation (θ2) is mounted on top of a rotation to implement the azimuth (ϕ2). The whole combined positioner (θ2 + ϕ2) needs to be completely independent and external to the elevation and azimuth for AoA1.

Figure 2-1 shows an approximate minimum Range Length (RL) for each antenna. RL for AoA1 is defined as 1m following the minimum range length for DFF systems, assuming a maximum antenna aperture D=5cm for a QZ=30cm, which was updated in clause 5.6.1 of TR 38.884 [3]:

Table 5.6.1-1: Minimum Range Length of DFF System for D = 5cm

| f [GHz] QZ [cm] | 24.25 | 30 | 40 | 50 | 52.6 | 71 |
| --- | --- | --- | --- | --- | --- | --- |
| 15 | 0.45 | 0.55 | 0.72 | 0.88 | 0.93 | 1.23 |
| 30 | 0.53 | 0.63 | 0.79 | 0.96 | 1.00 | 1.31 |

The RL for AoA2 is a best-case estimate based on the mechanical separation needed between the positioning system for AoA1 and AoA2. Considering this estimation, a ~ 3 to 3.5m chamber size (e.g. 3x3x3m) requiring approximately 9 m2 to 12 m2 of lab space with high ceiling requirements, would be the minimum to host this complex positioning system assuming DFF for both AoA. This represents a large increase in chamber size compare to the existing chambers deployed for conformance testing with 30cm QZ, which are in the range of 1.4 m2 to 5.3 m2 with 2m height.

This additional combined positioning system for AoA2 precludes any potential upgrade of existing test systems given the high complexity of integrating the two sets of positioning systems together.

Observation 1: the positioning system to implement full degrees of freedom for a 2 active AoA requires large chambers and are incompatible with existing FR2 conformance test systems.

Even if a dedicate test system is considered for 2 active AoA with full degrees of freedom, it will imply also a large degradation of the dynamic range due to the very long RF paths to reach each of the test antennas when compared with the existing FR2 conformance test systems based on IFF, for which the path loss is optimized since the measurement antenna (feed antenna to the CATR) is fixed. This degradation would make even the most basic requirements untestable.

Observation 2: the additional RF pathloss for a system implementing full degrees of freedom for a 2 active AoA implies large degradation of the dynamic range.

As mentioned in [4], this cannot be solved by usage of Near Field based methodologies since both active AoA must remain in Far Field conditions in other to perform Spherical Coverage and Beam Peak search properly.

In addition, the usage of DFF also has a major impact on the scalability of the methodology for different form factors (e.g. larger QZ size), Power Classes (e.g. antenna aperture larger than 5cm) or extending the frequency range (e.g. FR2-2, out-of-band measurements, etc.).

Observation 3: the usage of DFF for any of the 2 active AoA has a major impact on the scalability of the methodology.

It has to be noted as well that considering one or both AoA to be implemented as IFF only adds complexity to the positioning system due to the weight, size and feed positioning requirements of the CATR. This will require even more space than the example provided in Figure 2-1 and thus does not help with the RF pathloss degradation.

Observation 4: the usage of IFF for one or both AoA for a system implementing full degrees of freedom for a 2 active AoA implies even larger chambers and equivalent degradation of the dynamic range.

Given all these observations, it is proposed to consider not feasible the implementation of a system enabling full degrees of freedom for a 2 active AoA and focus on the reuse of legacy RRM test setup already designed for 2 active AoA as per TR 38.810 [5] and TS 38.508-1 [6].

Proposal 1: RAN4 to consider the implementation of a system enabling full degrees of freedom for 2 active AoA not feasible.

Proposal 2: RAN4 to focus on the reuse legacy RRM test setup already designed for 2 active AoA as per TR 38.810 and TS 38.508-1.

# Sequential test approach with test command

The concept of a test command to enable the testing for 2 active AoA was first introduced in [7]. This approach could be elaborated using the UE Beamlock Function (UBF) as the key element for the test procedure enabling the simultaneous connection over 2 AoA.

The following high-level procedure, based on RX beam peak direction search and Peak EIS measurement procedure in clauses K.1.2 and K.1.4 of TS 38.521-2 [8] respectively, can be considered.

- Perform Rx Beam peak search for AoA1.

- Connect the SS (System Simulator) with the DUT through the measurement antenna to form the RX beam towards the RX beam peak direction and respective polarization found for AoA1.

- SS activates the UE Beamlock Function (UBF) on AoA1. If necessary, switch the connection of the SS from the measurement antenna to the link antenna.

- Perform Rx Beam peak search for AoA2.

In our view, this is a smart approach and will enable a direct reuse of existing test systems although it may require improvements to the definition of UE Beamlock Function (UBF) and assumes that the UE performance for AoA2 is not dependent on the selection of AoA1.

If the latter is not true, the test procedure must be extended to test several times AoA2 for different directions of AoA1. In all cases the test procedure complexity, setup implications and test time are lower compared to the case with full degrees of freedom for a 2 active AoA.

Proposal 3: RAN4 to develop the sequential test approach with test command for testing UEs with multi-panel reception and 4DL layer.

# Baseline assumptions for requirements

In our understanding, the request to study the feasibility of an FR2 test system enabling full degrees of freedom for a 2 active AoA environment is assuming that all possible offset relations between AoA1 and AoA2 have the same probability of occurring in the field.

Even though, current assumption for multi-panel reception and 4DL layer is that the UE is connected to the same TRxP through two beams (or SSB indexes). Therefore, we could expect the system to pick AoA1 and AoA2 among the clusters with highest received power. Figure 4-1 shows a rough example of such a field case.

Figure 4-1: field case for 2 active AoA for UEs with multi-panel reception

Based on this rationale, we expect that the cases where 2 AoA connection can be achieved will happen with the most probability when the relative offset between angles (α in Figure 4-1) is relatively small and within a very restricted difference in elevation.

Observation 5: relative offset between the 2 active AoA in the field is expected to be small and have a restricted difference in elevation.

In order to have a clear objective for the test methodology on what cases must be covered to fit realistic implementations, it is essential to define the baseline assumptions used for the requirement definition.

Proposal 4: RAN4 to clarify the baseline assumptions used for requirement development (e.g. relative offset between AoA).

# Proposal

In this contribution, the following observations and proposals were made:

Observation 1: the positioning system to implement full degrees of freedom for a 2 active AoA requires large chambers and are incompatible with existing FR2 conformance test systems.

Observation 2: the additional RF pathloss for a system implementing full degrees of freedom for a 2 active AoA implies large degradation of the dynamic range.

Observation 3: the usage of DFF for any of the 2 active AoA has a major impact on the scalability of the methodology.

Observation 4: the usage of IFF for one or both AoA for a system implementing full degrees of freedom for a 2 active AoA implies even larger chambers and equivalent degradation of the dynamic range.

Observation 5: relative offset between the 2 active AoA in the field is expected to be small and have a restricted difference in elevation.

Proposal 1: RAN4 to consider the implementation of a system enabling full degrees of freedom for 2 active AoA not feasible.

Proposal 2: RAN4 to focus on the reuse legacy RRM test setup already designed for 2 active AoA as per TR 38.810 and TS 38.508-1.

Proposal 3: RAN4 to develop the sequential test approach with test command for testing UEs with multi-panel reception and 4DL layer.

Proposal 4: RAN4 to clarify the baseline assumptions used for requirement development (e.g. relative offset between AoA).

# References

- RP‑222652, “Revised SID for Study on NR frequency range 2 (FR2) Over-the-Air (OTA) testing enhancements”, Qualcomm Incorporated, 3GPP TSG RAN Meeting #97-e, September 2022

- R4-2214357, “WF on NR FR2 OTA testing enhancements”, Qualcomm Incorporated, 3GPP TSG RAN WG4 Meeting #104-e, August 2022

- 3GPP TR 38.884, v18.2.0 (2022-09)

- R4-2213627, “Discussion on FR2 methods for UEs with multi-panel reception”, Rohde & Schwarz, 3GPP TSG RAN WG4 Meeting #104-e, August 2022

- 3GPP TR 38.810, v16.6.1 (2020-11)

- 3GPP TS 38.508-1, v17.5.0 (2022-06)

- R4-2211549, “Views on FR2-1 RF OTA test for a device with multi-panel reception”, Anritsu Corporation, 3GPP TSG RAN WG4 Meeting #104-e, August 2022

- 3GPP TR 38.521-2, v16.12.0 (2020-07)