3GPP TSG-RAN WG4 #104bis-e	R4-2216645

October 10th ‒ 19th, 2022

Electronic Meeting

Agenda item:	7.4.3

Source: 	Qualcomm Incorporated

Title: 	5G broadcast core requirements verification

Document for:	Approval

# Introduction

One of the issues identified during the discussion of RAN4 #104e [1] was the need to verify the UE core requirements without an uplink to provide ACK/NACK feedback.  This contribution provides a proposal on how this can be done.

# Discussion

According to an agreed way forward [2] from RAN4 #104e, the following options were presented to address the issue of core requirement verification

- Option 1:

- Study/Identify existing indirect performance tests like BLER.

- If possible/necessary, adapt existing performance tests accordingly.

- If it turns out as infeasible/insufficient, start the development of alternative performance tests relying on downlink only.

- Option 2: Develop alternative performance tests relying on downlink only.

- Option 3: other

In this contribution, we explore option 1 to use a performance test.  Clause 10.4 of TS 36.101 [3] specifies performance requirements for FDD with 5G terrestrial broadcast.  The receive characteristic of MBMS is determined by a maximum BLER for a given average downlink SNR.  Two example requirements are copied below.

Table 10.4.1.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | MBSFN RS type | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-1 FDD | Type 1 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 18.5 | ≥ 2 |
| 2 | PCell | 10 | N/A | N/A | OP.1 FDD | AWGN | 1x1 | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.106-2 FDD | Type 2 | N/A | MBSFN channel model (Table B.2.6.3-1) | 1x1 | 1 | 20.2 | ≥ 2 |

Table 10.4.1.2-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | PCell | 10 | N/A | OP.1 FDD | AWGN | 1x2 low | N/A | N/A | N/A |
|  | MBMS Dedicated Cell | 10 | R.107 FDD | N/A | MBSFN channel model (Table B.2.6.4-1) | 1x2 low | 1 | 12.9 | ≥ 2 |

For these particular examples, a PCell is available for uplink feedback but in the context of the current work item, the UE is operating as a standalone ROM so no PCell is available.  Compared to a core requirement test such as reference sensitivity, the following differences can be observed

- The pass/fail criterion is a BLER rather than 95% maximum throughput.

- The signal is received at a relatively high signal power level, but has noise added to it in order to achieve a desired SNR.  For core requirements, no noise is added but the signal is received at a very low power level to test the noise figure of the receiver.

- A channel model is used, while core requirements use AWGN.

- The reference channel (identified by R.106-1 FDD, R.106-2 FDD, or R.107 FDD and specified in Annex A.3.8.1 of TS 36.101) uses a higher MCS with 16QAM or 64QAM code rate ½.  Core requirements use a reference channel with QPSK code rate 1/3.

The mapping from BLER to throughput is straightforward.  Since the entire transport block is discarded in case of error and no retransmission is available, the throughput is (1-BLER)*MaxTput.  Hence a 95% maximum throughput criterion is represented as 5% BLER.  Since there is no uplink channel, BLER must be reported by the UE directly to the test equipment during verification.  The details of reporting are left to RAN5, but it is noted that a similar approach of UE reporting for BLER counting has already been specified in Annex G.8.2 of TS 36.521 for sidelink as copied below.

Given that BLER is a suitable metric to verify core requirements, it is then recommended to define a BLER requirement that most closely follows the core requirement.  In other words, instead of SNR the BLER requirement should be specified at the reference sensitivity power level (and offset as specified for ACS and blocking).  The MCS should be chosen to be QPSK R=1/3 and the channel specified as AWGN.  New reference channel definitions may be needed for the PMCH, channel bandwidth, and SCS needed for 5G broadcast.

Proposal:  A BLER test is specified to verify core requirements.  The parameters (power levels, MCS, channel, etc) of the BLER test should be the same as those conventionally used for core requirement verification.

# Conclusion

In this contribution, a proposal was made to enable verification of UE 5G broadcast core specifications such as reference sensitivity, ACS, and blocking.  Traditionally, the tests have relied on uplink ACK/NACK to compute a throughput degradation as a function of received signal power level and interferer power level.  For 5G broadcast downlink only UE’s, since an uplink is not available, it is proposed to define a BLER requirement.  A counting log can be maintained by the UE and queried by the test equipment to measure BLER – this is the same approach already used for sidelink PSSCH BLER measurement.  The other parameters can remain the same to avoid the need for extensive simulation campaigns to determine SNR thresholds for different MCS and channels.

# Reference

- R4-2214239, “Email discussion summary for [104-e][128] LTE_terr_bcast_bands_UERF,” Moderator (Qualcomm Incorporated)

- R4-2214443, “WF on UE RF requirements for 5G broadcast,” SWR/EBU

- TS 36.101, “User Equipment (UE) radio transmission and reception,” v17.4.0