3GPP TSG-RAN WG4 Meeting #104b-e              	R4-2216691

Electrical Meeting, 10th Oct-19th Oct, 2022

Agenda item:	4.3.7.3.1

Source:	Samsung

Title:	View on BS demodulation requirement for NR extended to 71GHz

Document for:	Discussion

# 1	Introduction

In the last RAN4 meeting, the performance part of NR extended to 71GH was under discussed according to work plan. The related agreement on the general test scope and test setup was captured in the WF [1].

In this contribution, the view on general test scope and test setup for BS demodulation requirement is provided.

# 2	Discussion

In the last meeting, the following open issues are under discussed

SCS for requirements definition

- Option 1: 120 Hz and 480 kHz - Option 2: 120 kHz, 480 kHz and 960 kHz - Option 3: 120 kHz, 480 kHz and 960 kHz - 960KHz requirements are declared optional

Regarding SCS, 120KHz, 480KHz and 960KHz are supported from RAN1 design, with optional UE feature. In Rel-15, 240KHz SCS is supported, while there is no related requirement in RAN4.

From initial access perspective, as indicated in WID, 960KHz SCS for the SSB is not supported by UE, and 480 kHz is an optional SSB numerology for initial access for the UE.

Based on that, we would like to prioritize the requirement for UE with supported SCS for both data and initial access.

For this issue, there is no conclusion with several meeting cyclic, considering the schedule and time plan for this WI, we prefer to define the requirement with UE mandatory supported SCS

Proposal 1: RAN4 applies only 120KHz and 480KHz SCS for UL requirements definition.

CBW for requirements definition

- 100MHz CBW for 120KHz SCS - 400MHz CBW for 480KHz SCS - FFS for other cases including - 1600MHz for 480KHz - 400MHz and 2000MHz CBW for 960KHz SCS

Regarding the CBW for requirement, the minimum CBW can meet the test purpose for baseband processing verification. Meanwhile, based on the core requirement discussion, 1600MHz for 480KHz and 2000MHz for 960KHz are UE optional.

Considering the schedule and time plan for this WI, we prefer to define the requirement with UE mandatory supported CBW.

Proposal 2: RAN4 applies only 100MHz CBW with 120KHz SCS, 400MHz CBW with 480KHz SCS for PUSCH requirement.

# 3	Conclusion

In this contribution, the overview on the test scope and detail setup for BS demodulation requirement for NR extended to 71GHz are provided.

Proposal 1: RAN4 applies only 120KHz and 480KHz SCS for UL requirements definition.

Proposal 2: RAN4 applies only 100MHz CBW with 120KHz SCS, 400MHz CBW with 480KHz SCS for PUSCH requirement.

# References

R4-2214655, WF on PUSCH demodulation requirements for FR2-2, Nokia