3GPP TSG-RAN4 Meeting #104-bis-e	R4-2216743

# Electronic Meeting, 10 – 19 October, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR for test case for CG-SDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Perf |  |  |  |  |  | Date: |  |  | 2022-09-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test cases for CG-SDT in FR1 are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases are added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test cases for verifying CG-SDT RRM requirements in FR1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) A.6.2.1, A.6.2.1.1, A.6.2.1.2, A.6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

## A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)

A.6.2.1.1 Test purpose and Environment

The purpose of this test is to verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. In the test, UE is configured two CG-SDT configurations at different time. The test consists of five successive time periods, with time duration of T1, T2, T3, T4, T5, and T6 repectively. Figure A.6.2.1.1-1 shows the variation of the RSRP. Prior to the time point A, the UE shall be fully synchronized to PCell.

Figure A.6.2.1.1-1: RSRP variation model for CG-SDT testing

A.6.2.1.2 Test Parameters

There is one cells in the test, the FR1 PCell. The test parameters for the PCell are given in Table A.6.2.1.2-1, Table A.6.2.1.2-2, and Table A.6.2.1.2-3.

Table A.6.2.1.2-1: NR configuration for FR1 SSB

| Config | Description |
| --- | --- |
| 1 | FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.2.1.2-2 : General test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Duplex mode |  | FDD |
| TDD Configuration |  | N/A |
| BWchannel | MHz | 10: NRB,c = 52 |
| PDSCH Reference measurement channel |  | SR.1.1 FDD |
| RMSI CORESET Reference Channel |  | CR.1.1 FDD |
| Dedicated CORESET Reference Channel |  | CCR.1.1 TDD |
| SSB configuration |  | SSB.3 FR1 |
| OCNG Patterns |  | OP.1 |
| Initial BWP Configuration |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | SMTC.1 |
| DRX configuration |  | [DRX.3] |
| T1 | ms | [TBD] |
| T2 | ms | [TBD] |
| T3 | ms | [TBD] < 640 ms |
| T4 | ms | [TBD] |
| T5 | ms | [TBD] |
| T6 | ms | [640] ms |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |
| Propagation condition |  | AWGN |

Table A.6.2.1.2-3: SSB specific test parameters

| Parameter | Config | Unit | SSB#0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T4 | T5 |
| Note1 | 1 | dBm/15kHz | -98 |  |  |  |
| Note1 | 1 | dBm/SSB SCS | -95 |  |  |  |
|  | 1 | dB | 14 | 3 | 3 | -4 |
| SS RSRP Note2 | 1 | dBm/SSB SCS | -81 | -95 | -95 | -104 |
| Io Note3 | 1 | dBm/38.16 MHz | -49.79 | -62.2 | -62.2 | -64.49 |
|  | 1 | dB | 14 | 3 | 3 | -4 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

A.6.2.1.3 Test requirements

The UE behaviour in each time durations T1,T2,T3,T4,T5,and T6 shall be as follows:

At time point A, UE shall receive RRC release message with CG-SDT configurations and measure the reference RSRP.

During the period from time point A to time point B, the RSRP dropped within the RSRP change threshold.

UE shall measure RSRP during the period between time point B and time point C.

UE shall transmit CG-SDT during the period between time point C and time point D.

During the period from time point D and time point E, RSRP dropped beyond the RSRP change threshold.

UE shall measure RSRP during the period between time point B and time point F.

UE shall not transmit CG-SDT during the period between time point F and timepoint G.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change>