3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2216763

Electronic meeting, October 10th – 19th 2022

Agenda Item:	4.6.3.3

Source:	Ericsson

Title:	Discussions on RRM measurement relaxations

Document for:	Discussion

# 1	Introduction

Whether the UE is allowed to apply relaxed requirements in IDLE mode when it has failed to meet the cell selection criterion was discussed at last meeting with following outcome [1]:

Issue 2-1-3 Clarification on RRM relaxation applying conditions - Proposals - Option 1: The RedCap UE shall not relax measurements on any of the neighbour cells when it has failed to meet the S criterion; In detail add the phrase: “In this case the UE shall not relax measurements on any of the neighbour cells even if the UE is configured with any relaxed measurement criterion and has fulfilled that criterion.”, for the cases configured eDRX and non-configured eDRX in clause 4.2B.2.2 in TS 38.133 (Nokia Ericsson CMCC Intel) - Option 2: RAN4 not to capture the additional highlighted text from the WF in the RAN4 specifications (Apple Qualcomm vivo xiaomi MTK) - Option 3: “If the UE is configured with and has fulfilled the stationary and not-at-cell-edge criteria in sections 4.2B.2.10.3 and 4.2B.2.11.3 and if UE has failed to meet the S-criterion, then the UE shall not relax measurements on any of the neighbour cells. (Ericsson) Tentative agreements: No

In this contribution we further discuss the options and provide updated view.

# 2	Discussion

According to current requirements, the UE is required to initiate measurements of all neighbour cells indicated by the serving cell when it has failed to meet the cell suitability criterion. The related requirements are quoted be for reference [2]:

#### 4.2B.2.2 Measurement and evaluation of serving cell for RedCap UE ….. If the UE is not configured with eDRX_IDLE cycle and the UE has evaluated according to Table 4.2B.2.2-1 for 1 Rx RedCap or Table 4.2.2.2-1 for 2 Rx RedCap in Nserv_RedCap consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities.

The RedCap UE is allowed to operate in relaxed mode with respect to the neighbour cell measurmenets upon fulfilling the relaxation criteria. The problem is that, as per current specification, the RedCap UE can be operating in relaxed mode while not being able to fulfil the S criterion for the serving cell. This behaviour is not aligned with the serving cell evaluation requirements, see the quoted text above. When the UE has failed to meet the cell selection criterion, then it is critical for the UE to exit any power saving or relaxation mode and start to do measurements according to the non-relaxed procedures. At this stage, power saving should not be an issue as the UE may end up losing its connection totally. This shall be clarified in the current specification.

During the last meeting discussions, it was brought to our attention that same issue also exist for Rel-16 relaxation. While we do acknowledge this fact, our understanding is that this issue was overlooked in Rel-16 RRM relaxation work. In addition, it shall be noted that the level of relaxation introduced in Rel-17 RedCap is significantly higher compared to Rel-16 neighbour cell measurement relaxation. As an example, the RedCap UE is allowed to not measure on any of the neighbour cells for 4 hours when meeting multiple relaxation criteria compared to 1 hour in Rel-16 NR UE.

Observation 1	Whether UE can apply relaxed measurement when failing to meet the S-criterion was overlooked in Rel-16 RRM relaxation WI.

Observation 2	Rel-17 RedCap UE is allowed to not measure on any of the neighbour cells for 4 hours compared to the longest relaxation period of 1 hour for Rel-16 NR UE.

Moreover, it was mentioned that the issue of UE mistakenly enters relaxed or meets the relaxed requirements although it has failed to meet the S-criterion can be solved by NW configuration. A specific proposal where the NW adapts the value of SSearchThresholdP and SSearchThresholdQ, such that SSearchThresholdP >=0 SSearchThresholdQ >=0, then when Srxlev or Squal<0 was presented. The problem with this approach is that same parameters (SSearchThresholdP and SSearchThresholdQ) are used for evaluating all the relaxation criteria and this may lead to that the UE in some cases does not even enter the relaxed mode even if it has met the S-criterion.

Therefore, as a compromise proposal, we would like to focus on the case when the UE has met the multiple relaxation criteria (e.g. fulfilment of stationary and not-at-cell-edge criteria) which allows the UE to not measure on any of the neighbour cells for 4 hours. In this case, if the UE has failed to meet the S-criterion then UE shall not apply the relaxed measurement requirements.

- If the UE is configured with and has fulfilled multiple relaxation criteria that allows the UE to not measure for 4 hours and if UE has failed to meet the S-criterion, then the UE shall not relax measurements on any of the neighbour cells.

# Conclusion

In this contribution we have discussed the remaining issues of RRM relaxation work for RedCap UE. Based on the discussions we have made following observations and proposals:

Observation 1	Whether UE can apply relaxed measurement when failing to meet the S-criterion was overlooked in Rel-16 RRM relaxation WI.

Observation 2	Rel-17 RedCap UE is allowed to not measure on any of the neighbour cells for 4 hours compared to the longest relaxation period of 1 hour for Rel-16 NR UE.

- If the UE is configured with and has fulfilled multiple relaxation criteria that allows the UE to not measure for 4 hours and if UE has failed to meet the S-criterion, then the UE shall not relax measurements on any of the neighbour cells.

# References

R4-2214486, “WF on eDRX and RRM measurement relaxations requirements for Redcap”, vivo

TS 38.133 V17.6.0