3GPP RAN4 Meeting #104-bis-e	R4-2216795

# Electronic meeting, October 10 - 19, 2022

Agenda item:	4.3.2.1

Source: 	Qualcomm Incorporated

Title: 	60 GHz UE PRACH and PTRS capabilities

Document for:	Approval

# Introduction

In this paper we discuss PRACH and PTRS capabilities as they relate to FR2-2.

# PRACH for 480 and 960 kHz SCS

RAN1 has agreed that PRACH may use 120, 480, or 960 kHz SCS. We can specify the measurement time period in the same table for FR2-1 and -2.

In [1], RAN1 has designed the system to allow the UE to optionally support 480 kHz SCS using ul-FR2-2-SCS-480kHz-r17 and support 960 kHz SCS using ul-FR2-2-SCS-960kHz-r1. One aspect of supporting these SCSs is to support PRACH in the declared SCS.

#### 4.2.7.2b	FR2-2-AccessParamsPerBand

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ul-FR2-2-SCS-480kHz-r17 Indicates whether the UE supports the following: - PRACH with 480kHz SCS and length 139. - Transmission of 4800kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2. - Multi-PUSCH scheduling by single DCI for the operation with 480 kHz SCS. UE indicating support of this feature shall also indicate support of dl-FR2-2-SCS-480kHz-r17 and ul-FR2-2-SCS-120kHz-r17. | Band | No | N/A | N/A |
| ul-FR2-2-SCS-960kHz-r17 Indicates whether the UE supports the following: - PRACH with 960kHz SCS and length 139. - Transmission of 960kHz subcarrier spacing for UL data and control channels and reference signals in FR2-2. - Multi-PUSCH scheduling by single DCI for the operation with 960 kHz SCS. UE indicating support of this feature shall also indicate support of dl-FR2-2-SCS-960kHz-r17 and ul-FR2-2-SCS-120kHz-r17. | Band | No | N/A | N/A |

Except from [1] indicating 480 and 960 kHz PRACH support

Considering RAN4 has PRACH measurement definitions in 38.101-2 we should update the measurement definition tables to include the new SCS.

Proposal : PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown.

Table 6.3.3.4-1: PRACH ON power measurement period

| Format | SCS | Measurement period | Note |
| --- | --- | --- | --- |
| A1 | 60 kHz | 0.035677 ms |  |
|  | 120 kHz | 0.017839 ms |  |
|  | 480 kHz | 0.004460 ms |  |
|  | 960 kHz | 0.002230 ms |  |
| A2 | 60 kHz | 0.071354 ms |  |
|  | 120 kHz | 0.035677 ms |  |
|  | 480 kHz | 0.008919 ms |  |
|  | 960 kHz | 0.004460 ms |  |
| A3 | 60 kHz | 0.107031 ms |  |
|  | 120 kHz | 0.053516 ms |  |
|  | 480 kHz | 0.013379 ms |  |
|  | 960 kHz | 0.006690 ms |  |
| B1 | 60 kHz | 0.035091 ms |  |
|  | 120 kHz | 0.0175455 ms |  |
|  | 480 kHz | 0.004386 ms |  |
|  | 960 kHz | 0.002193 ms |  |
| B4 | 60 kHz | 0.207617 ms |  |
|  | 120 kHz | 0.103809 ms |  |
|  | 480 kHz | 0.025952 ms |  |
|  | 960 kHz | 0.012976 ms |  |
| A1/B1 | 60 kHz | 0.035677 ms for front X1 occasion 0.035091 ms for last occasion | X1 = [2,5] |
|  | 120 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion |  |
|  | 480 kHz | 0.004460 ms for front X1 occasion 0.004387 ms for last occasion |  |
|  | 960 kHz | 0.017839 ms for front X1occasion 0.017546 ms for last occasion |  |
| A2/B2 | 60 kHz | 0.071354 ms for front X2 occasion 0.069596 ms for last occasion | X2 = [1,2] |
|  | 120 kHz | 0.035677 ms for front X2 occasion 0.034798 ms for last occasion |  |
|  | 480 kHz | 0.008919 ms for front X2 occasion 0.008700 ms for last occasion |  |
|  | 960 kHz | 0.004460 ms for front X2 occasion 0.004350 ms for last occasion |  |
| A3/B3 | 60 kHz | 0.107031 ms for first occasion 0.104101 ms for second occasion |  |
|  | 120 kHz | 0.053515 ms for first occasion 0.052050 ms for second occasion |  |
|  | 480 kHz | 0.013379 ms for first occasion 0.013013 ms for second occasion |  |
|  | 960 kHz | 0.006689 ms for first occasion 0.006506 ms for second occasion |  |
| C0 | 60 kHz | 0.026758 ms |  |
|  | 120 kHz | 0.013379 ms |  |
|  | 480 kHz | 0.003345 ms |  |
|  | 960 kHz | 0.001672 ms |  |
| C2 | 60 kHz | 0.083333 ms |  |
|  | 120 kHz | 0.0416667 ms |  |
|  | 480 kHz | 0.010417 ms |  |
|  | 960 kHz | 0.005208 ms |  |
| NOTE: For PRACH on PRACH occasion start from begin of 0ms or 0.5 ms boundary, the measurement period will plus 0.032552 μs |  |  |  |

# PTRS configuration for 16QAM and 64QAM

PTRS configuration has as much impact on phase noise mitigation as the phase noise profile itself. This aspect is not related to the EVM calculator, but it in effect is a discussion on the side conditions for the EVM test. RAN2 signalling already exists (ptrs-DensityRecommendationSetUL) to allow the UE to convey to the network what it’s PTRS preferences are in terms of preferred MCS/BW thresholds. We propose to make a slight modification to the PTRS configuration note in 38.101-2 for n263, indicating the EVM test should follow the IE indication when determining how to configure PTRS for 16QAM and 64QAM.

The existing note in the EVM tables is ‘NOTE 1:	PTRS is configured for 16 QAM and 64 QAM’ and we propose to void the note and add ‘NOTE 2:  PTRS is configured according to the UE preference in ptrs-DensityRecommendationSetUL’

Proposal: Void NOTE 1 in FR2-2 EVM tables and add NOTE 2:  PTRS is configured according to the UE preference in ptrs-DensityRecommendationSetUL’

# Conclusions

Proposal : PRACH ON power measurement period table should be updated for 480 and 960 SCS as shown.

Proposal: Void NOTE 1 in FR2-2 EVM tables and add NOTE 2:  PTRS is configured according to the UE preference in ptrs-DensityRecommendationSetUL’

# References

[1] 3GPP TS 38.306 V17.2.0 ;User Equipment (UE) radio access capabilities