3GPP TSG-RAN WG4 Meeting #104bis-e 	R4-2216825

# Electronic Meeting, October 10 – October 19, 2022

Agenda Item:	6.8.3.2

Source: 	Ericsson

Title:	Discussion on L3 measurements and procedures

Document for:	Discussion

# Introduction

A new WI on Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception [1] was approved at RAN#96. This work item aims to introduce the requirements for UEs capable of multi-beam/chain simultaneous DL reception on a single component carrier in FR2-1 from two different directions to achieve improved RF, RRM and UE demodulation performance. A UE capability for such operation has been introduced in Rel-16 (simultaneousReceptionDiffTypeD-r16) [TS 38.306], as shown below. However, no RF, RRM, or performance requirements were defined in Rel-16 or Rel-17.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| simultaneousReceptionDiffTypeD-r16 Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal as specified in TS38.213 [11]. | Band | No | N/A | FR2 only |

The following two scenarios are to be considered in the WI:

- Dual TCI (same or different TRPs),

- Single TCI (Note [1]: The case of single TCI is handled as a second priority. Additional aspects related to single TCI can be further revisited).

The WI will cover RRM core and RRM performance parts, both impacting TS 38.133.

The WID [1] includes the following objective of the core part:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception with two different QCL TypeD RSs on single component carrier with up to 4-layer DL MIMO

  - Enhanced RF requirements:

    - Specify RF requirements, mainly spherical coverage requirements, for devices with simultaneous reception from different directions with different QCL TypeD RSs

    - The legacy spherical coverage requirement for reception from a single direction will be kept

    - PC3 will be prioritized, other power classes should be considered after the PC3 requirements framework is finalized

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - Enhanced RRM requirements:

    - The following requirements should be studied and specified if necessary:

      - L1-RSRP measurement delay

      - L3 measurement delay (both cell detection delay and measurement period can be considered)

        - The starting point is the enhancements related to L1-RSRP measurement enhancements

      - RLM and BFD/CBD requirements

      - Scheduling/measurement restrictions

      - TCI state switching delay with dual TCI

      - Receive timing difference between different directions (different QCL Type D RSs)

The WID [1] includes the following objective of the performance part:

- Introduce necessary requirement(s) for enhanced FR2-1 UEs with simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier

  - RRM Performance requirements and test cases corresponding to the new core requirements

  - RRM test cases to verify the UE behavior/functionalities needed to support this feature in case existing requirements are reused

  - UE demodulation requirements:

    - Specify performance requirements for enhanced FR2-1 UEs supporting up to 4 DL MIMO layers with dual TCI with different QCL typeD on a single component carrier.

      - Demod requirements on up to 4 DL MIMO layers

      - CSI requirements to support up to 4-layer with 2 TCIs and Rel-17 mTRP Type I codebook

    - Extension of requirements to cover intra-band CA can be considered after the single carrier requirements are defined

The agreed WF on RRM from RAN4#104-e is captured in [2].

# Discussion

In this WI [1], a UE simultaneously receives two RSs from different QCL type D infos. Since SSBs are never target RSs in QCL info but rather an associated RS, it can be assumed that CSI-RS based measurements and procedures are in the main scope of the WI [1]. Hence, RAN4 shall not assume simultaneous reception for SSB-based cell detection and L3 SSB-based measurements, which suggests that L3 measurements can be deprioritized in this WI [1].

- Proposal 1: Cell detection and L3 measurement period, SSB-based:

  - No simultaneous reception for SSB-only based measurements and procedures are assumed.

- Proposal 2: Deprioritize L3 measurements in the current WI.

# Summary

The following have been observed and proposed in this contribution.

- Proposal 1: Cell detection and L3 measurement period, SSB-based:

  - No simultaneous reception for SSB-only based measurements and procedures are assumed.

- Proposal 2: Deprioritize L3 measurements in the current WI.

# References

[1] RP-221753, WI description, Requirement for NR frequency range 2 (FR2) multi-Rx chain DL reception, Qualcomm Inc., June 2022.

[2] R4-2214344, WF on FR2 multi-Rx RRM, vivo, Aug. 2022.