3GPP TSG-RAN WG4 Meeting # 104-bis-e 	R4-2216891

Electronic Meeting, October 10 – October 19, 2022

Agenda item:			6.4.3

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [104-bis-e][307] FS_NR_BS_RF_evo

Document for:	Information

Introduction

This email thread discuss the SI on NR BS RF requirement evolution. The contributions are in agenda 6.4, which includes:

- Topic #1: Definition of FR2-1 multi-band BS

- Topic #2: Feasibility of FR2-1 multi-band BS

- Topic #3: Requirements of FR2-1 multi-band BS

Topic #1: Definition of FR2-1 multi-band BS

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215413 | CATT | Observation 1: FR1 definition of multi-band RIB can be re-used for FR2. Observation 2: 1) is BS with multi-band RIB; 2), 3) and 6) is BS without multi-band RIB; for 4) and 5) whether they are BS with multi-band RIB depends on whether transmitter or receiver have ability to process two or more carriers from the different bands in common active RF components simultaneously. Observation 3: Introducing the definition of multi-band BS for FR2 into specification to illustrate the possible implementations supported by this specification is reasonable. Observation 4: Definition of FR2 multi-band BS may be, If there is BS with multi-band RIB can be implemented, the FR2 multi-band BS can be defined as following: BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary: - All RIBs are single-band RIBs. - All RIBs are multi-band RIBs. - A combination of single-band RIBs and multi-band RIBs provides support of the BS type 2-O capability of operation in multiple operating bands. Otherwise, the FR2 multi-band BS can be defined as following: BS type 2-O may be capable of supporting operation in multiple operating bands with the following implementations at the radiated interface boundary: - All RIBs are single-band RIBs. |
| R4-2215575 | Nokia, Nokia Shanghai Bell | Proposal 1: To reuse the current FR1 definition of multi-band RIB for FR2. Proposal 2: No need to introduce the definition of multi-band BS for FR2 into specification. |
| R4-2216240 | Huawei, HiSilicon | Proposal 1: Scenario 2 and 3 should not be considered as multi-band BS. 2) Single-band transmitter and receiver 3) Configurable BS for different bands with the same hardware FR2 multi-band BS can be defined as below. FR2 multi-band BS: FR2 Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s). |
| R4-2216241 | Huawei, HiSilicon | TP for deployment scenarios |
| R4-2216397 | Ericsson | Observation 1 The term multi-band BS is not defined for FR1, but this does not prevent implementation of multi-band BS that consist of single band RIBs Observation 2 It may be possible that intermodulation effects could occur between panels operating on different bands within an FR2-1 BS. This could be investigated further but could also be solved by simply stating that all panels should be active during testing. Observation 3 There is not an obvious need to introduce the term “multi-band BS” for FR2-1 Observation 4 If multi-band RIB specific requirements are needed for FR2-1, then the definition of multi-band RIB from FR1 is sufficient. |
| R4-2216551 | ZTE Corporation | Proposal 2: to follow the same definition of FR1 multi-band RIB for FR2 multi-band RIB. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 1 –Definition of FR2-1 multi-band BS

Issue 1-1: Scenarios

1) Multi-band transmitter and/or receiver with common active RF components

2) Single-band transmitter and receiver

3) Configurable BS for different bands with the same hardware, i.e. only one band can be configured to operate at any time.

4) BS covers full-band or sub-band of band A and band B

5) BS covers consecutive spectrums with different band number, for example, n258+n261

6) BS covers two bands which have overlapping spectrums, for example, n258+n257, and BS support no overlapping frequency range in the two bands.

- Proposals:

  - Option 1: Scenarios 1), 4), 5) and 6) should be considered as multi-band BS while scenario 2) and 3) should not.

  - Option 2: All RIBs are single-band RIBs should also considered as multi-band BS

- Recommended WF

  - Discuss on which option is agreeable

Issue 1-2:  Definition of multi-band RIB

- Proposal:

  - To reuse the current FR1 definition of multi-band RIB for FR2.

- Recommended WF

  - Discuss whether the proposal is agreeable

Issue 1-3:  Definition of multi-band BS

- Proposals:

  - Option 1: No need to introduce the definition of multi-band BS for FR2 into specification

  - Option 2: FR2 multi-band BS can be defined as below and FFS whether need to introduce the definition for FR2 into specification

FR2 multi-band BS: FR2 Base Station characterized by the ability of its transmitter and/or receiver to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s).

  - Option 3:

If there is BS with multi-band RIB can be implemented, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with one of the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

-	All RIBs are multi-band RIBs.

-	A combination of single-band RIBs and multi-band RIBs provides support of the BS type 2-O capability of operation in multiple operating bands.

Otherwise, the FR2 multi-band BS can be defined as following:

BS type 2-O may be capable of supporting operation in multiple operating bands with the following implementations at the radiated interface boundary:

-	All RIBs are single-band RIBs.

- Recommended WF

  - Discuss on which option is agreeable

Issue 1-4:  TP for deployment scenarios

- Proposals:

  - R4-2216241

- Recommended WF

  - Discuss on TP in R4-2216241 to check whether it is agreeable.

Companies views’ collection for 1st round

Open issues

Collection of comments:

Issue 1-1: Scenarios

| Company | Comments |
| --- | --- |
| Huawei | Option 1 Multi-band BS should include common active RF components at TX and/or RX, i.e. at least one of them is multi-band RIB |
| Nokia | Prefer option 1. |
| Ericsson | Option 1 |
| Qualcomm | Support option 1 |
| ZTE | Support option 1 |
| CATT | Prefer option 1. |

Issue 1-2:  Definition of multi-band RIB

| Company | Comments |
| --- | --- |
| Huawei | Agree to reuse the current FR1 definition. |
| Nokia | Agree with proposal. |
| Ericsson | Agree |
| Qualcomm | Support to reuse current FR1 definition of multi-band RIB |
| ZTE | Agree with proposal. |
| CATT | Agree |

Issue 1-3:  Definition of multi-band BS

| Company | Comments |
| --- | --- |
| Huawei | Option 2 From the discussion of MB MSR, the definition of multi-band BS is different with BS capable of supporting operation in multiple operating bands. The latter has larger scope. |
| Nokia | Propose option 1. OK with option 3. For option 2, currently there is no definition in clause 3.1 of TS 38.141-2 for multi-band BS, only description in clause 4.8. |
| Ericsson | Option 1; we don’t really see the need to define the term Our understanding is that the discussion last time is that the term “multi-band BS” may be different to “multi-band RIB”. We don’t really think the term “multi-band BS is needed”. If it is defined, to avoid simply being parallel to the definition of “multi-band RIB” it could refer to a BS that transmit multiple bands even if some are through single band RIBs. But we don’t really need that discussion if we don’t define the term. |
| Qualcomm | Support option 1 as the definition of multi-band RIB should suffice. If not, then option 3 is ok. |
| ZTE | We also support the option 1 since mult-band RIB is sufficient. |
| CATT | OK with option 3. We also agree that there is only BS with multi-band RIB need specific additions or exclusions for requirement. |

Issue 1-4:  TP for deployment scenarios (R4-2216241)

| Company | Comments |
| --- | --- |
| Nokia | No need to mention CBM and IBM in RF, they should be discussed in RRM. |
| Ericsson | The claim that it gives benefits for site space should be removed; this may not always be the case for FR2. Also the claim that it supports hardware upgrade would apply equally to a BS implementing single band RIBs. Also non-wideband implementation as single band RIBs could support the TAE requirement. Regarding the example structures, it would be good to have some more discussion and study on which are actually feasible before including in the TR. |
| Qualcomm | Not clear the reason behind mentioning TAE requirements in the deployment scenarios clause. More explanation of the figures should be reflected in the text (e.g., pros and cons of separate vs shared antenna arrays) |
| ZTE | Just as other companies mentioned, more discussions are needed in the 2nd round. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1: Scenarios | Based on 1st round discussion, all companies are ok with Option 1. Moderator suggest to approve Option 1. - Option 1: Scenarios 1), 4), 5) and 6) should be considered as multi-band BS while scenario 2) and 3) should not. |
| Issue 1-2: Definition of multi-band RIB | Based on 1st round discussion, all companies support to reuse the current FR1 definition of multi-band RIB for FR2. Moderator suggest to approve the proposal. - Proposal: - To reuse the current FR1 definition of multi-band RIB for FR2. |
| Issue 1-3: Definition of multi-band BS | Based on 1st round discussion, 5 companies support option 1, 3 companies ok with option 3 and one company supports option 2. Moderator suggest to check whether Option 1 is agreeable for 2nd round. Option 1: No need to introduce the definition of multi-band BS for FR2 into specification |
| Issue 1-4: TP for deployment scenarios | Based on 1st round discussion, the TP is to be revised to capture companies’ comments. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on definition of FR2-1 multi-band BS | CATT |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Topic #2: Feasibility of FR2-1 multi-band BS

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215413 | CATT | Observation 5: Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in 26+28 GHz combinations is feasible. Observation 6: Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261, or with 26.3% FBW in frequency range 37-48 GHz which includes n260/n259/n262 is feasible. Observation 7: Multi-band AA (Antenna Array) with common radiated element with higher than 40.6% FBW in 28+39 GHz, 26+40 GHz, and 28+40 GHz combinations is not feasible. Observation 8: It is possible to use multiple separate single-band AAs (Antenna Array) for ultra-wide band combination, for example, one single-band AA is for frequency range 24-29 GHz which includes n257/n258/n261, and another single-band AA is for frequency range 37-48 GHz which includes n260/n259/n262. Observation 9: Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible. Observation 10: [20%] can be considered as the largest feasible FBW for an FR2 multi-band RIB dependent BS. |
| R4-2215576 | Nokia, Nokia Shanghai Bell | Observation 1: FR2 multi-band RIB should have same definition as FR1 multi-band RIB. Observation 2: Combination n261+n257 should be considered as single-band RIB following the current FR1 multi-band RIB definition as any BS bandwidth in n261 is a sub-band of n257. Observation 3: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% has directivity variation being less than (absolute) 1.7dB on the boresight direction. Whereas that of band combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26% have directivity variation being higher than (absolute) 2.2dB on the boresight direction. Observation 4: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% achieves better trade-off between directivity (of the lowest frequency) and maximum azimuth steering angle (of the highest frequency) compared to the other combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26%. Observation 5: High linearity and high efficiency wideband-PA is challenging to obtain due to the requirement on the high video bandwidth, e.g., up to 4GHz video bandwidth is already challenging for PA implementation. Observation 6: Multi-band signals going through same PA for combinations with the supported radio bandwidth < 6GHz may generate the IM3 and IM5 frequencies which falls into the operating frequency ranges which increase intermodulation distortion level. Observation 7: DPD is needed to minimize nonlinear distortions caused by PA, but such algorithms require high DAC’s sampling rate which is expensive or may be currently unavailable for band combinations with the supported radio bandwidth > 11.2 GHz (or FBW > 26%). Observation 8: Using single frequency-invariable PS per RF chain to do concurrent beamforming transmission for multi-band signals increases intra-band interference which impacts OTA receiver out-of-band blocking requirement while being energy inefficient. Observation 9: Using multiple frequency-invariable PSs per RF chain to do concurrent beamforming transmission for multi-band signals will increase size, cost and energy consumption for the FR2 radio which might not be of practical interest. Observation 10: Multi-band PS, i.e., frequency-selective PSs might not be soon commercially available yet. Observation 11: In general the use of phase shifter poses a challenge for practical implementations of FR2 multi-band RIB. Observation 12: The implementation of PS in beamforming architecture may impact the declaration of FR2 multi-band RIB. Proposal 1: The maximum supported bandwidth for FR2 multi-band RIB needs to be limited from the perspective of capabilities and performance of RF frontend to make the practical implementation feasible. Proposal 2: To clarify if the use of active phase shifters in analog/hybrid beamforming in FR2 multi-band RIB can impact to the declaration of multi-band RIB, e.g., FR2 RIB would be considered as FR2 multi-band RIB if active phase shifters are used to process two or more bands simultaneously. Proposal 3: To investigate further if the requirements of EVM, ACLR and EIRP accuracy, transmitter intermodulation requirement for FR2 multi-band RIB needs to be changed or relaxed compared to those for single band RIB. Proposal 4: There should be maximum radio bandwidth defined for FR2 multi-band BS with multi-band RIBs. Some requirements would need to be revisited. Observation 13: FR2 multi-band BS with multiple single-band RIBs is a more suitable solution for band combinations, e.g., those with the supported radio bandwidth > 9GHz. |
| R4-2216242 | Huawei, HiSilicon | Observation 1：Some recent literature shows PA cover 26-39GHz, 24-42GHz. Observation 2：For RF front-end, commercially available TRX chips cover 24-29.5GHz. 27-41GHz RX is implemented .A harmonic-selection technique is proposed to extend the receiver’s operating bandwidth up to 24.25-71GHz. Observation 3: State of the art device performance indicates that RF components and solutions are feasible to cover multi-band FR2 implementations. Observation 4：Traditional 20% FBW is not suitable to multi-band antenna scenarios. For some FR1 multi-band antenna FBW can be up to 61% with the acceptable electrical performance. Observation 5: FR2 MB BS can design each band antenna separately to achieve best performance meanwhile without significant increase in PCB area. Observation 6：Multi-band antenna element are feasible to cover FR2 frequency ranges. Observation 7：Optical lens antenna, such as Luneburg Lens antenna may be considered for FR2 MB BS. Observation 8：Phase shifter is demonstrated to cover a bandwidth of 23.8–30.4 GHz. Observation 9：Considering the coupling of beam management of different bands, multi-band shifter may not a good candidate. Single-band shifter is more suitable, especially for larger band gap case. Observation 10：Even if the single-band shifter is adopted in FR2 BS, it still belongs to the MB BS as long as the front-end use multi-band implementation. Proposal 1: MB BS supporting 26G+40G is feasible from RF front-end and antenna implementation perspective based on preliminary study. |
| R4-2216397 | Ericsson | Proposal 1 The Study Item does not consider wideband radio/antenna BS covering bands within covering bands from the 24-30GHz and 39-40GHz frequency ranges together. |
| R4-2216551 | ZTE Corporation | Observation 1: Pros; Cons. multi-band operation among different frequency groups; To support the inter-band CA with multi-band operation at BS side might draw more commercial interest from the operators and the corresponding UE RF requirement is also more complete; More challenging to support it with multi-band operation at BS side.. multi-band operation within the same frequency group; Easier due to its narrower supported frequency range; Draw less commercial interest from the operators at least from supporting the inter-band CA with multi-band operation.. Note 1: not sure to support the different bands from different operators with multi-band operation is practical implementation. Based on the legacy design, the AAU equipment is more or less targeted to single operator’s holding spectrum.. Observation 2: for multi-band operation across different frequency group, it is challenging to support by the same power amplifier or driver amplifier. Observation 3: for multi-band operation of example band combination 26+28 GHz: n258 + n261 it seems feasible to support by the same power amplifier or driver amplifier. Proposal 1: to focus the multi-band RIB within the same frequency group in Rel-18. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2 –Feasibility of FR2-1 multi-band BS

Issue 2-1: RF front-end

- Observations/proposals on feasibility study from contributions:

  - Observation 1: Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible.

  - Observation 2: High linearity and high efficiency wideband-PA is challenging to obtain due to the requirement on the high video bandwidth, e.g., up to 4GHz video bandwidth is already challenging for PA implementation.

  - Observation 3: Multi-band signals going through same PA for combinations with the supported radio bandwidth < 6GHz may generate the IM3 and IM5 frequencies which falls into the operating frequency ranges which increase intermodulation distortion level.

  - Observation 4：Some recent literature shows PA cover 26-39GHz, 24-42GHz.

  - Observation 5：For RF front-end, commercially available TRX chips cover 24-29.5GHz. 27-41GHz RX is implemented. A harmonic-selection technique is proposed to extend the receiver’s operating bandwidth up to 24.25-71GHz.

  - Observation 6: State of the art device performance indicates that RF components and solutions are feasible to cover multi-band FR2 implementations.

  - Observation 7: DPD is needed to minimize nonlinear distortions caused by PA, but such algorithms require high DAC’s sampling rate which is expensive or may be currently unavailable for band combinations with the supported radio bandwidth > 11.2 GHz (or FBW > 26%).

  - Observation 8: for multi-band operation across different frequency group, it is challenging to support by the same power amplifier or driver amplifier.

  - Observation 9: for multi-band operation of example band combination 26+28 GHz: n258 + n261 it seems feasible to support by the same power amplifier or driver amplifier.

- Recommended WF

  - Discuss on these observations and please indicate which observations would be acceptable as RAN4 common understanding

Issue 2-2: Antenna array

- Observations/proposals on feasibility study from contributions:

  - Observation 1: Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261, or with 26.3% FBW in frequency range 37-48 GHz which includes n260/n259/n262 is feasible.

  - Observation 2: Multi-band AA (Antenna Array) with common radiated element with higher than 40.6% FBW in 28+39 GHz, 26+40 GHz, and 28+40 GHz combinations is not feasible.

  - Observation 3: It is possible to use multiple separate single-band AAs (Antenna Array) for ultra-wide band combination, for example, one single-band AA is for frequency range 24-29 GHz  which includes n257/n258/n261, and another single-band AA is for frequency range 37-48 GHz which includes n260/n259/n262.

  - Observation 4: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% has directivity variation being less than (absolute) 1.7dB on the boresight direction. Whereas that of band combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26% have directivity variation being higher than (absolute) 2.2dB on the boresight direction.

  - Observation 5: Wideband antenna array for band combinations with the supported radio bandwidth < 9GHz or FBW < 20% achieves better trade-off between directivity (of the lowest frequency) and maximum azimuth steering angle (of the highest frequency) compared to the other combinations with the supported radio bandwidth > 11.2 GHz or FBW > 26%.

  - Observation 6：Traditional 20% FBW is not suitable to multi-band antenna scenarios. For some FR1 multi-band antenna FBW can be up to 61% with the acceptable electrical performance.

  - Observation 7: FR2 MB BS can design each band antenna separately to achieve best performance meanwhile without significant increase in PCB area.

  - Observation 8：Multi-band antenna element are feasible to cover FR2 frequency ranges.

  - Observation 9：Optical lens antenna, such as Luneburg Lens antenna may be considered for FR2 MB BS.

- Recommended WF

  - Discuss on these observations and please indicate which observations would be acceptable as RAN4 common understanding

Issue 2-3: Phase shifters

- Observations/proposals on feasibility study from contributions:

  - Observation 1: Using single frequency-invariable PS per RF chain to do concurrent beamforming transmission for multi-band signals increases intra-band interference which impacts OTA receiver out-of-band blocking requirement while being energy inefficient.

  - Observation 2: Using multiple frequency-invariable PSs per RF chain to do concurrent beamforming transmission for multi-band signals will increase size, cost and energy consumption for the FR2 radio which might not be of practical interest.

  - Observation 3: Multi-band PS, i.e., frequency-selective PSs might not be soon commercially available yet.

  - Observation 4: In general the use of phase shifter poses a challenge for practical implementations of FR2 multi-band RIB.

  - Observation 5: The implementation of PS in beamforming architecture may impact the declaration of FR2 multi-band RIB.

  - Observation 6：Phase shifter is demonstrated to cover a bandwidth of 23.8–30.4 GHz.

  - Observation 7：Considering the coupling of beam management of different bands, multi-band shifter may not a good candidate. Single-band shifter is more suitable, especially for larger band gap case.

  - Observation 8：Even if the single-band shifter is adopted in FR2 BS, it still belongs to the MB BS as long as the front-end use multi-band implementation.

- Recommended WF

  - Discuss on these observations and please indicate which observations would be acceptable as RAN4 common understanding

Issue 2-4: Frequency ranges/groups

- Proposals:

  - Proposal 1: The maximum supported bandwidth for FR2 multi-band RIB needs to be limited and defined.

  - Proposal 2: MB BS supporting 26G+40G is feasible from RF front-end and antenna implementation perspective based on preliminary study.

  - Proposal 3: The Rel-18 Study Item focus the multi-band RIB within the same frequency group, and does not consider multi-band RIB covering bands from the 24-30GHz and 39-40GHz frequency groups together.

- Recommended WF

  - Discuss on these proposals and please indicate which proposal would be acceptable as RAN4 agreements

Companies views’ collection for 1st round

Open issues

Collection of comments:

Issue 2-1: RF front-end

| Company | Comments |
| --- | --- |
| Huawei | Observation 1, 4, 5 and 6 indicate that RF front-end are feasible to cover multi-band FR2 bands. Observation 2, 3, 7 and 8: more studies are required, of course it is challenging for wide band but our paper shows some examples are feasible. And assuming a particular type of DPD implementation is a bit limiting, for FR2 DPD is not commonly adopted for ABF architecture. |
| Nokia | Observations 1, 2, 3, 5, 7, 8, 9 could be acceptable as common RAN4 understanding. Observations 4 and 6 are from literature, and we agree that whether they will be soon realistic are questionable. |
| Ericsson | Observation 1 is agreeable in principle, but we would need to add that then the same beam direction would need to be applied in all bands. Observation 2, 3 are OK Observation 4 is OK but we should add that more data would be needed on PA performance with multiple simultaneous bands, overall efficiency etc. Observation 5 Seems OK Observaiton 6 is too general Observation 7/8; generally OK, but needs to be formulated to consider that DPD is highly complex if needed and also the bandwidth of interfaces and data shifting needs to be mentioned. Observation 9 needs to be checked whether there are any issues with performance or DPD bandwidth etc. |
| ZTE | Reply Ericsson, for FR2 even without DPD, the ACLR requirement could be still achieved, maybe DPD is not the bottle neck. In short, we think that 28+40GHz multi-band BS might be not easy to be implemented at the current phase. |
| CATT | Generally, all observations are OK and can be acceptable to us. |

Issue 2-2: Antenna array

| Company | Comments |
| --- | --- |
| Huawei | Traditional 20% FBW criteria is not suitable to multi-band antenna scenarios. As shown in our paper, FR1 multi-band antenna FBW can be up to 61% with the acceptable electrical performance. For FR2 in our view Multi-band antenna element are feasible to cover FR2 frequency ranges and antenna array need further study. On the other hand, as indicated in observation 3 and 7 separate antenna array can be implemented for ultra-wide band combination, without significant increase in PCB area. |
| Nokia | Need to first clarify if understanding is from perspective of FR2 multi-band RIB or multi-band BS. For example, Observations 3 and 7 may not be acceptable from multi-band RIB perspective. |
| Ericsson | For an array with a large FBW, the performance will obviously be compromised to at least some degree compared to lower FBW. For multiple arrays, the question is then on the impact to power, noise figure etc. of the associated signal routing. For multiple arrays and lens etc., another question is whether filtering and matching are straightforward to achieve. Again the question is whether in the Rel-19 timescale it is mature to decide requirements. Observation 1 is OK Observation 2 is OK, or could be reformulated to state that such FBW would lead to a performance decrease. Observation 3 should state that there may be losses in power or NF and possibly non-linearity effects that need to be considered. Observation 4 seems OK Observation 5 OK Observation 6: What is meant by “acceptable electrical performance”. It should probably say “some degree of performance loss” Observation 7: May be better to state that “there is potential to design each antenna separately” and then that issues such as power of NF loss, matching etc. need to be considered. Observation 8 is too general Observation 9 is in principle OK to consider, but should capture that performance needs further evaluation. |
| Qualcomm | For observation 3, what would be the impact in terms of area increase when multiple single-band AAs are used? Observation 9, would the work here intend also to scope down the key enablers for FR2 MB BS? |
| CATT | From our side, The feasible FBW (or bandwidth) for wide-band antenna array is dependent on the trade-off between FBW and performance degradation. |

Issue 2-3: Phase shifters

| Company | Comments |
| --- | --- |
| Huawei | From the discussion, in our view, using multiple frequency-invariable PSs per RF chain to do concurrent beamforming transmission for multi-band signals offer the feasibility for MB BS. Due to the adoption of multi-band front-end, it can reduce the size and cost compared to single band RIB implementation. |
| Nokia | We noticed the term ‘multi-band phase shifter’ may be understood differently in Observations 3 and 7. Multi-band phase shifter in Observation 3 means frequency-selective phase shifter which can control phase/amplitude of each operating band independently, while that in Observation 7 means frequency-invariable PS covers wideband. |
| Ericsson | Clearly a solution would need to be found to enable separate beamforming in each band. Multiple PS may be suitable, although impacts to linearity, NF need to be carefully understood when considering requirements based on multiple PS for each band. Observations look generally OK |
| CATT | Generally, observations can be acceptable to us. It is dependent on Beamformer IC. |

Issue 2-4: Frequency ranges/groups

| Company | Comments |
| --- | --- |
| Huawei | From our study, MB BS supporting 26G+40G is feasible from RF front-end and antenna implementation perspective. Furthermore, we do not think there is a need to place any limits on frequency range/groups in the spec. FR1 has many bands which are not feasible for multi-band operation and yet there is not specific mention of what is and is not possible. The specifications exist and meeting them is left to implementation. We see no reason why this is different. Clearly from our paper we have shown the feasibility in the future that these bands could be implemented, it is perhaps ok to disagree but there isn’t need to limit based on different views on implementation. One possible reason to limit the study would be if it was proposed to make relaxations to requirements based on fractional BW. In this case then possibly this type of proposal is reasonable as if relaxations were based on implementation then it might be sensible to limit the difficultly of implementation. But we see no real justification to make these sort of relaxations as they impact system throughput. With no relaxations as with FR1 there is no need to mention fraction BW limits or limit the frequency range/groups. |
| Nokia | OK with Proposal 3, but still prefer Proposal 1 to be discussed as there is possibly >11GHz band combination in frequency group 37-48GHz which could be challenging. Proposal 3 should be stated specific to FR2 multi-band RIB. |
| Ericsson | Thanks Huawei and Nokia for the information and observations. We need to take care on what is relevant when discussing feasibility within the context of the SI. What is important is not necessarily whether it would be ever possible to build, but whether the level of knowledge is sufficient to set the requirements in the Rel-19 timescale. Clearly in academic research there is promising work on wideband amplifiers, receiver front ends, phase shifters etc., which suggests that building wideband FR2 may be possible. More work is needed to characterize the performance considering that in multi-band, wideband signals are transmitted/received in different parts of the spectrum simultaneously (i.e., not one band or the other), intermediate spurious emissions requirements need to be met, independent beamforming is needed etc. Also issues such as matching of a new array structure need to be taking into account, impact on output power or noise figure of filtering etc. Also algorithmic considerations such as DPD, CFR, receiver algorithms and the associated large bandwidths, sampling rates, interface sizes etc. For a commercial product, there is of course then the question whether the wideband implementation can become more efficient (considering power, complexity, cost, weight etc.) than two narrower bandwidth implementations. For RAN4, the question for wideband operation is whether a re-use of the FR1 approach is suitable and whether all of the requirements (for example, TX linearity related requirements or receiver blocking, IM etc.) are suitable. “Feasibility” in our view refers to whether in the release 19 timescale we will have progressed enough to make this decision. Hence our proposal to consider the 24-30 and 37-40 ranges separately for now in the Rel-19 timescale. |
| ZTE | We support the proposal 3. |
| CATT | Fine with proposal 3. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1: RF front-end | Based on 1st round discussion, observation 1 and 5 are ok as common RAN4 understanding, while companies hold different view on other observations. Moderator suggest to agree on observation 1 and 5. More discussion are needed for other observations. - Observation 1: Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible. - Observation 5: For RF front-end, commercially available TRX chips cover 24-29.5GHz. 27-41GHz RX is implemented. A harmonic-selection technique is proposed to extend the receiver’s operating bandwidth up to 24.25-71GHz. |
| Issue 2-2: Antenna array | Based on 1st round discussion, none of the observations can be agreed and more studies are needed. Nokia raised one clarification question on whether using multiple single-band antenna arrays for wide band combination can be considered for multi-band RIB which can be discussed for 2nd round. |
| Issue 2-3: Phase shifters | Based on 1st round discussion, none of the observations can be agreed and more studies are needed. Moderator suggest to discuss the following 2 updated observations for 2nd round. - Multi-band frequency selective phase shifter might not be soon commercially available but should not excluded specifically. - Multiple single band frequency selective phase shifter with common PA is feasible while the impact to requirements/performance need further study. |
| Issue 2-4: Frequency ranges/groups | Based on 1st round discussion, 4 companies support Proposal 1, and Proposal 1 and 2 each has one support. Moderator think proposal 3 does not follow the WID which includes the example band 26+28 GHz, 28+39 GHz, 26+40 GHz and 28+40 GHz, and suggest to further discuss it for 2nd round. - Proposal 1: The maximum supported bandwidth for FR2 multi-band RIB needs to be limited and defined. (Nokia) - Proposal 2: MB BS supporting 26G+40G is feasible from RF front-end and antenna implementation perspective based on preliminary study. (Huawei) - Proposal 3: The Rel-18 Study Item focus the multi-band RIB within the same frequency group, and does not consider multi-band RIB covering bands from the 24-30GHz and 39-40GHz frequency groups together. (Ericsson, Nokia, ZTE, CATT) |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on feasibility of FR2-1 multi-band BS | Huawei |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Topic #3: Requirements of FR2-1 multi-band BS

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215413 | CATT | Observation 11: Existing definitions of Inter RF Bandwidth gap and Base Station RF Bandwidth for multi-band operation can be reused for FR2 multi-band operation. Observation 12: The existing method for requirement for FR1 multi-band operation can be reused for that for FR2 multi-band operation. |
| R4-2216243 | Huawei, HiSilicon | The required actions to impliment FR2 multi-band have been identified as follows: Table 3-1: The required actions to implement FR2 multi-band Clause; Requirement; Actions for FR2 MB. OTA. 9.5.2.1; Transmitter OFF power; No change needed. 9.7.3.2; ACLR; Action 1: agree the sub-block gaps for the FR2 non-contiguous spectrum can be applied to FR2 multi-band. Action 2: Extend the ranges in note 3 and 4 to ensure the Inter RF bandwidth gaps are covered. Action 3: Discuss the clarification for the case different ACLR requirements applies to the two bands. Action 4: agree a set of sub-block gaps for FR2 channel BW’s for CACLR multi-band.. 9.7.4.2; OBUE; Observation 1: FR1 OTA multi-band definition referencing does not completely define the requirement. Action 5: agree the applicability inside the Inter RF Bandwidth gaps for FR2 (potentially copy FR1 where Inter RF Bandwidth gaps with Wgap < 2*ΔfOBUE).. 9.7.5.2.1; TX Spurious emissions; Action 6: apply the SE operating band exemption to FR2 multi-band. 9.7.5.2.4; Additional spurious emissions (co-existence); No action required.. 9.7.5.2.5; co-location; No action required.. 9.8.2; Tx intermodulation; Observation 2: The term multi-band RIBs should be used for the FR1 requirement. FR2 No action needed.. 10.5.1; ACS; Action 7: Apply multi-band requirements inside Inter RF Bandwidth gap if it as wide as the interfering signal in the same way as FR1.. 10.5.2; In-band blocking (general and narrow band); Action 8: Apply multi-band requirements inside Inter RF Bandwidth gap if it as wide as the interfering signal in the same way as FR1.. 10.6.2.1; Out of band blocking; Action 9: Apply the multi-band exceptions to FR2 minimum requirements sub-clause.. 10.7.2; Rx spurious emissions; Action 10: Apply the multi-band exceptions text to the BS type 2-O minimum requirements sub-clause.. 10.8.2; Receiver Intermodulation; Action 11: Apply multi-band requirements inside Inter RF Bandwidth gap if it as wide as the interfering signal in the same way as FR1.. |
| R4-2216397 | Ericsson | 6.2 Re-using FR1 multi-band methods In general, most multi-band methods applicable for FR1 requirements could be re-used for FR2-1. 6.3 Re-using FR1 exceptions The FR1 exceptions for spurious emissions, RX spurious emissions and out of band blocking, i.e. that the requirements do not apply in any of the bands supported by a multi-band RIB could be applied in FR2-1. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3 –Requirements of FR2-1 multi-band BS

Issue 3-1: General

- Proposals:

  - Proposal 1: Existing definitions of Inter RF Bandwidth gap and Base Station RF Bandwidth for multi-band operation can be reused for FR2 multi-band operation.

  - Proposal 2: The existing method for requirement for FR1 multi-band operation can be reused for that for FR2 multi-band operation

  - Proposal 3: The FR1 exceptions for spurious emissions, RX spurious emissions and out of band blocking could be applied in FR2-1

- Recommended WF

  - Discuss whether these proposals are agreeable

Companies views’ collection for 1st round

Open issues

Collection of comments:

Issue 3-1: General

| Company | Comments |
| --- | --- |
| Huawei | In general we are ok with these proposals as starting point. |
| Nokia | Ok with proposals 1 and 2. For proposal 3, what about the other requirements identified at last meeting in R4-2211811: transmit ON/OFF power, operating band unwanted emissions, transmitter/receiver intermodulation, in-band selectivity and blocking, could the FR1 exceptions be applied in FR2-1? |
| Ericsson | We agree with these as a starting point; there may still be a need to clarify whether requirement levels and definitions are suitable. |
| Qualcomm | Ok with the list of proposals. |
| ZTE | we are ok with these proposals as starting point. |
| CATT | OK with proposal 1 and 2. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 3-1: General | Based on 1st round discussion, companies are ok with Proposal 1 and 2 as a starting point. Moderator suggest to approve proposal 1 and 2 as a starting point: - Proposal 1: Existing definitions of Inter RF Bandwidth gap and Base Station RF Bandwidth for multi-band operation can be reused for FR2 multi-band operation. - Proposal 2: The existing method for requirement for FR1 multi-band operation can be reused for that for FR2 multi-band operation And check whether modified proposal 3 is agreeable. - Proposal 3: The FR1 exceptions for spurious emissions, RX spurious emissions and out of band blocking could be applied in FR2-1 as well as all other multi-band considerations made for FR1 for example transmit ON/OFF power, operating band unwanted emissions, transmitter/receiver intermodulation, in-band selectivity and blocking. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on requirements of FR2-1 multi-band BS | Nokia |

Discussion on 2nd round (if applicable)

| Issues | Company Comments |
| --- | --- |
|  | Company A: Company B: |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

Recommendations for Tdocs

1st round

New tdocs

| Title | Source | Comments |
| --- | --- | --- |
| WF on definition of FR2-1 multi-band BS | CATT |  |
| WF on feasibility of FR2-1 multi-band BS | Huawei |  |
| WF on requirements of FR2-1 multi-band BS | Nokia |  |

Existing tdocs

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-2215413 | General consideration on mmWave multi-band BS | CATT | Noted |  |
| R4-2215575 | Proposals on definition of FR2 multi-band BS | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215576 | Discussion on possible issues on performance of wideband RF and antenna architectures | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215768 | FR2 Multi-band BS | Murata Manufacturing Co Ltd. | withdrawn |  |
| R4-2216240 | Definition of FR2 multi-band BS | Huawei, HiSilicon | Noted |  |
| R4-2216241 | TP for deployment scenarios | Huawei, HiSilicon | Revised |  |
| R4-2216242 | Discussion of mmWave multi-band BS feasibility | Huawei, HiSilicon | Noted |  |
| R4-2216243 | Study on the FR1 multi-band methods | Huawei, HiSilicon | Noted |  |
| R4-2216397 | TP to TR 38.877: NR mm wave BS | Ericsson | Noted |  |
| R4-2216551 | Further discussion on FR2 multi-band operation | ZTE Corporation | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- |
| R4-210xxxx | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-210xxxx | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-210xxxx | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents

Annex

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Man Hung Ng | man_hung.ng@nokia.com |
| Ericsson | Tom Chapman | Thomas.chapman@ericsson.com |
| Qualcomm Incorporated | Mustafa Emara | memara@qti.qualcomm.com |
| CATT | Huiping Shan | shanhuiping@catt.cn |
| Huawei | Liehai Liu | liuliehai@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)