3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2216893

Electronic Meeting, 10– 19 October 2022

Agenda item:			6.16.4

Source:	Moderator (Ericsson)

Title:	Email discussion summary [104-bis-e][309] NR_LTE_EMC_enh

Document for:	Information

# Introduction

This agenda item will handle all contributions related to NR_LTE_EMC_enh aspects:

- BS EMC enhancements

- UE EMC enhancements

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: TBA

- 2nd round: TBA

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| Ericsson | Aurelian Bria (Moderator) | aurelian.bria@ericsson.com |
| Ericsson | Bing Li | Bing.li@ericsson.com |
| ZTE | Xiangwei Jing | Jing.xiangwei@zte.com.cn |
| Xiaomi | Rui Zhou | zhourui1@xiaomi.com |
| Huawei | Michal Szydelko | Michal.szydelko@huawei.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

# Topic #1: Work plan

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216488 | Ericsson | Work Plan proposal on the BS EMC enhancements (only Performance part) |
| R4-2216168 | Xiaomi | Work Plan proposal on the UE EMC enhancements |

## Open issues summary

### Sub-topic 1-1

Sub-topic description: BS EMC Enhancements work plan

- Recommended WF

  - TBA

### Sub-topic 1-2

Sub-topic description: UE EMC Enhancements work plan

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Sub topic 1-1

| Company | Comments |
| --- | --- |
| Huawei | For the BS part, we have realized that there is some issue with the TU budget in RP-220916. As the Rel-18 core work has just started in RAN4, we have already jumped to the Rel-18 discussions for this WI. We are not sure if this is the right approach, and would like to double-check with RAN4 leadership. Therefore, prefer to Return to the work plan for BS until this is clarified. |

Sub topic 1-2

| Company | Comments |
| --- | --- |
| XXX |  |

### CRs/TPs comments collection

NA

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Issue | Status summary |
| --- | --- |
| 1-1 | Tentative agreements: R4-2216488 Candidate options: as proposed in R4-2216488 Recommendations for 2nd round: Continue discussion, will be on agenda for GTW Oct 18th |
| 1-2 | Tentative agreements: R4-2216168 Candidate options: as proposed in R4-2216168 Recommendations for 2nd round: Continue discussion, will be on agenda for GTW Oct 18th |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Following the comment from Huawei, we are not in a hurry with BS EMC enhancements and the work plan can suffer some modifications.

UE EMC is currently focusing on the core part, while core part does not exist for BS EMC.

Work plans can be approved this meeting, and later adapted after a few rounds of discussion. Please bring your input until GTW.

# Topic #2: BS EMC Enhancements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215958 | Ericsson | Observation 1: By grouping GSM and NB-IoT in the narrowband category, WCDMA, LTE and NR in the wideband category, we have a starting point to develop a RAT-agnostic analysis based on the categorization. Observation 2: From the testing perspective, conducted emission, conducted immunity, and radiated immunity are independent of the RATs. For radiated emission, the RATs can be reduced to two RATs (one narrowband + one wideband), representing the worst case. Observation 3: From the performance criteria perspective, for narrowband RATs, NB-IoT is the worst case, comparing to GSM. For wideband RATs, NR/LTE is the worst case, comparing to WCDMA. Proposal 1: Using NB-IoT test result cover GSM, and LTE cover WCDMA, then we can reduce EMC MR test scope to maximum in three RATs combination; Furthermore, if NR can cover LTE, or vice versa, we can reduce MSR test scope to maximum two RATs combination, as alternative test method for MSR BS. |
| R4-2215959 | Ericsson | Observation 1: One characteristic of MSR BS is the ability of receiver and transmitter to process two or more carriers in common active RF components simultaneously. Proposal 1: To agree with using the following capability sets: CS1, CS2, CS4, CS5, CS8, CS10, CS11, CS16, and CS17 defined in TS 37.141 [2] for EMC testing of MSR BS, as presented in this contribution. Furthermore, the capability sets can be narrow down to CS2, CS8, CS11, and CS17 if the MSR BS supports NR, LTE and NB IoT SA. |
| R4-2215731 | ZTE | Observation 1: The NB-IoT standalone rated output power has same value as BS’s however for some bands like band 85, the rated output power is 43dBm. Observation 2: The maximum output power for GSM Base Transceiver Station(BTS) is 38dBm for medium range and 24 dBm for local area. Observation 3: The test configuration in TS37.113 and TS 37.114 are not aligned with TS37.141and TS37.145-1/2. Observation 4: CS18 and CS19 are missing in TS37.113. CSA3B is missing in TS37.114. Proposal 1: Only test NB-IoT standalone for the CS consisting both GSM and NB-IoT standalone. Proposal 2: The rated output power of UTRA, E-UTRA and NR are same with each other. Further study is required for the worst case of wideband RAT. Proposal 3: An alternative way is to align the test configuration between the CSx and CSy, in which the RATs in the CSx are the subset of the RATs in CSy. |

## Open issues summary

### Sub-topic 2-1

Sub-topic description: Principles for BS EMC test case optimization

Proposals:

- using existing TCs and not creating a new TC

- covers at least one wideband and one narrowband active

- NB-IoT covers GSM test case

- E-UTRA covers UTRA test case

- Recommended WF

  - TBA

### Sub-topic 2-2

Sub-topic description: Proposal for limited set of CS

Proposals:

- CS1, CS2, CS4, CS5, CS8, CS10, CS11, CS16, and CS17 defined in TS 37.141 [2] for EMC testing of MSR BS

- Capability sets can be narrow down to CS2, CS8, CS11, and CS17 if the MSR BS supports NR, LTE and NB IoT SA.

- Recommended WF

  - TBA

### Sub-topic 2-3

Sub-topic description: Proposal for an alternative way to reduce CSs

Proposal:

- An alternative way is to align the test configuration between the CSx and CSy, in which the RATs in the CSx are the subset of the RATs in CSy.

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Sub topic 2-1

| Company | Comments |
| --- | --- |
| Ericsson | Agree with ZTE’s proposal of using NB IoT to cover GSM. |
| ZTE | Agree with the top three issues, but for the last one, E-UTRA cover UTRA, further study is required. Not sure why does E-UTRA worse than UTRA. More evidences are required. Besides, the behaviour of NR should also be considered. |
| Huawei | The analysis shall start listing all possible RAT combinations for the MSR BS. Narrowband/wideband grouping does not reduce workload if BS supports only e.g. NBIoT and NR. Theoretical analysis of the narrow-band vs. wide-band RATs does not necessarily map to the worst case scenario for the radiated emissions. Its unclear how we can draw conclusions without extensive measurement campaigns – and even if that would be done, one can question that some future low-cost products may pass such simplified EMC testing, i.e. cheat. One potential solution that we can think of is based on some sore od manufacturer declaration, saying, e.g. that manufacturer declares that the worst case scenario for RATx + RATy, and testing this single case is sufficient to show compliance. We need to have also some discussion on the future-proofness: how we envision to incorporate future 6G into such framework. Similar, how to consider NTN IoT Rel-18 WI (narrowband 1RB signals) into this? Some workflow would be good to be derived. At the current stage we are not in a position to accept Proposal 1 from R4-2215958. Formal question to sub-topic 2-1: is the intention to process all 4 bullets as package? - using existing TCs and not creating a new TC: we need to have some more clarifications on this – agree not to define new TCs for EMC, but if we are going to reduce testing, it is expected that certain TCs will be tested only partially. Is this the final intention? - covers at least one wideband and one narrowband active: this may simplify testing only in case the BS supports multiple narrowband and/or multiple wideband RATs. - NB-IoT covers GSM test case : don’t agree with such conclusion based on a single set of product specific measurements - E-UTRA covers UTRA test case: don’t agree with such conclusion based on a single set of product specific measurements |
| Ericsson | @Huawei: “The analysis shall start listing all possible RAT combinations for the MSR BS. Narrowband/wideband grouping does not reduce workload if BS supports only e.g. NBIoT and NR.” Ericsson: All possible RAT combinations (all 19 CSs listed in TS 37.141) were analyzed in R4-2215959. There is no need to reduce workload for all RAT combinations. If BS supports only e.g., NB IoT and NR, we can leave as it is. We should focus more on the MSR BS that supports three or more RATs. “Theoretical analysis of the narrow-band vs. wide-band RATs does not necessarily map to the worst case scenario for the radiated emissions. Its unclear how we can draw conclusions without extensive measurement campaigns – and even if that would be done, one can question that some future low-cost products may pass such simplified EMC testing, i.e. cheat.” Ericsson: The proposals in Sub-topic 2-1 are based on both theoretical analysis and massive practical measurements by Ericsson. The measurement result shown in R4-2215958 is just an example. People should always be responsible for their own products, and never cheat. What is the definition of low-cost product? Why is it possible to cheat? The proposal is an alternative way of EMC testing. If a MSR BS declared supporting 5 RATs, even if only 3 RATs are tested (using the alternative way), the BS shall fulfill the requirements set for 5 RATs. “One potential solution that we can think of is based on some sore od manufacturer declaration, saying, e.g. that manufacturer declares that the worst case scenario for RATx + RATy, and testing this single case is sufficient to show compliance.” Ericsson: It is possible to use self-declaration. However, it is better to reach consensus within 3GPP what is the worst case scenario for EMC testing. “We need to have also some discussion on the future-proofness: how we envision to incorporate future 6G into such framework. Similar, how to consider NTN IoT Rel-18 WI (narrowband 1RB signals) into this? Some workflow would be good to be derived.” Ericsson: New RATs are FFS, not the scope of this WI. But it is good to keep it in mind during this WI. “Formal question to sub-topic 2-1: is the intention to process all 4 bullets as package? - using existing TCs and not creating a new TC: we need to have some more clarifications on this – agree not to define new TCs for EMC, but if we are going to reduce testing, it is expected that certain TCs will be tested only partially. Is this the final intention? - covers at least one wideband and one narrowband active: this may simplify testing only in case the BS supports multiple narrowband and/or multiple wideband RATs. - NB-IoT covers GSM test case : don’t agree with such conclusion based on a single set of product specific measurements - E-UTRA covers UTRA test case: don’t agree with such conclusion based on a single set of product specific measurements” Ericsson: It is OK to discuss them one by one. - The intention is to use existing TC of less RATs to cover TC of more RATs. What does it mean by “certain TCs will be tested only partially”? - If a BS supports only one narrowband RAT and one wideband RAT, it is possible to configure the BS and perform EMC test at once. Not meaningful to reduce RAT, since there is no significant difference regarding workload/time and cost consumption. Again, we should focus more on the MSR BS that supports three or more RATs. - Again, Ericsson’s proposals are based on massive measurements, the measurement shown in R4-2215958 is just an example. Please consider the analysis and observations in R4-2215958 as well as the observations in ZTE’s R4-2215731. If Huawei has doubt on the proposals in Sub-topic 2-1, welcome to modify or propose new ideas for discussion. |

Sub topic 2-2

| Company | Comments |
| --- | --- |
| ZTE | Wait for the decision in topic 2-1. It is a good way to implement the simplification, but need to check whether E-UTRA can cover UTRA. |
| Huawei | See comments to 2-1. At the current stage we are not in a position to accept Proposal 1 from R4-2215959. |

Sub topic 2-3

| Company | Comments |
| --- | --- |
| Ericsson | The idea is good, but the example maybe not appropriate for using G + W to cover G + W + L, more research needs to be done. |
| Huawei | How to prove that such theoretical analysis will not result in the removal of the worst case scenario from the conformance testing? |

### CRs/TPs comments collection

#### General

| tdoc number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Issue | Status summary |
| --- | --- |
| 2-1 2-2 2-3 | Tentative agreements: no tentantive agreements for this meeting Candidate options:- Recommendations for 2nd round: Continue discussion, some topics will be on agenda for GTW Oct 18th |

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Continue discussion on the main principles for optimizing the BS EMC testing:

- using existing TCs and not creating a new TC

  - look into the possibility to actually define a new TC

- which combination of wideband and narrowband carriers in different RATs are needed

- NB-IoT covers GSM test case – companies should provide input during next meetings

- E-UTRA covers UTRA test case - companies should provide input during next meetings

Conclusions will be captured in a WF lead by Ericsson – will be on the agenda of the GTW Oct 18th

# Topic #3: UE EMC Enhancements

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216166 | Xiaomi | Observation 1: The EN-DC part improvement within CCSA has been agreed to be carried while there is no progress yet. Observation 2: The EN regulation RMC selection for NR SA is based on TS 38.521-1 and only single carrier RMC is selected for NR SA and no CA is included. Observation 3: The EN regulation RMC selection for NR NSA is based on TS 38.521-3 while no EN-DC selection criteria is mentioned. Observation 4: A maximization of the reference sensitivity level for the EUT is used in EU regulation for establishing the communication link. Proposal 1: For immunity test, the maximum of reference sensitivity can be the starting point for test configuration simplification. |
| R4-2216167 | Xiaomi | Observation 1: For Chinese regulation, the EMC specification framework is for each RAT there is one specification to cover the requirement and there is one general rule and one specification for Multi-RAT. Observation 2: For EU regulation, that the arrangement of different test signals for different RATs are listed and for NR it has been separated into SA and NSA mode. Proposal: Down select NR CA and DC features as the only feature to be studied and enhanced in Rel-18 EMC enhancement WID. |

## Open issues summary

### Sub-topic 2-1

Sub-topic description: Starting point for immunity test

Proposal:

- For immunity test, the maximum of reference sensitivity can be the starting point for test configuration simplification.

- Recommended WF

  - TBA

### Sub-topic 2-2

Sub-topic description: Features to be studied

Proposal:

- Down select NR CA and DC features as the only feature to be studied and enhanced in Rel-18 EMC enhancement WID.

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

Sub topic 3-1

| Company | Comments |
| --- | --- |
| Ericsson | It is confusing by using the word "maximum", whereas the referred text is "attempt to maximize the reference sensitivity level". Could you please write the proposal more explicitly? Does the proposal mean you are trying to find the "minimum mean power received at the antenna connector"? Since it is for immunity test, maybe it is better to add 4.2.6.1 as a reference, which is more specific for test signals at receiver, referring to 4.2.1 for reference sensitivity level, and showing the purpose of your proposal. |
| ZTE | - Agree, reference sensitivity level can be taken as a starting point. But it may face some issues for UE with EN-DC feature. |
| Xiaomi | - Thanks for the comments. We share the similar understanding. The 4.2.1 sub-clause is from the EU regulation, indeed we haven’t starting at the spec yet but just some initial thoughts. |
| Huawei | Unclear what is the link between the refsens level and test configuration simplifications. Could proponents clarify? |

Sub topic 3-2

| Company | Comments |
| --- | --- |
| Ericsson | In the WID, there are more features to be studied, e.g., for NR, SUL, UL MIMO and V2X; for LTE, CA DC, the reason of removing the study on them should be provided. |
| ZTE | Agree with Ericsson. NR CA and DC can be studied first. Not sure why the other features are excluded. |
| Xiaomi | We have already provided the reason as that current regulation is not interested in it and considering the work load. |
| Huawei | We need to follow WID objectives. Fine to have work and features prioritization. To double-check how this relates to the UE work-plan. |

### CRs/TPs comments collection

#### General

| tdoc number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| Issue | Status summary |
| --- | --- |
| 3-1 | Tentative agreements: For immunity test, the maximum of reference sensitivity can be the starting point for test configuration simplification. Recommendations for 2nd round: capture the agreement in the WF (on the agenda for GTW) |
| 3-2 | Tentative agreements: start with EN-DC Recommendations for 2nd round: Continue discussion, capture agreements in WF |

## Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Continue discussion on the issues and capture the eventual agreements in the WF document lead by Xiaomi.

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on principles for BS EMC enhancements | Ericsson |  |
|  | WF on continuation of study phase for UE EMC enhancements | Xiaomi |  |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2216488 |  | Work Plan proposal on the BS EMC enhancements (only Performance part) | Ericsson | Return to | Check the TU allocation and Rel-18 timeline |
| R4-2216168 |  | Work Plan proposal on the UE EMC enhancements | Xiaomi | Return to | No comments received in round 1 |
| R4-2215958 |  | Discussion of BS EMC Enhancement for NR and LTE | Ericsson | Noted |  |
| R4-2215959 |  | Proposal for Optimization of EMC Test Configurations | Ericsson | Noted |  |
| R4-2215731 |  | Discussion on EMC BS enhancement R18 | ZTE | Noted |  |
| R4-2216166 |  | on the regulation study of UE EMC | Xiaomi | Noted |  |
| R4-2216167 |  | on the study phase of UE EMC | Xiaomi | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents