3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2216907

Electronic Meeting, 10– 19 October 2022

Agenda item:			4.6.6

Source:	Moderator (Ericsson)

Title:	Email discussion summary for [104-bis-e][323] NR_RedCap_Demod

Document for:	Information

# Introduction

This email discussion thread discusses the UE demodulation and CSI reporting requirements for RedCap. The target of the email discussion is summarized as follows:

- 1st round:

  - Discuss the remaining open issues

  - Collect the updated simulation results and align the simulation results

  - Review the draft CRs

- 2nd round:

  - Conclude the remaining open issues

  - Review the revised draft CRs.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| MediaTek | Licheng Lin | licheng.lin@mediatek.com |
| Ericsson | Kazuyoshi Uesaka | kazuyoshi.uesaka@ericsson.com |
| Apple | Rolando Bettancourt | rbettancourt@apple.com |
| Nokia | Juergen Hofmann | juergen.hofmann@nokia.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e., Company A (XX, XX)

# Topic #1: UE demodulation requirements

## Companies’ contributions summary

### PDSCH/SDR requirements (4.6.5.1.1)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215625 | Apple Inc. | Proposal 1: The requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements - can be applied for both 2Rx HD-FDD and 2Rx FD-FDD |
| R4-2216027 | Huawei, HiSilicon | Proposal 1: Define the same requirements for 1RX HD-FDD and FD-FDD |
| R4-2216175 | Qualcomm Incorporated | Simulation results. |
| R4-2216221 | Nokia, Nokia Shanghai Bell | Proposal 1: Include PDSCH results for alignment and for requirements specification into the simulation summary. |
| R4-2216232 | Ericsson | Observation 1: Difference between FD-FDD scheduling and HD-FDD scheduling about 70% of maximum throughput is negligible. Proposal 1: For RedCap PDSCH demodulation requirements, the same requirements can be applied for both HD-FDD and FD-FDD. |
| R4-2216234 | Ericsson | Summary of simulation results for RedCap |
| R4-2215628 | Apple | Draft CR: PDSCH demodulation requirements for RedCap |
| R4-2216028 | Huawei, HiSilicon | Draft CR: Corrections of RedCap SDR requirements |
| R4-2216230 | Ericsson | Draft CR: Correction of RedCap UE demodulation requirements |

### PDCCH/PBCH requirement (4.6.5.1.2)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2216176 | Qualcomm Incorporated | Simulation results. |
| R4-2216222 | Nokia, Nokia Shanghai Bell | Proposal 1: Include PDCCH results for alignment and for requirements specification into the simulation summary. |
| R4-2216223 | Nokia, Nokia Shanghai Bell | Proposal 1: Include PBCH results for alignment and for requirements specification into the simulation summary. |

## Open issues summary

### Sub-topic 1-1 Applied requirements for HD-FDD

Issue 1-1-1: Whether to apply the same requirements for both 2Rx HD-FDD and 2Rx FD-FDD

Way forward from the last meeting: Interested companies can evaluate if the same requirements can be applied for both 2Rx HD-FDD and 2Rx FD-FDD

- Proposals (Apple, Ericsson, Huawei)

  - Confirm the agreement in RAN4#104: the requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements

- Recommended WF

  - Confirm the agreement in RAN4#104: the requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 1-1-1: Whether to apply the same requirements for both 2Rx HD-FDD and 2Rx FD-FDD |
| Ericsson | Issue 1-1-1: Whether to apply the same requirements for both 2Rx HD-FDD and 2Rx FD-FDD Support the recommended WF. |
| QC | Issue 1-1-1 We support the WF |
| Huawei | Issue 1-1-1 We support the WF |
| Apple | Issue 1-1-1 We support the recommended WF |
| Nokia | Issue 1-1-1: Whether to apply the same requirements for both 2Rx HD-FDD and 2Rx FD-FDD We are fine with the recommended WF. |

### CRs/TPs comments collection

Companies can put comment provide comments for CRs by adding New comments and suggested changes with Markup in revised CRs for being easily understood. Draft CR files are available in inbox:

- [104-bis-e][323] NR_RedCap_Demod/CRs_demod/1-x_R4-221yyyy_v00.docx

| CR/TP number | Comments collection |
| --- | --- |
| R4-2215628 (Apple) | 1-1: Draft CR PDSCH demodulation requirements for RedCap |
|  | Ericsson: See 1-1_REV_R4-2215628_v01_Ericsson.docx Apple: Thank you Ericsson for your suggestions. These will be incorporated into our final draft CR |
|  | Company B |
| R4-2216028 (Huawei, HiSilicon) | 1-2: Draft CR: Corrections of RedCap SDR requirements |
|  | Ericsson: See 1-2_REV_R4-2216028_v01_Ericsson.docx\ Huawei@ Ericsson Thanks for comments, we agree with only scheduling PDSCH in full slots and the HD-FDD pattern proposed by you |
|  | Nokia: We agree to the proposed changes by Ericsson. The table header needs correction: “5.5.1-1 Additional test parameters for HD-FDD single carrier”. |
| R4-2216230 (Ericsson) | 1-3: draft CR: Correction of RedCap UE demodulation requirements |
|  | Moderator: Recommend to add the update of PDCCH/PBCH requirements also to capture the updated simulation results in R4-2216234. |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 1-1-1: Whether to apply the same requirements for both 2Rx HD-FDD and 2Rx FD-FDD | Agreements: - Confirm the agreement in RAN4#104: the requirements with HD-FDD are the same as the corresponding (full-duplex) FDD requirements Recommendation for 2nd round: No discussion |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

Note: The tdoc decisions shall be provided in Section 3 and this table is optional in case moderators would like to provide additional information.

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2215628 (Apple) | To be revised Capture the suggestion in the 1st round comment. Update the PDSCH demodulation requirements based on the simulation result summary. |
| R4-2216028 (Huawei, HiSilicon) | To be revised Capture the suggestion in the 1st round comment. |
| R4-2216230 (Ericsson) | To be revised Remove PDSCH demodulation part, which is merged to R4-2215628. Add PDCCH/PBCH demodulation requirements to capture the updated simulation results. |

## Discussion on 2nd round (if applicable)

# Topic #2: CSI reporting requirements

## Companies’ contributions summary

### CQI requirements (4.6.5.2.1)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215626 | Apple Inc. | Proposal 1: Introduce CQI/RI/PMI delay of 14ms, along with proposed Resource/Report configuration of [10/1] [10/9] respectively. Proposal 2: Introduce proposed Aperiodic Report Slot Offset 3 slots and CQI/RI/PMI delay of 6ms. |
| R4-2216029 | Huawei, HiSilicon | Simulation results. |
| R4-2216030 | Huawei, HiSilicon | Proposal 1: (Slightly preferred) Keep the previous agreement on the static channel matrix in the frequency domain, that is, H1RX=11,. Set SNR test point X=3dB lower than 2Rx test case. Proposal 2: Use following CSI feedback pattern for HD-FDD/FDD CQI requirements - CSI-RS periodicity and offset: 10/1 (Periodic) - CSI-Report periodicity and offset: 10/9 (Periodic) - CQI/RI/PMI delay: 14ms |
| R4-2216177 | Qualcomm Incorporated | Observation 1: CQI reporting performances in static condition for the same SNR are expected to be similar for 1 RX/Rank=1 and 2 RX/Rank=2. Observation 2: The agreed static channel matrix H = [1 1]; proposed for CQI simulation alignment when using the codebookSubsetRestriction = 000001 results in 3dB SNR gain, summing (with the same phase) 2 TX ports at the TE outpit; Observation 3: The current agreements in [3] regarding the test points for CQI reporting test in static condition for RedCap UEs with 1Rx is not motivated by an expectation in performance but by the combination of choices for Channel Matrix and Codebook Index in the test setup; Observation 4: A careful choice for the Static propagation condition channel matrix for 1 RX UE H1RX=1j;, extracted from Channel matrix already included in 38.101-4, Annex B.1 results in no additional SNR gain with the typical CodebookSubsetRestriction value agreed; Proposal 3: For UE Receiver with 1 RX, define the static channel matrix in the frequency domain as H1RX=1j; Proposal 4: If H1RX=1j is used, test points for CQI reporting test in static condition for RedCap UEs with 1 RX are introduced X=0 dB lower than 2 RX (same test point for 1 RX and 2 RX); |
| R4-2216224 | Nokia, Nokia Shanghai Bell | Proposal 1: Include CQI reporting results for alignment into the simulation summary. |
| R4-2216233 | Ericsson | Observation 1: Throughput ratio with CQI delay 14 slots is higher than the agreed gamma (1.05) with enough margin. Proposal 1: Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD: - CSI-RS periodicity and offset: 10/1 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: 14ms Observation 2: 1Rx UE can report the same CQI index as 2Rx UE if static channel is configured as H1RX=1j. Proposal 2: If RAN4 configures H1RX=1j as the channel matrix for 1Rx UE CQI reporting test in static condition, the SNR test points for 1Rx UE are same as 2Rx UE, that is, SNR=8/9dB and SNR=14/15dB. If RAN4 configures H1RX=11 as the channel matrix for 1Rx UE CQI reporting test in static condition, the SNR test points for 1Rx UE are 3dB lower than 2Rx UE, that is, SNR=5/6dB and SNR=11/12dB. |
| R4-2216703 | MediaTek Inc. | Proposal 1: For HD-FDD in static/fading condition, use the following configurations for CQI reporting tests: - CSI-RS periodicity and offset: 10/5 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: 10ms |
| R4-2216225 | Nokia, Nokia Shanghai Bell | Draft CR: Finalization of channel quality reporting requirements under static condition for RedCap |
| R4-2216231 | Ericsson | Draft CR: Correction of RedCap CSI reporting requirements |
| R4-2216429 | Qualcomm Incorporated | Draft CR: UE Demodulation Requirements CQI for static channel |

### PMI/RI requirements (4.6.5.2.2)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215627 | Apple Inc. | Moderator: Not found |
| R4-2216031 | Huawei, HiSilicon | Simulation results. |
| R4-2216032 | Huawei, HiSilicon | Proposal 1: Use following configuration for RI test: FR1 - FDD: - CSI-RS periodicity and offset is 10/1, - CSI reporting periodicity and offset is 10/9, - CSI delay is 14ms. - CQI table 1 - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1 - TDD: - CQI table 1 - CBW/SCS: 20MHz/30kHz - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1 FR2: Reuse the requirements |
| R4-2216704 | MediaTek Inc. | Proposal 1: RAN4 defines the MIMO correlation matrix for 4x1 ULA high as Observation 1: The TP ratio for the case of 14ms CQI/RI/PMI delay is slightly lower for the case of 10ms CQI/RI/PMI delay. Proposal 2: RAN4 uses 10ms CQI/RI/PMI delay to define RI requirements. |
| R4-2216033 | Huawei, HiSilicon | Draft CR: Corrections of RedCap PMI requirements |
| R4-2216226 | Nokia, Nokia Shanghai Bell | Draft CR: Finalization of Rank Indicator reporting requirements for RedCap |
| R4-2216706 | MediaTek inc. | Draft CR: Corrections to ReCap PMI requirements |

## Open issues summary

### Sub-topic 2-1	CQI reporting requirements

Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD

Way forward from the last meeting: Interested companies are encouraged to evaluate the performance difference between CQI delay 14ms and 10ms in RAN4#104-bis-e. If significant performance degradation is observed compared with CQI delay 10ms, RAN4 will revisit the CQI/RI/PMI delay.

- Observations

R4-2216703 (MediaTek) Table 1. CQI simulation results for 2T1R, HD-FDD in fading condition CQI delay=10ms; CQI delay=14m. SNR; Percentage of reported CQI Index not in {medCQI-1, medCQI+1}; TP ratio; BLER; Percentage of reported CQI Index not in {medCQI-1, medCQI+1}; TP ratio; BLER. 9; 0.52; 1.11; 0.06; 0.53; 1.06; 0.08. 10; 0.53; 1.11; 0.07; 0.54; 1.10; 0.09. 15; 0.57; 1.07; 0.10; 0.57; 1.02; 0.13. 16; 0.56; 1.08; 0.11; 0.56; 1.04; 0.13. R4-2216233 (Ericsson) Table 1 Simulation results of CQI fading teste (1Rx). CQI delay: 14 slots; CQI delay: 10 slots. Alpha (>0.20); BLER (>0.02); Gamma (>1.05); Alpha (>0.20); BLER (>0.02); Gamma (>1.05). 10; 0.57; 0.20; 1.21; 0.58; 0.13; 1.33. 16; 0.52; 0.22; 1.20; 0.53; 0.17; 1.31. R4-2216029 (Huawei) SNR(dB); CQI delay; TP ratio of follow CQI/ Median CQI; Median CQI; Prob of(Median CQI-1<CQI< Median CQI+1); BLER with follow CQI. 6; 10ms; 1.54; 7; 0.42; 0.16. 6; 14ms; 1.40; 7; 0.42; 0.21. 7; 10ms; 1.60; 8; 0.41; 0.16. 7; 14ms; 1.49; 8; 0.41; 0.19. Table 2-1: Simulation results for FR1 FDD 1RX

- Proposals

  - Option 1 (Apple, Ericsson, Huawei):

    - CSI-RS periodicity and offset: 10/1

    - CSI-Report periodicity and offset: 10/9

    - CQI/RI/PMI delay: 14ms

  - Option 2 (MediaTek):

    - CSI-RS periodicity and offset: 10/5

    - CSI-Report periodicity and offset: 10/9

    - CQI/RI/PMI delay: 10ms

- Recommended WF

  - Proponents of Option 2 argue the throughput ratio between follow CQI and fixed median CQI with Option 2 cannot satisfy the requirement γ > 1.05, although the proponent of Option 1 argues both the options can meet the requirements with enough margin.

  - Moderator would like to ask companies to check the simulation results, and discuss the options.

Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests

Way forward from the last meeting: Interested companies are encouraged to evaluate the options for the static channel matrix and SNR test point offset.

- Proposals

  - Option 1 (Qualcomm):

    - Set the static channel matrix in the frequency domain as H1RX=1j.

    - Set the same SNR test point for both 1Rx and 2Rx UE, that is, SNR=8/9dB and SNR=14/15dB.

  - Option 2 (Huawei):

    - Set the static channel matrix in the frequency domain as H1RX=11.

    - Set SNR test point X=3dB lower than 2Rx test case, that is, SNR=5/6dB and SNR=11/12dB.

- Recommended WF

  - RAN4 has already agreed to set H1RX=11, but it seems the proponents of Option 1 have strong preference. It is also observed either option does not affect to UE performance.

  - Moderator recommends Option 1 if no companies have technical issues.

### Sub-topic 2-2 PMI reporting requirements

Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests

- Proposals (MediaTek)

  - Define the MIMO correlation matrix for 4x1 ULA high as follows:

- Recommended WF

  - Agree with the proposal.

### Sub-topic 2-3 RI reporting requirements

Issue 2-3-1: Test configuration for RI reporting tests

- Proposals (Huawei)

  - Use following configuration for RI reporting test

    - FR1 FDD

      - CQI table 1

      - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1

    - FR1 TDD

      - CQI table 1

      - CBW/SCS: 20MHz/30kHz

      - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1

    - FR2

      - Reuse the requirements

- Recommended WF

  - Agree with the proposal

Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz

- Observations

R4-2216704 (MediaTek) Table 1. TP ratio for RI test CQI/RI/PMI delay 10ms; CQI/RI/PMI delay 14ms. FD-FDD; 1.2; 1.1. HD-FDD; 1.2; 1.1. TDD; 1.2; 1.1. R4-2216031 (Huawei) Table 2-1: Simulation results for FR1 RI test SNR(dB); TP ratio: Follow RI/Fixed RI=1. FDD/HD-FDD; TDD. 16; 1.36; 1.50. 17; 1.30; 1.43. 18; 1.33; 1.56. 19; 1.41; 1.58. 20; 1.46; 1.63.

- Proposals

  - Option 1 (Huawei): Set the CQI/RI/PMI delay to 14ms

    - CSI-RS periodicity and offset is 10/1,

    - CSI reporting periodicity and offset is 10/9,

    - CQI/RI/PMI delay is 14ms.

  - Option 2 (MediaTek): Set the CQI/RI/PMI delay to 10ms

    - CSI-RS periodicity and offset is 10/5,

    - CSI reporting periodicity and offset is 10/9,

    - CQI/RI/PMI delay is 10ms.

- Recommended WF

  - Collect more inputs.

  - Companies are encouraged to check the simulation results with different delay options.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests Issue 2-3-1: Test configuration for RI reporting tests Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz |
| MediaTek | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD We support Option 2. The delay indeed impacts the throughput ratio. According to results from our company and other companies, the throughput ratio degrades when the delay is set to 14ms other than 10ms. In legacy CQI test cases for FDD, the delay is set to 8ms. Therefore, we think 14ms is too long for UE to reflect the channel quality in fading channel. Also, it is noted that either Option 1 or Option 2 is a valid setting from gNB side. Therefore, we propose the delay to be 10ms for UE to have more margin. Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests We are OK to the recommended WF. Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests Support the recommended WF. Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz We support Option 2. Like the arguments for Issue 2-1-1, we support to use 10ms to define the requirements. |
| Ericsson | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD We slight prefer Option 1. According to our simulation results, the throughput ratio is 1.2 even if CQI delay is 14ms, we see there are enough margin. On the other hand, since the difference between option 1 and option 2 is only the scheduling configuration, we are also ok with Option 2. @MediaTek, according to your simulation results in R4-2216703, even if the CQI delay is 10ms, the throughput ratio is 1.07/1.08 for higher SNR test points. We think the margin is also small. Could you check your simulation results? Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests We are fine with Option 1. Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests Support the recommended WF. Issue 2-3-1: Test configuration for RI reporting tests Support the recommended WF. Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz We have also run the simulation and the result throughput ratio is as follows: CQI/RI/PMI delay 10ms; CQI/RI/PMI delay 14ms. FDD; 1.17; 1.06. According to our simulation results, as MediaTek points, the throughput ratio exceeds the requirements (1.05) with 14ms, but the margin is very small. We therefore think it is better to set the CQI/PMI/RI delay to 10ms. We also want to listen other companies’ view. We are also thinking we should align the CQI/PMI/RI parameters between CQI fading tests and RI tests. |
| Qualcomm | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD Considering that most of the companies have reported better performances with CQI delay=10ms and that it can be achieved simply with a change in scheduling, we are okay with option 2; Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests Support recommended WF; Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests Support recommended WF; Issue 2-3-1: Test configuration for RI reporting tests Support recommended WF; Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz Support argument as Issue 2-1-1 can be done. CQI/PMI/RI delay should be aligned between the two cases; |
| Huawei | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD We slightly prefer option 1. Our simulation results show no much performance degradation can be observed for extending the CSI delay to 14ms. We think for 10ms delay the requirements are too loose. Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests Now we support option 2. After double checking, we don’t see any technical reasons to violate the previous agreements. Based on our understanding, the combination gain at the receiver side not only depends on the channel matrix but also depends on the precoding matrix. Taking following as an example, the optimal PMI for H1RX=11 is PMI index 0, the optimal PMI for H1RX=1j is PMI index 3. In the real implementation, BS always choose optimal PMI matrix to match the MIMO channel. Considering, PMI index 0 has been selected for CQI test, we don't see any problem to configure H1RX=11 to maximum the gain, we think it is common. Another way is to change the PMI to adjust the gain at receiver side. We support option 2 Codebook index; PMI; Combination gain for H1RX=11; Combination gain for H1RX=1j. 0; 2; 1. 1; 1; 0. 2; 0; 1. 3; 1; 2. Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests Support recommended WF; Issue 2-3-1: Test configuration for RI reporting tests Support recommended WF; Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz Our simulation results are based on assumptions with 14ms CSI delay, we didn’t see any problem. We suggest to use 14ms to be aliened with CQI test |
| Apple | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD Initially, we slightly preferred a CQI delay of 14ms. However, several companies have reported performance results with a CQI delay of 10ms. We are also OK with following Option 2 since it’s simply a scheduling issue Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests We support the recommended WF Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests We support the recommended WF Issue 2-3-1: Test configuration for RI reporting tests We support the recommended WF Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz We support a scheduling that is consistent with the outcome of Issue 2-1-1. CSI reporting parameters should be aligned. This is, if CQI delay of 10ms is decided, then Issue 2-3-2 should lean towards Option 2 |
| Nokia | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD Option 1. We prefer to keep the assumptions on feedback scheduling pattern as agreed in RAN4 #104-e. As companies have shown by simulation, the throughput gain is sufficient. Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests We support the recommended WF. Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests We support the recommended WF. Issue 2-3-1: Test configuration for RI reporting tests We support the recommended WF. Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz Option 1. We prefer to keep this aligned to CQI feedback scheduling pattern given there is sufficient throughput gain according to simulations. |
| MediaTek2 | Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD @Huawei: We think 10ms is not too loose. RAN4 agreed to define CQI and RI requirements for RedCap using the same TP ratio as legacy requirements, which are “TP ratio = 1.05 for CQI requirements” and “TP ratio = 1.05 for RI requirements”. In the legacy test cases for CQI and RI, the CQI/PMI/RI delay are “8ms for FDD” and “9.5ms for TDD”. Considering RedCap TDD CQI and RI test cases, RAN4 use 9.5ms for CQI/PMI/RI delay, which is the same as legacy test cases for CQI and RI in TDD mode. For the newly introduced RedCap FD-FDD/HD-FDD requirements, RAN4 consider defining CQI/PMI/RI delay to be 14ms or 10ms due to the DDDSU scheduling pattern. It is noted that both 10ms and 14ms are larger than legacy 8ms/9.5ms. Also, compared to 14ms, we think 10ms is more close to 8ms/9.5ms. Therefore, if we use the same TP ratio as legacy requirements, it is reasonable to use 10ms delay which is closed to 8ms/9.5ms. |

### CRs/TPs comments collection

Companies can put comment provide comments for CRs by adding New comments and suggested changes with Markup in revised CRs for being easily understood. Draft CR files are available in inbox:

- [104-bis-e][323] NR_RedCap_Demod/CRs_CSI/2-x_R4-221yyyy_v00.docx

| CR/TP number | Comments collection |
| --- | --- |
| R4-2216225 (Nokia, Nokia Shanghai Bell) | 2-1: Draft CR 38.101-4 Finalization of channel quality reporting requirements under static condition for RedCap |
|  | MediaTek: For TDD cases, Table 6.2.1.2.1.1-1 and Table 6.2.2.2.1.5-1, we think CQI/RI/PMI delay should be 9.5ms. The reason is that we use TDD UL-DL pattern, FR1.30-1, which is the same as TDD UL-DL pattern in legacy TDD test cases, Table 6.2.2.2.2.1-1. |
|  | Nokia: Thanks MediaTek, we will update accordingly. |
| R4-2216231 (Ericsson) | 2-2: draft CR: Correction of RedCap CSI reporting requirements |
|  | Company A |
|  | Company B |
| R4-2216429 (Qualcomm Incorporated) | 2-3: draftCR for RedCap UE Demodulation Requirements CQI for static channel |
|  | Ericsson: It depends on the conclusion of Issue 2-1-1. |
|  | Company B |
| R4-2216033 (Huawei, HiSilicon) | 2-4: Draft CR: Corrections of RedCap PMI requirements |
|  | Ericsson: Propose to merge 2-2 R4-2216231, e.g., FRC and notes, Huawei: Thanks for the comments. I will merge it. |
|  | Company B |
| R4-2216226 (Nokia, Nokia Shanghai Bell) | 2-5: Draft CR 38.101-4 Finalization of Rank Indicator reporting requirements for RedCap |
|  | MediaTek: For TDD cases, Table 6.4.2.2.1-1, we think CQI/RI/PMI delay should be 9.5ms. The reason is that we use TDD UL-DL pattern, FR1.30-1, which is the same as TDD UL-DL pattern in legacy TDD test cases, Table 6.4.3.2-1. |
|  | Ericsson: For FDD case, CQI/RI/PMI delay depends on the conclusion of issue 2-3-2. |
|  | Nokia: Thanks MediaTek, we will update accordingly. |
| R4-2216706 (MediaTek inc.) | 2-6: Draft CR to TS38.101-4, Corrections to ReCap PMI requirements |
|  | Ericsson: It depends on the conclusion of Issue 2-2-1. We propose this draft CR focuses on channel correlation matrix in B.2.3.1.2 because Section 6.3.1 is overlapped with 2-4 R4-2216033, |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Issue 2-1-1: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD | Conclusion from GTW session 2022-10-13: - Proposals - Option 1: - CSI-RS periodicity and offset: 10/1 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: 14ms - Option 2: - CSI-RS periodicity and offset: 10/5 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: 10ms - Agreement: Option 2 agreed Recommendation for 2nd round: No discussion |
| Issue 2-1-2: Static channel matrix used for 1Rx UE and SNR test points for CQI reporting tests | Conclusion from GTW session 2022-10-13: - Proposals - Option 1: - Set the static channel matrix in the frequency domain as H1RX=[1j]. - Set the same SNR test point for both 1Rx and 2Rx UE, that is, SNR=8/9dB and SNR=14/15dB. - Option 2: - Set the static channel matrix in the frequency domain as H1RX=11 - Set SNR test point X=3dB lower than 2Rx test case, that is, SNR=5/6dB and SNR=11/12dB. - Agreement: Option 2 agreed. Recommendation for 2nd round: No discussion |
| Issue 2-2-1: MIMO correlation matrix for 1Rx UE PMI reporting tests | Agreements: Define the MIMO correlation matrix for 4x1 ULA high as follows: Recommendation for 2nd round: No discussion |
| Issue 2-3-1: Test configuration for RI reporting tests | Agreements: - Use following configuration for RI reporting test - FR1 FDD - CQI table 1 - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.1-1 - FR1 TDD - CQI table 1 - CBW/SCS: 20MHz/30kHz - Other parameters: Reuse the parameters in Test 2 in Table 6.4.2.2-1 - FR2 - Reuse the requirements Recommendation for 2nd round: No discussion |
| Issue 2-3-2: CQI/RI/PMI reporting delay for RI reporting tests for FDD 15kHz | Agreements: Same conclusion as CQI reporting test in Issue 2-1-1. Recommendation for 2nd round: No discussion |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2216225 (Nokia, Nokia Shanghai Bell) | To be revised Capture the comments in the 1st round. For FDD CQI reporting requirements, change the parameters as follows according to the agreements in the 1st round. CSI-RS periodicity and offset; slot; 10/5. NZP CSI-RS-timeConfig periodicity and offset; slot; 10/5. CSI-IM timeConfig periodicity and offset; slot; 10/5. CQI/RI/PMI delay; ms; 10. |
| R4-2216231 (Ericsson) | Merge to R4-2216033. |
| R4-2216429 (Qualcomm Incorporated) | Not pursued. Option 2 is agreed in Issue 2-1-2. |
| R4-2216033 (Huawei, HiSilicon) | To be revised Merge R4-2216231, e.g., FRC table and notes |
| R4-2216226 (Nokia, Nokia Shanghai Bell) | To be revised Capture the comments in the 1st round. For FDD RI reporting requirements, change the parameters as follows according to the agreements in the 1st round. CSI-RS periodicity and offset; slot; 10/5. NZP CSI-RS-timeConfig periodicity and offset; slot; 10/5. CSI-IM timeConfig periodicity and offset; slot; 10/5. CQI/RI/PMI delay; ms; 10. |
| R4-2216706 (MediaTek inc.) | To be revised Remove 6.3.1. |

## Discussion on 2nd round (if applicable)

# Recommendations for Tdocs

## 1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on RedCap UE demodulation and CQI reporting requirements | Ericsson | Capture all the agreements. |
|  | Big Draft CR to 38.101-4: Correction of RedCap UE demodulation and CSI reporting requirements | Ericsson | For email endorsement. |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-2215628 |  | Draft CR PDSCH demodulation requirements for RedCap | Apple | Revised |  |
| R4-2216028 |  | Draft CR: Corrections of RedCap SDR requirements | Huawei, HiSilicon | Revised |  |
| R4-2216230 |  | draft CR: Correction of RedCap UE demodulation requirements | Ericsson | Revised |  |
| R4-2216225 |  | Draft CR 38.101-4 Finalization of channel quality reporting requirements under static condition for RedCap | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2216231 |  | draft CR: Correction of RedCap CSI reporting requirements | Ericsson | Merged | Merged to R4-2216033 |
| R4-2216429 |  | draftCR for RedCap UE Demodulation Requirements CQI for static channel | Qualcomm Incorporated | Not pursued. |  |
| R4-2216033 |  | Draft CR: Corrections of RedCap PMI requirements | Huawei, HiSilicon | Revised |  |
| R4-2216226 |  | Draft CR 38.101-4 Finalization of Rank Indicator reporting requirements for RedCap | Nokia, Nokia Shanghai Bell | Revised |  |
| R4-2216706 |  | Draft CR to TS38.101-4, Corrections to ReCap PMI requirements | MediaTek inc. | Revised |  |
| R4-2216027 |  | Simulation results for RedCap PDSCH | Huawei, HiSilicon | Revised | The existing file is empty, there are some problems for uploading |
| R4-2216029 |  | Simulation results for RedCap CQI requirements | Huawei, HiSilicon | Revised | The existing file is empty, there are some problems for uploading |
| R4-2216030 |  | Discussions on remain issues for RedCap CQI requirements | Huawei, HiSilicon | Revised | The existing file is empty, there are some problems for uploading |
| R4-2216031 |  | Simulation results for RedCap RI requirements | Huawei, HiSilicon | Revised | The existing file is empty, there are some problems for uploading |
| R4-2216032 |  | Discussions on remain issues for RedCap RI requirements | Huawei, HiSilicon | Revised | The existing file is empty, there are some problems for uploading |
| R4-2216234 |  | Summary of simulation results for RedCap | Ericsson | Noted |  |
| R4-2215625 |  | Discussion on PDSCH/SDR requirements in RedCap | Apple | Noted |  |
| R4-2216175 |  | Simulation Results for Redcap UE Demodulation PDSCH | Qualcomm Incorporated | Noted |  |
| R4-2216221 |  | Simulation results for Redcap PDSCH | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2216232 |  | Open issues on UE demodulation requirements for RedCap | Ericsson | Noted |  |
| R4-2216176 |  | Simulation Results for Redcap UE Demodulation PDCCH/PBCH | Qualcomm Incorporated | Noted |  |
| R4-2216222 |  | Simulation results for Redcap PDCCH | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2216223 |  | Simulation results for Redcap PBCH | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2215626 |  | Discussion on CQI requirements in RedCap | Apple | Noted |  |
| R4-2216177 |  | RedCap UE Demodulation Requirements CQI | Qualcomm Incorporated | Noted |  |
| R4-2216224 |  | Simulation results for RedCap CQI reporting | Nokia, Nokia Shanghai Bell | Noted |  |
| R4-2216233 |  | Open issues on CSI reporting requirements for RedCap | Ericsson | Noted |  |
| R4-2216703 |  | Discussion on CQI requirements for RedCap | MediaTek inc. | Noted |  |
| R4-2215627 |  | Discussion on PMI/RI requirements in RedCap | Apple | Withdrawn |  |
| R4-2216704 |  | Discussion on PMIRI requirements for RedCap | MediaTek inc. | Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

## 2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents