3GPP TSG-RAN WG4 Meeting # 104-bis-e												R4-2216930

Electronic Meeting, 10–19 October 2022

Agenda item:			6.13.6

Source:	CMCC

Title:	Email discussion summary for [104-bis-e][219] NR_ATG_RRM

Document for:	Information

Introduction

This email discussion focuses on RRM core requirements for Rel-18 NR ATG, including agenda 6.13.5. The agreed way forward in previous meeting is R4-2214737.

The targets of email discussion for 1st round and 2nd round are:

•	1st round:

- Identify and discuss the requirements which different from ground-based systems

•	2nd round:

- Strive to conclude the RRM core requirements scope for ATG. Approve the WF.

It is appreciated that the delegates for this topic put their contact information in the table below.

Contact information

| Company | Name | Email address |
| --- | --- | --- |
| CMCC | Shiyuan Wang | wangshiyuan@chinamobile.com |
| LGE | Jin Woong Park | jinwoong.park@lge.com |
| Ericsson | Santhan Thangarasa | Santhan.thangarasa@ericsson.com |
| Qualcomm | Prashant Sharma | prasshar@qti.qualcomm.com |

Note:

- Please add your contact information in above table once you make comments on this email thread.

- If multiple delegates from the same company make comments on single email thread, please add you name as suffix after company name when make comments i.e. Company A (XX, XX)

Topic #1: General RAN4 RRM ATG related aspects

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215396 | CATT | Observation 1: The radio resource efficiency will be low, it should be decided by operator whether the TDD band is used for ATG operation. Observation 2: The UE assistance information, at least altitude, location, velocity, are needed if ATG UE is required to do pre-compensation of transmit timing and frequency. |
| R4-2215505 | CMCC | Proposal 1: First study ATG RRM requirements with ISD assumption of 100-200km. If RF session come to other conclusions beyond the assumption range, then come back to have more study on ISD related requirements. Proposal 2: Consider FDD 15kHz SCS and TDD 30kHz SCS as the basic assumption for ATG RRM study. Proposal 3: 20ms period TDD frame structure could be configured for ATG TDD 30kHz SCS, which the TDD pattern is 30D4S6U. Proposal 4: ATG WI should avoid to introduce new UAI reporting method. Observation 1: The ATG UAI reporting including altitude, location, propagation delay difference, flight path, speed, velocity reporting could be supported by existing approach in current NR spec. Observation 2: The ATG UAI reporting including flight path reporting couldn’t be supported by existing approach in current NR spec. Proposal 5: Don’t introduce flight path as ATG UAI. Proposal 6: No need to introduce altitude, location, speed, velocity as ATG UAI. Proposal 7: There is no necessity of introducing ATG UAI of propagation delay difference with the ISD assumption of 100-200km. The conclusion can be revisited after RF session draw the final conclusion of ISD. |
| R4-2215937 | LGE | Proposal 1: The propagation delay information between gNB and ATG UE should be known by gNB since TDD operation can be affected by the propagation delay which can be up to one slot duration depending on the ATG UE position. Proposal 2: For further discussion, RAN4 can categorize UE assistance information into RRM requirement: measurement, timing and mobility. For example, Measurement triggering/relaxation condition: altitude, distance between UE and BS, speed, velocity, etc. Measurement window configuration: propagation delay difference, propagation delay estimation, etc. Timing: propagation delay difference, speed, velocity, propagation delay estimation, etc. Handover: location, flight path, etc. XXX: yy, zz, etc. Proposal 3: RAN4 needs to study NW assistance information such as ATG BS location. |
| R4-2216276 | HW | Observation 1: The maximum range in ATG to be considered can be evaluated in Demod session with respective to the PRACH demodulation performance. |
| R4-2216769 | Ericsson | - RAN4 to include the ATG bands (agreed by RF group) into frequency band grouping section 3.5 in TS 38.133. - Assistance information related to unpredictable changes in flight operation mode can be two ways: 1) UE detecting and informing the NW, 2) NW informing the UE - Assistance information comprises at least changes in the UE altitude, flight path and speed, remaining service time for serving cell and side conditions can be FFS. Observation 3: The RAN4 TS 38.133 RRM specification is agnostic when it comes to FDD or TDD. RAN1 incompatibilities are restricted to LTE NTN IoT. RAN2 and RAN3 specifications seems to be compatible with NTN TDD. Observation 4: An ATG system needs a full slot or even several slots of GP, however the large ISD and beamforming might mitigate any issues with regards to GP for TDD. Observation 5: The guard period for BS to BS interference, for the DL to UL switch is mainly dimensioned by the Tprop_BS2BS, if and when there is unacceptable interference between BS. Observation 6: The guard period for UE to UE interference, for the DL to UL switch is mainly dimensioned by the 2*Tprop_cell edge, if and when there is unacceptable interference between nearby UE. |

Open issues summary

Sub-topic 1-1 general impactions due to ATG feature

Sub-topic description: In sub-topic 1-1-, we will discuss some fundamental assumptions and general impactions due to ATG feature

Open issues and candidate options before e-meeting:

Issue 1-1: ISD assumption

- Proposals

  - Option 1: First study ATG RRM requirements with ISD assumption of 100-200km. If RF session come to other conclusions beyond the assumption range, then come back to have more study on ISD related requirements. (CMCC)

  - Option 2: The maximum range in ATG to be considered can be evaluated in Demod session with respective to the PRACH demodulation performance. (HW)

- Recommended WF

  - Discuss the Options above

Issue 1-2: Duplex mode and SCS

- Proposals

  - Option 1: Consider FDD 15kHz SCS and TDD 30kHz SCS as the basic assumption for ATG RRM study. (CMCC)

- Recommended WF

  - Take Option 1 as the basic assumption for ATG RRM study.

Issue 1-3: TDD pattern

- Proposals

  - Option 1: 20ms period TDD frame structure could be configured for ATG TDD 30kHz SCS, which the TDD pattern is 30D4S6U. (CMCC)

  - Option 2: An ATG system needs a full slot or even several slots of GP, however the large ISD and beamforming might mitigate any issues with regards to GP for TDD.  (Ericsson)

    - The guard period for BS to BS interference, for the DL to UL switch is mainly dimensioned by the Tprop_BS2BS, if and when there is unacceptable interference between BS.

    - The guard period for UE to UE interference, for the DL to UL switch is mainly dimensioned by the 2*Tprop_cell edge, if and when there is unacceptable interference between nearby UE.

- Recommended WF

  - Discuss the Options above

Issue 1-4: Assistance information

Issue 1-4-1: UE Assistance information (UE detecting and informing the NW)

- Proposals

  - For UAI of altitude, location, velocity:

    - Option 1-1: The UE assistance information, at least altitude, location, velocity, are needed if ATG UE is required to do pre-compensation of transmit timing and frequency. (CATT)

    - Option 1-2: No need to introduce altitude, location, speed, velocity as ATG UAI (CMCC)

    - Option 1-3: Assistance information comprises at least changes in the UE altitude and speed, side conditions can be FFS. (Ericsson)

  - For UAI of flight path:

    - Option 2-1: Don’t introduce flight path as ATG UAI. (CMCC)

      - The ATG UAI reporting including flight path reporting couldn’t be supported by existing approach in current NR spec.

    - Option 2-2: Assistance information comprises at least changes in the flight path, side conditions can be FFS. (Ericsson)

  - For UAI of propagation delay information:

    - Option 3-1: There is no necessity of introducing ATG UAI of propagation delay difference with the ISD assumption of 100-200km. The conclusion can be revisited after RF session draw the final conclusion of ISD. (CMCC)

    - Option 3-2: The propagation delay information between gNB and ATG UE should be known by gNB since TDD operation can be affected by the propagation delay which can be up to one slot duration depending on the ATG UE position. (LGE)

  - For UAI of remaining service time for serving cell

    - Option 4-1: Assistance information comprises remaining service time for serving cell and side conditions can be FFS.

  - Option 5: For further discussion, RAN4 can categorize UE assistance information into RRM requirement: measurement, timing and mobility. For example, (LGE)

    - Measurement triggering/relaxation condition: altitude, distance between UE and BS, speed, velocity, etc.

    - Measurement window configuration: propagation delay difference, propagation delay estimation, etc.

    - Timing: propagation delay difference, speed, velocity, propagation delay estimation, etc.

    - Handover: location, flight path, etc.

  - Option 6: ATG WI should avoid to introduce new UAI reporting method. (CMCC)

- Recommended WF

  - Discuss the Options above

Issue 1-4-2: Network Assistance information (NW informing the UE)

- Proposals

  - Option 1: RAN4 needs to study NW assistance information such as ATG BS location. (LGE)

- Recommended WF

  - Discuss the Options above

Issue 1-5: Band table

- Proposals

  - Option 1: Proposal 1	RAN4 to include the ATG bands (agreed by RF group) into frequency band grouping section 3.5 in TS 38.133. (Ericsson)

- Recommended WF

  - Wait for the conclusion from RF.

Issue 1-6: Large cell impact on general RRM requirements

- Proposals

  - Option 1 (Ericsson): RAN4 to discuss the technical impact to RRM requirement due to the large serving cell radius in ATG system, such as IDLE mode mobility, CONNECTED mode mobility, and measurement procedure etc.

- Recommended WF

  - Discuss the option.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| CMCC | Issue 1-1: ISD assumption In our view, without ISD assumption, most of our discussion will be repeated without any progress. For Option 2, Demod session will be started in March next year, it will be too late if we wait the final conclusion in Demod session. To combine with Option 1 and Option 2, we further propose that: First study ATG RRM requirements with ISD assumption of 100-200km. If RF/Demod session come to other conclusions beyond the assumption range, then come back to have more study on ISD related requirements. Issue 1-2: Duplex mode and SCS We support the recommended WF. This assumption will help the discussion about TDD impaction, TA and so on. Issue 1-3: TDD pattern We think the Option 1 could address the observations and concerns in Option 2. Therefore, we think Option1 could be the baseline or initial assumption when discussing TDD impacted issues. Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) In high level, we support Option 6. For UAI of altitude, location, velocity: Option 1-2, even in NR NTN, such information is not allowed to report to BS, considering the information safety and so on. Besides, in ATG network, UE is in high-speed, these information keeps time-varying, how to report is unclear. For UAI of flight path: Option 2-1, based on our knowledge, The flight path reporting couldn’t be supported by existing approach in current NR spec. For UAI of propagation delay information: We support Option 3-1 For UAI of remaining service time for serving cell: Since UE’s speed is time varying, we are not sure if UE is able to derive a precise remaining service time. Therefore, we prefer not to introduce such UAI. Issue 1-4-2: Network Assistance information (NW informing the UE) We support Option 1. As proposed by Ericsson (Observation 2 in R4-2216769), we agree that the PositionVelocity-r17 in TS 38.331 can be used to signal path of BS relative to ATG UE Issue 1-5: Band table Support the recommended WF. Issue 1-6: Large cell impact on general RRM requirements Option 1 is ok, we think RAN4 has already identify the large cell impaction in Topic 2-5. |
| LGE | Issue 1-1: ISD assumption Support option 1 Issue 1-2: Duplex mode and SCS Support option 1 Issue 1-3: TDD pattern Does the option 1 propose that only one TDD pattern can be configured in ATG NW? Does the expression “BS to BS interference” means that the “ATG BS to ATG BS interference”? If yes, the interference level should be analyzed first to define TDD pattern and GP duration. And interference level depends on antenna tilt angle, ISD, and so on discussed in RF session. So, we think RAN4 can wait RF session’s conclusion. Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) For UAI of altitude, location and velocity, support option 1-1. For flight path, further discussion is needed. For propagation delay information, support option 3-2. For ATG UE, DL slot can be overlapped with UL slots due to the propagation delay. But, if gNBs have information of delay, the slots shall not be overlapped by gNB scheduling restriction. For remaining service time, clarification is needed for “remaining service time”. For option 5, in RAN4 various types of UAI are discussed. So, clarification and discussion, categorization is needed. For option 6, we think new UAI reporting method can be discussed and considered to resolve RRM issues in ATG network deployed with large cell coverage. So, it does not need avoid to introduce new UAI reporting method. Issue 1-4-2: Network Assistance information (NW informing the UE) We are open to discuss this issue since we think some information in issue 1-4-1 such as delay difference and location based measurement/HO can be obtained using assistance information from NW such as ATG BS location. Issue 1-5: Band table Okay with recommended WF Issue 1-6: Large cell impact on general RRM requirements Support option 1. |
| Huawei | Issue 1-1: ISD assumption Support option 1 and option 2. We think there is no significant impact of ISD on RRM requirements. Issue 1-2: Duplex mode and SCS We are wandering what is the impact on core requirements? This may be discussed as test configurations in perf stage. Issue 1-3: TDD pattern We think the TTD pattern does not have impacts on core requirements which is more appropriate to be discussed in perf stage. For the GP configuration, we agree with the observation that larger GP will be configured, but it is up to NW configuration, we also don’t think there is impact on core requirements. Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) We slightly prefer option 5 currently. It depends on whether per-compensate is needed for ATG UE. Issue 1-4-2: Network Assistance information (NW informing the UE) Same comments as 1-4-1, it is suggested to be discussed in particular requirements/behavior and decide what kind of information is needed or not. Issue 1-5: Band table Support the recommended WF. Issue 1-6: Large cell impact on general RRM requirements It expected to clarify what the impact is on RRM requirements of larger cell range. |
| Ericsson | Issue 1-1: ISD assumption The ISD affects the RRM requirements, and should therefore be discussed under RRM session. Therefore we disagree to option 2. Regarding option 1, our understanding is that RF session considers ISD ranges from 14 km to 200 km. The RRM requirements should be robust enough to work for the full range of ISD. Option 1 should be revised accordingly. Issue 1-2: Duplex mode and SCS FDD 15 kHz is fine. But why only TDD 30 kHz and not TDD 15 kHz? This needs to be discussed. Issue 1-3: TDD pattern Option 2: The TDD GP will only be needed if there is overlapping interference between ATG BS and between AG UE. This depends on how selective the beams are and how the sites are distributed. Option1: If a large GP is needed for TDD, then a longer switch period, like 20 ms proposed, will reduce the overhead, Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) Many of the options are related and can be merged. Option 1-1, 1-3, 2-2 and 4-1 are related and both options can be merged as follows: - the assistance information comprises changes in at least UE altitude, flight path, speed, location, remaining service time for the serving cell. Side conditions are FFS. On the issue related to propagation delay information, this issue is more suited to be discussed under as part of timing requirements. We agree that RAN4 needs to discuss the RRM requirements on how the assistance information are used and we are option to discuss the listed items in option 5, but RAN4 needs to first agree on the assistance information. Therefore, after the RRM requirements for those can be discussed. Issue 1-4-2: Network Assistance information (NW informing the UE) We disagree to option 1. Our view is that the assistance information discussed under issue 1-4-1 are sufficient for the performing the necessary RRM tasks. Issue 1-5: Band table Issue 1-6: Large cell impact on general RRM requirements We think one of the main differences between ATG to HST is ATG cell coverage is large. ATG UE will stay in the serving cell for a long time. In such situation, some normal mobility procedures are unnecessary. For example, in Idle mode, we can reduce the number of inter-frequencies for measurement in cell reselection when UE is in the cell center. |
| Qualcomm | Issue 1-1: ISD assumption We prefer Option 1. We can start with ISD of 100-200 km and if the assumption is not valid based on RF session conclusions, we can revisit which RRM requirements would be impacted. Issue 1-2: Duplex mode and SCS We don’t see how core requirements would be impacted by this. Issue 1-3: TDD pattern We don’t see how core requirements would be impacted by this. Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) For UE assistance information we first need to identify what assistance information is needed and whether such information can be provided. In that sense Option 5 seems like a good starting point. Issue 1-4-2: Network Assistance information (NW informing the UE) Why does the UE needs to know ATG UE’s location? Issue 1-5: Band table Recommended WF is fine. Issue 1-6: Large cell impact on general RRM requirements Fine with the proposal |
| ZTE | Issue 1-1: ISD assumption We support Option 1. Regarding to Option 2, the current preamble format design can not support some extreme large ISD case in ATG. So the assumed maximum range in ATG would impact the PRACH design or some additional pre-compensation from UE side is needed. However as CMCC referred, Demod session will be started in March next year, it will be too late. Issue 1-2: Duplex mode and SCS We are also wondering why different SCS assumption for FDD and TDD. Issue 1-3: TDD pattern We agree that large GP is needed for ATG frame structure, to control the overhead of GP, 20ms period is general fine to us. Referring to the exact frame structure, it is up to NW configuration. Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) To our understanding, when we talking about whether UE need to report the UAI or not, a key point is whether UE performs pre-compensation. If UE would perform time/frequency pre-compensation, the UAI such as altitude, location and velocity are almost useless for NW. Referring to the timing related UAI, such as propagation delay estimation, such info can help NW to identify the overlapping between DL slot and UL slot due to large propagation delay. However we concern under the situation of the high speed moving of UE, further more the flight path is not always fixed, whether the report can be received by NW in time enough. If not, it would be meaningless. Issue 1-4-2: Network Assistance information (NW informing the UE) We agree with Option 1. With the help of UE location and BS location info, UE can perform estimation and pre-compensation. Issue 1-5: Band table Fine with the recommended WF. Issue 1-6: Large cell impact on general RRM requirements Fine with Option 1. |
| CATT | Issue 1-1: ISD assumption By our understanding, WI expect extreme large cell coverage range (e.g., up to 300 kilometres) and flight speed (e.g., up to 1200km/h). Generally ISD should be up to 600km or 450km. 100~200km ISD may not be fulfil objective of the WI. If ATG doesn’t using NTN mechanism (timing pre-compensation), the PRACH can only support about up to 100km and ISD can be assumed as 100~200km. It is basically issue and should be decided firstly. It will impact the requirements of transmit timing, whether UE to do transmit timing advance pre-compensation, and whether Koffset is needed. Issue 1-2: Duplex mode and SCS Recommended WF is acceptable. Issue 1-3: TDD pattern We think NR physical specification is not defined TDD pattern. How to configure TDD pattern is network implementation. Issue 1-4: Assistance information Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) It can be same as TN. Additional information is not necessary. Issue 1-4-2: Network Assistance information (NW informing the UE) If the maximum coverage is less than 100km, additional information may not be necessary. If ATG supports maximum coverage is larger than 100km (e.g. up to 300km), the position of gNB should be informed to UE for transmit timing/frequency pre-compensation. Issue 1-5: Band table Agree recommended WF. Issue 1-6: Large cell impact on general RRM requirements If coverage is less than 100km, general RRM requirements may not be impacted. If coverage is larger than 100km, at least initial transmit timing error, Te should be updated. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic #1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Sub-topic 1-1 general impactions due to ATG feature

|  | Status summary |
| --- | --- |
| Issue 1-1 | Issue 1-1: ISD assumption Status summary: 4 companies support Option 1, 2 companies mentioned that Option 2 should also be considered. 1 company think the lower range should be 14km-200km to align with RF session. 1 company think that 300km cell range should be considered. Moderator give the tentative agreement as follows. To be mentioned, the current tentative agreement doesn’t preclude the ISD beyond 200km, just to focus 14-200km firstly, which is also align with current status in RF session. Tentative agreements: Start the discussion with ISD assumption of 14-200 km. If the assumption is not valid based on the conclusion from RF/Demod session, we can revisit the RRM requirements which would be impacted. Recommendations for 2nd round: Check whether the tentative agreement is agreeable. This issue will be discussed in Friday GTW session. |
| Issue 1-2 | Issue 1-2: Duplex mode and SCS 3 companies support Option 1, CMCC mentioned that this assumption will help the discussion about TDD impaction, TA and so on. 2 companies think this issue doesn’t have impacts on core requirements. Should be discussed in perf stage. 1 company would like to discuss why TDD 15kHz is not involved, 1 company don’t understand why different SCS assumption for FDD and TDD. Considering diverse interpretation about this issue, moderator suggest continue the discussion based on following candidate options in the 2nd round. Candidate options: - Option 1: Consider FDD 15kHz SCS and TDD 30kHz SCS as the basic assumption for ATG RRM study. (CMCC, CATT, LGE) - Option 2: Consider FDD 15kHz SCS and TDD 15kHz and 30kHz SCS as the basic assumption for ATG RRM study. (Ericsson) - Option 3: Discuss the Duplex mode and SCS in perf stage (HW, QC) - Other options are not precluded. Status summary: Recommendations for 2nd round: Continue the discussion based on above candidate options. |
| Issue 1-3 | Issue 1-3: TDD pattern 1 company would like to take Option 1 as the baseline assumption when discussing TDD impacted issues. 1 company need further clarification on Option 1 and Option 2. 2 company think this issue doesn’t have impacts on core requirements. 3 companies think the TDD pattern is up to NW configuration Candidate options: - Option 1: 20ms period TDD frame structure could be configured for ATG TDD 30kHz SCS, which the TDD pattern is 30D4S6U. (CMCC) - Option 2: An ATG system needs a full slot or even several slots of GP, however the large ISD and beamforming might mitigate any issues with regards to GP for TDD. (Ericsson) - The guard period for BS to BS interference, for the DL to UL switch is mainly dimensioned by the Tprop_BS2BS, if and when there is unacceptable interference between BS. - The guard period for UE to UE interference, for the DL to UL switch is mainly dimensioned by the 2*Tprop_cell edge, if and when there is unacceptable interference between nearby UE. - Option 3: No impaction on core requirements is observed, can be further discussed in Perf stage (HW, QC) - Option 4: Up to NW configuration. (HW, ZTE, CATT) - Other options are not precluded. Considering diverse interpretation about this issue, moderator suggest continue the discussion in 2nd round. Recommendations for 2nd round: Continue the discussion based on the above candidate options. |
| Issue 1-4-1 | Issue 1-4-1: UE Assistance information (UE detecting and informing the NW) 1 company support Option 6, while 1 Company opposes Option 6 5 companies support Option 5, to discuss the UAI together with specific RRM requirement. 1 company support to introduce the UAI of altitude, location, velocity, 1 company need FFS Company support to introduce the UAI of flight path, 1 company think further discussion is needed, 1 company need FFS 1 Company support to introduce the UAI of propagation delay information, Based on feedback from companies, moderator don’t see the possibility of conclude this issue in this meeting, and have two suggestions. First, discuss whether new UAI reporting method could be introduced separately. Second, discuss the necessity of UAI together with specific RRM requirement. Issue 1-4-1-1: whether new UAI reporting method could be introduced Candidate options: - Option 1: ATG WI should avoid to introduce new UAI reporting method. (CMCC) - Option 2: New UAI reporting method can be discussed and considered to resolve RRM issues in ATG network deployed with large cell coverage. (LGE) Issue 1-4-1-2: Specific UAI Tentative agreements: - Discuss the necessity and feasibility of UAI together with specific RRM requirement Recommendations for 2nd round: Continue the discussion of Issue 1-4-1-1 based on the above candidate options. Continue the discussion of specific UAI (Issue 1-4-1-2) in the discussion of related RRM requirements. |
| Issue 1-4-2 | Issue 1-4-2: Network Assistance information (NW informing the UE) 3 companies support Option 1. 2 companies disagree with Option 1 1 company suggest to be discussed in particular requirements/behavior. Similar as Issue 1-4-1-2, moderator suggest to discuss the NAI together with specific RRM requirement. Tentative agreements: - Discuss the necessity and feasibility of UAI together with specific RRM requirement Recommendations for 2nd round: Continue the discussion of specific NAI in the discussion of related RRM requirements. |
| Issue 1-5 | Issue 1-5: Band table All companies support the recommended WF Tentative agreements: - Wait for the conclusion from RF Recommendations for 2nd round: No more discussion. |
| Issue 1-6 | Issue 1-6: Large cell impact on general RRM requirements 5 companies support Option 1, 1 company need clarification of the specific impaction of larger cell range, Ericsson clarify that some normal mobility procedures are unnecessary in this larger cell range scenario. Based on moderator’s observation, Option 1 is very general, and it has already been discussed in Issue 2-1-1-2, 2-2-3x, 2-2-5x, other open issues are not precluded. Therefore, moderator suggest to continue the discussion in particular sub-topics and issues. Tentative agreements: - RAN4 to discuss the technical impact to RRM requirement due to the large serving cell radius in ATG system, such as IDLE mode mobility, CONNECTED mode mobility, and measurement procedure etc. Recommendations for 2nd round: No more discussion. |

Discussion on 2nd round (if applicable)

Topic #2: Mobility

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215396 | CATT | Proposal 1: Not to consider conditional handover for ATG UE at least in Rel-18. |
| R4-2215505 | CMCC | Observation 3: For ATG deployment method 1, when ISD is smaller than 117.76km, both legacy R15 requirements and HST requirements can’t support UE to complete cell-reselection. Proposal 8: If ATG deployment method 1 is adopted, whether and what DRX cycle length to configure is up to NW. As long as the ISD is smaller than 117.76km, UE is not required to fulfill the cell re-selection requirements for 2.56s DRX cycle length. Observation 4: If the typical ISD assumption is larger than 78.5km, we think the legacy R15 cell-reselection requirement can be reused for ATG. While if the typical ISD assumption is smaller than 64km, then the HST cell-reselection requirements should be reused for ATG. Proposal 9: With the basic ISD assumption of 100-200km, the legacy R15 cell-reselection requirement could be reused for ATG. The conclusion can be revisited after receiving final conclusion about ISD assumption and deployment approach from RF session. Proposal 10: Reuse the legacy cell-reselection rule and procedure. Proposal 11: The conclusion on MDT measurement requirements can be delayed until RAN4 decide how to define ATG cell-reselection measurement requirements. Proposal 12: For SDT RRM requirements, the legacy requirements can be reused, no need to define ATG specific requirements. Proposal 13: For NR ATG handover, reuse legacy NR HO requirements. Proposal 14: Introduce legacy CHO for ATG, the legacy R16 CHO delay requirements can be reused. Proposal 15: Introduce location-based CHO for ATG, the logic and signalling from R17 NTN could be reused, while the location-based CHO delay requirements should be revisited in ATG scenario. Proposal 16: For NR ATG RRC Re-establishment and RRC Connection Release with Redirection, reuse the legacy NR delay requirements. Proposal 17: For both 4-step and 2-step RA, reuse the legacy requirements for ATG. |
| R4-2215635 | Apple | Proposal 1: SDT is not considered unless the use case is clear for ATG. Proposal 2: Consider distance-based handover for ATG UE. |
| R4-2215937 | LGE | Proposal 4: Introduce distance based triggering as an additional condition for intra- and inter-frequency cell measurement considering ATG UE speed. Proposal 5: RAN4 is not going to define ATG specific requirements (Note by moderator: SDT) |
| R4-2216276 | HW | Observation 2: Considering larger cell range in ATG, the mobility requirements are less challenging than HST. Proposal 1: Existing requirement of intra-frequency and inter-frequency measurement requirements for cell re-selection can apply to ATG. Proposal 2: Existing intra-frequency HO and inter-frequency HO requirements can apply to ATG. Observation 3: Whether to consider CHO requirements for ATG depends on whether assistant information in NTN will be used in ATG. Proposal 3: Existing Re-establishment and RRC release with re-direction requirements can apply and no need to have ATG specific requirements. Proposal 4: Existing Random access requirements can apply and no need to have ATG specific requirements. |
| R4-2216481 | ZTE | Proposal 4: Reusing legacy R15 requirements of intra-frequency and inter-frequency measurements in cell re-selection is fine. Proposal 5: Not need to consider inter-RAT measurement for cell re-selection due to no commerical demand. Proposal 6: regular intra-frequency measurement and inter-frequency measurement similar as legacy is needed. (Note by moderator: For handover) Proposal 7: Considering the requirements for known case handover, re-using legacy legacy requirement for ATG UE is fine. Proposal 8: Not need to consider handover to unknown cell for ATG scenario. Proposal 9: Re-using the legacy RRC re-establishment requirements for ATG UE. |
| R4-2216769 | Ericsson | - RAN4 to discuss the technical impact to RRM requirement due to the large serving cell radius in ATG system, such as IDLE mode mobility, CONNECTED mode mobility, and measurement procedure etc. Discussions on measurement capability shall be postponed until the scenario is agreed in RF group. The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above a certain threshold which is FFS. For cell reselection, the A2G UE should resume the neighbor cell measurement in normal manner without any relaxation based on assistance information related to unpredictable changes in flight operation mode. - SDT is supported for A2G in Rel-18, and SDT requirements for ATG is FSS. - CHO is supported for A2G in Rel-18, and CHO requirements for ATG is FSS. - The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify_intra_NR and Tidentify_inter_NR. (Noted by moderator: for RRC_re-establishment) - Current definitions of known/unknown cells and side conditions in the RRC re-establishment requirements can be reused for ATG. - RAN4 to study the larger cell impact to RRC re-establishment, such as the target cell list and total delay requirement. - RAN4 to define RA requirements only for 4-step RA for ATG scenario. - Existing 4-step RA requirements are reused for ATG. - The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify-NR. - Current definitions/requirements on TRACH are reused for ATG. - RAN4 to study the larger cell impact to RRC connection release with redirection, such as the target candidate cell and total delay requirement. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 2-1 Mobility in RRC_IDLE/INACTIVE

Sub-topic description: Sub-topic 2-1 focus on the mobility requirements which are performed in RRC_IDLE and RRC_INACTIVE mode.

Open issues and candidate options before e-meeting:

Issue 2-1-1: Cell re-selection requirements

Issue 2-1-1-1: Cell re-selection measurement capability

- Proposals

  - Option 1: Discussions on measurement capability shall be postponed until the scenario is agreed in RF group. (Ericsson)

- Recommended WF

  - Keep the agreement in last meeting, which is:

  - Use current UE capability for NR intra-frequency measurement and NR inter-frequency measurement as the starting point.

    - Further study the capability after the scenario is clearer in the RF group.

Issue 2-1-1-2: Cell re-selection mechanism

- Proposals

  - Option 1: Reuse the legacy cell-reselection rule and procedure. (CMCC)

  - Option 2: Introduce distance based triggering as an additional condition for intra- and inter-frequency cell measurement considering ATG UE speed (LGE)

  - Option 3: (Ericsson)

    - The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above a certain threshold which is FFS.

    - For cell reselection, the A2G UE should resume the neighbor cell measurement in normal manner without any relaxation based on assistance information related to unpredictable changes in flight operation mode.

- Recommended WF

  - Discuss the Options above.

Issue 2-1-1-3: Cell re-selection measurement requirements

- Proposals

  - Option 1: With the basic ISD assumption of 100-200km, the legacy R15 cell-reselection requirement could be reused for ATG. The conclusion can be revisited after receiving final conclusion about ISD assumption and deployment approach from RF session. (CMCC)

    - Option 1-1:  If ATG deployment method 1 is adopted, whether and what DRX cycle length to configure is up to NW. As long as the ISD is smaller than 117.76km, UE is not required to fulfill the cell re-selection requirements for 2.56s DRX cycle length. (CMCC)

  - Option 2: Existing requirement of intra-frequency and inter-frequency measurement requirements for cell re-selection can apply to ATG. (HW, ZTE)

- Recommended WF

  - Reusing legacy R15 requirements of intra-frequency and inter-frequency measurement as the starting point, the conclusion can be revisited after receiving final conclusion about ISD assumption and deployment approach from RF session.

Issue 2-1-2: MDT

- Proposals

  - Option 1: The conclusion on MDT measurement requirements can be delayed until RAN4 decide how to define ATG cell-reselection measurement requirements. (CMCC)

- Recommended WF

  - Option 1

Issue 2-1-3: SDT

- Proposals

  - Option 1: For SDT RRM requirements, the legacy requirements can be reused, no need to define ATG specific requirements. (CMCC)

  - Option 2: RAN4 is not going to define ATG specific requirements. (LGE)

  - Option 3: SDT is not considered unless the use case is clear for ATG. (Apple)

  - Option 4: SDT is supported for A2G in Rel-18, and SDT requirements for ATG is FSS. (Ericsson)

- Recommended WF

  - Proponents are encouraged to clarify the use case and ATG specific impaction to SDT requirements

Sub-topic 2-2 Mobility in RRC_CONNECTED

Sub-topic description: Sub-topic 2-2 focus on the mobility requirements which are performed in RRC_CONNECTED

Open issues and candidate options before e-meeting:

Issue 2-2-1: Handover

Issue 2-2-1-1: NR Handover mechanism

- Proposals

  - Option 1: regular intra-frequency measurement and inter-frequency measurement similar as legacy is needed (ZTE)

- Recommended WF

  - Option 1

Issue 2-2-1-2: NR Handover requirement

- Proposals

  - Option 1: For NR ATG handover, reuse legacy NR intra-frequency HO and inter-frequency HO requirements. (CMCC, HW, ZTE (for known case))

  - Option 2: Not need to consider handover to unknown cell for ATG scenario. (ZTE)

- Recommended WF

  - Reuse legacy NR intra-frequency HO and inter-frequency HO requirements for known case, FFS the unknown case.

Issue 2-2-2: Conditional Handover

- Proposals

  - Option 1: Not to consider conditional handover for ATG UE at least in Rel-18. (CATT)

  - Option 2: Introduce legacy CHO for ATG (CMCC, Ericsson)

    - the legacy R16 CHO delay requirements can be reused. (CMCC)

    - CHO requirements for ATG are FSS (Ericsson)

  - Option 3: Introduce location-based CHO for ATG (CMCC, Apple)

    - Option 3-1: The logic and signalling from R17 NTN could be reused, while the location-based CHO delay requirements should be revisited in ATG scenario. (CMCC)

- Recommended WF

  - Discuss the Options above

Issue 2-2-3: SA: RRC Re-establishment

- Proposals

  - Option 1: Reuse the legacy NR delay requirements. (CMCC, HW, ZTE)

  - Option 2: Current definitions of known/unknown cells and side conditions in the RRC re-establishment requirements can be reused for ATG. (Ericsson)

  - Option 3: RAN4 to study the larger cell impact to RRC re-establishment, such as the target cell list and total delay requirement. (Ericsson)

  - Option 4: The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify_intra_NR and Tidentify_inter_NR. (Ericsson)

- Recommended WF

  - Reuse the current definitions of delay requirement, known/unknown cells and side conditions in the RRC re-establishment requirements, reuse the current definitions/requirements on TSI_NR, TPRACH TUL_grant, FFS the Tidentify_intra_NR and Tidentify_inter_NR.

Issue 2-2-3x: Large cell impact on RRC re-establishment

- Proposals

  - Option 1 (Ericsson): RAN4 to study the larger cell impact to RRC re-establishment, such as the target cell list and total delay requirement.

- Recommended WF

  - Discuss the option.

Issue 2-2-4: Random Access

- Proposals

  - Option 1: Existing Random access requirements can apply and no need to have ATG specific requirements. (CMCC, HW)

    - Option 1-1: RAN4 to define RA requirements only for 4-step RA for ATG scenario. Existing 4-step RA requirements are reused for ATG. (Ericsson)

- Recommended WF

  - Discuss the Option “Existing Random access requirements can apply and no need to have ATG specific requirements. The conclusion could be revisited after RF achieve final conclusion about ISD”

Issue 2-2-5: SA: RRC Connection Release with Redirection

- Proposals

  - Option 1: Reuse the legacy NR delay requirements. (CMCC, HW)

  - Option 2: Current definitions/requirements on TRACH are reused for ATG. (Ericsson)

  - Option 3: The outcome of cell detection performance under UE speed corresponding to ATG scenario is used to determine impact of Tidentify-NR. (Ericsson)

- Recommended WF

  - Reuse the current definitions of RRC Connection Release with Redirection delay requirement, definitions/requirements on TRACH, TSI-NR and TRRC_procedure_delay FFS the Tidentify-NR and total delay requirement.

Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection

- Proposals

  - Option 1 (Ericsson): RAN4 to study the larger cell impact to RRC connection release with redirection, such as the target candidate cell and total delay requirement.

- Recommended WF

  - Discuss the option.

Companies views’ collection for 1st round

Open issues

Sub topic 2-1 Mobility in RRC_IDLE/INACTIVE

| Company | Comments |
| --- | --- |
| CMCC | Issue 2-1-1-1: Cell re-selection measurement capability We support the Recommended WF. Issue 2-1-1-2: Cell re-selection measurement rule and procedure Option 1 can be the baseline. For Option 2, we are fine to consider the distance-based cell re-selection in ATG, FFS whether the NTN approach can be reused or not. For Option 3, as stated in our contribution, we think the high priority cell re-selection is also important in ATG, it should not be skipped. Issue 2-1-1-3: Cell re-selection measurement requirements We support the Recommended WF Issue 2-1-2: MDT We support the Recommended WF Issue 2-1-3: SDT We don’t have strong view about whether to introduce the SDT in ATG or not. However, if it is introduced, after double-check, we think whether the legacy SDT requirement can be reused for ATG may need to be considered together with UE based timing pre-compensation. If UE based timing pre-compensation is introduced, then the TA validation may not be needed. Details can be FFS. |
| LGE | Issue 2-1-1-1: Cell re-selection measurement capability Support the recommended WF Issue 2-1-1-2: Cell re-selection measurement rule and procedure For option 1, we think legacy cell reselection rule and procedure can be reused. Support option 2. In ATG, distance based measurement triggering with considering UE speed can be used. We are open to discuss option 3. Also, distance between serving cell and UE can be discussed together with coverage information of serving cell. Issue 2-1-1-3: Cell re-selection measurement requirements Support Recommended WF Issue 2-1-2: MDT Issue 2-1-3: SDT Support option 2 and 3. |
| Huawei | Issue 2-1-1-1: Cell re-selection measurement capability Fine with the recommended WF. Issue 2-1-1-2: Cell re-selection measurement rule and procedure Support option 1. For condition triggered Cell re-selection, we are not sure whether it is supported by RAN2 currently. Issue 2-1-1-3: Cell re-selection measurement requirements We support option 2 and fine with the recommended WF. The ISD is transparent to UE, we think there is no need to add such applicability rules about the DRX cycles. |
| Ericsson | Issue 2-1-1-1: Cell re-selection measurement capability We do not fully understand the recommend WF since the previous agreement does not state that the legacy measurement capability requirements are reused. However, we are fine to study the capability when the ATG scenario is more clear in RF group, e.g. after typical ISD has been agreed. Issue 2-1-1-2: Cell re-selection measurement rule and procedure We support option 3. ATG operating scenario is different from terrestrial operating scenario mainly because of the large cell size, UE speed and UE mobility behaviour (e.g. flight path). It is noted that the legacy IDLE mode cell re-selection requirements are defined assuming much smaller cell size, UE speed and UE mobility than ATG. Therefore the legacy IDLE mode cell re-selection requirements are not suitable and are unnecessarily complicated for ATG. We also think option 2 and 3 are partly related in the sense that both intend to define different cell re-selection requirements for ATG compared to legacy TN requirements. Issue 2-1-1-3: Cell re-selection measurement requirements Both options are related to previous issue 2-1-1-2. Our understanding is that the ISD assumed in RF group ranges from 14 km to 200 km. The RRM requirements should be robust enough to work for the full range of ISD. One compromise option can be that when the UE is required to measure on the neighbour cells then then the legacy requirements are applied, e.g. due to unpredictable changes detected in flight operation mode. However, the UE is not required to always measure on the neighbour cells based on the coverage information of the serving cell otherwise. Issue 2-1-2: MDT We support the recommended WF as it is too early to discuss MDT in this phase of the WI, and our understanding is that MDT is covered in this ATG WI. Issue 2-1-3: SDT We support option 4. We think option 1 and 4 are related, i.e. both options support defining SDT requirements for SDT. But the details can be further discussed. Firstly, we would like to clarify the motivation behind SDT. The purpose of using SDT is not to transmit passenger data as pointed out in R4-2215635. Instead, the intention is to transmit small data, e.g. those related to assistance information such as changes in flight path, altitude, speed etc. This allows the UE to transmit small data while in INACTIVE mode and reduces unnecessary complexity in switching to CONNECTED mode to transmit small data. Our understanding is also that the effort is quite small as we believe most of existing requirements can be reused and the TA validation mechanism can also be simplified. RAN4 should first agree on whether to support SDT and the details can be further discussed. |
| Qualcomm | Issue 2-1-1-1: Cell re-selection measurement capability Fine with the recommended WF Issue 2-1-1-2: Cell re-selection measurement rule and procedure We support Option 1 as a baseline but are okay to discuss some sort of neighbour cell measurement relaxations . |
| Apple | Issue 2-1-1-1: Cell re-selection measurement capability We are fine to study the measurement capability when the ATG scenario is clearer in RF group, e.g. after typical ISD has been agreed. So fine with the recommended WF. Issue 2-1-1-2: Cell re-selection measurement rule and procedure Option 1 can be the starting point. We are open to further study on ATG scenario specific measurements. Issue 2-1-1-3: Cell re-selection measurement requirements Ok with the moderator recommendation. defer the discussion after the scenario is clear in the RF sesson. Issue 2-1-2: MDT Ok with the recommended WF by moderator. Issue 2-1-3: SDT SDT is useful for RRC_INACTIVE state for TN. While for ATG, we are not sure that the ATG UE need to transfer to RRC inactive state often. Further clarification on the use case is needed before we decide that this feature is necessary. |
| ZTE | Issue 2-1-1-1: Cell re-selection measurement capability Fine with the Recommended WF. Issue 2-1-1-2: Cell re-selection measurement rule and procedure Support Option 1. We are open to discuss the distance-based cell re-selection. Issue 2-1-1-3: Cell re-selection measurement requirements Fine with the Recommended WF. Issue 2-1-2: MDT Regarding to MDT, we do not need any enhancement is necessary, if MDT is enabled and supported by ATG UE, just reuse legacy requirement is fine. Issue 2-1-3: SDT Prefer Option 1. |
| CATT | Issue 2-1-1: Cell re-selection requirements Issue 2-1-1-1: Cell re-selection measurement capability Agree recommended WF. Issue 2-1-1-2: Cell re-selection mechanism Agree Option 1 and option 3 Issue 2-1-1-3: Cell re-selection measurement requirements Agree recommended WF. Issue 2-1-2: MDT Agree recommended WF. Issue 2-1-3: SDT Agree recommended WF. We can support Option 1/2/3. |

Sub topic 2-2 Mobility in RRC_CONNECTED

| Company | Comments |
| --- | --- |
| CMCC | Issue 2-2-1-1: NR Handover rule and procedure Support Option 1. Issue 2-2-1-2: NR Handover requirement Support Option 1. We think there is no harm to involve the unknown case. Issue 2-2-2: Conditional Handover We support Option 2/2-1 and Option 3/3-1. Due to large propagation delay in ATG network, we think the CHO could decrease the risk of HO failure. Besides, based on our calculation of PL, when UE is near to cell edge, the variation of PL is not obvious. Therefore, location-based CHO is also needed. Issue 2-2-3: SA: RRC Re-establishment Support the recommended WF. Issue 2-2-3x: Large cell impact on RRC re-establishment We didn’t observe ATG impaction, we are open to further discuss. Issue 2-2-4: Random Access We are fine with the Option in Recommended WF. Issue 2-2-5: SA: RRC Connection Release with Redirection We support the recommended WF. Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection We are open to further discuss if companies observe valid ATG impaction. |
| LGE | Issue 2-2-1-1: NR Handover rule and procedure Option 1. Issue 2-2-1-2: NR Handover requirement Option 1. Issue 2-2-2: Conditional Handover Need more discussion with option 3 Issue 2-2-3: SA: RRC Re-establishment Option 1 Issue 2-2-3x: Large cell impact on RRC re-establishment Issue 2-2-4: Random Access Issue 2-2-5: SA: RRC Connection Release with Redirection Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection |
| Huawei | Issue 2-2-1-2: NR Handover requirement Support option 1. Issue 2-2-2: Conditional Handover Fine with option 2/3. Issue 2-2-3: SA: RRC Re-establishment Support proposal 1. Issue 2-2-3x: Large cell impact on RRC re-establishment More clarification is needed on the impact of large cell. Issue 2-2-4: Random Access Fine with recommended WF. Issue 2-2-5: SA: RRC Connection Release with Redirection Support proposal 1. Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection More clarification is needed on the impact of large cell. |
| Ericsson | Issue 2-2-1-1: NR Handover mechanism We agree that both intra-frequency and inter-frequency measurements are needed, but the ATG UE may not need to perform those regularly as a TN UE. This is similar to the discussed under issue 2-1-1-2. We would like to understand from the proponent of option 1, how this proposal affects the HO requirements or how they intend to capture it. Issue 2-2-1-2: NR Handover requirement We can agree to option 1 provided that it applies to the known target cell only. For unknown target cell more discussions are needed. Issue 2-2-2: Conditional Handover Our view is that CHO can be useful for ATG given that the ATG operating scenario is different from the legacy TN scenario. For example, CHO would allow the NW to identify and configure HO parameters triggering the UE to perform HO while taking into account the large cell size, UE speed, flight path etc. We support introducing Rel-16 and Rel-17 based CHO requirements for ATG, but the details of requirements need further analysis. Issue 2-2-3: SA: RRC Re-establishment Our view is that most of the existing requirements can be reused, but we would like to keep TSI_NR, Tidentify_intra_NR and Tidentify_inter_NR as FFS as they may depend on ATG channel conditions and its performance. Issue 2-2-3x: Large cell impact on RRC re-establishment It’s better to merge the discussion into the general issue 1-6 instead of duplicated discussion. Issue 2-2-4: Random Access For 4-step RA, we agree to option 1, i.e. the existing requirements can be reused excluding 2-step RA. We don’t think 2-step RA is a valid case for ATG scenario given the very large cell size. This should be clarified in the recommended WF. Issue 2-2-5: SA: RRC Connection Release with Redirection Our view is that most of the existing requirements can be reused, but we would like to keep TSI_NR, and Tidentify-NR as FFS as they may depend on ATG channel conditions and its performance. Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection It’s better to merge the discussion into the general issue 1-6 instead of duplicated discussion. |
| Apple | Issue 2-2-1-1: NR Handover rule and procedure Support Option 1. Issue 2-2-1-2: NR Handover requirement Ok with option 1 for known cell and FFS on option 2. Issue 2-2-2: Conditional Handover We think it’s beneficial to consider the difference of ATG network and TN network. New criteria can be considered to improve the system performance. Requirement frame work could be based on the legacy. While new criteria may be needed. Option 3 Issue 2-2-3: SA: RRC Re-establishment FFS Issue 2-2-3x: Large cell impact on RRC re-establishment FFS Issue 2-2-4: Random Access FFS. Premature to agree focus on 4 step RACH. Issue 2-2-5: SA: RRC Connection Release with Redirection FFS Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection FFS |
| ZTE | Issue 2-2-1-1: NR Handover rule and procedure Support Option 1. Issue 2-2-1-2: NR Handover requirement Support Option 1 and 2. For unknown case, For unknown case, according to legacy, if the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = Trs ms. In ATG scenario, NBs should be deployed to cover the whole air line. Since the trajectory is deterministic, so the common case is the target cell is known. So we believe handover to unknown cell is a corner case, which can be ignored, similar as in HST. Issue 2-2-2: Conditional Handover We support Option 1. We can not see clear necessity of CHO in ATG scenario. Issue 2-2-3: SA: RRC Re-establishment Support the recommended WF. Issue 2-2-3x: Large cell impact on RRC re-establishment We can not see different demand since of ATG scenario compared with legacy. Issue 2-2-4: Random Access Fine with the Recommended WF. Issue 2-2-5: SA: RRC Connection Release with Redirection Fine with the Recommended WF. Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection We are open to further discuss. |
| CATT | Issue 2-2-1: Handover Issue 2-2-1-1: NR Handover mechanism Agree recommended WF. Issue 2-2-1-2: NR Handover requirement Agree recommended WF. Issue 2-2-2: Conditional Handover Support Option1 and option 2 acceptable. Issue 2-2-3: SA: RRC Re-establishment Agree recommended WF. It may be better using requirements for HST. Issue 2-2-3x: Large cell impact on RRC re-establishment No impact on RRC re-establishment. Issue 2-2-4: Random Access Agree Option1. It may not be necessary to limit only 4-step random access applicable. Issue 2-2-5: SA: RRC Connection Release with Redirection Agree recommended WF. Prefer Option 1. Tidentify-NR is defined in measurement requirement for ATG. Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection We think no impact on RRC connection release with redirection. Whether delay is impacted is decided the measurement requirements for ATG. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Sub-topic 2-1 Mobility in RRC_IDLE/INACTIVE

|  | Status summary |
| --- | --- |
| Issue 2-1-1-1 | Issue 2-1-1-1: Cell re-selection measurement capability 7 companies support the recommended WF. 1 company points out that the previous agreement does not state that the legacy measurement capability requirements are reused. Based on Moderator’s observation, the recommended WF does not state that the legacy measurement capability requirements are reused either, therefore, we think the recommended WF can be the tentative agreement. Tentative agreements: - Use current UE capability for NR intra-frequency measurement and NR inter-frequency measurement as the starting point. - Further study the capability after the scenario is clearer in the RF group. Recommendations for 2nd round: No more discussion |
| Issue 2-1-1-2 | Issue 2-1-1-2: Cell re-selection mechanism 7 companies support to use Option 1 as the baseline. 5 companies support to further discuss Option 2. 1 company not sure whether Option 2 is supported by RAN2 currently. 4 companies support to further discuss Option 3. 1 company are disagree to Option 3. Based on comments, Option 1 can be used as the baseline and further discuss the relaxation cell-re-selection mechanism. Tentative agreements: - Use the legacy cell-reselection rule and procedure as the baseline. Further discuss whether and how to introduce the relaxation cell-re-selection mechanism in ATG. - Option 1: Introduce distance based triggering as an additional condition for intra- and inter-frequency cell measurement considering ATG UE speed - Option 2: - The A2G UE is allowed to not measure on the neighbour cells based on the coverage information of the serving cell e.g. if serving cell RSRP is above a certain threshold which is FFS. - For cell reselection, the A2G UE should resume the neighbor cell measurement in normal manner without any relaxation based on assistance information related to unpredictable changes in flight operation mode. Recommendations for 2nd round: Continue the discussion |
| Issue 2-1-1-3 | Issue 2-1-1-3: Cell re-selection measurement requirements 6 companies are fine with the recommended WF. 2 companies prefer Option 2. To combine companies’ comments, moderator give the tentative agreement as follows, you can further check in the 2nd round discussion. Tentative agreements: - Reusing legacy R15 requirements of intra-frequency and inter-frequency measurement - The conclusion can be revisited after receiving final conclusion about ISD assumption and deployment approach from RF session. - The requirements are applied when UE is required to measure on the target cells Recommendations for 2nd round: Provide your comment if the tentative agreement is not agreeable. |
| Issue 2-1-2 | Issue 2-1-2: MDT 4 companies are fine with the recommended WF. 1 company propose to reuse the legacy requirement. Since the legacy requirement reusing method is not precluded if we follow the recommended WF. Moderator take the recommended WF as tentative agreement: Tentative agreements: - The conclusion on MDT measurement requirements can be delayed until RAN4 decide how to define ATG cell-reselection measurement requirements. (CMCC) Recommendations for 2nd round: Provide your comment if the tentative agreement is not agreeable. |
| Issue 2-1-3 | Issue 2-1-3: SDT 1 company support to introduce SDT, 1 company oppose to introduce SDT, 1 company don’t have strong view. 1 company need FFS 2 companies think that the legacy requirement can be reused. The views from companies are quite diverse, moderator propose to continue the discussion with following candidate options. Candidate options: - Option 1: SDT is not supported for R18 ATG. - Option 1-1: RAN4 is not going to define ATG specific requirements - Option 2: SDT is supported for A2G in Rel-18 - Option 2-1: SDT requirements for ATG is FFS. - Option 2-2: RAN4 is not going to define ATG specific requirements - Option 3: FFS whether to introduce SDT or not, based on use case and so on. Recommendations for 2nd round: Continue the discussion based on the candidate options above. |

Sub-topic 2-2 Mobility in RRC_CONNECTED

|  | Status summary |
| --- | --- |
| Issue 2-2-1-1 | Issue 2-2-1-1: NR Handover mechanism 5 companies agree with Option 1. One company propose to consider the ATG UE may not need to perform those regularly as a TN UE Candidate options: - Option 1: regular intra-frequency measurement and inter-frequency measurement similar as legacy is needed (CMCC, LGE, Apple, ZTE, CATT) - Option 2: the ATG UE may not need to perform intra-frequency and inter-frequency measurements regularly as a TN UE (Ericsson) Recommendations for 2nd round: Continue the discussion. |
| Issue 2-2-1-2 | Issue 2-2-1-2: NR Handover requirement 3 companies support Option 1, 2 companies propose that for unknown target cell more discussions are needed. 1 company propose unknown case can be ignored, 1 company agree with the recommended WF. Moderator propose to continue the discussion with the following candidate options. Candidate options: - For the known case, reuse legacy NR intra-frequency HO and inter-frequency HO requirements, - For the unknown case - reuse legacy NR intra-frequency HO and inter-frequency HO requirements (HW, LGE, CMCC) - unknown case can be ignored. (ZTE) - FFS (Ericsson, Apple, CATT) Recommendations for 2nd round: Discussion can be continued. |
| Issue 2-2-2 | Issue 2-2-2: Conditional Handover 2 company support Option 1, 4 company support Option 2, 3 company support Option 3. 1 company propose Option 3 should be FFS Candidate options: - Option 1: Not to consider conditional handover for ATG UE at least in Rel-18. (CATT, ZTE) - Option 2: Introduce legacy CHO for ATG (CMCC, Ericsson, HW, CATT) - the legacy R16 CHO delay requirements can be reused. (CMCC) - CHO requirements for ATG are FSS (Ericsson) - Option 3: Introduce location-based CHO for ATG (CMCC, Apple, HW) - Option 3-1: The logic and signalling from R17 NTN could be reused, while the location-based CHO delay requirements should be revisited in ATG scenario. (CMCC) Recommendations for 2nd round: Continue the discussion |
| Issue 2-2-3 | Issue 2-2-3: SA: RRC Re-establishment 3 companies support the recommended WF. 2 company support Option 1. 1 company propose also keep TSI_NR as FFS. To summary companies’ comments, following candidate options are proposed by moderator: Candidate options: - Option 1: Reuse the current definitions of delay requirement, known/unknown cells and side conditions in the RRC re-establishment requirements, reuse the current definitions/requirements on TPRACH TUL_grant (CMCC, LGE, Ericsson, ZTE, CATT) - Option 1-1: Also reuse the current definitions/requirements on TSI_NR, Tidentify_intra_NR and Tidentify_inter_NR. (CMCC, LGE) - Option 1-2: FFS the TSI_NR, Tidentify_intra_NR and Tidentify_inter_NR. (Ericsson, CMCC, ZTE) - Option 1-3: using HST requirements for Tidentify_intra_NR and Tidentify_inter_NR. (CATT) - Option 2: FFS (Apple) Recommendations for 2nd round: Continue the discussion |
| Issue 2-2-3x | Issue 2-2-3x: Large cell impact on RRC re-establishment 3 companies don’t observe ATG impaction on RRC re-establishment. 2 companies are willing to FFS. Candidate options: - Option 1: RAN4 to study the larger cell impact to RRC re-establishment, such as the target cell list and total delay requirement. (Ericsson) - Option 2: No ATG impaction on RRC re-establishment (CMCC, ZTE, CATT) - Option 3: More clarification is needed on the impact and FFS (HW, Apple) Recommendations for 2nd round: Proponent are encouraged to share what is the specific impaction. |
| Issue 2-2-4 | Issue 2-2-4: Random Access For 4-step RA, all companies agree with using the existing requirements. For 2-step RA, companies have different views, Moderator give the candidate options below Candidate options: - For 4-step RA, existing requirements can apply and no need to have ATG specific requirements. The conclusion could be revisited after RF achieve final conclusion about ISD” (CMCC, HW, Ericsson, ZTE) - For 2-step RA - Include 2-step RA, apply existing requirements (CATT) - Exclude 2-step RA in R18 ATG (Ericsson) - FFS(Apple) Recommendations for 2nd round: Further discuss 2-step RA. |
| Issue 2-2-5 | Issue 2-2-5: SA: RRC Connection Release with Redirection 3 companies are fine with recommended WF, 2 companies support proposal 1, 1 company propose also keep TSI_NR as FFS, 1 company would like to FFS Candidate options: - Option 1: Reuse the current definitions of RRC Connection Release with Redirection delay requirement, definitions/requirements on TRACH, TSI-NR and TRRC_procedure_delay (CMCC, CATT, ZTE, HW) - Option 1-1: Reuse the legacy requirement for TSI-NR, Tidentify-NR and total delay requirement. (CMCC, CATT, HW) - Option 1-2: FFS the TSI-NR, Tidentify-NR and total delay requirement. (ZTE, Ericsson) - Option 2: FFS (Apple) Recommendations for 2nd round: Continue the discussion |
| Issue 2-2-5x | Issue 2-2-5x: Large cell impact on RRC Connection Release with Redirection 4 companies are open to discuss, 2 companies don’t observe valid ATG impaction. Candidate options: - Option 1: RAN4 to study the larger cell impact to RRC connection release with redirection, such as the target candidate cell and total delay requirement. (Ericsson) - Option 2: More clarification is needed and FFS (CMCC, HW, Apple, ZTE) - Option 3: No ATG impaction on Connection Release with Redirection (CMCC, CATT) Recommendations for 2nd round: Proponent are encouraged to clarify what is the ATG specific impaction. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #3: Timing and frequency adjustment

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215396 | CATT | Proposal 2: The Kmac should not be needed in ATG network, and the Koffset should be used to support up to 300km cell coverage range. Proposal 3: The ATG UE should do the compensation of transmit frequency and timing based on relative moving velocity and distance between UE and gNB. Proposal 4: The initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline. Proposal 5: The requirements of gradual timing adjustment for TN or NTN in Rel-17 can be used as baseline. Proposal 6: The open loop TA (UE specific TA) and close loop TA (TAC based adjustment) are necessity for TAG UE. Proposal 7: The legacy TN requirement for cell phase synchronization accuracy can be reused. Proposal 8: Not to define deriveSSB-IndexFromCell tolerance in Rel-18, or the legacy TN requirement can be reused. |
| R4-2215505 | CMCC | Proposal 18: Introduce the mechanism of Koffset in ATG system. The conclusion can be revisited after RF session draw the final conclusion about ISD and so on. Proposal 19: There is no need to introduce the UE based UL timing pre-compensation under the ISD assumption of 100-200km. The conclusion could be revisited after receiving the input about ISD and cell radius from RF session. Proposal 20: Reuse the legacy R15 TN requirement for initial transmit timing requirement Te with the assumption of ISD 100-200km. The issue should be reviewed after receiving the conclusion about ISD and cell radius from RF session. Proposal 21: The koffset mechanism may need to be introduced in timing advance adjustment delay requirement. Proposal 22: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS. Proposal 23: Reuse Cell phase synchronization accuracy 3µs requirement for ATG. Proposal 24: Do not update ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ tolerance requirements. It is up to network to decide whether to configure ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not, based the real deployment scenario, such as ISD. |
| R4-2215635 | Apple | Proposal 3: it is proposed to further study Alt 1~3 for cell phase timing misalignment Alt 1: a new offset shall be added on top of deriveSSB-IndexFromCell. deriveSSB-IndexFromCell is enabled/true, but the target cell for measurement shall be offset by the propagation time difference between serving and target cell. Alt 2: if propagation time difference between serving and target is unknown, all the case shall be treated as deriveSSB-IndexFromCell=disabled Alt 3: new signaling is introduced for cell phase timing misalignment, the time difference is indicated by using difference Ies |
| R4-2215937 | LGE | Proposal 6: Existing TA adjustment procedure can be reused for ATG, but other benefits such as signaling overhead reduction need to be investigated further by introducing timing pre-compensation. Proposal 7: Due to longer propagation delay, deriveSSB-IndexFromCell is not valid for ATG FR1 TDD. RAN4 can further discuss whether the applicability of deriveSSB-IndexFromCell according to the propagation delay information is valid. Proposal 8: Reuse legacy cell phase synchronization accuracy. |
| R4-2216276 | HW | Proposal 5: Existing Cell phase synchronization accuracy can apply to ATG. Observation 4: Whether UE shall do frequency pre-compensation should be evaluated in RF session about frequency error requirements. Observation 5: The legacy close-loop TA adjustment is sufficient to support ATG network. Proposal 6: Use the current timing adjustment procedure (close-loop TA adjustment) as baseline for timing requirements, and Tp and Tq shall be updated accordingly. Proposal 7: Existing deriveSSB-IndexFromCell tolerance can apply to ATG. RAN4 shall evaluate whether deriveSSB-IndexFromCell can be assumed always enabled for ATG. |
| R4-2216481 | ZTE | Observation 1: For some combination of frequency and SCS, SSB+TRS is feasible implementation for frequency offset tracking to support 1200km/h for ATG deployment. Observation 2: The solution of frequency offset tracking in NTN system can be considered as reference for ATG system when SSB+TRS is not sufficient for some combination of frequency and SCS. Proposal 1: Referring to whether need to introduce Koffset and Kmac identified in NTN into ATG system, for Kmac, not necessary since of no feeder link existence; for Koffset, considering the RTT for cell edge UE under extreme case, we are open to discuss the necessity. Proposal 2: UE based timing pre-compensation for PRACH or MsgA transmission is necessary for ATG CPE under the extreme case, i.e. 300 km cell coverage. Proposal 3: UE based frequency pre-compensation similar as in NTN can be reused for ATG CPE. |
| R4-2216769 | Ericsson | Observation 1: If NTN is used for ATG then ATG UE GNSS support enables calculation of NTA,UE-specific if orbit data corresponding to path of BS relative to ATG UE is provided. Observation 2: PositionVelocity-r17 in TS 38.331can be used to signal path of BS relative to ATG UE |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 3-1 General issues

Sub-topic description: Sub-topic 3-1 discuss the basic assumption and general issues about timing requirement

Open issues and candidate options before e-meeting:

Issue 3-1-1: The mechanism of Koffset and Kmac

- Proposals

  - Option 1: The Kmac should not be needed in ATG network, and the Koffset should be used to support up to 300km cell coverage range. (CATT)

  - Option 2: Introduce the mechanism of Koffset in ATG system. The conclusion can be revisited after RF session draw the final conclusion about ISD and so on. (CMCC)

  - Option 3: Referring to whether need to introduce Koffset and Kmac identified in NTN into ATG system, for Kmac, not necessary since of no feeder link existence; for Koffset, considering the RTT for cell edge UE under extreme case, we are open to discuss the necessity. (ZTE)

- Recommended WF

  - No need to consider Kmac in ATG network, FFS the Koffset based on cell coverage range.

Issue 3-1-2: Frequency offset tracking

- Proposals

  - Option 1: The solution of frequency offset tracking in NTN system can be considered as reference for ATG system when SSB+TRS is not sufficient for some combination of frequency and SCS. (ZTE)

- Recommended WF

  - Proponents are encouraged to give more clarification about what is the RRM impact of this frequency tracking method.

Issue 3-1-3: BS location

- Proposals

  - Option 1: PositionVelocity-r17 in TS 38.331can be used to signal path of BS relative to ATG UE (Ericsson)

- Recommended WF

  - Option 1

Sub-topic 3-2 Timing and frequency pre-compensation by UE

Sub-topic description: Sub-topic 3-2 discuss the timing compensation procedure in ATG

Open issues and candidate options before e-meeting:

Issue 3-2-1: Whether to introduce UE based Timing pre-compensation

- Proposals

  - Option 1: Use the current timing adjustment procedure (close-loop TA adjustment) as baseline for timing requirements (HW, LGE, CMCC)

    - Option 1-1: There is no need to introduce the UE based UL timing pre-compensation under the ISD assumption of 100-200km. The conclusion could be revisited after receiving the input about ISD and cell radius from RF session. (CMCC)

    - Option 1-2: UE based timing pre-compensation for PRACH or MsgA transmission is necessary for ATG CPE under the extreme case, i.e. 300 km cell coverage. (ZTE)

    - Option 1-3: other benefits such as signalling overhead reduction need to be investigated further by introducing timing pre-compensation. (LGE)

  - Option 2: The ATG UE should do the compensation of transmit timing based on relative moving velocity and distance between UE and gNB. (CATT)

- Recommended WF

  - Discuss the options above

Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation

- Proposals

  - Option 1: The ATG UE should do the compensation of transmit frequency based on relative moving velocity and distance between UE and gNB. (CATT)

  - Option 2: UE based frequency pre-compensation similar as in NTN can be reused for ATG CPE. (ZTE)

  - Option 3: Whether UE shall do frequency pre-compensation should be evaluated in RF session about frequency error requirements. (HW)

- Recommended WF

  - UE based frequency pre-compensation similar as in NTN can be reused in ATG, however, no RRM impact. Details can be further evaluated in RF session.

Sub-topic 3-3 Timing requirements

Sub-topic description: Sub-topic 3-3 discusses the timing requirements in ATG

Open issues and candidate options before e-meeting:

Issue 3-3-1: UE transmit timing

Issue 3-3-1-1: Initial transmit timing requirements Te

- Proposals

  - Option 1: The initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline. (CATT)

  - Option 2: Reuse the legacy R15 TN requirement for initial transmit timing requirement Te with the assumption of ISD 100-200km. The issue should be reviewed after receiving the conclusion about ISD and cell radius from RF session. (CMCC)

- Recommended WF:

  - This issue is highly related with Issue 3-2-1, moderator suggest to discuss Issue 3-2-1 first

Issue 3-3-1-2: Gradual timing adjustment

- Proposals

  - Option 1: The requirements of gradual timing adjustment for TN or NTN in Rel-17 can be used as baseline. (CATT)

  - Option 2: Tp and Tq shall be updated. (HW, CMCC)

    - Option 2-1: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS. (CMCC)

- Recommended WF:

  - Discuss the Options above

Issue 3-3-2: Timing advance

- Proposals

  - Option 1: The open loop TA (UE specific TA) and close loop TA (TAC based adjustment) are necessity for TAG UE. (CATT)

  - Option 2: The koffset mechanism may need to be introduced in timing advance adjustment delay requirement. (CMCC)

- Recommended WF

  - This issue is highly related with Issue 3-2-1, moderator suggest to discuss Issue 3-2-1 first

Issue 3-3-3: Cell phase synchronization accuracy

- Proposals

  - Option 1: The legacy TN requirement for cell phase synchronization accuracy can be reused. (CATT, CMCC, LGE, HW)

- Recommended WF

  - Reuse the legacy TN requirement for cell phase synchronization accuracy

Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance

- Proposals

  - Option 1: Not to define deriveSSB-IndexFromCell tolerance in Rel-18 (CATT)

  - Option 2: Legacy TN requirement can be reused (CATT)

    - Option 2-1: It is up to network to decide whether to configure ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not, based the real deployment scenario, such as ISD (CMCC)

    - Option 2-2: RAN4 shall evaluate whether deriveSSB-IndexFromCell can be assumed always enabled for ATG. (HW)

  - Option 3: it is proposed to further study Alt 1~3 for cell phase timing misalignment (Apple)

    - Alt 1: a new offset shall be added on top of deriveSSB-IndexFromCell. deriveSSB-IndexFromCell is enabled/true, but the target cell for measurement shall be offset by the propagation time difference between serving and target cell.

    - Alt 2: if propagation time difference between serving and target is unknown, all the case shall be treated as deriveSSB-IndexFromCell=disabled

    - Alt 3: new signaling is introduced for cell phase timing misalignment, the time difference is indicated by using difference Ies

  - Option 4: Due to longer propagation delay, deriveSSB-IndexFromCell is not valid for ATG FR1 TDD. RAN4 can further discuss whether the applicability of deriveSSB-IndexFromCell according to the propagation delay information is valid. (LGE)

- Recommended WF

  - Discuss the Options above.

Companies views’ collection for 1st round

Open issues

Sub topic 3-1 General issues

| Company | Comments |
| --- | --- |
| CMCC | Issue 3-1-1: The mechanism of Koffset and Kmac We support the recommended WF. For Koffset, based on our calculation, in 15kHz SCS scenario, if the cell radius larger than 150km, where the RTT will larger than 1ms=1slot, Koffset should be configured to compensate the RTT between UE and BS. In 30kHz SCS scenario, if the cell radius larger than 75km, where the RTT will larger than 0.5ms=1slot, Koffset should be configured to compensate the RTT between UE and BS. Considering ISD assumption of 100km-200km, the cell radius may exceed 75km. Therefore, Koffset mechanism shall be introduced in ATG network. Issue 3-1-2: Frequency offset tracking Issue 3-1-3: BS location We support Option 1. The BS location or reference location may be needed for frequency compensation, CHO and so on. |
| LGE | Issue 3-1-1: The mechanism of Koffset and Kmac Support recommended WF. Issue 3-1-2: Frequency offset tracking Issue 3-1-3: BS location Support option 1 |
| Huawei | Issue 3-1-1: The mechanism of Koffset and Kmac Fine with option 2 Issue 3-1-2: Frequency offset tracking Same question as recommended WF. Clarification is needed on the RRM impact. Issue 3-1-3: BS location Clarification is needed on the usage of this information for RRM purpose. |
| Ericsson | Issue 3-1-1: The mechanism of Koffset and Kmac The WF is OK Issue 3-1-2: Frequency offset tracking: NTN can be used. The NTN system orbit data broadcasted to UE enables UE to compensate Doppler at receive and transmit. Issue 3-1-3: BS location: Option 1. |
| ZTE | Issue 3-1-1: The mechanism of Koffset and Kmac Fine with the recommended WF. Issue 3-1-2: Frequency offset tracking For DL, generally SSB is used for initial frequency tracking. It is expected that the residual frequency offset can be handled by other RS, such as DMRS, PTRS or TRS. Therefore, SSB+TRS is feasible implementation for frequency offset tracking in ATG system. However under some combination of band and SCS, we concern SSB+TRS is not sufficient to realize frequency tracking in ATG. So UE pre-compensation is also helpful. Issue 3-1-3: BS location We support Option 1. The BS location or reference location may be needed for frequency compensation. |
| CATT | Issue 3-1-1: The mechanism of Koffset and Kmac Agree recommended WF。 Issue 3-1-2: Frequency offset tracking Agree recommended WF. We think the demodulation and measurement performance may be impacted by larger frequency offset. UE transmit frequency pre-compensation should be considered. Issue 3-1-3: BS location Agree PositionVelocity-r17 can be used for indicating gNB location. But the location should be in geodetic coordinate, not relative to ATG UE. |

Sub topic 3-2 Timing and frequency pre-compensation by UE

| Company | Comments |
| --- | --- |
| CMCC | Issue 3-2-1: Whether to introduce UE based Timing pre-compensation We support Option 1-1. Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation We support the recommended WF. |
| LGE | Issue 3-2-1: Whether to introduce UE based Timing pre-compensation We support option 1 and option 1-3. Also, depending on the ISD assumption, timing pre-compensation can be further discussed. Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation |
| Huawei | Issue 3-2-1: Whether to introduce UE based Timing pre-compensation Support option 1 Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation We do not observe RRM impact for frequency pre-compensation. |
| Ericsson | Issue 3-2-1: Whether to introduce UE based Timing pre-compensation: Option 1, current closed loop TA adjustment can indeed handle up to 100 km cells (option 1-1), ie ISD = 200 km, but only the long sequence PRACH format 0,1 or 2 can handle 100 km cell radius and then ΔfRA= 1.25 kHz => the system becomes sensitive to uncompensated Doppler. For 300 km cell range as stated in WID we need option 1-2, ie UE based timing pre-compensation, or some other compensation, option 2. Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation Option 2: UE based frequency pre-compensation similar as in NTN can be reused for ATG CPE. |
| ZTE | Issue 3-2-1: Whether to introduce UE based Timing pre-compensation Related with Issue 1-1, we can wait for the outcome of Issue 1-1. Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation General fine with the recommended WF. |
| CATT | Issue 3-2-1: Whether to introduce UE based Timing pre-compensation It is depended on assumption of maximum cell coverage supported for ATG. If it is larger than 100km, UE based timing pre-compensation should be introduced. Otherwise current timing adjustment procedure can be reused. Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation Agree recommended WF. |

Sub topic 3-3 Timing requirements

| Company | Comments |
| --- | --- |
| CMCC | Issue 3-3-1-1: Initial transmit timing requirements Te Support the recommended WF. Issue 3-3-1-2: Gradual timing adjustment May be this issue also depends on whether UE based pre-compensation will be introduced or not. If it is introduced, then NTN method can be reused. If it is not introduced, then Tp and Tq should involve the timing drift from UE movement. Issue 3-3-2: Timing advance Support the recommended WF. Issue 3-3-3: Cell phase synchronization accuracy Support the recommended WF. Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance In our understanding, Option 2-1/2-2 and Option 4 has similar meanings, they are not conflict to each other. One minor update on Option 2-1: It is up to network to decide whether to enable ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not, based the real deployment scenario, such as ISD Updated Option 2-1 is preferred. |
| LGE | Issue 3-3-1-1: Initial transmit timing requirements Te Support moderator’s suggestion Issue 3-3-1-2: Gradual timing adjustment Issue 3-3-2: Timing advance Support moderator’s suggestion. Issue 3-3-3: Cell phase synchronization accuracy Support option 1. Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance Support option 4. In ATG, deriveSSB-IndexFromCell is not always valid due to propagation delay. However, deriveSSB-IndexFromCell can be valid if the propagation delay information between serving and target is available. We think our proposal (option 4) can be discussed together with Alt 1 in option 3 (Apple). |
| Huawei | Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance We support option 2-1. There is no need to update the tolerance. Whether to enable this is up to NW configuration. |
| Ericsson | Issue 3-3-1-1: Initial transmit timing requirements Te If we use NTN as baseline with GNSS 50 m uncertainty, for timing compensation, then we need T_e_NTN, ie Option 1. The Te_NTN values assume 50 m GNSS uncertainty (and 30 m satellite pos uncertainty). If we decide to reuse rel-15 TN requirements, (option 2), then UE GNSS can not be used for timing compensation Issue 3-3-1-2: Gradual timing adjustment: The exact update of Tp and Tq depends if we use TN or NTN and the characteristics of ATG. Issue 3-3-2: Timing advance: We need option 1 if we go for 300 km cell range as in WID, ie the initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline. Option 2 is fine got us Koffset. Issue 3-3-3: Cell phase synchronization accuracy Recommended WF is fine, ie legacy TN requirement 3 µs. Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance So far we have not seen any evaluation showing impact on deriveSSB-IndexFromCell due to ATG. Without having done such evaluations, we are not sure whether the existing requirements can be reused. Therefore we propose to postpone the decision on this issue to allow more time for companies to study/investigate this issue as also pointed out by some companies. |
| Apple | Issue 3-3-1-1: Initial transmit timing requirements Te - Ok with moderator recommended WF. Ok to use NTN requirement as baseline and FFS on ATG specific details. Issue 3-3-1-2: Gradual timing adjustment - Tend to agree with Option 2. Details can be FFS. Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance Propose to have further study based on option 3. |
| ZTE | Issue 3-3-1-1: Initial transmit timing requirements Te Support the recommended WF. Issue 3-3-1-2: Gradual timing adjustment Agree with CMCC and Ericsson. Issue 3-3-2: Timing advance Support the recommended WF. Issue 3-3-3: Cell phase synchronization accuracy Support the recommended WF. Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance Support Option 2-1. |
| CATT | Issue 3-3-1: UE transmit timing Issue 3-3-1-1: Initial transmit timing requirements Te Agree recommended WF. Issue 3-3-1-2: Gradual timing adjustment This issue is related with Issue 3-2-1. If introducing UE transmite timing pre-compensation, the requirements for TN can be reused. Otherwise, Tp and Tq may be updated. Issue 3-3-2: Timing advance This issue is related with Issue 3-2-1 also. If coverage distance is less than 100km, the open loop TA and Koffset is not necessary. Issue 3-3-3: Cell phase synchronization accuracy Agree recommended WF. Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance It is proposaed that not defining the requirements or reused current TN requirement for Rel-18 ATG. DeriveSSB-IndexFromCell Enabled/disabled is decided by network. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Sub-topic 3-1 General issues

|  | Status summary |
| --- | --- |
| Issue 3-1-1 | Issue 3-1-1: The mechanism of Koffset and Kmac All companies support no need to consider Kmac in ATG network. For Koffset, moderator summary companies view as in Candidate Options. Candidate options: - For Kmac, No need to consider Kmac in ATG network - For Koffset, - Option 1: Introduce the mechanism of Koffset in ATG system. The conclusion can be revisited after RF session draw the final conclusion about ISD and so on. (HW, CMCC) - Option 2: FFS the Koffset. (Ericsson, ZTE, CATT) Recommendations for 2nd round: Consider the ISD assumption 14-200km, further discuss the mechanism of Koffset, the issue will be discussed in Friday GTW session. |
| Issue 3-1-2 | Issue 3-1-2: Frequency offset tracking Proponents provide the clarification that the demodulation and measurement performance may be impacted by larger frequency offset. UE transmit frequency pre-compensation should be considered. However, no RRM core requirements will be impacted based on moderator’s understanding. Therefore, the tentative agreement is as follows: Tentative agreements: - The solution of frequency offset tracking in NTN system can be considered as reference for ATG system when SSB+TRS is not sufficient for some combination of frequency and SCS. - UE transmit frequency pre-compensation could be considered in ATG - No RRM core requirements impaction. Recommendations for 2nd round: - Please check whether the tentative agreements are agreeable. The issue will be discussed in Friday GTW session. |
| Issue 3-1-3 | Issue 3-1-3: BS location 5 companies support Option 1. 1 companies disagree with Option 1. 1 company need clarification about usage. Candidate options: - Option 1: PositionVelocity-r17 in TS 38.331can be used to signal path of BS relative to ATG UE (Ericsson, CMCC, LGE, ZTE, CATT) - Option 1-1: the location should be in geodetic coordinate, not relative to ATG UE (CATT) - Option 2: FFS (HW) Recommendations for 2nd round: Consider the 1st round feedback about the usage of frequency compensation, CHO. Please check whether Option 1/Option 1-1 is agreeable. |

Sub-topic 3-2 Timing and frequency pre-compensation by UE

|  | Status summary |
| --- | --- |
| Issue 3-2-1 | Issue 3-2-1: Whether to introduce UE based Timing pre-compensation Candidate options: - Use the current timing adjustment procedure (close-loop TA adjustment) as baseline for timing requirements (CMCC, HW, LGE) - The conclusion could be revisited after receiving the input about ISD and cell radius from RF session. (CMCC) - Use the current timing adjustment procedure (close-loop TA adjustment) as baseline for timing requirements, other benefits such as signalling overhead reduction need to be investigated further by introducing timing pre-compensation. (LGE) - Based on ISD assumption (Ericsson, ZTE, CATT) Recommendations for 2nd round: Consider the ISD assumption 14-200km, further discuss whether Option 1 and Option 1-1 is agreeable. This issue will be treated in Friday GTW session |
| Issue 3-2-2 | Issue 3-2-2: Whether to introduce UE based Frequency pre-compensation Tentative agreements: - UE based frequency pre-compensation similar as in NTN can be reused in ATG, however, no RRM impact. Details can be further evaluated in RF session. (CATT, ZTE, CMCC) Recommendations for 2nd round: Provide your comments if the above tentative agreements are not agreeable. |

Sub-topic 3-3 Timing requirements

|  | Status summary |
| --- | --- |
| Issue 3-3-1-1 | Issue 3-3-1-1: Initial transmit timing requirements Te Candidate options: - Option 1: The initial transmit timing requirement Te need to be defined for ATG UE, and the requirements for NTN UE can be used as baseline. (CATT) - Option 2: Reuse the legacy R15 TN requirement for initial transmit timing requirement Te with the assumption of ISD 100-200km. The issue should be reviewed after receiving the conclusion about ISD and cell radius from RF session. (CMCC) Recommendations for 2nd round: Derive the conclusion based on Issue 3-2-1. The issue can be treated in Friday GTW session. |
| Issue 3-3-1-2 | Issue 3-3-1-2: Gradual timing adjustment Candidate options: - Option 1: The requirements of gradual timing adjustment for TN or NTN in Rel-17 can be used as baseline. (CATT) - Option 2: Tp and Tq shall be updated. (HW, CMCC) - Option 2-1: For ATG, the Tq_ATG and Tp_ATG should be 9.5*64*Tc in FR1 and UL SCS 15kHz and 30kHz SCS. (CMCC) Recommendations for 2nd round: Derive the conclusion based on Issue 3-2-1. |
| Issue 3-3-2 | Issue 3-3-2: Timing advance Candidate options: - Option 1: The open loop TA (UE specific TA) and close loop TA (TAC based adjustment) are necessity for TAG UE. (CATT) - Option 2: The koffset mechanism may need to be introduced in timing advance adjustment delay requirement. (CMCC) Recommendations for 2nd round: Derive the conclusion based on Issue 3-2-1. |
| Issue 3-3-3 | Issue 3-3-3: Cell phase synchronization accuracy Tentative agreements: - Reuse the legacy TN requirement for cell phase synchronization accuracy Recommendations for 2nd round: No more discussion |
| Issue 3-3-4 | Issue 3-3-4: deriveSSB-IndexFromCell and deriveSSB-IndexFromCell-inter tolerance Based on companies comments in 1st round, we further summarize the candidate options. Candidate options: - Option 1: Legacy TN requirement can be reused (CATT, CMCC, HW, ZTE) - Option 1-1: It is up to network to decide whether to enable ‘deriveSSB-IndexFromCell’ and ‘deriveSSB-IndexFromCell-inter’ or not, based the real deployment scenario, such as ISD (CMCC, HW, ZTE, CATT) - Option 2: it is proposed to further study Alt 1~3 for cell phase timing misalignment (Apple) - Alt 1: a new offset shall be added on top of deriveSSB-IndexFromCell. deriveSSB-IndexFromCell is enabled/true, but the target cell for measurement shall be offset by the propagation time difference between serving and target cell. (LGE) - Alt 2: if propagation time difference between serving and target is unknown, all the case shall be treated as deriveSSB-IndexFromCell=disabled - Alt 3: new signaling is introduced for cell phase timing misalignment, the time difference is indicated by using difference IEs - Option 3: Due to longer propagation delay, deriveSSB-IndexFromCell is not valid for ATG FR1 TDD. RAN4 can further discuss whether the applicability of deriveSSB-IndexFromCell according to the propagation delay information is valid. (LGE) - Option 4: FFS (Ericsson) Recommendations for 2nd round: Continue the discussion. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #4: Signalling characteristics

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215505 | CMCC | Proposal 25: Reuse the legacy RLM OOS evaluation period and legacy BFD evaluation period. Proposal 26: Further discuss whether the requirement for RLM In-sync evaluation period and Candidate beam detection evaluation period should be tightened or not in ATG network. Proposal 27: Reuse the legacy ‘Active BWP switching delay’, ‘Active TCI state switching delay’, ’Active downlink TCI state switching delay for unified TCI’, ’Active uplink TCI state switching delay for unified TCI’ and ‘Pathloss reference signal switching delay’ requirement for ATG. Proposal 28: Uplink spatial relation switch delay requirement will not be applicable for R18 ATG. Proposal 29: No need to introduce the pre-configured measurement gap for R18 ATG. |
| R4-2216276 | HW | Proposal 8: Existing requirements for RLM/BFD/ Active BWP switching delay/ Active TCI state switching delay/ Pathloss reference signal switching delay can apply to NTN without ATG specific impacts. |
| R4-2216481 | ZTE | Proposal 13: Reusing legacy requirements of RLM, BFD, LRP and BWP switching active TCI state/spatial relation switching are fine. |
| R4-2216769 | Ericsson | - Discussions on whether the legacy RLM requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance. - Discussions on whether the legacy BFD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance. - Discussions on whether the legacy CBD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance. - Discussions on whether the legacy L1-RSRP measurement requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance. - No need to define interruptions due to active BWP in section 8.2 of TS 38.133 for NR serving cells for EN-DC and NE-DC. - Existing active BWP switching requirements (excluding simultaneous and non-simultaneous BWP switching delay on multiple CCs) defined are reused for ATG. - Existing TCI state switching requirements are reused for ATG. - Existing uplink spatial relation switching delay requirements are reused for ATG. - Existing requirements on UE-specific CBW change are reused for ATG. - Pre-MG can be used in ATG network to improve the total system performance. - RAN4 to investigate conditions under which Pre-MG can be disabled/enabled. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 4-1 Signalling characteristics related requirements

Sub-topic description: Sub-topic 4-1 will discuss the signalling characteristics

Open issues and candidate options before e-meeting:

Issue 4-1-1: Radio Link Monitoring

- Proposals

  - Option 1: Reuse the legacy RLM OOS evaluation period. Further discuss whether the requirement for RLM In-sync evaluation period should be tightened or not in ATG network. (CMCC)

  - Option 2: Existing requirements for RLM can apply to ATG without ATG specific impacts. (HW, ZTE)

  - Option 3: Discussions on whether the legacy RLM requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance. (Ericsson)

- Recommended WF

  - Discuss the following option “Reuse the legacy RLM OOS evaluation period, FFS RLM In-sync evaluation period. The conclusion could be revisited after RF group has identified the channel model.”

Issue 4-1-2: Link Recovery Procedure

- Proposals

  - Option 1: Reuse the legacy BFD evaluation period. Further discuss whether the requirement for Candidate beam detection evaluation period should be tightened or not in ATG network. (CMCC)

  - Option 2: Existing requirements for BFD can apply to ATG without ATG specific impacts. (HW, ZTE)

  - Option 3: Discussions on whether the legacy BFD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance.  (Ericsson)

  - Option 4: Discussions on whether the legacy CBD requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance. (Ericsson)

- Recommended WF

  - Discuss the options.

Issue 4-1-3: Active BWP switching delay

- Proposals

  - Option 1: Reuse the legacy Active BWP switching delay requirement for ATG. (CMCC, HW, ZTE)

  - Option 2: No need to define interruptions due to active BWP in section 8.2 of TS 38.133 for NR serving cells for EN-DC and NE-DC. (Ericsson)

  - Option 3: Existing active BWP switching requirements (excluding simultaneous and non-simultaneous BWP switching delay on multiple CCs) defined are reused for ATG. (Ericsson)

- Recommended WF

  - Reuse the legacy Active BWP switching delay requirement for ATG. No need to define interruptions due to active BWP in section 8.2 of TS 38.133 for NR serving cells for EN-DC and NE-DC.

Issue 4-1-4: L1-RSRP measurement requirements

- Proposals

  - Option 1: Reusing legacy requirements of L1-RSRP measurement requirements (ZTE)

  - Option 2: Discussions on whether the legacy L1-RSRP measurement requirements can be reused for ATG is postponed until RF group has identified the channel model to evaluate the ATG performance. (Ericsson)

- Recommended WF

  - Discuss the following option “Reuse the legacy L1-RSRP measurement requirements for ATG. The conclusion could be revisited after RF group has identified the channel model.”

Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI

- Proposals

  - Option 1: Reuse the legacy Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI requirement for ATG. (CMCC)

  - Option 2: Existing requirements for Active TCI state switching delay can apply to ATG without ATG specific impacts. (HW, ZTE, Ericsson)

- Recommended WF

  - Reuse the legacy Active TCI state switching delay requirement for ATG.

  - Further check Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay measurement requirement.

Issue 4-1-6: Pathloss reference signal switching delay

- Proposals

  - Option 1: Reuse the legacy Pathloss reference signal switching delay requirement for ATG. (CMCC, HW)

- Recommended WF

  - Reuse the legacy Pathloss reference signal switching delay requirement for ATG.

Issue 4-1-7: Uplink spatial relation switch delay

- Proposals

  - Option 1: Uplink spatial relation switch delay requirement will not be applicable for R18 ATG. (CMCC)

  - Option 2: Reusing legacy requirements of spatial relation switching for ATG. (ZTE, Ericsson)

- Recommended WF

  - Discuss the options above

Issue 4-1-8: Pre-configured measurement gap

- Proposals

  - Option 1: No need to introduce the pre-configured measurement gap for R18 ATG. (CMCC)

  - Option 2: Pre-MG can be used in ATG network to improve the total system performance. RAN4 to investigate conditions under which Pre-MG can be disabled/enabled. (Ericsson)

- Recommended WF

  - Discuss the Options above

Issue 4-1-9: UE-specific CBW change requirements

- Proposals

  - Option 1: Existing requirements on UE-specific CBW change are reused for ATG. (Ericsson)

- Recommended WF

  - Existing requirements on UE-specific CBW change are reused for ATG.

Companies views’ collection for 1st round

Open issues

Sub topic 4-1 Signalling characteristics related requirements

| Company | Comments |
| --- | --- |
| CMCC | Issue 4-1-1: Radio Link Monitoring Support the recommended WF. Issue 4-1-2: Link Recovery Procedure We support the Option 1. If beam failure happens, it is beneficial that UE could detect candidate beam as soon as possible. Issue 4-1-3: Active BWP switching delay Support the recommended WF. Issue 4-1-4: L1-RSRP measurement requirements Support the Option recommended WF. Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI Support the first bullet in recommended WF. As for the second bullet, we think the legacy requirement can also be reused for Active downlink TCI state switching delay for unified TCI and Active uplink TCI state switching delay for unified TCI Issue 4-1-6: Pathloss reference signal switching delay Support the recommended WF. Issue 4-1-7: Uplink spatial relation switch delay Option 1, this feature is only for FR2. Issue 4-1-8: Pre-configured measurement gap Based on our understanding, per-confided MG could provide performance gain in BWP switch scenario, however, we don’t think there will be too frequency BWP switch in ATG network, so Option 1 is preferred. Issue 4-1-9: UE-specific CBW change requirements Support the recommended WF. |
| LGE | Issue 4-1-1: Radio Link Monitoring Option 1 Issue 4-1-2: Link Recovery Procedure Option 1 Issue 4-1-3: Active BWP switching delay Support recommended WF Issue 4-1-4: L1-RSRP measurement requirements Support recommended WF Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI Issue 4-1-6: Pathloss reference signal switching delay Issue 4-1-7: Uplink spatial relation switch delay Issue 4-1-8: Pre-configured measurement gap Issue 4-1-9: UE-specific CBW change requirements |
| Huawei | Issue 4-1-1: Radio Link Monitoring Fine with recommended WF Issue 4-1-2: Link Recovery Procedure Support option 2 but also fine with further discuss the channel model Issue 4-1-3: Active BWP switching delay Issue 4-1-4: L1-RSRP measurement requirements Fine with recommended WF Issue 4-1-6: Pathloss reference signal switching delay Fine with recommended WF. |
| Ericsson | Issue 4-1-1: Radio Link Monitoring We support option 3 because currently it is not known how the RLM evaluations are going to be impacted by the ATG channel model. For example, would the PDCCH performance will be same when the it is evaluated for RLM at 10 000 meter altitude compared to evaluation performed on the ground level as in legacy TN operation. In order to understand this, the channel model to use for evaluating the ATG performance need to be in place. This depends on the progress on RF group. In general, we are fine to reuse the existing framework and requirements but we are not ready to reach an agreement without knowing which channel model to use. Issue 4-1-2: Link Recovery Procedure Based on the arguments provided in issue 4-1-1, we support option 3 and 4 . Issue 4-1-3: Active BWP switching delay We support option 2 and 3. We don’t think the existing requirements can be directly reused since they apply to multiple cells but for ATG it was agreed to not support CA/DC in Rel-18. For example, the existing BWP switching requirements are defined for both simultaneous and non-simultenaous BWP switching on multiple CCs since Rel-16. Similarly, the legacy interruption requirements apply to multiple cells (CA/DC) which are not needed for ATG. Therefore option 1 is not relevant. Issue 4-1-4: L1-RSRP measurement requirements Based on the arguments provided in issue 4-1-1, we support 4-1-4. Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI We support option 2. We don’t see much difference between option 1 and 2 though. Issue 4-1-6: Pathloss reference signal switching delay This feature is related to eMIMO and RAN4 needs to first decide whether eMIMO is to be considered for ATG. If this agreement is reached, only then the detailed requirements can be discussed. Therefore option 1 should be discussed after RAN4 has reached an agreement on whether to support eMIMO for ATG. Issue 4-1-7: Uplink spatial relation switch delay Our understanding is that for FR2, certain assumptions on T L1-RSRP are made. However, the rest of the requirements are valid for both FR1 and FR2 and we think the FR1 requirements can be reused for ATG. However, we are open to discuss both options. Issue 4-1-8: Pre-configured measurement gap This is a measurement issue. Could moderator move it to Topic #5? In Rel-17, Pre-MG is introduced for dynamically switching between intra-frequency measurement with gap and intra-frequency without gap. We should also notice that one of the KPIs for ATG network is the throughput. If the MG can be disabled, it will help to improve the total system throughput. Thus, from our understanding, the Pre-MG can have benefits than legacy MG and will be used for ATG network. Issue 4-1-9: UE-specific CBW change requirements Option 1 is agreeable. |
| ZTE | Issue 4-1-1: Radio Link Monitoring Support the recommended WF. Issue 4-1-2: Link Recovery Procedure Support Option 1 and 2. Issue 4-1-3: Active BWP switching delay Support the recommended WF. Issue 4-1-4: L1-RSRP measurement requirements Support the recommended WF. Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI Support Option 2 and the first bullet in recommended WF. We are not sure whether we need to further check unified TCI. Issue 4-1-6: Pathloss reference signal switching delay Support the recommended WF. Issue 4-1-7: Uplink spatial relation switch delay We are open to discuss Option 1. Issue 4-1-8: Pre-configured measurement gap Support Option 1. Issue 4-1-9: UE-specific CBW change requirements Support the recommended WF. |
| CATT | Issue 4-1-1: Radio Link Monitoring Agree recommended WF. Prefer Option 2. Issue 4-1-2: Link Recovery Procedure Agree recommended WF. Prefer Option 2. Issue 4-1-3: Active BWP switching delay Agree recommended WF. Issue 4-1-4: L1-RSRP measurement requirements Agree recommended WF. Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI Agree recommended WF. Issue 4-1-6: Pathloss reference signal switching delay Agree recommended WF. Issue 4-1-7: Uplink spatial relation switch delay Prefer Option 2. Issue 4-1-8: Pre-configured measurement gap Option 1. Issue 4-1-9: UE-specific CBW change requirements Agree recommended WF. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Sub-topic 4-1 Signalling characteristics related requirements

|  | Status summary |
| --- | --- |
| Issue 4-1-1 | Issue 4-1-1: Radio Link Monitoring 2 companies support Option 1, 3 companies support Option 2, 1 company support Option 3. 2 companies also support the recommended WF. Since the recommended WF cover the common part and different part of both Option 1,2,3, moderator propose to continue the discussion based on the recommended WF. Tentative agreements: - For RLM OOS evaluation period, reuse the legacy requirement. - For RLM In-sync evaluation period, - Option 1: FFS - Option 2: Reuse the legacy requirement - The conclusion could be revisited after RF group has identified the channel model Recommendations for 2nd round: Continue the discussion based on the tentative agreement. |
| Issue 4-1-2 | Issue 4-1-2: Link Recovery Procedure 3 companies support Option 1, 3 companies support Option 2, 2 company support Option 3 and 4. moderator propose to continue the discussion based on following tentative agreement: Tentative agreements: - For BFD evaluation period, reuse the legacy requirement. - For CBD evaluation period, - Option 1: FFS (CMCC, ZTE, LGE) - Option 2: Reuse the legacy requirement (HW, ZTE, CATT) - The conclusion could be revisited after RF group has identified the channel model Recommendations for 2nd round: Continue the discussion based on the tentative agreement. |
| Issue 4-1-3 | Issue 4-1-3: Active BWP switching delay Based on the comments, moderator summarize the candidate options as follows: Candidate options: - No need to define interruptions due to active BWP in section 8.2 of TS 38.133 for NR serving cells for EN-DC and NE-DC. - FFS: - Option 1: Reuse the legacy Active BWP switching delay requirement for ATG. (CMCC, CATT, ZTE) - Option 2: Existing active BWP switching requirements (excluding simultaneous and non-simultaneous BWP switching delay on multiple CCs) defined are reused for ATG. Recommendations for 2nd round: Continue the discussion based on the candidate options. |
| Issue 4-1-4 | Issue 4-1-4: L1-RSRP measurement requirements All companies are fine with the recommended WF Tentative agreements: - Reuse the legacy L1-RSRP measurement requirements for ATG. The conclusion could be revisited after RF group has identified the channel model. Recommendations for 2nd round: No more discussion |
| Issue 4-1-5 | Issue 4-1-5: Active TCI state switching delay, Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay for unified TCI Tentative agreements: - Reuse the legacy Active TCI state switching delay requirement for ATG. - Further check Active downlink TCI state switching delay for unified TCI, Active uplink TCI state switching delay measurement requirement. Recommendations for 2nd round: Companies please provide your views on whether and how to consider unified TCI. |
| Issue 4-1-6 | Issue 4-1-6: Pathloss reference signal switching delay All companies except Ericsson agree with recommended WF. Moderator suggest to keep this issue open. Candidate options: - Option 1: Reuse the legacy Pathloss reference signal switching delay requirement for ATG. (CMCC, HW, ZTE, CATT) - Option 2: Check whether eMIMO is to be considered. (Ericsson) Recommendations for 2nd round: Continue the discussion |
| Issue 4-1-7 | Issue 4-1-7: Uplink spatial relation switch delay Companies share different views. Candidate options: - Option 1: Uplink spatial relation switch delay requirement will not be applicable for R18 ATG. (CMCC, ZTE) - Option 2: Reusing legacy requirements of spatial relation switching for ATG. (ZTE, Ericsson, CATT) Recommendations for 2nd round: Continue the discussion |
| Issue 4-1-8 | Issue 4-1-8: Pre-configured measurement gap Companies share different views. Candidate options: - Option 1: No need to introduce the pre-configured measurement gap for R18 ATG. (CMCC, ZTE, CATT) - Option 2: Pre-MG can be used in ATG network to improve the total system performance. RAN4 to investigate conditions under which Pre-MG can be disabled/enabled. (Ericsson) Recommendations for 2nd round: Continue the discussion |
| Issue 4-1-9 | Issue 4-1-9: UE-specific CBW change requirements No company disagree with the recommended WF Tentative agreements: - Existing requirements on UE-specific CBW change are reused for ATG. Recommendations for 2nd round: No more discussion |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #5: Measurement

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215505 | CMCC | Proposal 30: Reuse the legacy MG requriements and legacy measurement capability of Monitoring of multiple layers using gap for R18 ATG. Proposal 31: For measurement capability of maximum allowed layers for multiple monitoring, reuse the current legacy intra and inter frequency measurement capability as the baseline. the total number of effective carrier frequency layers that UE shall be capable of monitoring update to 8. Proposal 32: Take 2 searchers as the assumption when defining the CSSF requirement. Proposal 33: The legacy NR intra-frequency measurements requirement, NR inter-frequency measurements requirement and CSI-RS based L3 measurements requirement can be reused for R18 ATG. Proposal 34: Don’t consider L1-RSRP measurement for a cell with different PCI from serving cell in R18 ATG. Proposal 35: Reuse the legacy delay requirement for NR measurements with autonomous gaps for R18 ATG. |
| R4-2215635 | Apple | Proposal 4: The measurement window and measurement GAP need to be redesigned based on the propagation time difference between serving gNB and neighbor gNB. |
| R4-2216276 | HW | Observation 6: There is no gap impact to support ATG. Proposal 9: Existing intra-frequency and inter-frequency measurement requirements can apply. For intra-frequency measurement without measurement gap, CSSFintra = 1 for ATG. |
| R4-2216481 | ZTE | Proposal 10: It is not necessary to specify the upper bound of DRS cycle for ATG system. Proposal 11: Legacy intra/inter-frequency measurement can be baseline for ATG CEP. For CSSF, the CSSF under RedCap single carrier case can be a reference. Proposal 12: We can not see obvious necessity of positioning measurement for ATG if UE based frequency pre-compensation is supported. |
| R4-2216769 | Ericsson | - Only FR1 MG will be used in Rel-18 ATG network. - The intra-frequency measurement requirement needs to be updated, such as no deactivated SCell measurement, no SCCs, PSCell measurement. - CSSF outside gap needs to be updated. - CSI-RS measurement is unnecessary in ATG network. - RAN4 to study the trade-off between Inter-frequency measurement within MG and the data throughput due to large cell coverage. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 5-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 5-1-1: Measurement gap

- Proposals

  - Option 1: Reuse the legacy MG requirements for R18 ATG. (CMCC, HW)

  - Option 2: The measurement window and measurement GAP need to be redesigned based on the propagation time difference between serving gNB and neighbor gNB. (Apple)

  - Option 3: Only FR1 MG will be used in Rel-18 ATG network. (Ericsson)

- Recommended WF

  - Discuss the options above

Issue 5-1-2: Measurement capability

- Proposals

  - Option 1: Reuse the legacy measurement capability of Monitoring of multiple layers using gap for R18 ATG. (CMCC)

  - Option 2: For measurement capability of maximum allowed layers for multiple monitoring, reuse the current legacy intra and inter frequency measurement capability as the baseline. the total number of effective carrier frequency layers that UE shall be capable of monitoring update to 8. (CMCC)

- Recommended WF

  - Discuss the Options above

Issue 5-1-3: CSSF

- Proposals

  - Option 1: Take 2 searchers as the assumption when defining the CSSF requirement. (CMCC)

  - Option 2: For intra-frequency measurement without measurement gap, CSSFintra = 1 for ATG. (HW)

  - Option 3: For CSSF, the CSSF under RedCap single carrier case can be a reference. (ZTE)

  - Option 4: CSSF outside gap needs to be updated. (Ericsson)

- Recommended WF

  - Discuss the Options above

Issue 5-1-4: Intra-frequency measurements requirement

- Proposals

  - Option 1: The legacy NR intra-frequency measurements requirement can be reused for R18 ATG. (CMCC, HW, ZTE)

  - Option 2: The intra-frequency measurement requirement needs to be updated, such as no deactivated SCell measurement, no SCCs, PSCell measurement. (Ericsson)

- Recommended WF

  - Discuss the Options above

Issue 5-1-5: Inter-frequency measurements requirement

- Proposals

  - Option 1: The legacy NR inter-frequency measurements requirement can be reused for R18 ATG. (CMCC, HW, ZTE)

  - Option 2: RAN4 to study the trade-off between Inter-frequency measurement within MG and the data throughput due to large cell coverage. (Ericsson)

- Recommended WF

  - Discuss the Options above

Issue 5-1-6: CSI-RS based L3 measurements

- Proposals

  - Option 1: The legacy CSI-RS based L3 measurements requirement can be reused for R18 ATG. (CMCC)

  - Option 2: CSI-RS measurement is unnecessary in ATG network (Ericsson)

- Recommended WF

  - Discuss the Options above

Issue 5-1-7: L1-RSRP measurement for a cell with different PCI from serving cell

- Proposals

  - Option 1: Don’t consider L1-RSRP measurement for a cell with different PCI from serving cell in R18 ATG. (CMCC)

- Recommended WF

  - Don’t consider L1-RSRP measurement for a cell with different PCI from serving cell in R18 ATG

Issue 5-1-8: NR measurements with autonomous gaps

- Proposals

  - Option 1: Reuse the legacy delay requirement for NR measurements with autonomous gaps for R18 ATG. (CMCC)

- Recommended WF

  - Discuss the option.

Issue 5-1-9: positioning measurement

- Proposals

  - Option 1: We can not see obvious necessity of positioning measurement for ATG if UE based frequency pre-compensation is supported. (ZTE)

- Recommended WF

  - No need to consider positioning measurement for ATG.

Companies views’ collection for 1st round

Sub topic 5-1

| Company | Comments |
| --- | --- |
| CMCC | Issue 5-1-1: Measurement gap We agree with Option 3 that only FR1 MG will be used in R18 ATG. However, no explicit spec impaction will be introduced. Therefore, we think Option 1 could be a baseline. For Option 2, in our analysis, with ISD assumption 100-200km, the current MG and SMTC could cover propagation time difference between cells. We are open to further discuss if we miss anything. Issue 5-1-2: Measurement capability Both Option 1 and Option 2 is proposed by us, and we support it. With high capability CPE, we don’t see difficulty for UE to support TN UE measurement capability for intra-frequency and inter-frequency measurement. Issue 5-1-3: CSSF Option 1 and Option 2 are not conflict options. We support both of them. Issue 5-1-4: Intra-frequency measurements requirement Option 1 is preferred. We don’t see the necessity of the update in Option 2. The legacy requirement can also be applied for non-CA/DC scenario. Issue 5-1-5: Inter-frequency measurements requirement Option 1. Although data throughput loss will be observed when UE perform inter-frequency measurement, UE still have to perform the necessary measurement. Issue 5-1-6: CSI-RS based L3 measurements Option 1 is preferred. Issue 5-1-7: L1-RSRP measurement for a cell with different PCI from serving cell We support the recommended WF. Issue 5-1-8: NR measurements with autonomous gaps Option 1. We support to introduce this feature in ATG. It could help to save the cost of adding and optimizing neighbour cell relationship through ANR, which may be higher in ATG network. The legacy delay requirement can be reused for ATG. Issue 5-1-9: positioning measurement Support the recommended WF. |
| LGE | Issue 5-1-1: Measurement gap Generally support option 1. But we are open to discuss option 2 Issue 5-1-2: Measurement capability Option 1 Issue 5-1-3: CSSF Issue 5-1-4: Intra-frequency measurements requirement Option 1 Issue 5-1-5: Inter-frequency measurements requirement Option 1. Open to discuss option 2 Issue 5-1-6: CSI-RS based L3 measurements Issue 5-1-7: L1-RSRP measurement for a cell with different PCI from serving cell Issue 5-1-8: NR measurements with autonomous gaps Issue 5-1-9: positioning measurement Option 1. We think position can be obtained by GNSS capability. |
| Huawei | Issue 5-1-2: Measurement capability Fine with option 1and 2 Issue 5-1-3: CSSF We think current CSSF can apply with clarification of option 2. Issue 5-1-4: Intra-frequency measurements requirement Support option 1, the difference between option 1 and option 2 are only how to capture the requirements. Issue 5-1-5: Inter-frequency measurements requirement More clarification on option 2 is needed. |
| Ericsson | Issue 5-1-1: Measurement gap Option 3 For option 1, can we agree to say ‘legacy MG requirements as a baseline, and RAN4 can further discuss the possible changes if RAN4 identifies a valid issue for MG’? For option 2, we don’t see too much difference between ATG and NTN. In NTN, we don’t define any new type MG. We don’t think new MG is needed for ATG. Issue 5-1-2: Measurement capability Issue 5-1-3: CSSF We don’t see any contradiction among these options. We’re fine with all of them. Option 1,2,3,4: Fine Issue 5-1-4: Intra-frequency measurements requirement Both Option 1 and 2 are fine. Issue 5-1-5: Inter-frequency measurements requirement Option 2. From our understanding, inter-frequency measurement isn’t much important in ATG since UE will stay in a large cell for a long time while. Furthermore, the possible target cells are determined to network planning and dedicated flight path which can be known to UE in advance. RAN4 should further study how to improve the ATG UE throughput and not impact the ATG mobility. Issue 5-1-6: CSI-RS based L3 measurements Option 2. We don’t think CSI-RS measurement need to be considered in 1st release for ATG UE. Issue 5-1-7: L1-RSRP measurement for a cell with different PCI from serving cell Agree with the recommended WF. Issue 5-1-8: NR measurements with autonomous gaps Fine with option 1. Issue 5-1-9: positioning measurement We don’t think RAN4 has sufficient time unit to discuss and introduce positioning measurement requirements for ATG in Rel-18. Also, we are not sure whether it is part of the WID. |
| Apple | Issue 5-1-1: Measurement gap - We support option 2. But are open for further discussion and check whether the NTN scheme is applicable for ATG. Issue 5-1-2: Measurement capability - Option 1 and Option 2 could be a good starting point. Propose to further check after the scenario becomes clearer. Issue 5-1-3: CSSF Would like to clarify whether inter-frequency without GAP is in the scope or not? We can decide CSSF after we have conclusion on inter-frequency without GAP. Issue 5-1-4: Intra-frequency measurements requirement This need to be discussed later after we concluded 5-1-1/2/3. Issue 5-1-5: Inter-frequency measurements requirement This need to be discussed later after we concluded 5-1-1/2/3. |
| ZTE | Issue 5-1-1: Measurement gap We agree with Option 3 that only FR1 MG will be used in R18 ATG. Issue 5-1-2: Measurement capability Support both Option 1 and Option 2. Issue 5-1-3: CSSF We do not see any conflict between all of them, support all options. Issue 5-1-4: Intra-frequency measurements requirement Support Option 1. Issue 5-1-5: Inter-frequency measurements requirement Support Option 1. Issue 5-1-6: CSI-RS based L3 measurements We are open to both Option 1 and 2. Issue 5-1-7: L1-RSRP measurement for a cell with different PCI from serving cell Support the recommended WF. Issue 5-1-8: NR measurements with autonomous gaps Support Option 1. Issue 5-1-9: positioning measurement Support the recommended WF. |
| CATT | Issue 5-1-1: Measurement gap Support Option 1 and option 3. Issue 5-1-2: Measurement capability Support Option 1. Issue 5-1-3: CSSF Support Option 1. (can be reused legacy) Issue 5-1-4: Intra-frequency measurements requirement Support Option 1. Issue 5-1-5: Inter-frequency measurements requirement Support Option 1. Issue 5-1-6: CSI-RS based L3 measurements Support Option 1. Issue 5-1-7: L1-RSRP measurement for a cell with different PCI from serving cell Agree recommended WF. Issue 5-1-8: NR measurements with autonomous gaps Support option 1. Issue 5-1-9: positioning measurement Agree recommended WF. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

|  | Status summary |
| --- | --- |
| Issue 5-1-1 | Issue 5-1-1: Measurement gap Based on companies comments, moderator summarize the candidate Options as follows: Candidate options: - Option 1: Reuse the legacy MG requirements as baseline for R18 ATG. (CMCC, HW, CATT, Ericsson) - RAN4 can further discuss the possible changes if RAN4 identifies a valid issue for MG - Option 2: The measurement window and measurement GAP need to be redesigned based on the propagation time difference between serving gNB and neighbor gNB. (Apple, LGE) - Option 3: Only FR1 MG will be used in Rel-18 ATG network. (Ericsson, ZTE, CATT) Recommendations for 2nd round: Continue the discussion |
| Issue 5-1-2 | Issue 5-1-2: Measurement capability Tentative agreements: - Reuse the legacy measurement capability of Monitoring of multiple layers using gap for R18 ATG. - For measurement capability of maximum allowed layers for multiple monitoring, reuse the current legacy intra and inter frequency measurement capability as the baseline. the total number of effective carrier frequency layers that UE shall be capable of monitoring update to 8. - The agreements can be further checked after the scenario becomes clearer in RF session Recommendations for 2nd round: Please provide the comment if you disagree with the tentative agreement. |
| Issue 5-1-3 | Issue 5-1-3: CSSF Based on comments, following Candidate options are summarized. Candidate options: - Option1: For intra-frequency measurement without measurement gap, CSSFintra = 1 for ATG. - Option 2: FFS, further consider whether inter-frequency without GAP is in the scope or not (Apple) Recommendations for 2nd round: Continue the discussion. |
| Issue 5-1-4 | Issue 5-1-4: Intra-frequency measurements requirement Candidate options: - Option 1: The legacy NR intra-frequency measurements requirement can be reused for R18 ATG. (CMCC, HW, ZTE, LGE, Ericsson) - Option 2: This need to be discussed later after we concluded 5-1-1/2/3. (Apple) Recommendations for 2nd round: Continue the discussion. |
| Issue 5-1-5 | Issue 5-1-5: Inter-frequency measurements requirement Candidate options: - Option 1: The legacy NR inter-frequency measurements requirement can be reused for R18 ATG. (CMCC, HW, ZTE, CATT) - Option 2: RAN4 to study the trade-off between Inter-frequency measurement within MG and the data throughput due to large cell coverage. (Ericsson, LGE) - Option 3: This need to be discussed later after we concluded 5-1-1/2/3. (Apple) Recommendations for 2nd round: Continue the discussion. |
| Issue 5-1-6 | Issue 5-1-6: CSI-RS based L3 measurements Candidate options: - Option 1: The legacy CSI-RS based L3 measurements requirement can be reused for R18 ATG. (CMCC, CATT) - Option 2: CSI-RS measurement is unnecessary in ATG network (Ericsson) Recommendations for 2nd round: Continue the discussion. |
| Issue 5-1-7 | Issue 5-1-7: L1-RSRP measurement for a cell with different PCI from serving cell Tentative agreements: - Don’t consider L1-RSRP measurement for a cell with different PCI from serving cell in R18 ATG Recommendations for 2nd round: No more discussion. |
| Issue 5-1-8 | Issue 5-1-8: NR measurements with autonomous gaps Tentative agreements: - Reuse the legacy delay requirement for NR measurements with autonomous gaps for R18 ATG. Recommendations for 2nd round: No more discussion. |
| Issue 5-1-9 | Issue 5-1-9: positioning measurement Tentative agreements: - No need to consider positioning measurement for ATG. Recommendations for 2nd round: No more discussion. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Topic #6: Specifiction documentaion

Main technical topic overview. The structure can be done based on sub-agenda basis.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2215505 | CMCC | Proposal 36: Add ATG related requirements in the current corresponding section, similar as HST |
| R4-2216276 | HW | Proposal 10: Add ATG specific requirements in the current corresponding section, similar as HST. |

Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

Sub-topic 6-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 6-1-1: How to involve ATG RRM core requirements in TS38.133

- Proposals

  - Option 1: Add ATG related requirements in the current corresponding section, similar as HST (CMCC, HW)

- Recommended WF

  - Discuss the option.

Companies views’ collection for 1st round

Open issues

Sub topic 6-1

| Company | Comments |
| --- | --- |
| CMCC | Issue 6-1-1: How to involve ATG RRM core requirements in TS38.133 We prefer Option 1, because we only see several updates for ATG at the current stage. |
| Huawei | Support option 1 as most requirements are same. |
| Ericsson | We think issue 6-1-1 can be postponed until RAN4 has made more progress in identifying the RRM impact. Therefore we disagree to option 1. |
| Qualcomm | We agree with Ericsson that this could be postponed to be discussed at a later stage. |
| ZTE | Support Option 1 |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

|  | Status summary |
| --- | --- |
| Issue 6-1-1 | Issue 6-1-1: How to involve ATG RRM core requirements in TS38.133 Tentative agreements: - Postpone this issue until RAN4 has made more progress in identifying the RRM impact. Recommendations for 2nd round: No more discussion in this meeting. |

Discussion on 2nd round (if applicable)

Moderator can provide summary of 2nd round here. Note that recommended decisions on tdocs should be provided in the section titled ”Recommendations for Tdocs”.

Recommendations for Tdocs

1st round

New tdocs

| New Tdoc number | Title | Source | Comments |
| --- | --- | --- | --- |
|  | WF on NR ATG RRM core requirements | CMCC | To capture agreements |
|  | LS on … | ZZZ | To: RAN_X; Cc: RAN_Y |

Existing tdocs

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics incl. existing and new tdocs.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- For new LS documents, please include information on To/Cc WGs in the comments column

- Do not include hyper-links in the documents

2nd round

| Tdoc number | Revised to | Title | Source | Recommendation | Comments |
| --- | --- | --- | --- | --- | --- |
| R4-22xxxxx |  | CR on … | XXX | Agreeable, Revised, Merged, Postponed, Not Pursued |  |
| R4-22xxxxx |  | WF on … | YYY | Agreeable, Revised, Noted |  |
| R4-22xxxxx |  | LS on … | ZZZ | Agreeable, Revised, Noted |  |

Notes:

- Please include the summary of recommendations for all tdocs across all sub-topics.

- For the Recommendation column please include one of the following:

  - CRs/TPs: Agreeable, Revised, Merged, Postponed, Not Pursued

  - Other documents: Agreeable, Revised, Noted

- Do not include hyper-links in the documents