3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217098

Electronic Meeting, 10 October – 19 October 2022

Source:	Skyworks Solutions, Inc., Huawei, HiSilicon

Title:	TP for TR 37.718-11-21 to update DC_(n)3-n8

Agenda item:			5.1.1 UE RF Requirements

Document for:	Approval

# 1. Introduction

This contribution updates the MSD test point of previous text proposal [1] on DC band combination of LTE band B3 with NR bands n3 and n8 for TR 37.718-11-21[2] based on WF [3].

# 2. References

[1]		R4-2215089, “TP for TR 37.718-11-21: DC_(n)3AA-n8A”,  Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #104-e.

[2]		Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)

[3]		R4-2217097, “Way forward on MSD to Scell for intra-band contiguous with IMD overlap”, Skyworks Solutions, Inc., Murata Manufacturing Co Ltd., 3GPP TSG-RAN WG4 Meeting #104-bis-e.

# 3. Text Proposal

---Start of changes---

## 6.x	DC_(n)3-n8

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3AA-n8A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configurations | E-UTRA and NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth For DL [MHz] |
| DC_(n)3AA-n8A | DC_(n)3AA1 DC_3A_n8A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE1: Only single switched UL is supported. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

The studies are already performed for DC_(n)3AA and DC_3A_n8A, in TR 37.717-11-11 and TR 37.716-11-11, respectively.

### 6.x.4	∆TIB and ∆RIB values

For DC_(n)3-n8, the TIB,c and RIB,c values are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_(n)3-n8 | 3 | 0.3 |
|  | n8 | 0.3 |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_(n)3-n8 | 3 | 0 |
|  | n8 | 0 |

### 6.x.5	MSD

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_(n)3AA-n8A | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | IMD5 |
|  | n3 | N/A | 5 | N/A | 1835 | 4.5 | IMD5 |
|  | n8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |

----- End of TP -----