3GPP TSG-RAN WG4 Meeting # 104bis-e   										  			R4-2217106

Electronic Meeting, October 10 – October 19, 2022

Source:	CMCC

Title:	TP for 38.718-02-01 CA_n3A-n41C with UL_n41C

Agenda item:	5.10.2

Document for:	Approval

# 1 Background

In last meeting, a TP CA_n3A-n41C with UL_n41C was proposed but finally postponed. This contribution provides updated text proposal on the NR CA band combination CA_n3A-n41C with UL_n41C with different MSD value compared with in [1].

# 2 Text Proposal

##### ---Start of changes---

## 6.X	CA_n3-n41

### 6.X.1	Common for 1 band UL and 2 bands UL CA

#### 6.X.1.1 Operating bands for CA

Table 6.X.1.1-1: CA band combination CA_n3-n41

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n3-n41 | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

#### 6.X.1.2	Channel bandwidths per operating band for CA

Table 6.X.1.2-1: Supported bandwidths per CA band combination CA_n3-n41

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41C | CA_n41C | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | CA_n41C_BCS0 |  |

#### 6.X.1.3	UE Co-existence studies

The harmonic / harmonic mixing issue have been analyzed in previous release.

IMD order for contiguous UL CA as below.

WGap = lowest band edge of n41 - highest DL band edge of n3 = 2496 – 1880 = 616 MHz.

Min(maxUL aggregated BW, UL band bandwidth) = Min(190, 194) = 190 MHz.

IMD order = 2*ceil(WGap /Min(maxUL aggregated BW, UL band bandwidth) + 1 = 9.

If full n41C bandwidth is considered (190MHz aggregated BW), band n3 is subject to IMD9. But since band n3 is not present in regions where n41 uses the full bandwidth, at current stage we don’t need to consider this IMD9 issue.

If 160MHz aggregated bandwidth is considered for the IMD calculation. lowest IMD9 is centred around 1884.2MHz which is just missing the top of n3. Then there is no MSD using the n41 spectrum with aggregated 160MHz BW, e.g. in  China (also valid for Japan). A test point with N/A MSD is as follows with a note on n41 spectrum

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n3-n41 | n3 | N/A | 5 | N/A | 1877.5 | N/AX | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 RBSTART= 0 | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 RBSTART= 272 | 2625 |  |  |  |
| NOTE X: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |

#### 6.X.1.4	∆TIB and ∆RIB values

The TIB,c and RIB have been specified in current spec.

#### 6.X.1.5	REFSENs requirements

#### As analysied before, the MSD is as below.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc | UL/DL BW | UL | DL Fc (MHz) | MSD | Duplex mode |  |
|  |  | (MHz) | (MHz) | CLRB |  | (dB) |  |  |
| CA_n3-n41 | n3 | N/A | 5 | N/A | 1877.5 | N/AX | FDD | IMD9 |
|  | n41 | 2545 | 60 | 1 RBSTART= 0 | 2545 | N/A | TDD | N/A |
|  |  | 2625 | 100 | 1 RBSTART= 272 | 2625 |  |  |  |
| NOTE X: Applicable when n41 spectrum is restricted to 2515-2675MHz |  |  |  |  |  |  |  |  |

#### 6.X.1.6	OOB blocking exception requirements

There is no OOB blocking exception requirements

##### ---End of changes---

# Reference

[1]	R4-2213126, TP for 38.718-02-01 CA_n3A-n41C with UL_n41C, Huawei, HiSilicon