3GPP TSG-RAN WG4 Meeting # 104b-e 										R4-2217203

Electronic Meeting, 10th-19th, Oct., 2022

Title:	WF on FeMIMO RRM impact for unified TCI state

Agenda item:			4.5.1

Source:	Intel

Document for:	Approval

# Way Forward on Unified TCI state

### Sub-topic 1-1 Active UL TCI state

Issue1-1-1a If source RS in UL TCI state is in the DL active TCI list:

- Agreements:

  - No additional time/frequency tracking for the source RS in UL TCI state is needed during TCI state switch

Issue1-1-1b If source RS in UL TCI state is not in the DL active TCI list:

- Proposals:

  - Proposal 1: No additional time/frequency tracking is needed

  - Proposal 2: Additional time/frequency tracking is needed

  - Proposal 3: No requirement for the case. Adding applicability rules for current UL TCI switching when source RS in active UL TCI state is a subset of source RS in DL active TCI list

  - Proposal 4: Check with RAN1

### Sub-topic 1-2 MAC CE based TCI state Switching delay requirements

Issue 1-2-1 Joint TCI switching delay requirement for DL TCI state switch

- Proposals:

  - Proposal 1(Intel, MTK, vivo, Apple, Samsung,vivo, Qualcomm, ZTE):

    - Remove the square bracket:

-   In case of joint TCI state switch, UE is not expected to receive on DL before UE completes the DL and UL TCI state switch.

  - Proposal 2(Nokia):

    - For joint TCI state switch, if the UL TCI state switch delay exceeds the DL TCI state switch delay, the UE is required to receive in DL up to THARQ before it completes UL TCI state switch.

Issue 1-2-2 MAC-CE based UL TCI state switching delay when SSB is indicated as PL-RS in UL TCI state for FR2

- Proposals

  - Proposal 1(Apple, Samsung, Huawei):

    - When PL-RS in UL TCI state switch is SSB in FR2, longer delay is expected.

  - Proposal 2(Huawei, Apple, Samsung):

    - If no consensus can be achieved in RAN4, we suggest that there is no requirements when SSB is indicated as PL-RS in UL TCI state in FR2.

  - Proposal 3(Intel):

    - When SSB is indicated as PL-RS in UL TCI state for FR2, the total delay is:

-    n+THARQ + 3ms + NM* (Tfirst_target-PL-RS + 7*Ttarget_PL-RS + 2ms)

  - Proposal 4(MTK, vivo, Ericsson, ZTE, Qualcomm):

    - Reuse the existing delay requirement of MAC CE based UL TCI state switch.

  - Proposal 5(Nokia):

    - known conditions:

    - The UE shall be able to transmit uplink signal with the target TCI state in the slot n+THARQ + 3Nslotsubframe,µ + NM* (1*Ttarget_PL-RS + Tprocessingms) / NR slot length.

Where:

-	NM = 1, if the target PL-RS is not maintained by the UE, 0 otherwise.

-	PL-RS is considered maintained if the DL RS associated with the UL TCI state is in the active TCI state list.

### Sub-topic 1-3 Common TCI state switching in CA case

Issue 1-3-1 Common TCI state switching delay requirement

- Agreements:

  - No further spec change is needed

### Sub-topic 1-4 TCI state list update delay

Issue 1-4-1 Whether to consider unknown TCI state in the TCI state list

- Proposals

  - Proposal 1(Samsung, Apple, MTK, Huawei,vivo):

    - Longer delay applies if any TCI state is unknown in TCI state list update. Active TCI state list can contain known and unknown TCI states.

  - Proposal 2(Ericsson, Nokia, ZTE,vivo):

    - Define the detailed delay requirement