3GPP TSG-RAN WG4 Meeting #104bis-e	R4-2217204

Electronic Meeting, October 10 – 19, 2022

Title: 	WF on FeMIMO RRM requirements for inter-cell beam management

Source: 	Huawei, HiSilicon

Agenda item:	4.5.1

Document for:	Approval

Introduction

This contribution is to capture the agreements for the email discussion for Rel-17 FeMIMO RRM in RAN4 #104bis-e meeting.

Way-forward

Sub-topic 2-1: Sharing factor

Issue 2-1-1 Sharing factor design

- Proposals:

  - Proposal 1(Intel, Huawei, vivo, Apple):

    - Remove the bracket in the corresponding CR.

  - Proposal 2(Ericsson, MTK):

    - RAN4 to agree following sharing factor for CDP

- For FR1:

  - PCDP= Ntotal_CDP / Noutside_MG_CDP

- For FR2:

  - if Navailable,SSB_CDP_SMTC_MG = 0,

    - If measurement occasions of SSB CDP is also used for L3 measurements which are measured outside gap, then PCDP = Psharing SMTC * Psharing SSB * Ntotal_CDP / Noutside_MG_CDP

    - Else, PCDP = Psharing SSB * Ntotal_CDP / Noutside_MG_CDP

    - Where, Psharing SSB = N, where N is the number overlapping SSB from different cells.

  - If Navailable,SSB_CDP_SMTC_MG ≠ 0

    - PCDP = Psharing SSB * Ntotal / Navailable,SSB_CDP_SMTC_MG

    - RAN4 to agree following sharing factor for SC

- For FR1:

  - PSC= Ntotal_SC / Noutside_MG_SC

- For FR2:

  - if Navailable,SSB_SC_SMTC_MG = 0,

    - If measurement occasions of SSB CDP is also used for L3 measurements which are measured outside gap, then PSC = Psharing SMTC * Psharing SSB * Ntotal_SC / Noutside_MG_SC

    - Else, PSC = Psharing SSB * Ntotal_SC / Noutside_MG_SC

    - Where, Psharing SSB = N, where N is the number overlapping SSB from different cells.

  - If Navailable,SSB_SC_SMTC_MG ≠ 0

    - PSC = Psharing SSB * Ntotal_SC / Navailable,SSB_SC_SMTC_MG

Sub-topic 2-2: Scheduling Restriction

Issue 2-2-1 Scheduling restriction for dynamic TDD

- Proposals:

  - Proposal 1(Apple, Ericsson):

    - RAN4 need not discuss the scheduling restriction for dynamic TDD as its already captured in RAN1 specification.

  - Proposal 2(vivo):

    - Do not introduce scheduling restriction for dynamic TDD when L1-RSRP measurement on cell with different PCI overlaps with serving cell UL slots. Clarify longer L1 measurement delay is expected for this case.

  - Proposal 3(MTK, ZTE, Samsung):

    - Introduce scheduling restriction for dynamic TDD on serving cell UL symbols which fully or partially (because of TA) overlaps with the SSB for L1-RSRP measurement on cell with different PCI.

  - Proposal 3a(Samsung, ZTE):

    - Introduce scheduling restriction for dynamic TDD when L1-RSRP measurement on the cell with different PCI. It is enough to add the scheduling restriction on 1 symbol before SSB and one symbol after SSB.

  - Proposal 3b(ZTE,Samsung):

    - For the scheduling restriction due to L1-RSRP measurement on cell with different PCI, reusing the scheduling restriction due to L1-RSRP measurement on serving cell is fine. Whether the adjacent symbol before and after SSB should be restricted, which should be aligned with the specification for L1-RSRP measurement on serving cell.

Issue 2-2-2: Whether to define scheduling restriction for non-serving cell

- Agreements

  - Not to define scheduling restriction for non-serving cell

Sub-topic 2-3: Applicability of ICBM feature

Issue 2-3-1 Applicability of ICBM feature

- Proposals:

  - Proposal 1(Apple, MTK, Ericsson, Samusng, Intel, Huawei):

    - RAN4 not extend ICBM requirements for concurrent R17 Wis in Release 17. It can be postponed to further release.

  - Proposal 2(vivo, CMCC):

    - Confirm that R17 requirements for inter-cell L1 measurements can be applicable to FR1 HST. The square brackets related to FR1 HST should be removed.

    - Confirm that R17 requirements for inter-cell L1 measurements can be applicable to FR2 HST, with the assumption that only one active UE panel is used.

    - Clarify in TS 38.133 that there is no R17 requirements when inter-cell L1 measurements and R17 enhance gap related features are configured simultaneously to one UE.

Sub-topic 2-4: SSB and PDCCH/PDSCH are overlapped on the same RE

Issue 2-4-1: Whether any clarification or update is needed in RAN4 spec when SSB and PDCCH/PDSCH are overlapped on the same RE

- Agreements

  - Whether to define the requirement of overlap between SSB and PDCCH/PDSCH in the same RE should wait for RAN1 conclusion.

Sub-topic 2-5: Measurement restriction

Issue 2-5-1 Measurement restriction for SSB based L1-RSRP

- Proposals:

  - Proposal 1(Huawei):

    - The measurement restrictions are applied between SC SSB for RLM/BFD/CBD and CDP SSB for L1-RSRP.

    - The measurement restrictions are applied between CDP SSB for BFD/CBD and SC SSB for L1-RSRP.

  - Proposal 2(Ericsson, Intel):

    - Further study the possibility of sharing under some scenarios

Sub-topic 2-6: Applicability of FR 2-2

Issue 2-6-1 Applicability of FR 2-2

- Agreements

  - FR 2-2 is not applicable to R17 inter cell beam management.