3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217246

Electronic Meeting, 10th ‒ 19th October, 2022

Title: 	WF on RRM impacts and general aspects for multi-Rx

Agenda Item:	6.8.3

Source: 	vivo

Document for:	Approval

- Topic #1: Analysis of RRM imapct

  - Sub-topic 1-1: Scope and scenarios

<Agreement >:

- Joint combination of multi-Rx chain with NR-U, RedCap and NTN are not in the scope of the WI.

<Agreement >:

- RRM requirement discussion shall be focused on the case with different QCL TypeD RSs on a single component carrier.

- FFS whether UE can be configured with multiple component carriers, including intra-band CCs and/or inter-band CCs, but multi-Rx chain is enabled on only one of the component carriers.

<Agreement >:

- Clarify the SSB and CSI-RS based simultaneous L1 measurement scenario before making conclusion on scenarios for Rel-18 multi-Rx DL reception, i.e., for simultaneous reception of different QCL type D RSs, what RSs combination(s) can be considered and which scenario(s) it is aimed at.

<Way forward >:

Issue 1-1-1: Scope of the WI

- Proposals

  - Option 1 (NTT DOCOMO, LGE, CMCC, Nokia, ZTE, vivo, Huawei): RRM discussion have not to be related to 4-layer MIMO directly, i.e., RRM enhancement thanks to multi-Rx chain which is not relevant to 4-layer MIMO should be discussed.

  - Option 2 (Qualcomm): The WID scope on RRM is focused on 4-layer MIMO only.

  - Option 3: It is not in the scope of the WI to define panel or RX chain specific behaviours with RX panel control signal for DL.

Issue 1-1-3: Scenarios for Rel-18 multi-Rx DL reception

- Proposals

  - Option 1 (vivo, Nokia, ZTE, CMCC, Xiaomi, LGE): Both intra-cell and inter-cell multi-TRP operation are supported for multi-Rx chain UE in the WI

  - Option 2 (Samsung, vivo, NTT DOCOMO, Huawei): Working on inter-cell operation with TRPs located within reasonable intercell distance after intra-cell multi-TRP operation work is completed.

  - Option 3 (MTK, Qualcomm, OPPO): Not to consider inter-cell mTRP operation in R18 multi-Rx UE

Issue 1-1-4: Support of single-DCI and/or multi-DCI multi-TRP operation

- Proposals

  - Option 1 (vivo, LGE, Xiaomi, Samsung, CMCC, Nokia, ZTE, Ericsson, NTT DOCOMO): Define RRM requirements depending on UE behaviour for single-DCI and multi-DCI multi-TRP operation

    - Option 1a (vivo, Xiaomi, Nokia): Multi-DCI multi-TRP operation should not be precluded from RRM requirements perspective.

    - Option 1b (LGE, Nokia): Consider RRM requirements such as scheduling restrictions and interruption for single DCI and multi-DCI based multi-TRP operations.

    - Option 1d (Samsung, NTT DOCOMO): For RRM requirements specified for UE supporting simultaneous DL reception from different directions, both UE behaviour for single-DCI and multi-DCI multi-TRP operation, while single-DCI is adopted as a basline to be studied first.

  - Option 2 (OPPO, Huawei, Samsung): Whether to down-select some scenarios of multi-TRP can be further discussed after general assumption with respect to TRP are clarified

  - Option 3 (Qualcomm, MTK): Define RRM requirements for single-DCI multi-TRP operation only.

Issue 1-1-5: Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel

- Proposals

  - Option 1 (Samsung, Xiaomi, NTT DOCOMO, MTK, Intel, Huawei): Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to achieve two independent signals from the same or nearly the same direction is not the scope of this work item.

  - Option 2 (vivo, Qualcomm, LGE, Nokia, Ericsson): How Spatial MIMO (either spatial diversity or spatial multiplexing) is used is up to NW and UE implementation. It is not necessary to explicitly preclude spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to achieve two independent signals from the same or nearly the same direction from the WI

  - Option 3 (Apple, OPPO, Nokia, ZTE, Samsung, NTT DOCOMO, Huawei, Intel, LGE, Ericsson): Spatial MIMO (either spatial diversity or spatial multiplexing) by using one panel to receive two independent signals from the same or nearly the same direction is up to RF conclusion.

Issue 1-1-6: Simultaneous L3 measurements and L1 measurements

- Proposals

  - Option 1 (MTK, Huawei, Intel, NTT DOCOMO, Qualcomm, Ericsson): In R18 multi-Rx, UE is not required to perform both L3 measurements and L1 measurements at a time.

  - Option 2 (vivo, OPPO, Nokia, ZTE, Xiaomi, Apple): RAN4 to identify use cases for simultaneous L3 measurements and L1 measurements and study the feasibility

  - Option 3 (CMCC, Nokia, ZTE): To consider simultaneous L3 measurements and L1 measurements with multi-RX chain reception

    - Further check whether Klayer1_measurement of 1.5 can be removed (or Klayer1_measurement = 1)

  - Sub-topic 1-2: General aspects

<Agreement >:

- UE behaviour and capability of handling Rx signal level difference between two channels may be discussed in RF/demodulation session.

- Enhanced RRM requirements for multi-Rx chain UE should maintain the existing accuracy requirements.

<Agreement >:

- Whether to define RRM requirements for activation delay from a single antenna panel to multi-antenna panels should be decided after RF conclusion.

<Way forward >:

Issue 1-2-1: Definition of “simultaneous reception”

- Proposals

  - Option 1 (Ericsson): In this WI, the two RSs are considered to be received simultaneously, if their instances are received in the same or overlapping OFDM symbols, which may occur in one, some, or all RS occasions during the measurement period.

  - Option 2 (Nokia): In this WI, the two RSs, RS and data, or 2 data signals, are considered to be received simultaneously, if their instances are received in the same or overlapping OFDM symbols, which may occur in one, some, or all RS occasions during the measurement period.

Issue 1-2-2: Scenarios for “simultaneous reception”

- Proposals

  - Option 1 (ZTE): Under the assumption of non-overlapped or very limited partial overlapped between the beam direction coverage of each panel, all following candidate combinations are possible:

    - Simultaneous fine beam reception from panel A and fine beam reception from panel B

    - Simultaneous coarse beam reception from panel A and coarse beam reception from panel B

    - Simultaneous fine beam reception from panel A and coarse beam reception from panel B

    - Simultaneous coarse beam reception from panel A and fine beam reception from panel B

  - Option 2 (Ericsson): RAN4 agrees on the deployment scenarios A-D for discussing the requirements for simultaneous reception from two different directions:

    - Scenario A: simultaneous reception of RS0 and RS2, same TRP, same SSB index, same cell

    - Scenario B: simultaneous reception of RS0 and RS10, different TRPs, same SSB index, same cell

    - Scenario C: simultaneous reception of RS0 and RS3, same TRP, different SSBs, same cell

    - Scenario D: simultaneous reception of RS0 and RS13, different TRPs, different SSBs, same cell.

Issue 1-2-4: UE architectures

- Proposals

  - Option 2 (Nokia):

    - Multi Rx architecture with above consideration, to assume that each Rx chain will need to process at an independent FFT window.

    - Multi Rx architecture to assume that each Rx chain can perform independent RRM measurements or demodulation tasks.

    - Multi Rx architecture to assume that each Rx chain can perform independent RRM measurements on 2 Rx chains.

    - Independent timing loops are supported in multi-RX for intra and inter cell mobility measurements.

  - Option 4 (vivo, MTK, Ericsson, Qualcomm): It is not necessary to have a general conclusion on multi-Rx chain architecture.

  - Option 5 (NTT DOCOMO, Samsung, Xiaomi, Ericsson, ZTE, LGE, Apple, Huawei, MTK, OPPO): Should be discussed in RF session.

Issue 1-2-6: Beam management

- Proposals

  - Option 1 (vivo): Principle of defining beam management related requirements for IBM can be extended for multi-Rx chain

  - Option 2 (Nokia): Independent beam management between multiple Rx chains on the same carrier is assumed in this work item.

  - Option 3: Further clarify which beam management requirements are used for multi-RX WI.

  - Note: Company is encouraged to check if this issue is covered by other issues already.

Issue 1-2-8: RRM impact of the UE behaviour using a single antennal panel

- Proposals

  - Option 1 (LGE): Consider UE measurement behaviour for L1 or L3 related measurement when UE receive the DL signals from multiple TRP using a single antenna panel.

  - Option 2 (NTT DOCOMO, Samsung, Intel, OPPO, vivo): Needs RF study conclusion

  - Option 3 (Nokia, LGE): L1 and L3 measurement delays and measuring/scheduling restrictions for measurements need to be considered when using single antenna panel on multi Rx capable UEs.

  - Option 4 (Qualcomm, Ericsson, Huawei, Xiaomi, MTK, ZTE): No need to discuss

Issue 1-2-9: Whether and how to define power saving related requirements

- Proposals

  - Option 1 (vivo, Apple, LGE, Ericsson, NTT DOCOMO, Huawei, Samsung, Xiaomi, ZTE): No power saving specific requirements are considered in the WI.

    - Power saving can be one aspect to be considered when specifying RRM requirements, e.g., if a UE always needs to keep two panels active.

  - Option 3 (Apple, MTK, LGE, ZTE, LGE): RAN4 should discuss if a UE capable of multi-RX reception should inform the network that it does not support two AoA reception, so the network knows the UE does not turn on or off this capability arbitrarily.

  - Option 3a (Nokia, Apple): RAN4 should discuss how a UE capable of multi-RX reception should inform the network when switching from 2 Rx chains to 1 Rx chain, and vice versa, so the network knows the UE does not turn on or off this capability arbitrarily.

  - Option 5 (Nokia): RAN4 to study if use of single Rx chain or multiple Rx chain can be adapted over time.

Issue 1-2-11: Scheduling restriction/availability requirements on group-based L1 measurements

- Proposals

  - Option 1 (Qualcomm): RAN4 to update scheduling restriction/availability requirements on group-based L1 measurements taking into account the following aspects:

    - For the case where RS for group based L1-RSRP measurement is CSI-RS which is QCLed with active TCI state for PDCCH/PDSCH and not in a CSI-RS resource set with repetition ON, and N=1,

      - If UE Rx beam for the measurement cannot achieve the same directivity gain as the gain that can be obtained for non-group based measurement,

        - There is a scheduling restriction due to (L1-RSRP) measurement performed based on the CSI-RS if the measurement resource is configured for group-based measurements from multiple TRPs.

      - Otherwise,

        - There is no scheduling restriction due to (L1-RSRP) measurement performed based on the CSI-RS from the other TRP if the measurement resource is configured for group-based measurements from multiple TRPs.

      - FFS on whether and how to signal a necessity of scheduling restriction.

  - Sub-topic 1-3: Receive timing difference

<Agreement >:

- Not to discuss UL transmission in R18 FR2 multi-Rx WI.

<Way forward >:

Issue 1-3-1: Receive timing difference

- Proposals

  - Option 1 (Apple, Xiaomi, Qualcomm, Intel, MTK, LGE, OPPO, Ericsson): The receive timing difference between different directions is within CP in R18.

    - Option 1b (Qualcomm, Apple): Receive time difference for configured different QCL Type D RSs is not larger than CP. FFS whether and how much additional margin within the CP length is needed.

  - Option 2 (vivo, Huawei, Samsung, CMCC, ZTE): The receive timing difference between different directions is within CP at least. FFS whether to define requirements with timing difference larger than CP.

  - Option 3 (Nokia): Consider receive time differences larger than CP.

    - The multi-RX UE can support independent time and frequency tracking for each Rx chain.

  - Option 4 (Tentative agreement in GTW): The receive timing difference considered for data reception and L1 measurements between different directions (2AOAs) is within CP

    - Receive time differences between any configured different QCL Type D RSs is not larger than CP.

    - FFS whether to define requirements with timing difference larger than CP.

  - Sub-topic 1-4: Applicability and conditions

<Agreement >:

- RRM requirement of simultaneous DL reception from different directions shall be defined based on the applicable condition to be specified in UE RF session.

  - FFS the relation between the applicability condition and RRM requirement impacts

<Agreement >:

- To clarify the requirement in new section that R18 multi-Rx is applicable for FR2-1 only, i.e., no FR2-2.

<Way forward >:

Issue 1-4-2: Applicability of new requirements to different QCL types

- Proposals

  - Option 1 (vivo, MTK, Apple, OPPO): FFS whether requirements defined for QCL type-D only are also applicable when QCL type D is configured together with QCL type A/C

  - Option 2 (Samsung): Whether or not the new RRM requirement for the feature of simultaneous DL reception from different directions shall be applicable is irrelevant to the TCI configuration of other QCL type.

  - Option 3 (Ericsson, ZTE, Samsung): In addition to QCL type D, the two RSs may also be configured with QCL type A or QCL type C.

Issue 1-4-3: Applicability of requirements for serving cell

- Proposals

  - Option 1 (Ericsson, Nokia, vivo): The simultaneously received RSs may also be from a non-serving cell

  - Option 2 (NTT DOCOMO): Down-prioritized.

Issue 1-4-5: Detectable condition of RS signals

- Proposals

  - Option 1 (MTK): For detectable condition, all RSs in the same QCL chain for the target TCI state should remain detectable during the entire measurement/switch period.

  - Sub-topic 1-5: UE capabilities

<Way forward >:

Issue 1-5-1: Clarification/understanding on R16 UE capabilitiy simultaneousReceptionDiffTypeD

- Proposals

  - Option 1 (vivo, LGE, Nokia): RAN4 to clarify the usage of existing UE capability simultaneousReceptionDiffTypeD and further discuss if additional UE capability is needed depending on progress of RRM requirements.

  - Option 2a (Samsung):

    - RAN4 shall not introduce new, but reuse Rel-16 UE capability IE simultaneousReceptionDiffTypeD-r16, to indicate enhanced FR2-1 UEs supporting simultaneous DL reception from different directions with different QCL TypeD RSs on a single component carrier.

    - If UE support Rel-16 simultaneousReceptionDiffTypeD-r16, UE supports to perform simultaneous measurement based on SSBs with different QCL TypeD indications.

  - Option 3 (OPPO, ZTE): New UE capability of supporting simultaneous reception from different directions with different QCL type D RSs in R18 is preferred.

  - Option 4 (Qualcomm, Apple, MTK, Ericsson, Intel): Rel-16 UE capability simultaneousReceptionDiffTypeD-r16 is applicable only for PDSCH reception.

Issue 1-5-2: UE capability of simultaneous reception of measured RS and data

- Proposals

  - Option 1 (Huawei): For R18 multi-Rx DL reception, the enhanced RRM requirements can be developed based on the following principles:

    - UE can be assumed to support simultaneous data receptions with two different beam directions.

    - UE does not support simultaneous data receptions and L1 measurements with different beam directions.

    - UE does not support simultaneous data receptions and L3 measurements.

  - Option 2 (Samsung, LGE, ZTE): For UE capability of simultaneous reception of measured RS and data, if UE support simultaneousReceptionDiffTypeD-r16, it should support:

    - simultaneous reception of L1 measured RS and PDCCH/PDSCH/TRS/CSI-RS for CQI

    - simultaneous reception of L3 measured RS and PDCCH/PDSCH/TRS/CSI-RS for CQI

  - Option 3 (MTK, Qualcomm, vivo, Huawei): RAN4 to further study how to guarantee that network can know when to apply schedule restriction or when not to.

  - Option 4 (OPPO): UE capability of simultaneous reception of L1/L3 RS and data can be decided after the feasibility issues of L1 and L3 enhancements are concluded.

  - Option 5 (Nokia, Xiaomi): For R18 multi-Rx DL reception, the enhanced RRM requirements can be developed based on the following principles:

    - UE can be assumed to support simultaneous data receptions with two different beam directions.

    - UE supports simultaneous data receptions and L1 measurements with different beam directions.

    - UE supports simultaneous data receptions and L3 measurements with different beam directions.

    - UE supports simultaneous L1 and L3 measurements with different beam directions.

  - Option 6 (Ericsson): for requirements definition, RAN4 to consider

    - UE supports simultaneous data receptions and L1 measurements with different beam directions.

    - UE supports simultaneous L1 measurements with different beam directions.

Issue 1-5-3: UE capability of simultaneousRxDataSSB-DiffNumerology

- Proposals

  - Option 1 (MTK, Xiaomi, ZTE, Huawei, Xiaomi): The existing simultaneousRxDataSSB-DiffNumerology IE can be re-used in R18 multi-panel WI

  - Sub-topic 1-6: RRM Testability

<Way forward >:

Issue 1-6-1: AoA setup for multi-Rx chain

- Proposals

  - Option 1 (Samsung, R&S): RAN4 RRM session use the test parameters required given in 6.2.1.4.1 of TR 38.810 as the starting point to discussion on 2AoA setup needed for RRM performance requirement for UE supporting simultaneous DL reception from from different directions with different QCL TypeD RSs:

  - Note: The issue will be further discussed when there is TU for perf part of the WI.

- Topic #2: L3 measurement related

<Way forward >:

Issue 2-1-1: Feasibility/necessity of enhancing requirements for L3 measurements

- Proposals

  - Option 1 (MTK, Intel, Huawei, Qualcomm, OPPO, Ericsson, Samsung, NTT DOCOMO): No L3 measurement requirements enhancement in R18 multi-Rx chain WI.

    - Option 1a (MTK, Intel): Not to enhance L3 measurement requirement in R18 multi-Rx chain WI.

    - Option 1b (Huawei): For R18 multi-Rx DL reception, it is suggested not to enhance L3 measurement requirements.

      - For R18 multi-Rx DL reception, UE is not assumed to support simultaneous L3 measurements with two different beam directions.

      - In R18, it is suggested not to enhance the beam sweeping factor for L3 measurement requirements due to searcher limitation.

      - For R18 multi-Rx DL reception, UE is not assumed to support simultaneous L1 measurements and L3 measurements and the sharing factor between L1 and L3 needs to be kept in L3 measurement requirements.

      - For R18 multi-Rx DL reception, the enhancement on scheduling restrictions due to L3 measurements is not considered.

    - Option 1c (Qualcomm): RAN4 to not discuss the following items for RRM requirement enhancements under the work item of FR2 multi-Rx chain DL reception:

      - RRM requirement enhancements that require

        - more than two cell searchers for cell and SSB detection and SSB measurements

        - L3 measurements by using concurrently activated multiple Rx panels, e.g. FR2 SCell activation delay reduction

      - Idle mode measurements

    - Option 1d (OPPO): Except L3 measurement delay reduction, other impacts on L3 measurements can be studied based on the enhancements on L1 measurements.

    - Option 1e (Ericsson) 1: Deprioritize L3 measurements in the current WI.

      - Cell detection and L3 measurement period, SSB-based:

        - No simultaneous reception for SSB-only based measurements and procedures are assumed.

  - Option 2 (CMCC, LGE, Xiaomi, vivo, ZTE, Nokia): L3 measurement requirements are enhanced in R18 multi-Rx chain WI.

    - Option 2a (CMCC): For L3 measurement in connected mode, all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG) need to be considered for the delay improvement with multi-beam simultaneous reception.

    - Option 2b (LGE):

      - L3 measurement enhancements should be considered since L3 measurement through multi-antenna panels is a natural UE behaviour when multi-antenna panels are activated.

      - Introduce the following case for L3 measurement enhancement first

        - For L3 measurement in connected mode, including all the existing scenarios (intra-frequency measurements without MG, intra-frequency measurements with MG, inter-frequency measurement with MG, inter-frequency measurement without MG).

    - Option 2c (Xiaomi): To agree on the following L3 measurement requirement enhancement:

      - Handover to FR2-1 unknown cell

      - FR2-1 unknown Scell activation

      - L3 measurement in connected mode, including intra-frequency measurement with and without MG.

    - Option 2d (vivo):

      - FR2 SCell activation delay for unknown SCell can be enhanced by reducing L1-RSRP measurement time and/or cell search time for multi-Rx chain UE.

      - Handover delay, PSCell addition/change delay and SCG activation delay for FR2 unknown target cell/PSCell can be enhanced by reducing cell search time for multi-Rx chain UE.

      - FFS if enhancement of L3 measurements in connected mode for multi-Rx chain simultaneous DL reception is feasible.

      - A new capability, which is different from simultaneousReceptionDiffTypeD-r16 or other L1 measurement related UE capability, should be introduced for L3 procedure/measurement delay reduction for UE supporting multi-Rx chain simultaneous reception, if L3 measurement enhancement is agreed to be specified.

    - Option 2e (ZTE): Referring to whether L3 measurement can be enhanced by multi-panel reception, we believe same logic as the enhancement for L1 measurement can be referenced and would not lead to much additional workload.

    - Option 2f (Nokia):

      - RAN4 to consider the following requirements for multi Rx enhancements in RRC connected mode

        - a.	L3 measurement in connected mode

        - b.	Handover to FR2-1

        - c.	RRC re-establishment

        - d.	Intra-frequencey measurements

        - e.	Inter-frequency measurements

      - RAN4 to focus on L3 enhancements for requirements in RRC connected.

        - RAN4 to discuss the need of L3 enhancements for requirements in RRC idle and inactive in a later phase of the work item.