3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2217249

# Electronic Meeting, Oct 10 – Oct 19, 2022

Agenda item:	6.9

Source:	Apple

Title:	WF on R18 eFeRRM - FR2 SCell activation enhancement

Document for:	Approval

# Topic #2: L3 part enhancement for FR2 SCell activation (6.9.2.1)

  - Sub-topic 2-1 Scenarios for FR2 SCell activation enhancement

Issue 2-1-1: Scenarios/status/categories for FR2 SCell activation enhancement

- Option 1 (Qualcomm, Nokia, vivo(1st and 3nd bullet), ZTE):

  - Unknown SCell in FR2 needs to split in two categories for FR2 SCell activation delay reduction purpose. 1) completely unknown SCell, 2) semi-unknown SCell.

  - SCell activation delay reduction should be applied for semi-unknown SCell scenario where SCell is activated from deactivated state.

  - Optional UE capability to indicate the required SSBs to be measured during AGC and cell search before RSRP reporting from UE side for semi-unknown scenario.

- Option 2 (Nokia, ZTE):

  - The availability of a valid L3 measurement result at the time of SCell activation shall be considered to reduce the SCell activation delay.

  - The UE indication on the up-to-date L3 measurement status of the to-be-activated SCell is introduced to reduce the FR2 unknown SCell activation delay.

- Option 3 (Ericsson, QC(scenario 1), Nokia, ZTE):

  - RAN4 to discuss and specify SCell activation for following two cases.

    - Scenario1: SCell is unknown due to the fact that UE did not sent measurement report to gNB in last X seconds.

    - Scenario2: SCell is unknown due to the fact that UE may be measuring it for first time.

- Option 4 (last meeting agreement) (Apple, LGE, Xiaomi, HW, OPPO, CTC, MTK): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI

  - FR2 unknown Scell without intra-band serving cell is considered for 1st phase.

  - The extension of the enhancement solutions to FR1 can also be discussed.

- Option 5 (Moderator):

  - RAN4 to use option 4 as baseline to discuss the enhancement, and the new UE categorization based on option 1/2/3 can be discuss case-by-case in enhancement issues to determine whether and how to use such new category for enhancement.

Agreement:

- RAN4 to use option 4 as baseline to discuss the enhancement, and further unknown SCell scenario categorization can be discussed case-by-case in enhancement issues to determine whether and how to use such new category for enhancement.

- Option 4 (last meeting agreement): RAN4 to prioritize at least FR2 unknown SCell delay reduction in the 1st phase of the WI

  - FR2 unknown Scell without intra-band serving cell is considered for 1st phase.

  - The extension of the enhancement solutions to FR1 can also be discussed.

- FFS on option 1/2/3 and other options are not precluded.

  - Sub-topic 2-2 Beam related enhancement for L3 part

Issue 2-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

FFS:

- Option 1: RAN4 to introduce the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 SCell activation.

  - Option 1a: for RX beam sweeping factor reduction, the agreements on reduced RX beam sweeping factor for Rel-17 FR2 HST or Rel-17 positioning enhancement can be considered as baseline.

  - Option 1b: Introduce the UE capability to support Rx beam sweeping factor can be less than 8 (i.e., 1, 2, 4, 6) for AGC settling and cell detection during unknown FR2 Scell activation.

- Option 2:

  - Since L3 part is the first procedure after RF retuning, reducing RX beam sweeping factor at this stage could have negative impact on the performance of AGC and cell search procedure, which is a significant sacrifice to enhance the SCell activation delay.

- Option 3: RAN4 to consider introducing the UE capability to support the UE Rx beam sweeping factor less than 8 for FR2 Scell activation which is subject to the discussion on feasible conditions and scenarios.

Issue 2-2-2: Beam sweeping factor enhancement related with WI of FR2 multi-Rx chain DL reception

Agreement：

- In 1st phase, RAN4 not to consider to leverage conclusions from multi-Rx chain DL reception WI to FR2 SCell activation enhancement in R18 eFeRRM WI.

- In 2nd phase, if the multi-Rx chain DL reception WI has corresponding conclusions for measurement delay reduction of single carrier case, RAN4 to discuss whether or how to leverage conclusions from multi-Rx chain DL reception WI to FR2 Scell activation enhancement in R18 eFeRRM WI.

  - Sub-topic 2-3 AGC/Cell measurement/synchronization sample number related enhancement for L3 part

Issue 2-3-1: Align the understanding of cell measurement /synchronization /AGC in the existing FR2 unknown SCell activation delay requirement

FFS：

- Option 1(Intel, CMCC, Ericsson, OPPO, Nokia, ZTE): RAN4 should align the understanding of cell measurement/synchronization/AGC/T/F tracking in the existing FR2 unknown SCell activation delay requirements

  - Option 1a (QC, HW, OPPO, Nokia, vivo, ZTE, CTC, MTK): in existing FR2 unknown SCell activation case:

    - TFirstSSB_MAX + 15*TSMTC_MAX: AGC (1 sample for coarse AGC and 1 sample for fine AGC)

    - 8*Trs: cell search, time/frequency synchronization, time/frequency tracking

- Option 2 (Apple, Xiaomi): Option 1 is not needed.

Issue 2-3-2: enhancement of “TFirstSSB_MAX + 15*TSMTC_MAX” part of current FR2 unknown SCell activation delay

FFS:

- Option 1 (Apple, LGE, CMCC, Ericsson(for semi-unknown), OPPO, Nokia, ZTE): reduce the sample number to 1 for L3 measurement/synchronization during unknown FR2 Scell activation (with -2dB SINR side condition); i.e., change TFirstSSB_MAX + 15*TSMTC_MAX to TFirstSSB_MAX + 7*TSMTC_MAX.

- Option 2 (Ericsson(for semi-unknown), Nokia, ZTE): RAN4 to discuss the potential of simplified AGC settling with 1 sample.

- Option 3 (HW, vivo, MTK): The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown Scell activation scenario.

- Option 4 (Ericsson(for semi-unknown), Nokia, ZTE): based on option 3 in issue 2-1-1

  - For scenario 1, RAN4 to agree only one sample is required for AGC. That means TFirstSSB_MAX + N1*TSMTC_MAX.

Issue 2-3-3: enhancement of “8*Trs” part of current FR2 unknown SCell activation delay

FFS:

- Option 1 (Apple, QC (for semi-unknown), OPPO, Nokia, ZTE): for unknown R18 FR2 SCell activation enhancement, RAN4 to further discuss remove or reduce L3 T/F tracking time (8 Trs) after concluding beam sweeping factor and PSS/SSS detection enhancement.

- Option 2 (CMCC, OPPO, ZTE): if timing information can be acquired in the component of AGC/Cell synchronization (i.e. TFirstSSB_MAX + 15*TSMTC_MAX ), 8*Trs can be removed.

- Option 3 (vivo, MTK) The sample number of L3 part is 3 (2 for AGC and 1 for cell search), which is already small and can’t be reduced any further especially for the unknown SCell activation scenario.

  - Sub-topic 2-4 RS related enhancement for L3 part

Issue 2-4-1: RS related enhancement of FR2 unknown SCell activation

FFS:

- Option 1 (Xiaomi, HW, OPPO, CTC): The A-TRS can be used for L3 measurement for FR2 unknown Scell activation if the QCL information of the A-TRS is provided to UE.

  - Option 1a (Xiaomi, ZTE, CTC): If the triggered A-TRS is QCL-ed with the SSB of inter-band SpCell or one of inter-band active serving cell, A-TRS is configured for AGC adjustment, cell search and fine timing tracking for FR2 unknown Scell activation.

  - Option 1b (ZTE, CTC): The extension of A-TRS based unknown FR1 Scell activation into inter-band scenario can also be introduced.

- Option 2 (Apple, QC, LGE, Intel, Ericsson, MTK): AP-CSI-RS and/or A-TRS based fast Scell activation is not apply to L3 part for unknown FR2 Scell activation enhancement.

- Option 3 (Nokia): based on option 2 in issue 2-1-1:

  - RAN4 to study the solution to reduce the unknown Scell activation delay based on the A-TRS.

  - A-TRS can be triggered based on the latest measurement status at UE.

- Option 4 (Xiaomi, HW, ZTE, CTC):

  - UE can obtain coarse timing information via QCL TypeC to inter-band serving cell subject to gNB configuration where the cell detection can be skipped.

  - UE can obtain beam information via QCL TypeD to inter-band serving cell subject to gNB configuration.

  - For FR2 unknown Scell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms.

  - Sub-topic 2-5 Others for L3 part

Issue 2-5-1: Other enhancement for L3 part in FR2 unknown SCell activation

FFS:

- Proposal 1 (Ericsson, QC(for semi-unknown)): RAN4 to study the use of SSB periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation.

  - Proposal 1a (Apple): RAN4 to study the use of SMTC periodicity instead of SMTC_MAX for coarse and fine AGC measurement for unknown SCell activation.

  - Proposal 1b (HW, Nokia, MTK):the proposal 1 is already in existing spec.

- Proposal 2 (Ericsson): RAN4 to agree that UE sends a beam information report to during or at the end of L3 part of the procedure. Contents of the beam information report are FFS.

# Topic #3:	L1 part enhancement for FR2 SCell activation (6.9.2.2)

  - Sub-topic 3-1 Enhancement for L1-RSRP

Issue 3-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Agreement:

- If L1-RSRP measurement of FR2 unknown SCell activation is not skipped, RAN4 to consider Rx beam sweeping factor reduction for L1-RSRP

  - Details are FFS

Issue 3-1-2: Sample number enhancement in L1-RSRP measurement of FR2 unknown SCell activation

Agreements:

- If L1-RSRP measurement is needed, the sample number of PHY filtering cannot be reduced since M=1 is used for L1-RSRP measurement in FR2 unknown Scell activation requirement.

Issue 3-1-3: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation?

Agreements:

- RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

Issue 3-1-4: Prioritization enhancement for L1-RSRP measurement of FR2 unknown SCell activation

Agreement:

- L1-RSRP measurement is performed in non-DRX mode even DRX is configured.

- FFS on whether prioritization needs to be changed between L1-RSRP measurements and L3 measurements during FR2 unknown SCell activation

  - Sub-topic 3-2 TCI related enhancement for L1 part

Issue 3-2-1: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation

FFS:

- Option 1 (Apple, QC, Intel, HW, ZTE, MTK): Fine timing tracking is still needed no matter whether TCI activation is skipped or not.

- Option 2 (Apple, QC, Intel, Xiaomi, HW, OPPO, vivo, ZTE, MTK): Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing.

- Option 3 (Ericsson): RAN4 to confirm, UE timing assumption when performing L1-RSRP measurements.

- Option 4 (Apple): for unknown R18 FR2 SCell activation enhancement, skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused.

Issue 3-2-2: TCI activation enhancement during FR2 unknown SCell activation

FFS:

- Option 1 (Apple, Intel, CMCC, Xiaomi, HW, OPPO, CTC, MTK): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report.

  - Option 1a(Apple, Intel, HW, CTC): for FR2 unknown SCell activation enhancement, the TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report if no MAC CE or RRC indication for TCI is sent to UE.

- Option 2 (vivo): During the Scell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the Scell activation delay can be reduced accordingly.

- Option 3 (QC, Ericsson, Nokia, vivo, ZTE): Gain in delay reduction is quite small for skipping TCI state indication and RAN4 not consider TCI state skipping.

  - Sub-topic 3-3 Aperiodic RS related enhancement for L1 part

Issue 3-3-1: Aperiodic RS for TFineTiming during FR2 unknown SCell activation

Agreement:

- A-TRS can be configured for fine timing tracking after TCI state activation, and the A-TRS is QCL-ed with the selected SSB index.

Issue 3-3-2: Aperiodic RS for L1-RSRP measurement during FR2 unknown SCell activation

FFS:

- Option 1 (Apple): Use AP CSI-RS for L1-RSRP measurement if UE can indicate the completion of L3 stage or can indicate the readiness of L1 measurement.

- Option 2 (Ericsson, LGE, Nokia):

  - RAN4 to study usage of AP-RS and A-TRS for L1-RSRP measurement.

  - RAN4 to send LS to RAN1 if A-TRS can be used for L1-RSRP measurement. If it cannot be used as it is, RAN4 to ask if it can be enhanced to support L1-RSRP measurement on the A-TRS.

  - Sub-topic 3-4 SP CSI-RS and periodic CSI-RS related enhancement for L1 part

  - Sub-topic 3-5 Other enhancement for L1 part

Issue 3-5-1: Other enhancement for L1 part

FFS:

- Proposal 1 (NTT DoCoMo): FR2 SCell activation delay requirement should be studied under the assumption that UE performs L1-RSRP measurements for a cell with different PCI from serving cell.

- Proposal 2 (QC, Nokia, vivo): When Scell is semi-unknown to UE, NW can trigger UE to report RSRP before L1 RSRP measurement and report. The framework can be illustrated as following steps while UE perform the legacy FR2 Scell activation procedure.

  - Step 1. NW configure either periodic or aperiodic (event-based) report config before Scell activation command.

  - Step 2. NW trigger preconfigured report config. (e.g MAC-CE can trigger the report config)

  - Step 3: UE report latest RSRP measurement for target Scell

  - Step 4: UE can skip L1-RSRP measurement and report when UE reports RSRP measurement with associated SSB index during step3.

  - Step 5: Either A-TRS based or SSB-based for fine time tracking. TCI activation command and CSI-RS recourse activation for CQI reporting.

- Proposal 3 (Xiaomi, HW, ZTE, CTC): For FR2 unknown SCell activation without intra-band serving cell, up to gNB configuration, when the CSI-RS for CQI and TCI state of PDCCH/PDSCH is associated with the A-TRS, and the QCL source of A-TRS is configured as SSB in inter-band active serving cell (type C/D), A-TRS can be used for fine timing and the requirements is TFirstATRS + 5ms.

# Topic #4:	Other enhancements for FR2 SCell activation (6.9.2.3)

  - Sub-topic 4-1 Other possible enhancement for FR2 SCell activation

Issue 4-1-2: SCell without SSB for inter-band FR2 unknown SCell activation

FFS:

- Proposal 1 (ZTE, CTC): The FR2 SCell being activated without SSB is able to extend into inter-band scenario, and the delay reduction can be significant under certain conditions.

  - Proposal 1a (ZTE, CTC): SCell activation delay requirement in FR2 inter-band CA could be optimized down to 3ms via ScellwithoutSSB.

- Proposal 2 (ZTE, CTC): The extension of FR1 SCell activation without SSB into inter-band scenario can also be introduced.

  - Proposal 2a (ZTE, CTC): SCell activation delay requirement in FR1 inter-band CA could be optimized down to 3ms via ScellwithoutSSB.

Issue 4-1-3: known/unknown condition enhancement for FR2 SCell activation

FFS:

- Proposal 1 (Nokia): The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be removed to avoid the SCell unnecessarily being considered as unknown.

- Proposal 2 (Apple, HW, Ericsson, ZTE, Nokia):RAN4 to further discuss the known condition extension to facilitate SCell activation.

- Proposal 3 (QC, Ericsson): RAN4 not to consider mandatory aperiodic reports before SCell activation

Issue 4-1-4: Applying the timing/beam information of active serving cell on FR2 inter-band

FFS:

- Proposal 1 (HW, ZTE, CTC): Under certain condition when the timing/beam information of active serving cell on FR2 inter-band is able to be applied for to-be-activated SCell, the SCell activation delay can be further reduced to 3ms (e.g., Tactivation_time=3ms).

- Proposal 2 (HW, ZTE, CTC): Similar as the enhancement for fast SCell activation in Re-17, for unknown FR2 target SCell case, if the timing and beam can be obtained from an active serving cell in another band, then the cell search and L1-RSRP can be ignored.

Issue 4-1-5: Scheduling enhancement for FR2 unknown SCell activation

FFS:

- Proposal 1 (Nokia): The potential to enable earlier data transmission within the activation period shall be studied.

- Proposal 2 (Nokia): The UE shall be scheduled by the network immediately after L1-RSRP reporting.

- Proposal 3 (Nokia): The UE is allowed to be scheduled by the network after L3 measurement during cell detection when activating an FR2 unknown SCell.

Issue 4-1-6: CBRA for FR2 unknown SCell activation

FFS:

- Option 1 (MTK): Contention based random access (CBRA) can be used to activate first unknown SCell in one band, which can remove the need for L1-RSRP measurement and TCI state indication, and as a result, enhance the overall activation delay for unknown SCell.

- Option 2 (Apple, QC, LGE, Ericsson, vivo): RAN4 not to consider CBRA for SCell activation.