3GPP TSG-RAN WG4 Meeting #104-bis-e									                 R4-2217254

Electronic Meeting, 10– 19 October 2022

Agenda item:			6.12.6

Source:	Samsung

Title:	WF on tunnel deployment and UL timing adjustment for FR2 HST enhancement

Document for:	Approval

# Introduction

In RAN#95e meeting, the Rel-18 RAN4-led work item on enhanced NR support for high speed train scenario in FR2 has been approved [RP-220985], which has been further updated in [RP-222272].

This document provides way forwards on tunnel deployment and UL timing adjustment for FR2 HST enhancement, based on the outcomes of email discussion summary for [104-bis-e][218] NR_HST_FR2_enh_RRM.

# Way Forward on Tunnel Deployment for FR2 HST

## General assumption for tunnel deployment

Agreement:

- On the assumption for train-roof-mounted CPE:

  - For the feasibility study of tunnel scenarios, the assumed parameters for train-roof-mounted CPE UE in Rel-17 WI can be reused:

    - DUE_height: 5m

    - UE panel: N=4, M=4 with 2 polarizations

- On the assumption on SCS:

  - Only consider 120 kHz SCS for HST FR2 evaluations and requirements definition.

Way Forward:

- On the assumption of transmission scheme:

  - Further study the transmission scheme of the tunnel deployment scenario,

    - FFS SFN scheme and other multi-TRP schemes should be considered with tunnel deployment scenario.

    - FFS bi-directional and uni-directional RRH deployment for tunnel scenario

## Key parameters for tunnel deployment

Way Forward:

- RAN4 discuss and study the key parameters below as baseline assumption for tunnel deployment by considering feasibility study of tunnel scenarios:

  - Ds: the distance separation between two neighboring RRH sites. FFS options including:

    - Option 1: Ds = 500m

    - Option 2: Ds = 700m

  - Dmin: the minimum distance between RRH site and train track. FFS options including:

    - Option 1: Dmin = 0m

    - Option 2: Dmin = 2m

  - DRRH_height: determined/limited by tunnel height and RRH deployment method

  - Tunnel dimensions: such as tunnel shape, height, width etc.

    - Option 1: 7.6 meters in diameter for a 2 track tunnel

      - Accordingly, DRRH_height is assumed to be 7.4m

    - Option 2: 5.5 meters in diameter for a single tunnel

      - Accordingly, DRRH_height is assumed to be 5.3m

    - Other options are not precluded

  - gNB RRH and antenna panel element assumption.

    - FFS the number of RRHs per BBU,

      - Option 1: 4 RRHs per BBU,

      - Option 2: from 1 to 4 RRHs per BBU

      - Other options are not precluded

    - 1 beam per RRH panel

    - RRH Antenna Element Assumption for RRH side is the same as Rel-17: [Mg, Ng, M, N, P] = [1, 1, 8, 8, 2].

## Reference channel model for tunnel scenario

Way Forward:

- RAN4 further study the reference channel model for tunnel scenario:

  - FFS LoS propagation assumption is valid in the tunnel deployment.

  - FFS LoS UMi street canyon channel mode for the RRM evaluations of HST FR2 tunnel deployment.

  - FFS using multi-path fading channel model with strong LoS component for the performance evaluation of HST FR2 tunnel deployment.

  - FFS the solution about the problem about the significant performance degradation when UE is under RRH due to larger delay spread than CP.

  - FFS The tunnel pathloss model, fading model and link budget will be the same as scenario A (LoS).

## Mobility issue for tunnel scenario

Way Forward:

- RAN4 to discuss the mobility issue when the train is travelling in the direction opposite to the serving beam orientation, especially, in the case when RRH are close to the track, i.e., in tunnel deployments.

## Expectation from tunnel deployment scenario study

Agreement:

- Based on the study on tunnel scenario, at least the following targets can be expected:

  - FR2 HST tunnel scenario channel model

    - FFS the necessity of new channel model FR2 HST tunnel scenario

  - Typical FR2 HST deployment scenario for tunnel scenario

    - FFS the feasible parameters for FR2 HST deployment for tunnel scenario

  - Identified the requirements that might be impacted by the tunnel scenario, if any.

# Way Forward on UL Timing Adjustment Solution for FR2 HST

## General view on UL timing adjustment solution

Way Forward:

- RAN4 continue to discuss UL timing adjustment solution, including explicit NW signalling assistance in Rel-18, based upon Option 3 and 4 captured in WF R4-2120416.

  - Both RRC and MAC-CE based solutions are FFS.

    - FFS whether to embedded spatial similarity (QCL-like relation for beams across RRHs) information by ordering the SSB index (to RRH mapping) signaled in RRC solution

## Relationship with 2 TA enhancement in Rel-18 MIMO

Agreement:

- Two TA enhancement is not in the scope of Rel-18 FR2 HST enhancement workitem.

## Analysis on the impact of large propagation delay jump

Way Forward:

- RAN4 to discuss the existence of potential impacts of large jump in propagation delay on UE MAC of UL TA adjustment and timeAlignemntTimer in the case of inter-RRH TCI state switch.

# Appendix: Agreement from WF R4-2120416

<From approved WF R4-2120416> For one shot large uplink timing adjustment Moderator note: Highlight part is agreed during RAN4 GTW session - It is up to network configuration to enable one shot large uplink timing adjustment mechanism - RAN4 will further study if additional flag, e.g., unidirectional flag on top of general FR2 HST scenario flag is needed to enable one shot large uplink timing adjustment - RAN4 will further study the network configuration means to disable one shot large uplink timing adjustment. - If one shot large uplink timing adjustment is disabled, existing uplink timing adjustment, i.e., RA based mechanism, and related existing RAN4 requirements will be applied when needed - Introduce a mechanism for one shot large uplink timing adjustment for FR2 HST scenarios with UE allowed to adjust uplink timing beyond Tq - FFS for conditions and additional network assistance for UE to apply one shot large uplink timing adjustment. - The following options can be considered for triggering condition and network assistance - Option 1: No condition except DL timing difference: - UE will apply one shot large timing adjustment if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - Option 2: TCI switching without network assistance: - UE will apply one shot large timing adjustment on TCI switching occasion if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - Option 3: TCI switching with network assistance of indication of inter-RRH and UE large DL timing change detection - UE will apply one shot large timing adjustment on TCI switching between RRH occasion if UE measurement on DL timing difference is larger than certain threshold. - FFS for how to define the threshold - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH. - Option 4: TCI switching with network assistance of indication of inter-RRH but without UE large DL timing change detection - UE will apply one shot large timing adjustment on TCI switching between RRH occasions - FFS for detailed network indication of inter-RRH. One example could be a flag in MAC-CE command came with TCI state switch command, or could be SSB index and order per RRH. - Performance degradation and impact to signalling design shall be discussed for above procedures - RAN4 will further discuss the accuracy performance and testing issues based on conclusion of above procedures