3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217280

Electronic Meeting, 10th ‒ 19th October, 2022

Title: 	WF on RAN task on BWP operation without restriction

Agenda Item:	9.1

Source: 	vivo

Document for:	Approval

- Topic #1: Analysis of options for BWP operation without restriction

  - Sub-topic 1-1: High-level analysis of the options

<Agreement >:

<During GTW on 13th Oct., following agreements were reached.>

- Agreements

  - RAN4 works on the below aspects/criteria for highest-level analysis on options for UE performing RLM/BFD/BM when CD-SSB is outside active BWP

    - RRM requirements impact (Spec impact) / workload in RAN4

    - Mobility performance impact

    - Throughput impact (Data interruption)

    - UE power consumption / UE complexity

<Agreement >:

Options from RP-221911 are further split as below for high-level analysis.

- Option A) Perform BM/RLM/BFD based on CSI-RS within active BWP

- Option B) Perform BM/RLM/BFD based on SSB outside active BWP

  - Option B-1) UE’s capability not requiring additional measurement gap for BM/RLM/BFD

    - Option B-1-1) Using larger BW covering SSB outside active BWP without interruptions

    - Option B-1-2) Using larger BW covering SSB outside active BWP with interruptions

    - Option B-1-3) Using a separate RF chain without interruptions

    - Option B-1-4) Using a separate RF chain with interruptions

  - Option B-2) BM/RLM/BFD on SSB outside BWP within measurement gaps

    - Option B-2-1) Shared MG or NCSG for RLM/BFD/BM and L3 measurement

    - Option B-2-2) Dedicated MG or NCSG for RLM/BFD/BM measurements

- Option C) NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

<Agreement >:

<During GTW on 18th Oct., following agreements were reached.>

Agreement:

- Framework wise option 3 is agreed to be used for Issue 1-1-3-1, Issue 1-1-3-2, Issue 1-1-3-3, Issue 1-1-3-4 as baseline to develop high-level analysis for options for BWP operation without restrictions in Rel-18 for the report to RAN plenary.

  - Note: How the high-level analysis is structured/formulated in the reply LS can be further discussed.

- The contents in the current table in option 3 are for information; and further technical input is expected in the next meeting from companies

<Way forward >:

Issue 1-1-3-1: High-level analysis of options on RRM requirements impact/workload in RAN4

- Proposals

  - Option 1:

| Options | RRM requirements impact/workload in RAN4 |
| --- | --- |
| Option A) | - CSI-RS based RLM/BFD/BM requirements are already specified. - Timing requirements based on SSB outside active BWP need further discussion. |
| Option B-1-1) | - Applicability of existing requirements for RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to defined. |
| Option B-1-2) | - Applicability of existing requirements for SSB based RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to be specified - Interruption requirements need to be developed additionally to allow UE for switching, or - Interruption requirements with NCSG is developed so that UE is allowed for switching and interruption length and location is known to NW. |
| Option B-1-3) | - Applicability of existing requirements for SSB based RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to be specified - No interruption is allowed. - UE May need to fallback to larger BW when there is no vacant/separate RF available under certain band combinations |
| Option B-1-4) | - Applicability of existing requirements for SSB based RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to be specified - Interruption requirements need to be developed. - UE May need to fallback to larger BW when there is no vacant/separate RF available under certain band combinations |
| Option B-2-1) | New requirements should be developed for the gap sharing mechanism. - Requirements for gap-based RLM - Requirements for gap-based BFD - Requirements for gap-based BM - CCSF for measurements within gaps - Gap sharing mechanism for L1 measurements and L3 measurements. - Others? |
| Option B-2-2) | New requirements should be developed for L1 measurements with dedicated measurement gaps. - Requirements for gap-based RLM - Requirements for gap-based BFD - Requirements for gap-based BM - CCSF for measurements within gaps - Gap collision handling between L1 gap and L3 gap - Others? |
| Option C) | Option 1: BM/RLM/BFD measurement with NCD-SSB can work with existing RAN4 requirements. No new requirements are needed. Option 2: Applicability of existing requirements based on CD-SSB (SSB in existing requirements), i.e., SSB based RLM/BFD/BM and timing requirements, to NCD-SSB |

  - Option 2:

| Solutions | RRM requirements impact/workload in RAN4 |
| --- | --- |
| Option A) | None |
| Option B-1-1) | Minor |
| Option B-1-2) | Minor |
| Option B-1-3) | Minor |
| Option B-1-4) | Minor |
| Option B-2-1) | Medium |
| Option B-2-1) | Medium |
| Option C) | None |

  - Option 3: Combination of option 1 and option 2.

    - Note 1: The table below is framework for high-level analysis. The contents in the table are as example for information.

    - Note 2: For technical analysis, option 1 can be used as starting point.

    - Note 3: For summary, option 2 can be used as starting point.

| Options | RRM requirements impact/workload in RAN4 |  |
| --- | --- | --- |
|  | Technical analysis | Summary |
| Option A) | - CSI-RS based RLM/BFD/BM requirements are already specified. - FFS: Timing requirements based on SSB outside active BWP need further discussion. | FFS |
| Option B-1-1) | - Applicability of existing requirements for RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to defined. | FFS |
| Option B-1-2) | - Applicability of existing requirements for SSB based RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to be specified - Interruption requirements need to be developed additionally to allow UE for switching, or - Interruption requirements with NCSG is developed so that UE is allowed for switching and interruption length and location is known to NW. | FFS |
| Option B-1-3) | - Applicability of existing requirements for SSB based RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to be specified - No interruption is allowed. - UE May need to fallback to larger BW when there is no vacant/separate RF available under certain band combinations | FFS |
| Option B-1-4) | - Applicability of existing requirements for SSB based RLM/BFD/BM measurement, including applicability of measurement restrictions and scheduling restrictions, need to be specified - Interruption requirements need to be developed. - UE May need to fallback to larger BW when there is no vacant/separate RF available under certain band combinations | FFS |
| Option B-2-1) | New requirements should be developed for the gap sharing mechanism. - Requirements for gap-based RLM - Requirements for gap-based BFD - Requirements for gap-based BM - CCSF for measurements within gaps - Gap sharing mechanism for L1 measurements and L3 measurements. - Others? | FFS |
| Option B-2-2) | New requirements should be developed for L1 measurements with dedicated measurement gaps. - Requirements for gap-based RLM - Requirements for gap-based BFD - Requirements for gap-based BM - CCSF for measurements within gaps - Gap collision handling between L1 gap and L3 gap - Others? | FFS |
| Option C) | - Option 1: BM/RLM/BFD measurement with NCD-SSB can work with existing RAN4 requirements. No new requirements are needed. - Option 2: Applicability of existing requirements based on CD-SSB (SSB in existing requirements), i.e., SSB based RLM/BFD/BM and timing requirements, to NCD-SSB | FFS |

Issue 1-1-3-2: High-level analysis of options on mobility performance impact

- Proposals

  - Option 1:

| Options | Mobility performance impact |
| --- | --- |
| Option A) | - Intra-frequency measurement is performed within gap |
| Option B-1-1) | - Intra-frequency measurement is performed without gap - Inter-frequency measurement can be performed without gap when inter-frequency SSB is within the larger BW |
| Option B-1-2) | - Intra-frequency measurement is performed without gap |
| Option B-1-3) | - Intra-frequency measurement is performed without gap |
| Option B-1-4) | - Intra-frequency measurement is performed without gap |
| Option B-2-1) | - Intra-frequency measurement is performed within gap, and gap is shared with RLM/BFD/BM measurements. There could be mobility performance degradation. |
| Option B-2-2) | - Intra-frequency measurement is performed within gap, and gap could be collided with L1 gap for RLM/BFD/BM measurements. There could be mobility performance degradation. |
| OptionC) | - Intra-frequency measurement is performed without gap |

  - Option 2:

| Options | Mobility performance impact |
| --- | --- |
| Option A) | No |
| Option B-1-1) | No |
| Option B-1-2) | No |
| Option B-1-3) | No |
| Option B-1-4) | No |
| Option B-2-1) | Yes |
| Option B-2-1) | Yes |
| Option C) | No |

  - Option 3: Combination of option 1 and option 2.

    - Note 1: The table below are framework for high-level analysis. The contents in the table are as example for information.

    - Note 2: For technical analysis, option 1 can be used as starting point.

    - Note 3: For summary, option 2 can be used as starting point.

| Options | Mobility performance impact |  |
| --- | --- | --- |
|  | Technical analysis | Conclusion |
| Option A) | - Intra-frequency measurement is performed within gap | FFS |
| Option B-1-1) | - Intra-frequency measurement is performed without gap | FFS |
| Option B-1-2) | - Intra-frequency measurement is performed without gap | FFS |
| Option B-1-3) | - Intra-frequency measurement is performed without gap | FFS |
| Option B-1-4) | - Intra-frequency measurement is performed without gap | FFS |
| Option B-2-1) | - Intra-frequency measurement is performed within gap, and gap is shared with RLM/BFD/BM measurements. There could be mobility performance degradation. | FFS |
| Option B-2-2) | - Intra-frequency measurement is performed within gap, and gap could be collided with L1 gap for RLM/BFD/BM measurements. There could be mobility performance degradation. | FFS |
| Option C) | - Intra-frequency measurement is performed without gap | FFS |

Issue 1-1-3-3: High-level analysis of options on throughput impact (data interruption)

- Proposals

  - Option 1:

| Options | Throughput impact (Data interruption) |
| --- | --- |
| Option A) | - As measurement gap for intra-frequency measurement needs to be configured, UE cannot be scheduled during measurement gap. |
| Option B-1-1) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. |
| Option B-1-2) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. - Interruptions would cause throughput loss. |
| Option B-1-3) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. |
| Option B-1-4) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. - Interruptions would cause throughput loss. |
| Option B-2-1) | - UE cannot be scheduled within gap. - UE can be scheduled within ML for NCSG gap. |
| Option B-2-2) | - UE cannot be scheduled within gap for L1 and L3 measurements. - UE can be scheduled within ML of NCSG gap for L1 measurements. |
| Option C) | - UE can always be scheduled if no inter-frequency measurement with gap being configured |

  - Option 2:

| Options | Throughput impact (Data interruption) |
| --- | --- |
| Option A) | None |
| Option B-1-1) | None |
| Option B-1-2) | None |
| Option B-1-3) | None |
| Option B-1-4) | None |
| Option B-2-1) | None |
| Option B-2-1) | Additional interruption |
| Option C) | None |

  - Option 3: Combination of option 1 and option 2.

    - Note 1: The table below are framework for high-level analysis. The contents in the table are as example for information.

    - Note 2: For technical analysis, option 1 can be used as starting point.

    - Note 3: For summary, option 2 can be used as starting point.

| Options | Throughput impact (Data interruption) |  |
| --- | --- | --- |
|  | Technical analysis | Conclusion |
| Option A) | - As measurement gap for intra-frequency measurement needs to be configured, UE cannot be scheduled during measurement gap. | FFS |
| Option B-1-1) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. | FFS |
| Option B-1-2) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. - Interruptions would cause throughput loss. | FFS |
| Option B-1-3) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. | FFS |
| Option B-1-4) | - As gap is not needed for intra-frequency measurement, UE can always be scheduled if no other inter-frequency measurement with gap is configured. - Interruptions would cause throughput loss. | FFS |
| Option B-2-1) | - UE cannot be scheduled within gap. - UE can be scheduled within ML for NCSG gap. | FFS |
| Option B-2-2) | - UE cannot be scheduled within gap for L1 and L3 measurements. - UE can be scheduled within ML of NCSG gap for L1 measurements. | FFS |
| Option C) | - UE can always be scheduled if no inter-frequency measurement with gap being configured | FFS |

Issue 1-1-3-4: High-level analysis of options on UE power consumption / UE complexity

- Proposals

  - Option 1:

| Options | UE power consumption / UE complexity |
| --- | --- |
| Option A) | - UE works in active BWP. - No RF retuning is needed for CSI-RS based RLM/BFD/BM. - FFS whether RF retuning is needed for UE to meet timing requirements. - As periodicity of CSI-RS is shorter than that of SSB typically, CSI-RS based RLM/BFD/BM measurement period is shorter when no DRX is configured, which would cause higher power consumption. |
| Option B-1-1) | - UE works in larger BW than active BWP. - No RF retuning is needed. |
| Option B-1-2) | - UE works in larger BW than active BWP. - RF retuning is needed for UE to switch between larger BW and active BWP. |
| Option B-1-3) | - UE needs to always turn on vacant/separate RF chain |
| Option B-1-4) | - UE needs to periodically turn on vacant/separate RF chain |
| Option B-2-1) | - UE works in active BWP. |
| Option B-2-2) | - UE works in active BWP. |
| Option C) | - UE works in active BWP - RF retuning is not needed |

  - Option 2:

| Options | UE power consumption / UE complexity |
| --- | --- |
| Option A) | Low |
| Option B-1-1) | High |
| Option B-1-2) | Medium |
| Option B-1-3) | High |
| Option B-1-4) | Medium |
| Option B-2-1) | Low |
| Option B-2-1) | Low |
| Option C) | Low |

  - Option 3: Combination of option 1 and option 2.

    - Note 1: The table below are framework for high-level analysis. The contents in the table are as example for information.

    - Note 2: For technical analysis, option 1 can be used as starting point.

    - Note 3: For summary, option 2 can be used as starting point.

| Options | UE power consumption / UE complexity |  |
| --- | --- | --- |
|  | Technical analysis | Conclusion |
| Option A) | - UE works in active BWP. - No RF retuning is needed for CSI-RS based RLM/BFD/BM. - FFS whether RF retuning is needed for UE to meet timing requirements. - As periodicity of CSI-RS is shorter than that of SSB typically, CSI-RS based RLM/BFD/BM measurement period is shorter when no DRX is configured, which would cause higher power consumption. | FFS |
| Option B-1-1) | - UE works in larger BW than active BWP. - No RF retuning is needed. | FFS |
| Option B-1-2) | - UE works in larger BW than active BWP. - RF retuning is needed for UE to switch between larger BW and active BWP. | FFS |
| Option B-1-3) | - UE needs to always turn on vacant/separate RF chain | FFS |
| Option B-1-4) | - UE needs to periodically turn on vacant/separate RF chain | FFS |
| Option B-2-1) | - UE works in active BWP. - RF retuning is also needed in measurement gap. | FFS |
| Option B-2-2) | - UE works in active BWP. - RF retuning is also needed in measurement gap. | FFS |
| Option C) | - UE works in active BWP - RF retuning is not needed | FFS |

Issue 1-1-4: Considerations for option A

- Proposals

  - Option 1 (Intel):

    - RAN4 to discuss the necessity of “Introduction of timing requirements based on CSI-RS” to meet the existing UE transmit error requirements for FG 6-1a support with CSI-RS based solution.

  - Option 2 (CMCC, Apple, CATT, Ericsson, MTK, OPPO):

    - Existing specs can support CSI-RS based BM/RLM/BFD within active BWP. No additional work is required.

  - Option 3 (Spreadtrum):

    - CSI-RS for TRS might be required for time and frequency tracking in the case of SSB outside active BWP.

  - Option 4 (vivo, Qualcomm, Spreadtrum):

    - For a UE supporting FG 6-1a bwp-WithoutRestriction to perform CSI-RS based RLM/BFD/BM, it needs further discussion how UE can meet timing requirements when SSB is outside active BWP.

  - Sub-topic 1-2: General

<Agreement >:

- No need to discuss solution for RLM/BFD/BM when CD-SSB is outside active BWP for Rel-17 in RAN4.

<Way forward >:

Issue 1-2-1: Which new solution(s) are down-selected to be supported in Rel-18

- Proposals

  - Option 1 (Qualcomm, Vodafone, CMCC, Ericsson, Nokia):

    - Option B-1 is feasible, i.e. L1 measurements (RLM/BM/BFR) outside active DL BWP are feasible without measurement gap and interruptions

    - A new UE capability for Option B-1 can be introduced, e.g.

| Capability-x-y-z Indicates support of SSB-based RLM, SSB-based BFD (if supported), SSB-based CBD (if supported), and SSB-based L1-RSRP/L1-SINR measurement (if supported) using SSB that is outside active DL BWP. The SSB is still within the bandwidth of the configured UE-specific carrier. The UE supporting this Capability-x-y-z shall also support bwp-WithoutRestriction. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |

    - Expected specification changes are expected minimal and limited to TS38.300, TS38.213, and TS38.133.

  - Option 2 (OPPO):

    - Any solution is not precluded at this stage, and support to discuss in R18 FeRRM WI

  - Option 3 (CATT):

    - It is typical case that a Rel-18 UE already support FG1-7 and 2-24, so Option a) which does not require any specification effort is generally preferable.

    - If Option A is not sufficient in Rel-18, RAN4 can further discuss the extension of NCD-SSB to non-Redcap Ues in Rel-18. Requirements for Rel-17 Redcap Ues can be taken as a starting point.

  - Option 4 (Apple, OPPO, MTK):

    - RAN4 shall rely on CSI-RS based approach in R17.

    - 3GPP can consider studying the following solutions in R18 RRM enhancement.

      - Perform BM/RLM/BFD based on SSB outside active BWP

        - UE’s capability to operate using larger BW covering SSB outside active BWP, or a UE that is equipped with a separate RF chain

        - BM/RLM/BFD on SSB outside BWP are performed with shared MG or NCSG for L3 measurement, or dedicated MG or NCSG for RLM/BFD/BM measurements.

      - NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible with existing RAN4 specifications for BM/RLM/BFD

  - Option 5 (Spreadtrum, vivo, CMCC):

    - NCD-SSB is used for the UEs to support” FG 6-1a BWP without restriction”

  - Option 6 (MTK):

    - RAN4 suggest ranking the methods under study from the best option to the worst option as:

- Perform BM/RLM/BFD based on CSI-RS within active BWP.

- NCD-SSB approach which would work with existing UE hardware architectures (FG6-1) and be compatible.

- Perform BM/RLM/BFD based on SSB outside active BWP:

- Using Legacy rel-15/rel-16 MG.

- Using NCSG.

- Enlarge BW or using additional RF.

  - Option 8 (Huawei):

    - Further consider option b-ii) and option c) for bwp-WithoutRestriction in Rel-18.

      - Option b-ii): there is no CD- or NCD-SSB in the active BWP, and UE measures SSB outside BWP for RLM/BFD/BM, and gap or interruption is allowed for RF re-tuning.

      - Option c): there is no CD-SSB in the active BWP, and NW configures NCD-SSB within active BWP for UE to perform RLM/BFD/BM.