3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217372

Electronic Meeting, August 15th ‒ 26th, 2022

Title: 	WF on PUSCH demodulation requirements for FR2-2

Agenda Item:	9.14.9

Source: 	Nokia, Nokia Shanghai Bell

Document for:	Approval

# PUSCH requirements

## Sub-topic 1 Channel bandwidth and SCS

Sub-carrier spacings for PUSCH requirements

<Agreement>: 120 kHz and 480 kHz

Channel bandwidth for PUSCH requirements with 120 kHz SCS

<Agreement> Define PUSCH BS demodulation requirements for 120kHz SCS with 100MHz and 400MHz

- Further discuss test applicable rules considering the mandatory CHBW sets for BS

Channel bandwidth for PUSCH requirements with 480 kHz SCS

<Agreement> Define PUSCH BS demodulation requirements for 480kHz SCS with 400MHz

## Sub-topic 2 Channel model

Channel model for 16 QAM for PUSCH requirements

<Way forward>.

  - Option 1: Update channel model for MCS 16 with 120 kHz SCS as

    - For 1x2 Low, use TDLA30-650

    - For 2x2 Low, use TDLA30-650

  - Option 2: Update channel model for MCS 16 with 120 kHz SCS as

    - For 1x2 Low, use TDLA30-650

    - For 2x2 Low, use TDLD30-650

  - Option 3: Update channel model for MCS 16 with 120 kHz SCS as

    - For 1x2 Low, use TDLA30-650

    - For 2x2 Low, use TDLD30-200

  - Option 4: Update channel model for MCS 16 with 480 kHz SCS as

    - For 1x2 Low, use TDLA30-650

    - For 2x2 Low, use TDLD30-200

  - Option 5: Update channel model for MCS 16 with 120 kHz SCS and 480kHz SCS:

    - For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW.

    - For 2x2 Low, use TDLD30-200 for 100MHz CBW, TDLD10-200 for 400MHz CBW.

## Sub-topic 3 Configuration and test cases PUSCH demodulation requirements

MCS and number Tx/Rx branches for PUSCH requirements

<Way forward> Proposals regarding 64 QAM requirements.

  - Option 1: MCS 20 with 1T2R Low and MCS 18 with 2T2R Low

    - Note: If the final link budget agreement indicate 20Db SNR limit could be applied for FR2-2

  - Option 2: MCS20 with 1T2R and 2T2R Low

PTRS configuration

<Agreement> Disabled PTRS for QPSK for both CP-OFDM and DFT-S-OFDM.

Parameters for PUSCH requirements

< Agreement>:

List of test cases for FR2-2 PUSCH

| SCS (kHz) | CBW (MHz) | MCS | Channel model | Antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 120 | 100 | 4 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 16 | TDLA30-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD30-200Note 1 | 2x2 Low | 70% of maximum TP |
| 120 | 100 | 20 Note 2 | TDLD30-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD10-200Note 1 | 2x2 Low | 70% of maximum TP |
| 120 | 400 | 20 Note 2 | TDLD30-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 4 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 16 | TDLA10-650 | 1x2 Low | 70% of maximum TP |
|  |  |  | TDLD10-200Note 1 | 2x2 Low | 70% of maximum TP |
| 480 | 400 | 20 Note 2 | TDLD10-200 | 1x2 Low | 70% of maximum TP |
|  |  |  |  | 2x2 Low | 70% of maximum TP |
| NOTE 1: Confirmation on the channel model for MCS16 with 2x2 is pending NOTE 2: Confirmation on the use of MCS18 of MCS20 with 2x2 is pending |  |  |  |  |  |