3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216012

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Discussions on remain issues on FR2-2 PDSCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.2.1

Document for:	Discussion

Background

As per [1], there are some issues for FR2-2 PDSCH demodulation requirements. In this paper, we provide our views.

Discussions

MCS and propagation conditions

One issues is how to select MCS, propagation conditions and RB allocation.  The criteria for the selection can be summarized as follows:

- Criteria 1: Performance degradation due to phase noise is less than 1dB with the agreed PN compensation technique

- Criteria 2: The target SNR should be lower than maximum DL testable SNR specified by RF

We copy our simulation results for all candidate options as follows:

Table 2-2: Simulation results for FR2-2 PDSCH requirements

| CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size | SNR(dB)@70% of max TP | Maximum DL testable SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 100/120 | MCS 4/Rank1 | TDLA30-200 | 2x2 ULA Low | 66 | -2.4 | 9.8 |
| 100/120 | MCS13/Rank1 | TDLA30-200 | 2x2 ULA Low | 66 | 6.0 | 9.8 |
| 100/120 | MCS13/Rank1 | TDLD30-200 | 2x2 ULA Low | 66 | 5.2 | 9.8 |
| 100/120 | MCS17/Rank1 | TDLD30-200 | 2x2 ULA Low | 33 | 9.0 | 13 |
| 400/480 | MCS 4/Rank1 | TDLA10-200 | 2x2 ULA Low | 66 | -2.8 | 2.6 |
| 400/480 | MCS13/Rank1 | TDLD10-200 | 2x2 ULA Low | 16 | 5.4 | 10 |
| 400/480 | MCS17/Rank1 | TDLD10-200 | 2x2 ULA Low | 16 | 9.1 | 10 |

Based on our previous analysis, all cases with MCS 17 can satisfy the criteria1. Hence we should put our attention to criteria 2.

- 120kHz SCS

We suggest to select TDLD30-200 instead of TDLA30-200 for MCS13 to leave more much margin for testability.

- 480kHz SCS

For MCS 17, the target SNR is very closed to testable SNR. Hence we suggest to not consider MCS17.

Therefore, we propose the cases listed in Table 2-1 for requirements definition:

Table 2-1: Proposed cases for FR2-2 PDSCH demodulation requirements

| CBW(MHz) / SCS (kHz) | MCS/Rank | Channel Model | Antenna Configuration and Correlation Matrix | RB Allocation Size |
| --- | --- | --- | --- | --- |
| 100/120 | MCS 4/Rank1 | TDLA30-200 | 2x2 ULA Low | 66 |
| 100/120 | MCS13/Rank1 | TDLD30-200 | 2x2 ULA Low | 66 |
| 100/120 | MCS17/Rank1 | TDLD30-200 | 2x2 ULA Low | 33 |
| 400/480 | MCS 4/Rank1 | TDLD10-200 | 2x2 ULA Low | 66 |
| 400/480 | MCS13/Rank1 | TDLD10-200 | 2x2 ULA Low | 16 |

Proposal 1: Consider cases listed in Table 2-1 for PDSCH requirements definition

Partial bandwidth allocation position in the CBW

Similar as small RB requirements definition in Rel15, we think it is feasible to put the tested RB in the center of carrier.

Proposal 2: Put the tested RBs in the center of the carrier:

Whether RAN4 should introduce requirements for 30% of peak throughput

Requirements for 30% of peak throughput is used for verifying whether UE supporting maximum number of HARQ processes and HARQ combination. However, there is no big impact on this capability with change of frequency. Hence we suggest to deprioritize this test and only focus on 70% of peak throughput.

Proposal 3: Deprioritize the 30% of peak throughput and only focus on 70% of peak throughput

Conclusion

In this paper, we provide our views on remain issues on FR2-2 PDSCH requirements definition. The proposals are:

Proposal 1: Consider cases listed in Table 2-1 for PDSCH requirements definition

Proposal 2: Put the tested RBs in the center of the carrier:

Proposal 3: Deprioritize the 30% of peak throughput and only focus on 70% of peak throughput

Reference

[1]   R4-2214390 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated