3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216016

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Discussions on open issues for SDR requirements for FR2-2

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.2.3

Document for:	Discussion

Background

As per [1], RAN 4 listed many open issues for FR2-2 SDR requirements. In this paper, we provide our views.

Discussions

It is noted that we capture our simulation results in appendix.

Schedule and feedback patterns for FR1+FR2-2 CA scenario

PDSCH CA requirements are defined based on the assumptions all ACK/NACK are transmitted on PCell CC. However, for SDR requirements, the data rate will be lower if ACK/NACK are transmitted on PCell CC since UE should wait for UL slot of Pcell to transmit ACK/NACK. Hence we suggest that all ACK/NCK are transmitted on separately rather than on Pcell.

Proposal 1: For FR1+FR2-2 CA scenario, all ACK/NACK should be transmitted separately on each CC

Rank

- Option 1: Rank 1 and Rank 2; - Option 2: Rank 1 only;

One issue is number of MIMO layers. Based on our understanding, rank1 and rank2 are typical configurations when UE are operating in FR2-2. Also, RAN 4 defined rank 1 and rank 2 for FR2 SDR requirements, we proposed to cover both rank 1 and rank 2.

Proposal 2: Define FR2-2 SDR requirements for both rank 1 and rank 2.

SCS

We propose to only consider 120kHz SCS with following reasons:

- 480kHz and 960kHz are optional for UE and 120kHz is mandatory for UE

- 120kHz SCS is more sensitive to ICI caused by phase noise. RAN 4 should consider minimum requirements

- Maximum DL testable SNR is quite low for 480kHz. I.e. <-20dB for 1600MHz and -0.6dB for 400MHz, it is meaningless to test SDR requirements with such low maximum DL testable SNR.

Observation 1: 480kHz and 960kHz SCS are optional for UE and 120kHz is mandatory for UE

Observation 2: 120 kHz SCS is more sensitive to ICI caused by phase noise which is related to minimum requirements.

Observation 3: Maximum DL testable SNR is quite low for 480kHz. I.e. <-20dB for 1600MHz and -0.6dB for 400MHz. SDR is untestable under such quite low SNR.

Proposal 3: Only consider 120kHz SCS for SDR test

PN Compensation

- Option 1: Same as PDSCH test cases based on outcome of Issue 2-1-12 - Option 2: CPE compensation only

CPE compensation is basic algorithm when UE is operating on FR2-2 band which is corresponding to minimum performance requirements. Since RAN4 always consider minimum requirements, we propose to only consider PN compensation.

Proposal 4: Only consider CPE compensation.

Changes to Maximum MCS for FR2-2 64QAM, Rank1

- Proposals: - Option 1: MCS 24 for Rank1 and rank2; - Option 2: MCS 27 for Rank 1;

Based on our simulation results, 85% of peak throughput can’t be reached when MCS is larger than 25 and we find that target SNR for MCS25 is quite high (26.5dB), we propose to configure maximum MCS to MCS24.

Proposal 5: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 1.

Changes to Maximum MCS for FR2-2 64QAM, Rank2 (if defined)

Decide based on simulation results in the next meeting - Option 1: MCS22 for Scaling factors 1 and 0.8. - Other Options not precluded.

Based on our simulation results, 85% of peak throughput can’t be reached when MCS is larger than 25 and we find that target SNR for MCS25 is quite high (25.6dB), we propose to configure maximum MCS to MCS24.

Proposal 6: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 2.

Minimum MCS for FR2-2 MCS to SNR mapping

- Proposals: - Option 1: MCS 0 for Rank1 and rank2; - Option 2: Depending on maximum Modulation Format: Maximum Modulation Format; Minimum MCS. 6; 14. 4; 10. 2; 4.

Minimum MCS is MCS13 in MCS to SNR mapping table in FR2 SDR requirements definition. However, in FR2-2, minimum MCS should be lower since the maximum DL testable SNR is quite low. In FR2-2, maximum DL testable SNR is 7.7dB for 100MHz CBW and -0.6dB for 400MHz CBW.  Considering the worse case -0.6dB, we propose to set minimum MCS for FR2-2 MCS to SNR mapping to 0.

Proposal 7: Set MCS0 for minimum MCS for FR2-2 MCS to SNR mapping.

Conclusion

In this paper we provide our views on SDR requirements definition. The observations and proposals are:

Proposal 1: For FR1+FR2-2 CA scenario, all ACK/NACK should be transmitted separately on each CC

Proposal 2: Define FR2-2 SDR requirements for both rank 1 and rank 2.

Observation 1: 480kHz and 960kHz SCS are optional for UE and 120kHz is mandatory for UE

Observation 2: 120 kHz SCS is more sensitive to ICI caused by phase noise which is related to minimum requirements.

Observation 3: Maximum DL testable SNR is quite low for 480kHz. I.e. <-20dB for 1600MHz and -0.6dB for 400MHz. SDR is untestable under such quite low SNR.

Proposal 3: Only consider 120kHz SCS for SDR test

Proposal 4: Only consider CPE compensation.

Proposal 5: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 1.

Proposal 6: Configure MCS24 for Scaling factor 1 for FR2-2 64QAM Rank 2.

Proposal 7: Set MCS0 for minimum MCS for FR2-2 MCS to SNR mapping.

Appendix

Table 2-2: Simulation results for SNR@85% of max TP

| MCS | 120kHz/100MHz |  | MCS | 120kHz/400MHz |  |
| --- | --- | --- | --- | --- | --- |
|  | Rank 1 | Rank 2 |  | Rank 1 | Rank 2 |
| 0 | -7.3 | -4.2 | 0 | -7.0 | -4.3 |
| 1 | -6.3 | -3.2 | 1 | -6.3 | -3.3 |
| 2 | -5.3 | -2.3 | 2 | -5.3 | -2.3 |
| 3 | -4.3 | -1.2 | 3 | -4.3 | -1.3 |
| 4 | -3.3 | -0.3 | 4 | -3.3 | -0.3 |
| 5 | -2.3 | 0.7 | 5 | -2.3 | 0.7 |
| 6 | -1.3 | 1.7 | 6 | -1.3 | 1.7 |
| 7 | -0.3 | 2.7 | 7 | -0.3 | 2.7 |
| 8 | 0.7 | 3.7 | 8 | 0.7 | 3.7 |
| 9 | 1.6 | 4.7 | 9 | 1.6 | 4.7 |
| 10 | 1.7 | 4.7 | 10 | 1.7 | 4.7 |
| 11 | 2.7 | 5.7 | 11 | 2.7 | 5.7 |
| 12 | 3.7 | 6.7 | 12 | 3.7 | 6.7 |
| 13 | 4.7 | 7.7 | 13 | 4.7 | 7.7 |
| 14 | 5.7 | 8.7 | 14 | 5.7 | 8.7 |
| 15 | 6.7 | 9.7 | 15 | 6.7 | 9.7 |
| 16 | 7.7 | 10.7 | 16 | 7.7 | 10.7 |
| 17 | 7.8 | 11.3 | 17 | 8.0 | 11.2 |
| 18 | 8.7 | 11.7 | 18 | 8.7 | 11.7 |
| 19 | 9.7 | 12.9 | 19 | 9.8 | 13.0 |
| 20 | 10.7 | 14.4 | 20 | 11.4 | 14.6 |
| 21 | 11.8 | 15.6 | 21 | 12.5 | 15.7 |
| 22 | 13.7 | 16.8 | 22 | 13.9 | 17.2 |
| 23 | 15.6 | 19.0 | 23 | 16.4 | 19.8 |
| 24 | 17.2 | 21.3 | 24 | 18.5 | 22.1 |
| 25 | 20.9 | 26.8 | 25 | 25.6 | Inf |
| 26 | Inf | Inf | 26 | Inf | Inf |
| 27 | Inf | Inf | 27 | Inf | Inf |
| 28 | Inf | Inf | 28 | Inf | Inf |

Reference

[1]  R4-2214390 WF on FR2-2 UE demodulation requirements. Qualcomm.