3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2210619

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results on FR2-2 CQI requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.3.7.2.4

Document for:	Discussion

Background

As per [1], we provide our simulation results for FR2-2 CQI requirements in this paper.

Simulation results

The simulation results for CQI distribution and BLER requirements are captured in Table 2-1 (Without Phase noise modelled) and Table 2-2(With Phase noise). We use bandwidth/SCS 100MHz/120kHz and reuse the test parameters of Rel-15 FR2 CQI AWGN test with CQI table 1 is assumed.

Table 2-1: Simulation results without phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 1 | 100% | 0 | 0 | 100% |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 87.5% |
| 12 | 10 | 100% | 0 | 0 | 88.9% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 0 | 24.0% |
| 18 | 13 | 100% | 0 | 0 | 53.8% |

Table 2-2: Simulation results with phase noise modelled

| SNR | Median CQI | Prob(Median CQI-1<CQI< Median CQI+1) | BLER of Median CQI-1 | BLER of Median CQI | BLER of Median CQI+1 |
| --- | --- | --- | --- | --- | --- |
| -6 | 0 | 100% | / | / | 0.07 |
| -4 | 2 | 100% | 0 | 0 | 100% |
| -2 | 3 | 100% | 0 | 0 | 100% |
| 0 | 4 | 100% | 0 | 0 | 100% |
| 2 | 5 | 100% | 0 | 0 | 100% |
| 4 | 6 | 100% | 0 | 0 | 100% |
| 6 | 7 | 100% | 0 | 0 | 100% |
| 8 | 8 | 100% | 0 | 0 | 100% |
| 10 | 9 | 100% | 0 | 0 | 100% |
| 12 | 10 | 100% | 0 | 0 | 100% |
| 14 | 11 | 100% | 0 | 0 | 100% |
| 16 | 12 | 100% | 0 | 11% | 100% |
| 18 | 13 | 100% | 0% | 100% | 100% |

Conclusion

In this paper we provide our simulation results for FR2-2 CQI requirements.

Reference

[1]   R4-2214390 WF on FR2-2 UE demodulation requirements. Qualcomm Incorporated