3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216034

# Electronic Meeting, Aug.15th - 26th ,2022

Title: 	Simulation results and discussions on remain issues on Rel-17 URLLC PF0 sub-slot repetition

Source: 	Huawei, HiSilicon

Agenda item:       4.7.3.1

Document for:	Discussion

Background

As per [1], RAN4 agreed to introduce PF0 requirements with sub-slot repetition:

Agreements: PUCCH format: PF0. Antenna configuration: 1T2R. SCS/CBW: 30kHz/10MHz. Channel model: TDLC-300-100 Low. UCI information bits: 1bit, HARQ-ACK payload. Number of PRBs: 1 PRB. PUCCH repetitions across “14-symbol slot” boundaries: No. Symbol level PUCCH resource configuration [Number of PUCCH symbols per sub-slot]: - Symbols per sub-slot / PUCCH length = 2 - Sub-slot repetitions (nrofSlots) = 2 - Sub-slot length (subslotLengthForPUCCH-r16) = 7 - First symbol of PUCCH (startingSymbolIndex) = 5. (sub)Slot frequency hopping: - Hop between each sub-slot, i.e., interSlotFrequencyHopping=enabled. - Do not hop within each PUCCH allocation, i.e., intraslotFrequencyHopping=disabled. - First PRB prior to frequency hopping = 0. - First PRB after frequency hopping = The largest PRB index – (Number of PRBs – 1). Other parameters: - Group and sequence hopping = neither - Hopping ID =0 - Initial cyclic shift = 0. KPIs (Issue 1-2-2) - Use Prob(DTX to ACK)<1%. - FFS: Prob(ACK miss)<1%. - FFS: Prob(PUCCH NACK to ACK bits) < 0.1%.

In this paper, we provide our simulation based on above simulation assumptions and provide our views on test metric.

Simulation results

The simulation results are captured in Figure 2-1:

Figure 2-1: Simulation results for PF0 sub-slot based repetition

Table 2-1: Summary of simulation results

| Number | Number of | Propagation conditions | Number | Channel bandwidth/SCS | Metric | Target SNR |
| --- | --- | --- | --- | --- | --- | --- |
| of TX antennas | demodulation branches | and correlation matrix (annex J) | of OFDM symbols |  |  |  |
| 1 | 2 | TDLC300-100 Low | 1 | 10MHz/30kHz | Prob(ACK miss=1%) | -2.0dB |
|  |  |  |  |  | Prob(NACK to ACK=0.1%) | -5.2dB |

The target SNR for Prob(ACK miss=1%) is -2.0dB and the target SNR for Prob(NACK to ACK=0.1%) is -5.2dB. We can observe that the target SNR for metric with ACK miss is further higher than that with NACK to ACK. Hence we propose to only define the requirements for Prob(DTX to ACK)<1% and Prob(ACK miss)<1%

Observation 1: The target SNR for metric with ACK miss is further higher than that with NACK to ACK

Proposal 1: Only define the requirements with Prob(DTX to ACK)<1% and Prob(ACK miss)<1%

Conclusion

In this paper we provide our simulation results for PF0 sub-slot repetition requirements. The observation and proposal are:

Observation 1: The target SNR for metric with ACK miss is further higher than that with NACK to ACK

Proposal 1: Only define the requirements with Prob(DTX to ACK)<1% and Prob(ACK miss)<1%

Reference

[1]  R4-2214396  WF on enhanced IIoT and URLLC support demodulation and CSI requirements. Nokia, Nokia Shanghai Bell