3GPP TSG-RAN WG4 Meeting # 104-bis-e	R4-2216029

# Electronic Meeting, Oct.10th –19th ,2022

Title: 	Simulation results for RedCap CQI requirements

Source: 	Huawei, HiSilicon

Agenda item:       4.6.5.2.1

Document for:	Discussion

Background

As per [1], RAN4 left one issue on CSI delay for FDD/HD-FDD CQI test. Interested companies can check the performance for two different CSI feedback pattern:

Agreement: CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD - Configure the following parameters for CQI feedback scheduling pattern in static/fading condition (periodic CSI reporting) for both FD-FDD and HD-FDD: - CSI-RS periodicity and offset: 10/1 - CSI-Report periodicity and offset: 10/9 - CQI/RI/PMI delay: [14ms] - Interested companies are encouraged to evaluate the performance difference between CQI delay 14ms and 10ms in RAN4#104-bis-e. If significant performance degradation is observed compared with CQI delay 10ms, RAN4 will revisit the CQI/RI/PMI delay - If CQI delay 10ms is applied, consider CSI periodicity and offset is set to 10/5.

In this paper, we provide our simulation results for these two CSI feedback pattern.

Simulation results

The simulation results are based on two CSI pattern:

- CSI pattern 1:

  - CSI-RS periodicity and offset: 10/1

  - CSI-Report periodicity and offset: 10/9

  - CQI/RI/PMI delay: 14ms

- CSI pattern 2:

  - CSI-RS periodicity and offset: 10/5

  - CSI-Report periodicity and offset: 10/9

  - CQI/RI/PMI delay: 10ms

The simulation results for FR1 FDD 1RX are captured in Table 2-1:

Table 2-1: Simulation results for FR1 FDD 1RX

| SNR(dB) | CQI delay | TP ratio of follow CQI/ Median CQI | Median CQI | Prob of(Median CQI-1<CQI< Median CQI+1) | BLER with follow CQI |
| --- | --- | --- | --- | --- | --- |
| 6 | 10ms | 1.54 | 7 | 0.42 | 0.16 |
| 6 | 14ms | 1.40 | 7 | 0.42 | 0.21 |
| 7 | 10ms | 1.60 | 8 | 0.41 | 0.16 |
| 7 | 14ms | 1.49 | 8 | 0.41 | 0.19 |

The simulation results for FR1 FDD 2RX are captured in Table 2-2:

Table 2-2: Simulation results for FR1 FDD 2RX

| SNR(dB) | CQI delay | TP ratio of follow CQI/ Median CQI | Median CQI | Prob of(Median CQI-1<CQI< Median CQI+1) | BLER with follow CQI |
| --- | --- | --- | --- | --- | --- |
| 6 | 10ms | 1.34 | 9 | 0.58 | 0.19 |
| 6 | 14ms | 1.25 | 9 | 0.58 | 0.23 |
| 7 | 10ms | 1.27 | 9 | 0.47 | 0.18 |
| 7 | 14ms | 1.20 | 9 | 0.47 | 0.22 |

Conclusion

In this paper, we provide our simulation results for RedCap CQI test.

Reference

[1]   WF on RedCap demodulation and CQI reporting requirements. Ericsson