3GPP TSG-RAN WG4 Meeting #104-bis-e	R4-2217451

Electronic Meeting, 10 ‒ 19 October, 2022

Agenda item:			6.4.2

Source:	Huawei, HiSilicon

Title:	WF on feasibility of FR2-1 multi-band BS

Document for:	Approval

Way forward

Issue 2-1: RF front-end

Agreement:

- RAN4 reached consensus on below observations:

  - Multi-band beamformer IC with common active RF components with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261 is feasible.

  - For RF front-end, commercially available TRX chips cover 24-29.5GHz. 27-41GHz RX is implemented. A harmonic-selection technique is proposed to extend the receiver’s operating bandwidth up to 24.25-71GHz.

Issue 2-2: Antenna array

Agreement:

- RAN4 reached consensus on below observation

  - Multi-band AA (Antenna Array) with common radiated element with 19.5% FBW in frequency range 24-29 GHz which includes n257/n258/n261, or with 26.3% FBW in frequency range 37-48 GHz which includes n260/n259/n262 is feasible at least from antenna array aspect; other aspects FFS.

Issue 2-3: Phase shifters

Agreement:

- RAN4 reached consensus on below observations:

  - Multi-band frequency selective phase shifter might not be soon commercially available but should not be excluded specifically at this stage

  - Multiple single band frequency selective phase shifter with common PA is possible while the impact to requirements/performance needs further study.

Issue 2-4: Frequency ranges/groups

Agreement:

- The maximum supported bandwidth and the combinations of supported band combinations/groups for FR2 multi-band RIB may be defined based on the feasibility study.