# 3GPP TSG-RAN WG4 Meeting # 104bis-e                                    R4-2217709

# Electronic Meeting, October 10 – October 19, 2022

# Source: 	ZTE, [..]

# Title: 	WF on CA_n5-n28

# Agenda Item:	6.2.3.2

# Document for:	Approval

- Background

Studying inter-band NR CA_n5-n28 band combination is one of the objectives of FS_NR_700800900 SID, which was extensive discussed in the recent two meetings.

[Information #1]: Agreements in RAN4 #104 meeting

The agreements in R4-2214445 for CA_n5-n28 were:

- UE RF architecture assumption

2 antenna, 3 antenna. The antenna number is the total number of antennas to support Main UL/DL and diversity DL for all bands

- CBW assumption

n5: 5, 10, 15, 20

n28: 5, 10, 15, 20, 25, 30

- Feasibility issues need to be analysed

The following issues can be analyzed in the feasibility study.

  - The MSD due to cross band isolation

- Other observations and proposals

The following proposal is agreed: No IMD for 2UL CA n5-n28

- UL configuration

The CBW of UL configuration CA_n5-n28 is the same as the DL. The same RB allocation with single band REFSENS UL configuration is considered as the starting point.

[Information #2]: 1st round email discussion in RAN4 #104-bis-e meeting

The moderator summary for 1st round email discussion can be found in [1], where the summary for CA_n5-n28 can be found in the table 1. It can be see there are some issues needed to be discussed in 2nd round.

Table: 1st round summary for CA_n5-n28 from [1]

|  | Status summary |
| --- | --- |
| Issue 3-1: Whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28 | According to the 1st round discussion, some companies think 2UL can reuse 1UL requirement and no new requirements are needed. Some companies think 2UL cross band ISO issue should be revisited. We have agreed in last meeting that no IMD issues for 2UL CA. So moderator recommends to revisit the 2UL cross band MSD in next meeting. Tentative agreements: Revisit the 2UL cross band MSD compared with 1UL CA in next meeting. Recommendations for 2nd round: Confirm the above tentative agreement in the WF. |
| Issue 3-2: Whether both 2 antenna and 3 antenna should be allowed for the requirement analysis | GTW agreement: - Both 2 and 3 antenna architectures will be analysed in the study item - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements. Recommendations for 2nd round: Capture the above agreement in the WF, no further discussion in 2nd round |
| Issue 3-3: RF parameters for requirements analysis | The comments for this issue are not many, but moderator would like to provide some guideline for future meetings. Tentative agreements: No agreement. Candidate options: Companies are encouraged to provide the RF parameters when the feasibility study is conducted. Recommendations for 2nd round: Confirm the above candidate option in the 2nd round. |
| Issue 3-4: Other observations and proposals | Companies provided detail comments on the detail analysis. There were some comments that CA_n5-n28 and DC_28-n5 are already specified. Tentative agreements: No agreement. Candidate options: Companies are encouraged to provide the MSD analysis for CA_n5-n28 considering the already specified 1UL CA_n5-n28 and DC_28-n5. Recommendations for 2nd round: Confirm the above candidate option in the WF. |

In terms of the moderator recommendations for 2nd round, there are three tentative agreement or candidate options should be confirm in this WF.

- Way forward

Issue 3-1: Whether the requirements for 1UL CA_n5-n28 can be reused for 2UL CA_n5-n28

Agreements: Revisit the 2UL cross band MSD compared with 1UL CA in next meeting.

Issue 3-2: Whether both 2 antenna and 3 antenna should be allowed for the requirement analysis

Agreement:

- Both 2 and 3 antenna architectures will be analysed in the study item

  - It will be decided in WI phase which one of two UE architectures will be used to specify the requirements.

Issue 3-3: RF parameters for requirements analysis

Agreements: Companies are encouraged to provide the RF parameters when the feasibility study is conducted.

Issue 3-4: Other observations and proposals

Agreements: Companies are encouraged to provide the MSD analysis for CA_n5-n28 considering the already specified 1UL CA_n5-n28 and DC_28-n5.

- Architecture assumption and multiplexer isolation performance needs to be clarified

References

[1] R4-2216961, Email discussion summary for [104-bis-e][125] FS_NR_700800900, Moderator (CATT)