# 3GPP TSG-RAN WG4 Meeting # 104-bis-e					R4-2217751

# Electronic Meeting, 10th  –19th October, 2022

Source:		Ericsson

Title:			TP for some IoT NTN RF requirement

Agenda item:		7.5.5

Document for:		Approval

# Introduction

In [1], the draft TS 36.102 is agreed and In this paper, we present some of RF requirements based on our companion discussion paper.

# TP proposal

### 6.4.1A	Frequency error for UE category M1

For category M1 UE, the basic measurement interval of modulated carrier frequency is 1 UL timeslot (0.5ms). The UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift based on received ephemeris information of the SAN in IE EphemerisInfo (TS 36.331), its own location and UL carrier frequency signalled to the UE by the SAN (according to TS36.300 clause 16.14.2).

For category M1 FD-FDD UEs and for category M1 HD-FDD UEs with continuous uplink transmissions of duration ≤ 64 ms, the mean value of basic measurements of UE pre-compensated modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of one time slot (0.5 ms) compared with [the ideally pre-compensated reference uplink carrier frequency signalled to the UE by the SAN.

For category M1 FD-FDD UEs and for category M1 HD-FDD UEs with continuous uplink transmissions of duration > 64 ms, the mean value of basic measurements of UE pre-compensated modulated carrier frequency shall be accurate within the limits in Table 6.4.1A-1 observed over a period of one time slot (0.5 ms) compared with [the ideally pre-compensated reference uplink carrier frequency signalled to the UE by the SAN.

Note: TE needs to compensate the ideally pre-compensated doppler frequency.

Editor’s note:

- The exact wording to describe the reference UL carrier frequency needs further discussion.

- The requirements with respect to segments and repetitions needs to be discussed and added

Table 6.4.1A-1: Frequency error requirement for HD-FDD UE category M1

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

When the UE supports segmented transmissions, and segmented transmission is configured [IE xxx TS 36.331], the UE shall apply same pre-compensation for the duration of the UL segment defined by Nsegmentprecompensation.

## 7.3.1A	Minimum requirements (QPSK) for UE category M1

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [A.2.2 and A.3.2] (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex [A.5.1.1]) with parameters specified in Table 7.3.1A-1 and Table 7.3.1A-2 for category M1.

Table 7.3.1A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 255 | [-102.7] | FDD |
| 256 | [-102.2] | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5- in TS 36.101. |  |  |

Table 7.3.1A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 255 | [-103.5] | HD-FDD |
| 256 | [-103] | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 in TS 36.101. |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1A-1/Table 7.3.1A-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1A-3.

NOTE:	Table 7.3.1A-3 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex [G](informative).

Table 7.3.1A-3: FDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band | NRB | Duplex Mode |
| --- | --- | --- |
| 255 | 61 | FDD and HD-FDD |
| 256 | 61 | FDD and HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1 in TS 36.102). |  |  |

# References

- R4-2215118, Draft spec for TS 36.102, MediaTek Inc