| 3GPP TR 37.718-11-21 V0.3.0 (2022-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	9

2	References	9

3	Definitions of terms, symbols and abbreviations	10

3.1	Terms	10

3.2	Symbols	10

3.3	Abbreviations	10

4	Background	10

4.1	TR Maintenance	10

5	Rel-18 DC of x bands (x = 1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL): General Part	11

5.1	UE RF architectures	11

5.2	General treatment of ∆TIB and ∆RIB values	11

6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	11

6.1	 DC_38_n28-n78	11

6.1.1	 Operating bands for DC	11

6.1.2 	Configuration for DC	12

6.1.3	 Co-existence studies	12

6.1.4	 ∆TIB and ∆RIB values	18

6.1.4	 MSD requirements	18

6.2	DC_(n)3-n8	19

6.2.1	Operating bands for DC	19

6.2.2	Channel bandwidths per operating band for DC	19

6.2.3	Co-existence studies	20

6.2.4	∆TIB and ∆RIB values	20

6.2.5	MSD	20

6.3	DC_(n)3-n77	20

6.3.1	Operating bands for DC	20

6.3.2	Channel bandwidths per operating band for DC	21

6.3.3	Co-existence studies	21

6.3.4	∆TIB and ∆RIB values	21

6.3.5	MSD	22

6.4	 DC_(n)3-n257	22

6.4.1	Operating bands for DC	22

6.4.2	Configuration for DC	22

6.4.3	Co-existence studies	22

6.4.4	∆TIB and ∆RIB values	22

6.4.5	MSD	23

6.5	 DC_1_n8-n257	23

6.5.1	Operating bands for DC	23

6.5.2	Configuration for DC	23

6.5.3	Co-existence studies	23

6.5.4	∆TIB and ∆RIB values	24

6.5.5	MSD	24

6.6	 DC_1_n77-n257	24

6.6.1	Operating bands for DC	24

6.6.2	Configuration for DC	25

6.6.3	Co-existence studies	25

6.6.4	∆TIB and ∆RIB values	25

6.6.5	MSD	25

6.7	 DC_3_n77-n257	26

6.7.1	Operating bands for DC	26

6.7.2	Configuration for DC	26

6.7.3	Co-existence studies	26

6.7.4	∆TIB and ∆RIB values	26

6.7.5	MSD	27

6.8	 DC_8_n3-n257	27

6.8.1	Operating bands for DC	27

6.8.2	Configuration for DC	27

6.8.3	Co-existence studies	27

6.8.4	∆TIB and ∆RIB values	27

6.8.5	MSD	28

6.9	 DC_3-n26-n78	28

6.9.1	 Operating bands for DC	28

6.9.2 	Channel bandwidths per operating band for DC	28

6.9.3	 Co-existence studies	29

6.9.4	 ∆TIB and ∆RIB values	30

6.9.4	 MSD requirements	30

6.10	 DC_7-n26-n78	31

6.10.1	 Operating bands for DC	31

6.10.2 	Channel bandwidths per operating band for DC	31

6.10.3	 Co-existence studies	31

6.10.4	 ∆TIB and ∆RIB values	34

6.10.4	 MSD requirements	34

6.11	 DC_20_n1-n75	35

6.11.1	Operating bands for DC	35

6.11.2	Channel bandwidths per operating band for DC	35

6.11.3	UE co-existence studies	35

6.11.4	∆TIB and ∆RIB values	36

6.11.5	MSD	37

6.12	 DC_7_n1-n75	37

6.12.1	Operating bands for DC	37

6.12.2	Channel bandwidths per operating band for DC	38

6.12.3	UE co-existence studies	38

6.12.4	∆TIB and ∆RIB values	39

6.12.5	MSD	39

6.13	 DC_1-n26-n78	39

6.13.1	 Operating bands for DC	39

6.13.2 	Channel bandwidths per operating band for DC	40

6.13.3	 Co-existence studies	40

6.13.4	 ∆TIB and ∆RIB values	43

6.13.4	 MSD requirements	43

6.14	 DC_3_n38-n40	44

6.14.1	 Operating bands for DC	44

6.14.2 	Configuration for DC	44

6.14.3	 Co-existence studies	44

6.14.4	 ∆TIB and ∆RIB values	46

6.14.5	 MSD requirements	47

6.15	DC_41_n1-n78	47

6.15.1	Configuration for DC	47

6.15.2 	Channel bandwidths per operating band for DC	47

6.15.3	Co-existence studies	49

6.15.4	 ∆TIB and ∆RIB values	50

6.15.5	MSD requirements	51

6.16	 DC_8_n38-n40	51

6.16.1	 Operating bands for DC	51

6.16.2 	Configuration for DC	51

6.16.3	 Co-existence studies	52

6.16.4	 ∆TIB and ∆RIB values	54

6.16.5	 MSD requirements	55

6.17	 DC_3_n1-n75	55

6.17.1	Operating bands for DC	55

6.17.2	Channel bandwidths per operating band for DC	55

6.17.3	UE co-existence studies	56

6.17.4	∆TIB and ∆RIB values	56

6.17.5	MSD	56

7	Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	57

7.1	 DC_1-38_n28-n78	57

7.1.1	 Operating bands for DC	57

7.1.2 	Configuration for DC	57

7.1.3	 ∆TIB and ∆RIB values	57

7.2	 DC_3-38_n28-n78	58

7.2.1	 Operating bands for DC	58

7.2.2 	Configuration for DC	58

7.2.3	 ∆TIB and ∆RIB values	59

7.3	 DC_1_(n)3-n257	59

7.3.1	Operating bands for DC	59

7.3.2	Configuration for DC	60

7.3.3	Co-existence studies	60

7.3.4	∆TIB and ∆RIB values	60

7.3.5	MSD	60

7.4	 DC_1-3_n26-n78	61

7.4.1	 Operating bands for DC	61

7.4.2 	Channel bandwidths per operating band for DC	62

7.4.3	 ∆TIB and ∆RIB values	62

7.5	 DC_1-7_n26-n78	62

7.5.1	 Operating bands for DC	62

7.5.2 	Channel bandwidths per operating band for DC	63

7.5.3	 ∆TIB and ∆RIB values	63

7.6	 DC_3-7_n26-n78	64

7.6.1	 Operating bands for DC	64

7.6.2 	Channel bandwidths per operating band for DC	66

7.6.3	 ∆TIB and ∆RIB values	66

7.7	DC_1-38_n7-n78	67

7.7.1	Operating bands for DC	67

7.7.2	Configuration for DC	67

7.7.3	Co-existence studies	67

7.7.4	∆TIB and ∆RIB values	67

7.7.5	MSD	68

7.8	 DC_7-20_n1-n75	68

7.8.1	Operating bands for DC	68

7.8.2	Channel bandwidths per operating band for DC	68

7.8.3	UE co-existence studies	69

7.8.4	∆TIB and ∆RIB values	69

7.8.5	MSD	69

7.9	 DC_3-20_n1-n75	69

7.9.1	Operating bands for DC	69

7.9.2	Channel bandwidths per operating band for DC	70

7.9.3	UE co-existence studies	70

7.9.4	∆TIB and ∆RIB values	70

7.9.5	MSD	70

7.10	 DC_3-7_n1-n75	71

7.10.1	Operating bands for DC	71

7.10.2	Configuration for DC	71

7.10.3	UE co-existence studies	71

7.10.4	∆TIB and ∆RIB values	71

7.10.5	MSD	72

7.11	 DC_1-7_n75-n78	72

7.11.1	Operating bands for DC	72

7.11.2	Configuration for DC	73

7.11.3	UE co-existence studies	73

7.11.4	∆TIB and ∆RIB values	73

7.11.5	MSD	73

7.12	 DC_3-41_n1-n78	74

7.12.1	Operating bands for DC	74

7.12.2	Channel bandwidths per operating band for DC	74

7.12.3	UE co-existence studies	74

7.12.4	∆TIB and ∆RIB values	75

7.12.5	MSD	75

7.13	 DC_8-41_n1-n78	75

7.13.1	Operating bands for DC	75

7.13.2	Channel bandwidths per operating band for DC	76

7.13.3	UE co-existence studies	76

7.13.4	∆TIB and ∆RIB values	76

7.13.5	MSD	77

7.14	 DC_20-41_n1-n78	77

7.14.1	Operating bands for DC	77

7.14.2	Channel bandwidths per operating band for DC	78

7.14.3	UE co-existence studies	78

7.14.4	∆TIB and ∆RIB values	78

7.14.5	MSD	78

7.15	 DC_3-7_n75-n78	79

7.15.1	Operating bands for DC	79

7.15.2	Configuration for DC	79

7.15.3	UE co-existence studies	79

7.15.4	∆TIB and ∆RIB values	79

7.15.5	MSD	80

7.16	 DC_1-3_n75-n78	80

7.16.1	Operating bands for DC	80

7.16.2	Configuration for DC	81

7.16.3	UE co-existence studies	81

7.16.4	∆TIB and ∆RIB values	81

7.16.5	MSD	81

8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	82

8.1	 DC_1-3-38_n28-n78	82

8.1.1	 Operating bands for DC	82

8.1.2 	Configuration for DC	83

8.1.3	 ∆TIB and ∆RIB values	83

8.2	 DC_1-3-7_n26-n78	84

8.2.1	 Operating bands for DC	84

8.2.2 	Channel bandwidths per operating band for DC	85

8.2.3	 ∆TIB and ∆RIB values	85

8.3	 DC_3-8-41_n1-n78	86

8.3.1	Operating bands for DC	86

8.3.2	Channel bandwidths per operating band for DC	87

8.3.3	UE co-existence studies	87

8.3.4	∆TIB and ∆RIB values	87

8.3.5	MSD	88

8.4	 DC_3-20-41_n1-n78	88

8.4.1	Operating bands for DC	88

8.4.2	Channel bandwidths per operating band for DC	89

8.4.3	UE co-existence studies	89

8.4.4	∆TIB and ∆RIB values	89

8.4.5	MSD	90

8.5	 DC_3-7-20_n1-n75	90

8.5.1	Operating bands for DC	90

8.5.2	Channel bandwidths per operating band for DC	91

8.5.3	UE co-existence studies	91

8.5.4	∆TIB and ∆RIB values	91

8.5.5	MSD	91

8.6	 DC_1-3-7_n75-n78	92

8.6.1	Operating bands for DC	92

8.6.2	Channel bandwidths per operating band for DC	92

8.6.3	UE co-existence studies	92

8.6.4	∆TIB and ∆RIB values	92

8.6.5	MSD	93

9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part	93

9.1	DC_UA-VA-WA-XA_nYA-nZ	93

9.1.1	Operating bands for DC	93

9.1.2	Channel bandwidths per operating band for DC	93

9.1.3	Co-existence studies	93

9.1.4	∆TIB and ∆RIB values	93

9.1.5	MSD	94

Annex A: Change history	95

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document is a technical report for Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) under Rel-18 time frame. The purpose is to gather the relevant background information and studies to address DC band combinations of inter-band CA for LTE xDL/1UL (x=1,2,3,4) and  inter-band CA for NR 2DL/1UL in Rel-18 as shown in Table 1-1.

Table 1-1: Release 18 Dual Connectivity band combination of LTE xDL/1UL (x=1,2,3,4) + inter-band NR 2DL/1UL

| Combination |  |  |
| --- | --- | --- |
| LTE X Bands (x=1,2,3,4) Combinations |  | NR 2 Bands Combinations |
| LTE 1 Band | Band x | CA_nY-nZ |
| LTE 2 Bands | CA_w-x | CA_nY-nZ |
| LTE 3 Band | CA_v-w-x | CA_nY-nZ |
| LTE 4 Bands | CA_u-v-w-x | CA_nY-nZ |

This TR contains a general part and band-specific combination part. The actual requirements are added to the corresponding technical specifications.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 30.007: “Guideline on WI/SI for new Operating Bands”

[3]	3GPP TS 38.101-1: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: “NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background

The present document is a technical report for DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) under Rel-18 time frame. It covers both the UE and BS side. The document is divided in two different parts:

- 	General part: This part covers BS and UE specific requirements which are independent of band combinations.

- 	Specific band combination part:  This part covers each band combination and its specific issues independently from each other (i.e. one subclause is defined per band combination)

The specific band combination parts are independent and therefore, the working speed also differs.

4.1	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

5	Rel-18 DC of x bands (x = 1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL): General Part

5.1	UE RF architectures

RAN4 considers the same example RF architecture which is described in section 5.1 in TR37.863-01-01.  One different point is that LTE and NR can be integrated into one chip solution from Rel-16.

To support multiple component carriers in LTE and NR, RAN4 considers specific RF components such as Diplexer, Triplexer, Quadplexer, and Hexaplexer to derive self-interference issues according to the band combinations of x bands(x = 1,2,3,4) LTE inter-band CA(xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL).

5.2	General treatment of ∆TIB and ∆RIB values

For the x bands(x = 1,2,3,4) LTE inter-band CA(xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL), RAN4 should consider reusing the agreed additional insertion losses for all DC band combinations in Rel-16 when new RF components are not introduced to support this basket WI. If the new RF components are introduced, then more detailed description will be captured according to the specific DC band combinations.

6	Dual Connectivity band combinations of LTE 1 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

6.1		DC_38_n28-n78

6.1.1	 Operating bands for DC

Table 6.1.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_38_n28-n78 | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.1.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_38A_n28A-n78A | DC_38A_n28A DC_38A_n78A |

6.1.2 	Configuration for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_38A_n28A-n78A |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.1.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_38_n28 are shown below.

Table 6.1.3-1: The harmonic and IMD products caused by UL DC_38_n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 703 | 748 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 1406 | 1496 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 2109 | 2244 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 10280 | 10480 | 2812 | 2992 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 12850 | 13100 | 3515 | 3740 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1917 | 1822 | 3273 | 3368 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 4392 | 4537 | 1214 | 1074 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5843 | 5988 | 3976 | 4116 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6962 | 7157 | 511 | 326 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 8413 | 8608 | 4679 | 4864 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3644 | 3834 | 6546 | 6736 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 422 | 192 | 9777 | 9532 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2896 | 3131 | 6454 | 6214 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5382 | 5612 | 10983 | 11228 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7249 | 7484 | 9116 | 9356 |

The harmonic and IMD products caused by UL DC_38_n78 are shown below.

Table 6.1.3-2: The harmonic and IMD products caused by UL DC_38_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2570 | 2620 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 5140 | 5240 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 7710 | 7860 | 9900 | 11400 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 680 | 1230 | 5870 | 6420 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1340 | 1940 | 3980 | 5030 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8440 | 9040 | 9170 | 10220 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3910 | 4560 | 7280 | 8830 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 11010 | 11660 | 12470 | 14020 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2460 | 1360 | 11740 | 12840 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12630 | 10580 | 7180 | 6480 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6260 | 4660 | 1260 | 110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15770 | 17820 | 13580 | 14280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 15040 | 16640 | 14310 | 15460 |

The IMD2 and IMD4 interference of UL DC_38_n28 may have an impact on the Rx of band n78.

The IMD2 and IMD5 interference of UL DC_38_n78 may have an impact on the Rx of band n28.

6.1.4	 ∆TIB and ∆RIB values

Referring to DC_41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.1.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_38_n28-n78 | 0.3 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.1.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_38-n28-n78 | - | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.1.4	 MSD requirements

Since DC_38_n28-n78 share the similar RF architecture of DC_41_n28-n78, the MSD requirements can be specified as below, referring to DC_41A_n28A-n78A.

Table 6.1.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n28 | 745 | 5 | 25 | 798 | N/A | N/A |
| DC_38A_n28A-n78A | n78 | 3360 | 10 | 25 | 3360 | 28.2 | IMD29 |
|  | 38 | 2615 | 5 | 25 | 2615 | N/A | N/A |
|  | n28 | 730 | 5 | 25 | 785 | 30.8 | IMD24 |
|  | n78 | 3400 | 10 | 25 | 3400 | N/A | N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 9: This band is subject to IMD4 also which MSD is not specified. |  |  |  |  |  |  |  |

6.2	DC_(n)3-n8

6.2.1	Operating bands for DC

Table 6.2.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3AA-n8A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |

Table 6.2.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)3AA-n8A | DC_(n)3AA1 DC_3A_n8A |
| NOTE1: Only single switched UL is supported. |  |

6.2.2	Channel bandwidths per operating band for DC

Table 6.2.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n8A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 70 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n8 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.2.3	Co-existence studies

The studies are already performed for DC_(n)3AA and DC_3A_n8A, in TR 37.717-11-11 and TR 37.716-11-11, respectively.

6.2.4	∆TIB and ∆RIB values

For DC_(n)3-n8, the TIB,c and RIB,c values are given in the tables below.

Table 6.2.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n8 | 0.3 | - | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.2.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n8 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.2.5	MSD

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_(n)3AA-n8A | 8 | 897.5 | 5 | 25 | 942.5 | N/A | N/A |
|  | 3 | N/A | 5 | N/A | 1837.5 | [6.4] | IMD5 |
|  | n3 | 1747.5 | 5 | 25 | 1842.5 | [6.4] | IMD5 |

6.3	DC_(n)3-n77

6.3.1	Operating bands for DC

Table 6.3.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3AA-n77A | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |

Table 6.3.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)3AA-n77A | DC_(n)3AA1 DC_3A_n77A |
| DC_(n)3AA-n77(2A) | DC_(n)3AA1 DC_3A_n77A |
| NOTE1: Only single switched UL is supported. |  |

6.3.2	Channel bandwidths per operating band for DC

Table 6.3.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_(n)3AA-n77A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
| DC_(n)3AA-n77(2A) |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 250 |
|  |  | n3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n77 | See CA_n77(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 in TS38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.3.3	Co-existence studies

The studies are already performed for DC_(n)3AA and DC_3A_n77A, in TR 37.717-11-11 and TR 37.863-01-01, respectively.

6.3.4	∆TIB and ∆RIB values

For DC_(n)3_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.3.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n77 | - | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.3.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n77 | - | 0.2 | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.3.5	MSD

MSD analysis is already available for DC_(n)3AA and DC_3A_n77A, in TR 37.717-11-11 and TR 37.863-01-01, respectively.

6.4	 DC_(n)3-n257

6.4.1	Operating bands for DC

Table 6.4.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_(n)3-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.4.2	Configuration for DC

Table 6.4.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_(n)3AA-n257A DC_(n)3AA-n257G DC_(n)3AA-n257H DC_(n)3AA-n257I DC_(n)3AA-n257J DC_(n)3AA-n257K DC_(n)3AA-n257L DC_(n)3AA-n257M | DC_(n)3AA1 DC_3A_n257A |
| NOTE1: Only single switched UL is supported. |  |

6.4.3	Co-existence studies

The studies are already performed for DC_(n)3AA in TR 37.717-11-11.

No study is needed for DC_3A_n257A, due to large frequency separation.

6.4.3	Co-existence studies

The studies are already performed for DC_(n)3AA in TR 37.717-11-11.

No study is needed for DC_3A_n257A, due to large frequency separation.

6.4.4	∆TIB and ∆RIB values

For DC_(n)3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.4.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_(n)3-n257 | - | 0.6 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.4.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_(n)3-n257 | - | 0.2 | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.4.5	MSD

MSD analysis is already available for DC_(n)3AA in TR 37.717-11-11.

No MSD analysis is needed for DC_3A_n257A, as there is no coexistence issue.

6.5	 DC_1_n8-n257

6.5.1	Operating bands for DC

Table 6.5.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n8-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.5.2	Configuration for DC

Table 6.5.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n8A-n257A DC_1A_n8A-n257G DC_1A_n8A-n257H DC_1A_n8A-n257I DC_1A_n8A-n257J DC_1A_n8A-n257K DC_1A_n8A-n257L DC_1A_n8A-n257M | DC_1A_n8A DC_1A_n257A |

6.5.3	Co-existence studies

For DC_1A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_1A_n8A is already given in TR 37.716-11-11.

6.5.4	∆TIB and ∆RIB values

For DC_1_n8-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.5.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1_n8-n257 |  | 0.6 |  | 0.3 |  | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.5.4-2: ΔRIB

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_1_n8-n257 |  | 0.2 |  | - |  | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.5.5	MSD

MSD analysis is already available for DC_1A_n8A is already given in TR 37.716-11-11.

No MSD analysis is needed for DC_1A_n257A, as there is no coexistence issue.

6.6	 DC_1_n77-n257

6.6.1	Operating bands for DC

Table 6.6.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n77-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.6.2	Configuration for DC

Table 6.6.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_n77A-n257J DC_1A_n77A-n257K DC_1A_n77A-n257L DC_1A_n77A-n257M DC_1A_n77A-n257A | DC_1A_n77A DC_1A_n257A |

6.6.3	Co-existence studies

For DC_1A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_1A_n77A is already given in TR 37.863-01-01.

6.6.4	∆TIB and ∆RIB values

For DC_1_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.6.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1_n77-n257 |  | 0.6 |  | 0.8 |  | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.6.4-2: ΔRIB

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_1_n77-n257 |  | 0.2 |  | 0.5 |  | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.6.5	MSD

MSD analysis is already available for DC_1A_n77A is already given in TR 37.863-01-01.

No MSD analysis is needed for DC_1A_n257A, as there is no coexistence issue.

6.7	 DC_3_n77-n257

6.7.1	Operating bands for DC

Table 6.7.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_n77-n257 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.7.2	Configuration for DC

Table 6.7.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n77A-n257G DC_3A_n77A-n257H DC_3A_n77A-n257I DC_3A_n77A-n257J DC_3A_n77A-n257K DC_3A_n77A-n257L DC_3A_n77A-n257M DC_3A_n77(2A)-n257A DC_3A_n77(2A)-n257G DC_3A_n77(2A)-n257H DC_3A_n77(2A)-n257I DC_3A_n77(2A)-n257J DC_3A_n77(2A)-n257K DC_3A_n77(2A)-n257L DC_3A_n77(2A)-n257M | DC_3A_n77A DC_3A_n257A |

6.7.3	Co-existence studies

For DC_3A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_3A_n77A is already given in TR 37.863-01-01.

6.7.4	∆TIB and ∆RIB values

For DC_3_n77-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.7.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1_n77-n257 |  | 0.6 |  | 0.8 |  | 0 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.7.4-2: ΔRIB

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_1_n77-n257 |  | 0.2 |  | 0.5 |  | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.7.5	MSD

MSD analysis is already available for DC_3A_n77A is already given in TR 37.863-01-01.

No MSD analysis is needed for DC_3A_n257A, as there is no coexistence issue.

6.8	 DC_8_n3-n257

6.8.1	Operating bands for DC

Table 6.8.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n3-n257 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

6.8.2	Configuration for DC

Table 6.8.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n3A-n257A DC_8A_n3A-n257G DC_8A_n3A-n257H DC_8A_n3A-n257I DC_8A_n3A-n257J DC_8A_n3A-n257K DC_8A_n3A-n257L DC_8A_n3A-n257M | DC_8A_n3A DC_8A_n257A |

6.8.3	Co-existence studies

For DC_8A_n257A there is no coexistence issue, as frequency separation is high.

The coexistence studies of DC_8A_n3A is already given in TR 37.716-11-11.

6.8.4	∆TIB and ∆RIB values

For DC_1_n8-n257, the TIB,c and RIB,c values are given in the tables below.

Table 6.8.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_8_n3-n257 |  | 0.3 |  | 0.3 |  | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.8.4-2: ΔRIB

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_8_n3-n257 |  | - |  | - |  | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.8.5	MSD

MSD analysis is already available for DC_8A_n3A is already given in TR 37.716-11-11.

No MSD analysis is needed for DC_8A_n257A, as there is no coexistence issue.

6.9		DC_3-n26-n78

6.9.1	 Operating bands for DC

Table 6.9.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-n26-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.9.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-n26A-n78A | DC_3A_n26A DC_3A_n78A |
| DC_3C_n26A-n78A | DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |

6.9.2 	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-n26A-n78A |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3C_n26A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.9.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_3_n26 are shown below.

Table 6.9.3-1: The harmonic and IMD products caused by UL DC_3_n26

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5130 | 5355 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 6840 | 7140 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 971 | 861 | 2524 | 2634 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 157 | 12 | 2571 | 2756 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3338 | 3483 | 4234 | 4419 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1942 | 1722 | 5048 | 5268 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 657 | 837 | 4281 | 4541 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4152 | 4332 | 5944 | 6204 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6326 | 5991 | 1686 | 1471 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7654 | 7989 | 4966 | 5181 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3727 | 3432 | 873 | 1128 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6758 | 7053 | 5862 | 6117 |

The harmonic and IMD products caused by UL DC_3_n78 are shown below.

6.9.4	 ∆TIB and ∆RIB values

Referring to DC_3-5_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.9.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-n26-n78 |  | 0.6 |  | 0.6 |  | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.9.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_3-n26-n78 |  | 0.2 |  | 0.2 |  | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.9.4	 MSD requirements

Referring to CA_n3-n5-n78, the MSD requirements can be specified as below:

Table 6.9.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n26A-n78A DC_3C_n26A-n78A | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n26 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n78 | 3408 | 10 | 50 | 3408 | 16.1 | IMD3 |
|  | 3 | 1730 | 5 | 25 | 1825 | N/A | N/A |
|  | n26 | 839 | 5 | 25 | 884 | N/A | N/A |
|  | n78 | 3512 | 10 | 50 | 3512 | 4.5 | IMD5 |

6.10		DC_7-n26-n78

6.10.1	 Operating bands for DC

Table 6.10.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-n26-n78 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.10.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_7A-n26A-n78A | DC_7A_n26A DC_7A_n78A |
| DC_7C_n26A-n78A | DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |

6.10.2 	Channel bandwidths per operating band for DC

Table 6.1.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_7A_n26A-n78A |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_7C_n26A-n78A |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.10.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_7_n26 are shown below.

Table 6.10.3-1: The harmonic and IMD products caused by UL DC_7_n26

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 10000 | 10280 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 12500 | 12850 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1756 | 1651 | 3314 | 3419 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 942 | 802 | 4151 | 4326 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4128 | 4268 | 5814 | 5989 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3512 | 3302 | 6628 | 6838 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 128 | 47 | 6651 | 6896 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4942 | 5117 | 8314 | 8559 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9466 | 9151 | 896 | 686 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10814 | 11129 | 5756 | 5966 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6082 | 5802 | 2453 | 2698 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9128 | 9408 | 7442 | 7687 |

The harmonic and IMD products caused by UL DC_7_n78 are shown below.

Table 6.10.3-2: The harmonic and IMD products caused by UL DC_7_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 5000 | 5140 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 7500 | 7710 | 7500 | 7710 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 10000 | 10280 | 13200 | 15200 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 12500 | 12850 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1300 | 730 | 5800 | 6370 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1200 | 1840 | 4030 | 5100 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8300 | 8940 | 9100 | 10170 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2600 | 1460 | 11600 | 12740 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3700 | 4410 | 7330 | 8900 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10800 | 11510 | 12400 | 13970 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12700 | 10630 | 6980 | 6200 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15700 | 17770 | 13300 | 14080 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6400 | 4760 | 1110 | 100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14900 | 16540 | 14100 | 15310 |

The IMD2 and IMD4 interference of UL DC_7_n26 may have an impact on the Rx of band n78.

The IMD2 and IMD5 interference of UL DC_7_n78 may have an impact on the Rx of band n26.

6.10.4	 ∆TIB and ∆RIB values

Referring to DC_5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.10.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_7_n26-n78 |  | 0.6 |  | 0.6 |  | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.10.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_7_n26-n78 |  | 0.2 |  | 0.2 |  | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.10.4	 MSD requirements

Referring to CA_n7-n7-n78, the MSD requirements can be specified as below:

Table 6.10.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n26A-n78A DC_7C_n26A-n78A | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n26 | 834 | 5 | 25 | 879 | 30.2 | IMD2 |
|  | n78 | 3429 | 10 | 50 | 3429 | N/A | N/A |
|  | 7 | 2525 | 5 | 25 | 2645 | N/A | N/A |
|  | n26 | 830 | 5 | 25 | 875 | 3.3 | IMD5 |
|  | n78 | 3350 | 10 | 50 | 3350 | N/A | N/A |
|  | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n26 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3375 | 10 | 50 | 3375 | 29.7 | IMD2 |
|  | 7 | 2550 | 5 | 25 | 2670 | N/A | N/A |
|  | n26 | 835 | 5 | 25 | 880 | N/A | N/A |
|  | n78 | 3430 | 10 | 50 | 3430 | 9.7 | IMD4 |

6.11	 DC_20_n1-n75

6.11.1	Operating bands for DC

Table 6.11.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_20_n1-n75 | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

6.11.2	Channel bandwidths per operating band for DC

Table 6.11.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_20A_n1A-n75A | DC_20A_n1A | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.11.3	UE co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_20_n1-n75 with dual uplink on DC_20_n1 are listed in the table below.

Table 6.11.3-1: The harmonic and IMD products caused by UL DC_20_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 832 | 862 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1664 | 1724 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2496 | 2586 | 5760 | 5940 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1058 | 1148 | 2752 | 2842 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 316 | 196 | 2978 | 3128 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3584 | 3704 | 4672 | 4822 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 516 | 606 | 4898 | 5108 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4416 | 4566 | 6592 | 6802 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 2116 | 2296 | 5504 | 5684 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7088 | 6818 | 1528 | 1348 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8512 | 8782 | 5248 | 5428 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4276 | 4036 | 1254 | 1464 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7424 | 7664 | 6336 | 6546 |

Based on co-existence studies, IMD5 from UL 20_n1 may impact DL n75.

6.11.4	∆TIB and ∆RIB values

For DC_20_n1-n75, the TIB,c and RIB,c values can reuse the values of DC_20-32_n1 as given in the tables below.

Table 6.11.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_20_n1-n75 |  | 0.3 |  | 0.5 |  | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.11.4-2: ΔRIB,c

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_20_n1-n75 |  | - |  | - |  | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.11.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_20A-32A_n1A.

Table 6.11.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_20A-n1A_n75A | n1 | 1950.5 | 5 | 50 | 2140.5 | N/A | N/A |
|  | 20 | 852.5 | 5 | 25 | 811.5 | N/A | N/A |
|  | n75 | N/A | 5 | N/A | 1459.5 | 4.0 | IMD5 |

6.12	 DC_7_n1-n75

6.12.1	Operating bands for DC

Table 6.12.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7_n1-n75 | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

6.12.2	Channel bandwidths per operating band for DC

Table 6.12.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A_n1A-n75A | DC_7A_n1A | 7 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.12.3	UE co-existence studies

The 2nd and 3rd order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products for the UE coexistence analysis of DC_7A_n1A-n75A with dual uplink on DC_7A_n1A are listed in the table below.

Table 6.12.3-1: The harmonic and IMD products caused by UL DC_7_n1

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2500 | 2570 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3840 | 3960 | 5000 | 5140 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 7500 | 7710 |
| Two tone 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 520 | 650 | 4420 | 4550 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1270 | 1460 | 3020 | 3220 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6340 | 6530 | 6920 | 7120 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | ¦3*fy_low – fx_high¦ | ¦3*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3190 | 3370 | 5520 | 5790 |
| Two-tone 4th order IMD products | ¦3*fx_low + fy_low¦ | ¦3*fx_high + fy_high¦ | ¦3*fy_low + fx_low¦ | ¦3*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8260 | 8510 | 9420 | 9690 |
| Two-tone 4th order IMD products | ¦(2*fy_low – 2*fx_high)¦ | ¦(2*fy_high – 2*fx_low)¦ | ¦2*fx_low + 2*fy_low¦ | ¦2*fx_high + 2*fy_high¦ |
| IMD frequency limits (MHz) | 1040 | 1300 | 8840 | 9100 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8360 | 8020 | 5420 | 5110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11920 | 12260 | 10180 | 10490 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3870 | 3540 | 940 | 620 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11340 | 11670 | 10760 | 11080 |

Based on co-existence studies, IMD3 from UL 7_n1 may impact DL n75.

6.12.4	∆TIB and ∆RIB values

For DC_7A_n1A-n75A, the TIB,c and RIB,c values can reuse the values of DC_7A-32A_n1A as given in the tables below.

Table 6.12.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_7A_n1A-n75A |  | 0.6 |  | 0.5 |  | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.12.4-2: ΔRIB,c

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_7A_n1A-n75A |  | - |  | - |  | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.12.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_7A-32A_n1A.

Table 6.12.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n1A-n75A | n1 | 1977.5 | 5 | 25 | 2167.5 | N/A | N/A |
|  | 7 | 2502.5 | 5 | 25 | 2622.5 | N/A | N/A |
|  | 75 | N/A | 5 | N/A | 1454.5 | 15.2 | IMD3 |

6.13		DC_1-n26-n78

6.13.1	 Operating bands for DC

Table 6.13.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.13.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-n26A-n78A | DC_1A_n26A DC_1A_n78A |

6.13.2 	Channel bandwidths per operating band for DC

Table 6.13.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.13.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_1_n26 are shown below.

Table 6.13.3-1: The harmonic and IMD products caused by UL DC_1_n26

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 814 | 849 | 1920 | 1980 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 1628 | 1698 | 3840 | 3960 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 2442 | 2547 | 5760 | 5940 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 3256 | 3396 | 7680 | 7920 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 4070 | 4245 | 9600 | 9900 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1166 | 1071 | 2734 | 2829 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 352 | 222 | 2991 | 3146 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3548 | 3678 | 4654 | 4809 |
| Two-tone 4th order IMD products | ¦2*fx_low –2*y_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2332 | 2142 | 5468 | 5658 |
| Two-tone 4th order IMD products | ¦3*fx_low –1*fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – *fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 462 | 627 | 4911 | 5126 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4362 | 4527 | 6574 | 6789 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7106 | 6831 | 1476 | 1276 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + *fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 8494 | 8769 | 5176 | 5376 |
| Two-tone 5th order IMD products | ¦2*fx_low – *fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – *fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 4312 | 4062 | 1293 | 1518 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 7388 | 7638 | 6282 | 6507 |

The harmonic and IMD products caused by UL DC_1_n78 are shown below.

Table 6.13.3-2: The harmonic and IMD products caused by UL DC_1_n78

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 3300 | 3800 |
| 2nd harmonics frequency limits | 2* fy_low | 2* fy_high | 2*fx_low | 2*fx_high |
| 2nd harmonics frequency limits(MHz) | 3840 | 3960 | 6600 | 7600 |
| 3rd harmonics frequency limits | 3* fy_low | 3* fy_high | 3*fx_low | 3*fx_high |
| 3rd harmonics frequency limits (MHz) | 5760 | 5940 | 9900 | 11400 |
| 4th harmonics frequency limits | 4* fy_low | 4* fy_high | 4*fx_low | 4*fx_high |
| 4th harmonics frequency limits (MHz) | 7680 | 7920 | 13200 | 15200 |
| 5th harmonics frequency limits | 5* fy_low | 5* fy_high | 5*fx_low | 5*fx_high |
| 5th harmonics frequency limits (MHz) | 9600 | 9900 | 16500 | 19000 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1880 | 1320 | 5220 | 5780 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 40 | 660 | 4620 | 5680 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 7140 | 7760 | 8520 | 9580 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3760 | 2640 | 10440 | 11560 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 | 2640 | 7920 | 9480 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 | 9740 | 11820 | 13380 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 13280 | 11220 | 4620 | 3880 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 | 17180 | 10980 | 11720 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 7560 | 5940 | 660 | 1840 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 | 15360 | 12360 | 13540 |

The IMD3 interference of UL DC_1_n26 may have an impact on the Rx of band n78.

The IMD5 interference of UL DC_1_n78 may have an impact on the Rx of band n26.

6.13.4	 ∆TIB and ∆RIB values

Referring to DC_1-5_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.13.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-n26-n78 |  | 0.3 |  | 0.6 |  | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |  |  |  |

Table 6.13.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration8 |  |  |  |  |
| DC_1-n26-n78 |  | 0.2 |  | 0.2 |  | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |  |  |  |

6.13.4	 MSD requirements

Referring to CA_n1-n5-n78, the MSD requirements can be specified as below:

Table 6.13.4-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n26A-n78A | n1 | 1950 | 5 | 25 | 2140 | N/A | N/A |
|  | n26 | 830 | 5 | 25 | 875 | N/A | N/A |
|  | n78 | 3610 | 10 | 50 | 3610 | 15.7 | IMD3 |
|  | 1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | n26 | 840 | 5 | 25 | 885 | 3.1 | IMD5 |
|  | n78 | 3405 | 10 | 50 | 3405 | N/A | N/A |

6.14		DC_3_n38-n40

6.14.1	 Operating bands for DC

Table 6.14.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n38-n40 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.14.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A_n38A-n40A | DC_3A_n38A DC_3A_n40A |

6.14.2 	Configuration for DC

Table 6.14.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A_n38A-n40A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.14.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_3_n38 are shown below.

Table 6.14.3-1: The harmonic and IMD products caused by UL DC_3_n38

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 12850 | 13100 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10710 | 15420 | 15720 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12495 | 17990 | 18340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 785 | 910 | 4280 | 4405 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 800 | 1000 | 3355 | 3530 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5990 | 6190 | 6850 | 7025 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2510 | 2785 | 5925 | 6150 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1820 | 1570 | 8560 | 8810 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7700 | 7975 | 9420 | 9645 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8770 | 8495 | 4570 | 4220 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 4440 | 4140 | 215 | 110 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11990 | 12265 | 9410 | 9760 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 11130 | 11430 | 10270 | 10595 |

The harmonic and IMD products caused by UL DC_3_n40 are shown below.

Table 6.14.3-2: The harmonic and IMD products caused by UL DC_3_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 1710 | 1785 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3570 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5355 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7140 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8925 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 10260 | 10710 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 11970 | 12495 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 515 | 690 | 4010 | 4185 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1020 | 1270 | 2815 | 3090 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5720 | 5970 | 6310 | 6585 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 2730 | 3055 | 5115 | 5490 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1380 | 1030 | 8020 | 8370 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7430 | 7755 | 8610 | 8985 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 7890 | 7415 | 4840 | 4440 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 3780 | 3330 | 755 | 330 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10910 | 11385 | 9140 | 9540 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10320 | 10770 | 9730 | 10155 |

There are no intermodulation products of DC_3_n38 falling inside the RX band of n40.

There are no intermodulation products of DC_3_n40 falling inside the RX band of n38

6.14.4	 ∆TIB and ∆RIB values

Referring to CA_n3A-n38A-n40A, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.14.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n38-n40 | 0.5 | 0.53 | 0.5 |
| NOTE 3: The requirement is applied for UE transmitting on the frequency range of 2515 – 2690 MHz. NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.14.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n38-n40 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.14.5	 MSD requirements

All MSD requirements for DC_3_n38-n40 are already covered by constituent fallback modes. No additional MSD requirement is needed.

## 6.15	DC_41_n1-n78

6.15.1	Configuration for DC

Table 6.15.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_41_n1-n78 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 6.15.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_41A_n1A-n78A | DC_41A_n1A DC_41A_n78A |
| DC_41C_n1A-n78A | DC_41A_n1A DC_41A_n78A |

6.15.2 	Channel bandwidths per operating band for DC

Table 6.15.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  |  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  |  | E-UTRA and NR Band |  | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_41A_n1A-n78A |  |  | 41 |  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |  | 170 |
|  |  |  | n1 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_41C_n1A-n78A |  |  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  |  | n1 |  | 15 | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |  |
|  |  |  | n78 |  | 15 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |  |
|  |  |  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |  |

6.15.3	Co-existence studies

Table 6.15.3-1: Band 41 and Band n78 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 3300 | 3800 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1304 | 610 | 5796 | 6490 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 1192 | 2080 | 3910 | 5104 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 8292 | 9180 | 9096 | 10290 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2608 | 1220 | 11592 | 12980 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 3688 | 4770 | 7210 | 8904 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 10788 | 11870 | 12396 | 14090 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 12704 | 10510 | 7460 | 6184 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15696 | 17890 | 13284 | 14560 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 6408 | 4520 | 1470 | 112 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 14892 | 16780 | 14088 | 15670 |

Table 6.15.3-2: Band 41 and Band n1 IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 1920 | 1980 |
| Two tone 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 516 | 770 | 4416 | 4670 |
| Two-tone 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3012 | 3460 | 1150 | 1464 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 6912 | 7360 | 6336 | 6650 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1032 | 1540 | 8832 | 9340 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 5508 | 6150 | 3070 | 3444 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9408 | 10050 | 8256 | 8630 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 5424 | 4990 | 8840 | 8004 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10176 | 10610 | 11904 | 12740 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 948 | 380 | 4230 | 3528 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 10752 | 11320 | 11328 | 12030 |

As can be seen in Table 6.15.3-1 there are IMD4 impact from UL 41_n78 affecting DL band 1.

As can be seen in Table 6.15.3-2 there are IMD3 impact from UL 41_n1 affecting DL band n78.

### 6.15.4		∆TIB and ∆RIB values

For DC_41_n1-n78, the TIB,c and RIB,c values are reused from DC_1-41_n78 and are given in the tables below.

Table 6.15.4-1:ΔTIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_41_n1-n78 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.15.4-2:ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_41_n1-n78 | - | - | 0.5 |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

### 6.15.5	MSD requirements

MSD value band n1 are reused from DC_1A-41A_n78A.

MSD value band n78 is reused from DC_1A_n41A-n78A.

Table 6.15.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_41A_n1A-n78A | n1 | 1950 | 5 | 25 | 2140 | 9.3 | IMD4 |
| DC_41C_n1A-n78A | 41 | 2640 | 5 | 25 | 2640 | N/A | N/A |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | N/A |
|  | n1 | 1970 | 5 | 25 | 2160 | N/A | N/A |
|  | 41 | 2650 | 10 | 25 | 2650 | N/A | N/A |
|  | n78 | 3330 | 10 | 50 | 3330 | 19.6 | IMD3 |

6.16		DC_8_n38-n40

6.16.1	 Operating bands for DC

Table 6.16.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_8_n38-n40 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |

Table 6.16.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A_n38A-n40A | DC_8A_n38A DC_8A_n40A |

6.16.2 	Configuration for DC

Table 6.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_8A_n38A-n40A |  | 8 | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  | 150 |
|  |  | n38 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  | n40 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

6.16.3	 Co-existence studies

The harmonic and IMD products caused by UL DC_8_n38 are shown below.

Table 6.16.3-1: The harmonic and IMD products caused by UL DC_8_n38

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 880 | 915 | 2570 | 2620 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 5140 | 5240 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 7710 | 7860 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 10280 | 10480 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 12850 | 13100 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 | 5490 | 15420 | 15720 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 6160 | 6405 | 17990 | 18340 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1655 | 1740 | 3450 | 3535 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 860 | 740 | 4225 | 4360 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4330 | 4450 | 6020 | 6155 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 20 | 175 | 6795 | 6980 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3480 | 3310 | 6900 | 7070 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 5210 | 5365 | 8590 | 8775 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 9600 | 9365 | 1090 | 900 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 6100 | 5880 | 2395 | 2600 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 11160 | 11395 | 6090 | 6280 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 9470 | 9690 | 7780 | 7985 |

The harmonic and IMD products caused by UL DC_8_n40 are shown below.

Table 6.16.3-2: The harmonic and IMD products caused by UL DC_8_n40

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL Frequency [MHz] | 880 | 915 | 2300 | 2400 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1760 | 1830 | 4600 | 4800 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2640 | 2745 | 6900 | 7200 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3520 | 3660 | 9200 | 9600 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 4400 | 4575 | 11500 | 12000 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 5280 | 5490 | 13800 | 14400 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 6160 | 6405 | 16100 | 16800 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1385 | 1520 | 3180 | 3315 |
| 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 640 | 470 | 3685 | 3920 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 4060 | 4230 | 5480 | 5715 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 240 | 445 | 5985 | 6320 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 3040 | 2770 | 6360 | 6630 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4940 | 5145 | 7780 | 8115 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 8720 | 8285 | 1360 | 1120 |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5440 | 5070 | 1855 | 2160 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 10080 | 10515 | 5820 | 6060 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 8660 | 9030 | 7240 | 7545 |

The 5th order IMD product 2*2572MHz – 3*912MHz = 2408MHz will have a BW of the interference of 25MHz, which falls inside n40 upper channel.

There are no intermodulation products of DC_8_n40 falling inside the RX band of n38.

6.16.4	 ∆TIB and ∆RIB values

Referring to CA_n8A-n38A-n40A, ΔTIB,c and ΔRIB,c can be specified as below.

Table 6.16.4-1: ΔTIB,c due to EN-DC(three bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_8_n38-n40 | 0.3 | 0.3 | 0.3 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.16.4-2: ΔRIB,c due to EN-DC (three bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_8_n38-n40 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.16.5	 MSD requirements

Referring to CA_n8-n38-n40, the MSD requirement that addresses the 5th order product has been omitted. The reason being that CA_ n38-n40 has Note 9 applied meaning it is only applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. Following same reasoning no MSD have been defined for the IMD5.

6.17	 DC_3_n1-n75

6.17.1	Operating bands for DC

Table 6.17.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

6.17.2	Channel bandwidths per operating band for DC

Table 6.17.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A_n1A-n75A | DC_3A_n1A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 120 |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C_n1A-n75A | DC_3C_n1A | 3 |  | 15 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  | 140 |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

6.17.3	UE co-existence studies

Based on co-existence studies, IMD3 from UL 3_n1 may impact DL n1.

Based on co-existence studies, IMD3 and IMD5 from UL 3_n1 may impact DL n75

6.17.4	∆TIB and ∆RIB values

For DC_3_n1-n75, the TIB,c and RIB,c values can reuse the values of DC_1_n3-n75 as given in the tables below.

Table 6.17.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |
| DC_3_n1-n75 | 0.5 | 0.5 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_66_(n)12 the band order from left to right is 12, 66 and n12. |  |  |  |

Table 6.17.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)7 |  |  |
| --- | --- | --- | --- |
|  | Component band in order of bands in configuration8 |  |  |
| DC_3_n1-n75 | - | - | - |
| NOTE 7: “-” denotes ΔRIB,c = 0. NOTE 8: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_5_(n)12 the band order from left to right is 5, 12 and n12. |  |  |  |

6.17.5	MSD

Based on co-existence studies MSD need to be defined. MSD values are reused from DC_1_n3-n75.

Table 6.17.5-1: MSD test points for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)NR or E-UTRA

| Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n75A | n75 | N/A | 5 | 25 | 1480 | 15.2 | IMD34 |
|  | n1 | 1960 | 5 | 25 | 2150 | N/A | N/A |
|  | 3 | 1720 | 5 | 25 | 1815 | N/A | N/A |
| DC_3C_n1A-n75A | n75 | N/A | 5 | 25 | 1495 | [FFS] | IMD3 |
|  | n1 | 1975 | 5 | 25 | 2165 | N/A | N/A |
|  | 3 | 1730 1749.8 | 20 20 | 1(RBSTART=20) 1(RBSTART=79) | 1825 1844.8 | N/A N/A | N/A N/A |
| NOTE 4: This band is subject to IMD5 also which MSD is not specified. |  |  |  |  |  |  |  |

7	Dual Connectivity band combinations of LTE 2 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

7.1		DC_1-38_n28-n78

7.1.1	 Operating bands for DC

Table 7.1.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-38_n28-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.1.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_38A_n28A DC_38A_n78A |

7.1.2 	Configuration for DC

Table 7.1.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-38A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.1.3	 ∆TIB and ∆RIB values

Referring to DC_1-41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.1.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-38_n28-n78 | 0.5 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.1.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-38_n28-n78 | - | - | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.2		DC_3-38_n28-n78

7.2.1	 Operating bands for DC

Table 7.2.1-1: LTE 2 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-38_n28-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.2.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-38A_n28A-n78A | DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_3C-38A_n28A-n78A | DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |

7.2.2 	Configuration for DC

Table 7.2.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-38A_n28A-n78A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_3C-38A_n28A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.2.3	 ∆TIB and ∆RIB values

Referring to DC_3-41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.2.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-38_n28-n78 | 1.0 | 0.3 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.2.3-2: ΔRIB,c due to EN-DC (fourbands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-38_n28-n78 | 0,5 | 0.4 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.3	 DC_1_(n)3-n257

7.3.1	Operating bands for DC

Table 7.3.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1_(n)3-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n275 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |

7.3.2	Configuration for DC

Table 7.3.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A_(n)3AA-n257A DC_1A _(n)3AA-n257G DC_1A _(n)3AA-n257H DC_1A _(n)3AA-n257I DC_1A _(n)3AA-n257J DC_1A _(n)3AA-n257K DC_1A _(n)3AA-n257L DC_1A _(n)3AA-n257M | DC_1A_n3A DC_1A_n257A DC_(n)3AA1 DC_3A_n257A |
| NOTE1: Only single switched UL is supported. |  |

7.3.3	Co-existence studies

The studies are already performed for DC_1A_n3A in TR 37.716-11-11.

No study is needed for DC_1A_n257A, due to large frequency separation.

The studies are already performed for DC_(n)3AA in TR 37.717-11-11.

No study is needed for DC_3A_n257A, due to large frequency separation.

7.3.4	∆TIB and ∆RIB values

For DC_(n)3-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.3.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1_(n)3-n257 | - | - | 0.6 | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.3.4-2: ΔRIB

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1_(n)3-n257 | - | - | 0.2 | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.3.5	MSD

MSD analysis is already available for DC_(n)3AA and DC_1A_n3A, in TR 37.717-11-11 and TR 37.716-11-11, respectively.

No MSD analysis is needed for DC_1A_n257A  and DC_3A_n257A, as there is no coexistence issue.

7.4		DC_1-3_n26-n78

7.4.1	 Operating bands for DC

Table 7.4.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.4.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A |
| DC_1A-3C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A |

7.4.2 	Channel bandwidths per operating band for DC

Table 7.4.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

7.4.3	 ∆TIB and ∆RIB values

Referring to DC_1-3_5-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration13 |  |  |  |  |  |
| DC_1-3_n26-n78 |  | 0.6 |  | 0.6 | 0.3 |  | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

Table 7.4.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration12 |  |  |  |  |  |
| DC_1-3_n26-n78 |  | 0.2 |  | 0.2 | - |  | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

7.5		DC_1-7_n26-n78

7.5.1	 Operating bands for DC

Table 7.5.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.5.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |

7.5.2 	Channel bandwidths per operating band for DC

Table 7.5.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-7C_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

7.5.3	 ∆TIB and ∆RIB values

Referring to DC_1-5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.5.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration13 |  |  |  |  |  |
| DC_1-7_n26-n78 |  | 0.6 |  | 0.6 | 0.6 |  | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

Table 7.5.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration12 |  |  |  |  |  |
| DC_1-7_n26-n78 |  | 0.2 |  | 0.2 | 0.2 |  | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

7.6		DC_3-7_n26-n78

7.6.1	 Operating bands for DC

Table 7.6.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n26-n78 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.6.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-7A_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_3A_n26A DC_7A_n26A |
| DC_3A-7C_n26A-n78A | DC_3A_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_7A_n26A DC_7C_n26A |
| DC_3C-7A_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A |
| DC_3C-7C_n26A-n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A DC_7C_n78A DC_3A_n26A DC_3C_n26A DC_7A_n26A DC_7C_n26A |

7.6.2 	Channel bandwidths per operating band for DC

Table 7.6.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n26A-n78A |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3A-7C_n26A-n78A |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3C-7A_n26A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_3C-7C_n26A-n78A |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

7.6.3	 ∆TIB and ∆RIB values

Referring to DC_3-5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 7.6.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration13 |  |  |  |  |  |
| DC_3-7_n26-n78 |  | 0.6 |  | 0.6 | 0.6 |  | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

Table 7.6.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration12 |  |  |  |  |  |
| DC_3-7_n26-n78 |  | 0.2 |  | 0.2 | 0.2 |  | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

7.7	DC_1-38_n7-n78

7.7.1	Operating bands for DC

Table 7.7.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-38_n7-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.7.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-38A_n7A-n78A | DC_1A_n78A |

7.7.2	Configuration for DC

Table 7.7.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

| DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-38A_n7A-n78A | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n7 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.7.3	Co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.7.4	∆TIB and ∆RIB values

For DC_1-38_n7-n78, the TIB,c and RIB,c values are given in the tables as below.

Table 7.7.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-38_n7-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.7.4-2: ΔRIB, c due to EN-DC (four bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-38_n7-n78 | 0.2 | - | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.7.5	MSD

All MSD requirements for DC_1-38_n7-n78 are already covered by constituent fallback modes. No additional MSD requirement is needed.

7.8	 DC_7-20_n1-n75

7.8.1	Operating bands for DC

Table 7.8.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_7-20_n1-n75 | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

7.8.2	Channel bandwidths per operating band for DC

Table 7.8.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A | 7 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

7.8.3	UE co-existence studies

Co-existence studies of DC_7-20_n1-n75 are already covered by the fallback configurations, DC_7_n1-n75 and DC_20_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.8.4	∆TIB and ∆RIB values

For DC_7-20_n1-n75, the TIB,c and RIB,c values are reused from CA_1-7-20_32, and are given in the tables below.

Table 7.8.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration13 |  |  |  |  |  |
| DC_7-20_n1-n75 |  | 0.7 |  | 0.3 | 0.7 |  | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

Table 7.8.4-2: ΔRIB,c

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration12 |  |  |  |  |  |
| DC_7-20_n1-n75 |  | - |  | - | - |  | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

7.8.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.9	 DC_3-20_n1-n75

7.9.1	Operating bands for DC

Table 7.9.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

7.9.2	Channel bandwidths per operating band for DC

Table 7.9.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-20A_n1A-n75A | DC_3A_n1A DC_20A_n1A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C-20A_n1A-n75A | DC_3A_n1A DC_3C_n1A DC_20A_n1A | 3 |  | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

7.9.3	UE co-existence studies

Co-existence studies of DC_3-20_n1-n75 are already covered by the fallback configurations, DC_3_n1-n75 and DC_20_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.9.4	∆TIB and ∆RIB values

For DC_3-20_n1-n75, the TIB,c and RIB,c values are reused from CA_1-3-20_32, and are given in the tables below.

Table 7.9.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration13 |  |  |  |  |  |
| DC_3-20_n1-n75 |  | 0.5 |  | 0.3 | 0.5 |  | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

Table 7.9.4-2: ΔRIB,c

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration12 |  |  |  |  |  |
| DC_3-20_n1-n75 |  | - |  | - | - |  | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

7.9.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.10	 DC_3-7_n1-n75

7.10.1	Operating bands for DC

Table 7.10.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

7.10.2	Configuration for DC

Table 7.10.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n1A-n75A | DC_3A_n1A DC_7A_n1A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 140 |
|  |  | 7 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
| DC_3C-7A_n1A-n75A | DC_3C_n1A DC_3A_n1A DC_7A_n1A | 3 |  | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1 in TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  | 160 |
|  |  | 7 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

7.10.3	UE co-existence studies

Co-existence studies of DC_3-7_n1-n75 are already covered by the fallback configurations, DC_3_n1-n75 and DC_7_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.10.4	∆TIB and ∆RIB values

For DC_3-7_n1-n75, the TIB,c and RIB,c values are reused from CA_1-3-7_32, and are given in the tables below.

Table 7.10.4-1: ΔTIB,c

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration13 |  |  |  |  |  |
| DC_3-7_n1-n75 |  | 0.6 |  | 0.6 | 0.6 |  | - |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

Table 7.10.4-2: ΔRIB,c

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration12 |  |  |  |  |  |
| DC_3-7_n1-n75 |  | - |  | - | - |  | - |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |  |  |  |

7.10.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.11	 DC_1-7_n75-n78

7.11.1	Operating bands for DC

Table 7.11.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-7_n75-n78 | 1 | 1920MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 7 | 2500MHz |  | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n75 |  | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300MHz | – |  | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |

7.11.2	Configuration for DC

Table 7.11.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-7A_n75A-n78A | DC_1A_n78A DC_7A_n78A | 1 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.11.3	UE co-existence studies

Co-existence studies of DC_1-7_n75-n78 are already covered by the fallback configurations, DC_1-7_n78, DC_1_n75-n78 and DC_7_n75-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.11.4	∆TIB and ∆RIB values

For DC_1-7_n75-n78, the TIB,c and RIB,c values are reused from DC_1-7-32_n78, and are given in the tables below.

Table 7.11.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-7_n1-n75 | 0.2 | 0.2 | - | 0.5 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.11.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-7_n1-n75 | 0.6 | 0.6 | - | 0.8 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.11.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.12	 DC_3-41_n1-n78

7.12.1	Operating bands for DC

Table 7.12.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 3 | 1710 MHz |  | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 41 | 2496 MHz |  | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.12.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_41A_n1A DC_41A_n78A |

7.12.2	Channel bandwidths per operating band for DC

Table 7.12.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-41A_n1A-n78A DC_3A-3A-41A_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 190 |  |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| DC_3A-41C_n1A-n78A DC_3A-3A-41C_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 210 |  |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

7.12.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.12.4	∆TIB and ∆RIB values

For DC_3-41_n1-n78, the TIB,c, and RIB,c values are reused from DC_1-3-41_n78 and given in the tables below.

Table 7.12.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.6 | 0.5 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.12.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_3-41_n1-n78 DC_3-3-41_n1-n78 | 0.2 | - | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.12.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.13	 DC_8-41_n1-n78

7.13.1	Operating bands for DC

Table 7.13.1-1: DC band combination of NR 2DL/1UL + LTE 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_8-41_n1-n78 | 8 | 880 MHz |  | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz |  | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.13.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_8A-41A_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_8A-41C_n1A-n78A | DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |

7.13.2	Channel bandwidths per operating band for DC

Table 7.13.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |  |
| DC_8A-41A_n1A-n78A | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 180 |  |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| DC_8A-41C_n1A-n78A | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 200 |  |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

7.13.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.13.4	∆TIB and ∆RIB values

For DC_8-41_n1-n78, the TIB,c and RIB,c values are reused from DC_7-8_n1-n78 and given in the tables below.

Table 7.13.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_8-41_n1-n78 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.13.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_8-41_n1-n78 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.13.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.14	 DC_20-41_n1-n78

7.14.1	Operating bands for DC

Table 7.14.1-1: DC band combination of NR 2DL/1UL + LTE 2DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_20-41_n1-n78 | 20 | 832 MHz |  | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 41 | 2496 MHz |  | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 7.14.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_20A-41A_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_20A-41C_n1A-n78A | DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |

7.14.2	Channel bandwidths per operating band for DC

Table 7.14.2-1: Supported bandwidths per DC band combination of LTE 2DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |  |
| DC_20A-41A_n1A-n78A | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 190 |  |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| DC_20A-41C_n1A-n78A | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 210 |  |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

7.14.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.14.4	∆TIB and ∆RIB values

For DC_20-41_n1-n78, the TIB,c, and RIB,c values are reused from DC_1-20-n41_n78 and given in the tables below.

Table 7.14.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_20-41_n1-n78 | 0.3 | 0.5 | 0.5 | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.14.4-2: ΔRIB,

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_20-41_n1-n78 | - | - | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.14.5	MSD

There is no need to specify additional MSD requirements for this UL DC configuration.

7.15	 DC_3-7_n75-n78

7.15.1	Operating bands for DC

Table 7.15.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7_n75-n78 | 3 | 1710MHz |  | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz |  | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n75 |  | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300MHz | – |  | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |

7.15.2	Configuration for DC

Table 7.15.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_3A-7A_n75A-n78A | DC_3A_n78A DC_7A_n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 7 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.15.3	UE co-existence studies

Co-existence studies of DC_3-7_n75-n78 are already covered by the fallback configurations, DC_3-7_n78, DC_3_n75-n78 and DC_7_n75-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.15.4	∆TIB and ∆RIB values

For DC_3-7_n75-n78, the TIB,c and RIB,c values are reused from DC_3-7-32_n78, and are given in the tables below.

Table 7.15.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_3-7_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.15.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-7_n75-n78 | 0.2 | 0.2 | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.15.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

7.16	 DC_1-3_n75-n78

7.16.1	Operating bands for DC

Table 7.16.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3_n75-n78 | 1 | 1920MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710MHz |  | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | n75 |  | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |
|  | n78 | 3300MHz | – |  | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |

7.16.2	Configuration for DC

Table 7.16.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A_n75A-n78A | DC_1A_n78A DC_3A_n78A | 1 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n75 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

7.16.3	UE co-existence studies

Co-existence studies of DC_1-3_n75-n78 are already covered by the fallback configurations, DC_1-3_n78, DC_1_n75-n78 and DC_3_n75-n78. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

7.16.4	∆TIB and ∆RIB values

For DC_1-3_n75-n78, the TIB,c and RIB,c values are reused from DC_1-3-32_n78, and are given in the tables below.

Table 7.16.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)12 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration13 |  |  |  |
| DC_1-3_n75-n78 | 0.6 | 0.6 | - | 0.8 |
| NOTE 12: “-” denotes ΔTIB,c = 0. NOTE 13: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

Table 7.16.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)11 |  |  |  |
| --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration12 |  |  |  |
| DC_1-3_n75-n78 | - | - | - | 0.5 |
| NOTE 11: “-” denotes ΔRIB,c = 0. NOTE 12: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_30-66-(n)5 the band order from left to right is 5, 30, 66 and n5. |  |  |  |  |

7.16.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8	Dual Connectivity band combinations of LTE 3 bands DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

8.1		DC_1-3-38_n28-n78

8.1.1	 Operating bands for DC

Table 8.1.1-1: LTE 3 bands DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-38_n28-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
|  | n28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.1.1-2: Inter-band EN-DC configurations within FR1 (five bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |
| DC_1A-3C-38A_n28A-n78A | DC_1A_n28A DC_1A_n78A DC_3C_n78A DC_3A_n28A DC_3A_n78A DC_38A_n28A DC_38A_n78A |

8.1.2 	Configuration for DC

Table 8.1.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-38A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 170 |
|  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
| DC_1A-3C-38A_n28A-n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n28 | 15 | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.1.3	 ∆TIB and ∆RIB values

Referring to DC_—1-41_n28-n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 8.1.3-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-38_n28-n78 | 0.5 | 0.6 | 0.3 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.1.3-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-38_n28-n78 | - | 0.2 | - | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.2		DC_1-3-7_n26-n78

8.2.1	 Operating bands for DC

Table 8.2.1-1: LTE 1 band DL/1UL + NR 2 bands DL/1UL DC operating bands

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n26-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.2.1-2: Inter-band EN-DC configurations within FR1 (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3C-7A_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7A_n78A |
| DC_1A-3A-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3A_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |
| DC_1A-3C-7C_n26A-n78A | DC_1A_n26A DC_1A_n78A DC_3A_n26A DC_3C_n26A DC_3A_n78A DC_3C_n78A DC_7A_n26A DC_7C_n26A DC_7A_n78A DC_7C_n78A |

8.2.2 	Channel bandwidths per operating band for DC

Table 8.2.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  | DC operating / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-38A_n28A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 190 |
|  |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C-38A_n28A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C-38A_n28A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 210 |
|  |  | 3 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
| DC_1A-3C-7C_n26A-n78A |  | 1 | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |  |  |  | 230 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 0 in Table 5.6A.1-1 from TS 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | n26 | 15 | 5 | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 253 | 303 |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | 10 | 15 | 20 | 30 | 40 | 40 | 50 |  |  |  |  |  |  |
|  |  |  | 30 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |
|  |  |  | 60 |  | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 |  |

8.2.3	 ∆TIB and ∆RIB values

Referring to DC_1-3-5-7_n78, ΔTIB,c and ΔRIB,c can be specified as below.

Table 8.2.3-1: ΔTIB,c due to EN-DC(five bands)

| Inter-band EN-DC configuration |  | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |  |  |
| DC_1-3-7_n26-n78 |  | 0.5 |  | 0.6 | 0.6 |  | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |  |  |  |

Table 8.2.3-2: ΔRIB,c due to EN-DC (five bands)

| Inter-band EN-DC configuration |  | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Component band in order of bands in configuration7 |  |  |  |  |  |  |
| DC_1-3-7_n26-n78 |  | 0.2 |  | 0.2 | 0.2 |  | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |  |  |  |

8.3	 DC_3-8-41_n1-n78

8.3.1	Operating bands for DC

Table 8.3.1-1: DC band combination of NR 2DL/1UL + LTE 3DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 3 | 1710 MHz |  | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 8 | 880 MHz |  | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 41 | 2496 MHz |  | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.3.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_8A_n1A DC_8A_n78A DC_41A_n1A DC_41A_n78A |

8.3.2	Channel bandwidths per operating band for DC

Table 8.3.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-8A-41A_n1A-n78A DC_3A-3A-8A-41A_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 200 |  |
|  | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| DC_3A-8A-41C_n1A-n78A DC_3A-3A-8A-41C_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 220 |  |
|  | 8 |  | 15 | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

8.3.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.3.4	∆TIB and ∆RIB values

For DC_3-8-41_n1-n78, the TIB,c and RIB,c values are reused from DC_3-7-8_n1-n78 and given in the tables below.

Table 8.3.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 0.5 | 0.6 | 0.6 | 0.6 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.3.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-8-41_n1-n78 DC_3-3-8-41_n1-n78 | 0.2 | 0.2 | 0.2 | 0.2 | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.3.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.4	 DC_3-20-41_n1-n78

8.4.1	Operating bands for DC

Table 8.4.1-1: DC band combination of NR 2DL/1UL + LTE 3DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | 3 | 1710 MHz |  | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 20 | 832 MHz |  | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 41 | 2496 MHz |  | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n1 | 1920 MHz |  | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – |  | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

Table 8.4.1-2: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |
| DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A | DC_3A_n1A DC_3A_n78A DC_20A_n1A DC_20A_n78A DC_41A_n1A DC_41A_n78A |

8.4.2	Channel bandwidths per operating band for DC

Table 8.4.2-1: Supported bandwidths per DC band combination of LTE 3DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band |  | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |  |
| DC_3A-20A-41A_n1A-n78A DC_3A-3A-20A-41A_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 210 |  |
|  | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
| DC_3A-20A-41C_n1A-n78A DC_3A-3A-20A-41C_n1A-n78A | 3 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  | 230 |  |
|  | 20 |  | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n1 |  | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |
|  | n78 |  | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |

8.4.3	UE co-existence studies

Co-existence studies of this DC LTE inter-band 2DL/1UL + inter-band NR 2DL/1UL are already covered in the constituent fall-back modes. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.4.4	∆TIB and ∆RIB values

For DC_3-20-41_n1-n78, the TIB,c, and RIB,c values are reused from DC_1-3-20_n41-n78 and given in the tables below.

Table 8.4.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | 0.5 | 0.3 | 0.5 | 0.5 | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.4.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-20-41_n1-n78 DC_3-3-20-41_n1-n78 | - | - | - | - | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.4.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.5	 DC_3-7-20_n1-n75

8.5.1	Operating bands for DC

Table 8.5.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_3-7-20_n1-n75 | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 20 | 832MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

8.5.2	Channel bandwidths per operating band for DC

Table 8.5.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_7A-20A_n1A-n75A | DC_3A_n1A DC_7A_n1A DC_20A_n1A |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 160 |
|  |  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 20 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

8.5.3	UE co-existence studies

Co-existence studies of DC_3-7-20_n1-n75 are already covered by the fallback configurations, DC_3-7-20_n1, DC_3_n1-n75, DC_7_n1-n75, DC_20_n1-n75. Therefore, there is no additional harmonic and intermodulation impact for the additional band receiver.

8.5.4	∆TIB and ∆RIB values

For DC_3-7-20_n1-n75, the TIB,c and RIB,c values are reused from CA_1-3-7-20-32, and are given in the tables below.

Table 8.5.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-20_n1-n75 | 0.7 | 0.7 | 0.3 | 0.7 | - |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.5.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_3-7-20_n1-n75 | - | - | - | - | - |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.5.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

8.6	 DC_1-3-7_n75-n78

8.6.1	Operating bands for DC

Table 8.6.1-1: DC band combination of NR 2DL/1UL + LTE 1DL/1UL

| E-UTRA and NR DC Band | E-UTRA and NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-3-7_n75-n78 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 3 | 1710MHz | – | 1785 MHz | 1805 MHz | – | 1885 MHz | FDD |
|  | 7 | 2500MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | n78 | 3300MHz | – | 3800 MHz | 3300MHz | – | 3800 MHz | TDD |
|  | n75 | N/A |  |  | 1432 MHz | – | 1517 MHz | SDL |

8.6.2	Channel bandwidths per operating band for DC

Table 8.6.2-1: Supported bandwidths per DC band combination of LTE 1DL/1UL + NR 2DL/1UL

|  |  | DC operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | UL Configuration |  | E-UTRA and NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Maximum aggregated bandwidth [MHz] |
| DC_1A-3A-7A_n75A-n78A | DC_1A_n78A DC_3A_n78A DC_7A_n78A |  | 1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 210 |
|  |  |  | 3 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 7 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n75 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  | n78 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  |  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |

8.6.3	UE co-existence studies

Co-existence studies of DC_1-3-7_n75-n78 are already covered by the fallback configurations.

8.6.4	∆TIB and ∆RIB values

For DC_1-3-7_n75-n78, the TIB,c and RIB,c values are reused from DC_1-3-7-32_n78, and are given in the tables below.

Table 8.6.4-1: ΔTIB,c

| Inter-band EN-DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n75-n78 | 0.7 | 0.7 | 0.7 | - | 0.8 |
| NOTE 6: “-” denotes ΔTIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

Table 8.6.4-2: ΔRIB,c

| Inter-band EN-DC configuration | ΔRIB,c for E-UTRA band / NR band (dB)6 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Component band in order of bands in configuration7 |  |  |  |  |
| DC_1-3-7_n75-n78 | 0.3 | 0.3 | 0.3 | - | 0.5 |
| NOTE 6: “-” denotes ΔRIB,c = 0. NOTE 7: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively, such as for DC_2-30-66-(n)5 the band order from left to right is 2, 5, 30, 66 and n5. |  |  |  |  |  |

8.6.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

9	Dual Connectivity band combinations of LTE 4 band DL/1UL + NR 2 bands DL/1UL: Specific Band Combination Part

9.1	DC_UA-VA-WA-XA_nYA-nZ

9.1.1	Operating bands for DC

9.1.2	Channel bandwidths per operating band for DC

9.1.3	Co-existence studies

9.1.4	∆TIB and ∆RIB values

9.1.5	MSD

Annex A: Change history

| Date | Meeting | TDoc | CR | Rev | Subject/Comment | Old version | New version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2022-08 | RAN4#104e | R4-2212174 |  |  | TR37.718-11-21 Skeleton for Rel-18 DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL) |  | 0.0.1 |
| 2022-08 | RAN4#104e | R4-2212175 |  |  | TR 37.718-11-21 v0.1.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18 | 0.0.1 | 0.1.0 |
| 2022-10 | RAN4#104-bis-e | R4-2215383 |  |  | TR 37.718-11-21 v0.2.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18 | 0.1.0 | 0.2.0 |
| 2022-11 | RAN4#105 | R4-2218026 |  |  | TR 37.718-11-21 v0.2.0 TR Update: LTE(xDL/1UL)+ NR(2DL/1UL) DC in Rel-18 | 0.2.0 | 0.3.0 |