3GPP TSG-RAN WG4 Meeting#105	R4-2218065

Toulouse, France 14th Nov – 18th Nov, 2022

Source: 	Qualcomm Incorporated

Title: 	Simulation Results on NTN UE PDSCH Demodulation Requirements

Agenda item:	6.2.6.3.1

Document for:	Discussion

- Introduction

During RAN4#104bis-e, simulation assumptions for PDSCH demodulation requirements were finalized [1]. In this paper, we share our simulation results for defining PDSCH demodulation requirements.

- Requirements for NTN UE PDSCH Tests

Simulation assumption for PDSCH performance with disabled HARQ process

- Proposals

  - Option 1(Ericsson, Qualcomm, Apple, Nokia): Set the number of HARQ Processes as: 4 disabled, 12 enabled in 16 HARQ processes with re-Tx disable for all HARQ processes.

  - Option 2 (Huawei): Keep previous agreements that all HARQ feedback are disabled with 16 HARQ processes with re-Tx disable for all HARQ processes

Based on the agreed channel models for NTN-TDL-A and NTN-TDL-C, we choose SNR requirements at 70% peak throughput for MCS 4 and MCS13 along with different HARQ capabilities [1]. Besides, as agreed during RAN4#104-e [2], we assume that the Doppler shift due to satellite mobility is fully compensated by UE. Furthermore, in the case of disabled HARQ scenario, we have included results for both options 1 and 2, where it was agreed that re-transmission can be assumed to be disabled for all HARQ processes for both options. Our results suggest that there is no performance difference between these two options. Below we share our simulation results for both NTN-TDLA and NTN-TDLC channels.

Table 1: Alignment SNR requirements for NTN PDSCH Demodulation

| Channels | MCS4 |  |  |  | MCS13 |
| --- | --- | --- | --- | --- | --- |
|  | 16HARQ | 32HARQ | Disabled HARQ |  | 16HARQ |
| NTN-TDLA100-200 | -2.5 | N/A | Option 1 | Option 2 | N/A |
|  |  |  | -1.7 | -1.7 |  |
| NTN-TDLC5-200 k_factor=8.05 dB | N/A | -3.2 | N/A |  | 4.6 |

Table 2: Impairment SNR requirements for NTN PDSCH Demodulation

| Channels | MCS4 |  |  |  | MCS13 |
| --- | --- | --- | --- | --- | --- |
|  | 16HARQ | 32HARQ | Disabled HARQ |  | 16HARQ |
| NTN-TDLA100-200 | -0.5 | N/A | Option 1 | Option 2 | N/A |
|  |  |  | 0.3 | 0.3 |  |
| NTN-TDLC5-200 k_factor=8.05 dB | N/A | -1.2 | N/A |  | 6.6 |

- Conclusions

In this paper, we share our simulation results for PDSCH requirements based on the agreed simulation assumptions.

# References

[1] R4-2217344, “WF for NTN demodulation requirements - general and PDSCH,” 3GPP TSG RAN WG4 Meeting #104bis-e, Online, Oct 10 – Oct 19, 2022.

[2] R4-2214386, “WF for NTN demodulation requirements - general and PDSCH,” 3GPP TSG RAN WG4 Meeting #104-e, Online, Aug 15 – Aug 26, 2022.