3GPP TSG-RAN WG4 Meeting # 105		R4-2218187

Toulouse, France, November 14 – November 18, 2022

Agenda item:	6.5.2

Source:	Apple

Title:	On Remaining Issues for FeMIMO Performance Requirements

Release:	Rel-17

Document for:	Discussion

# 1. Introduction

In RAN4#104-bis-e performance testcases for FeMIMO were discussed and way forward [1] was agreed.  In this contribution we present our views on open issues on performance requirements FeMIMO.

# 2. Discussion

  - 2.1 Unified TCI State switching requirements

The open issues related to Unified TCI switching delay requirements are:

  - Whether PL-RS is maintained or not maintained in joint TCI test cases

  - How to configure maintain/ not-maintained PL-RS

  - How to define PL-RS of target TCI

  - RRC configuration of the cell of additional PCI

PL-RS configuration

The open issues related to PL-RS configuration in [1]:

Issues 1-1-1-b/Issue 1-1-3: Whether PL-RS is maintained or not maintained in joint TCI test cases and how to config?

- Option 1: Pathloss RS is maintained in joint TCI test cases

  - Option 1a: If only two TCI states in total are configured to UE during the test, UE can be assumed to maintain the PL-RS in the two TCI states. If more than 4 TCI states in total are configured to UE, then whether a PL-RS in a TCI state is maintained is up to UE implementation.

  - Option 1b: implicit configuration for PL-RS, for the known TCI state test cases, PL-RS can be maintained

- Option 2: Pathloss RS is not maintained in joint TCI test cases

  - Option 2a: In the test cases, only define the test cases for PL-RS is not maintained. For the test setup, configure a new RS as PL-RS, it is a “not maintained PL-RS”

  - Option 2b: In the test cases, only define the test cases for PL-RS is not maintained. If only 1 active TCI state is configured, and with TCI switch a new TCI state and new PL-RS is activated, then it would be PL-RS not maintained. The PL-RS and RS in UL/Joint TCI state should be the same or QCL Type D with each other, and the target UL /joint TCI should be configured for L1 measurement to ensure known state.

  - Option 2c: PL-RS is included in UL TCI state and PL-RS is identical to source RS in target UL TCI state. it’s the first activation for the UL TCI state. L1-RSRP based on the RS has been reported before.

Issue 1-1-2: How to define PL-RS of target TCI?

- Option 1 (vivo)

- RAN4 design test cases for unified TCI by configuring that PL RS of target TCI is not QCL-D with the any PL RS of the TCI in the currently activated TCI list.

- Option 2 (Huawei, Apple)

- In the test cases, the PL-RS of the target TCI shall be QCL type D with the source RS of the target TCI.

Based on RAN1 agreements and confirmation from reply LS from RAN1 [2], for PL-RS to be maintained, it must be part of the active UL TCI state prior to the TCI state switch. The number of TCI states in active TCI state list for DL or UL is based on UE capability. If UE is capability doesn’t support more than 1 active UL TCI in the TCI state list, then only 1 active UL TCI state can be configured, and we cannot configure a test case with PL-RS maintained prior to the UL or Joint TCI state switch.

Observation #1: Configuring test case with target PL-RS maintained is subject to additional UE capability of supporting > 1 active TCI states in active joint/UL TCI list.

Since configuring test case with target PL-RS maintained prior to TCI state switch is subject to additional UE capability, we propose to introduce test case for joint TCI with target PL-RS not maintained.

Proposal #1: Configure target PL-RS as ‘not maintained’ in Joint TCI state switching test case

Proposal #2: PL-RS is configured as not-maintained by not including it in active TCI state list for UL or Joint TCI

The target PL-RS can be the same as the RS for target joint TCI state or QCL Type D to the RS in the target joint TCI state. For simplicity of test configuration, the same RS can be configured as RS in joint TCI state and PL-RS.

Observation #2: The target PL-RS is configured same as the RS in target UL/joint TCI state or QCL-Type D to RS in target TCI state.

Proposal #3: PL-RS of target TCI state is defined as the RS in UL / Joint TCI state for simplifying the test configuration.

RRC configuration for cell of additional PCI

There was a discussion in RAN4#104-bis-e whethere the RRC configuration should be provided for cell with additional PCI:

Issue 1-2-2: Whether to provide RRC configuration of the cell of additional PCI?

- Option 1

- Prefer not to provide RRC configuration of the cell of additional PCI in the test cases since they are not actually used by UE.

- Option 2

- Others

For DL TCI state switch for unified TCI, we agreed to include 1 test case with TCI state switch to cell with additional PCI. The RRC configuration of the configured cells in any test case is provided as part of the test set up. If cell with different PCI is configured in the test case, then we would need to include the complete configuration set up for it. This configuration would not be part of the serving cell RRC configuration.

Proposal #4: The RRC configuration of cell with different PCI is specified for cell configuration in test case, but not as part of RRC configuration for serving cell

  - 2.2 TRP specific BFD and LR test cases

The open issues related TRP specific BFR test cases are:

Issue 3-1-1: Whether intra-cell TRP or inter-cell TRP specific BFR test cases are designed?

- Option 1: Design all TRP specific BFR test cases as intra cell. (Intel, Apple, Huawei, MediaTek)

- Option 2: For TRP specific BFR test cases, for 4 CSI-RS based TCs, design intra cell, for 2 SSB based TCs, design the test case as one serving cell and additionalPCI (Samsung)

- Option 3: For 2 SSB based TCs, design intra-cell, but for CSI-RS TCs, design inter-cell. (, Ericsson)

The feature of TRP specific BFR is applicable to both intra-cell and inter-cell mTRP scenarios. For verifying the requirements for TRP specific BFR it doesn’t make much difference if the TRPs are from same PCI as serving cell or one of them is from different PCI, except for the test configuration.

Observation #3:  For the purpose of verifying core requirements of TRP specific BFR, there is no difference whether intra-cell or inter-cell mTRP is configured.

We think it is sufficient to configure the test cases as intra-cell mTRP for TRP specific BFR.

Proposal #5: Design all test cases as intra-cell for TRP specific BFR.

In [1] we have the following agreement for beam recovery configuration for TRP specific BFR test cases:

Issue 3-1-2: Beam recovery method configured in the test case Agreements: - For BFR on SCells, dedicated BFR resource is configured and not configured for different test cases - For BFR on SpCells, CFRA based BFR is configured.

The above agreement suggests that only on SCells dedicated BFR resource is configured or not configured, and not for SpCells. For Rel-17 TRP specific BFR dedicated BFR resource can be configured for SpCells as well.

From TS 38.213:

On check of the existing requirements, we found that the TRP specific BFR test cases for PCell or PSCell don’t have dedicated BFR resource specified and is incomplete.

Observation #4: The BFR resource configuration is incomplete for TRP specific BFR tests with PSCell or PCell.

Based on the above observation, we summarized the current test case configuration and proposed modification in the table below.

Table 1: BFR Resource for TRP specific BFR test cases

| Scenario | Key Configurations | Clause Number | BFR resource config | Proposed Modification |
| --- | --- | --- | --- | --- |
| EN-DC + FR1 + PSCell | SSB based + Non-DRX | A.4.5.5.7 | Not specified | Configure dedicated BFR resource |
| EN-DC + FR2 + PSCell | CSI-RS based + DRX | A.5.5.5.8 | Not specified | Configure dedicated BFR resource |
| EN-DC + FR1 + SCell | CSI-RS-based BFD and SSB-based LR + Non-DRX | A.4.5.5.8 | Specified as not configured | Test parameters are incorrect |
| NR SA + FR1 + PCell | CSI-RS based + DRX | A.6.5.5.7 | Not specified | Configure dedicated BFR resource |
| NR SA + FR2 + PCell | SSB based + Non-DRX | A.7.5.5.10 | Not specified | Configure dedicated BFR resource |
| NR SA + FR2 + SCell | CSI-RS based + DRX | A.7.5.5.9 | Specified as configured | None |

Proposal #6: Configure dedicated BFR resource for TRP specific BFR test cases with PCell and PSCell.

# 3. Conclusion

In this paper, we provide our views on open issues on performance requirements FeMIMO. Our observations and proposals are captured below:

Unified TCI

Observation #1: Configuring test case with target PL-RS maintained is subject to additional UE capability of supporting > 1 active TCI states in active joint/UL TCI list.

Proposal #1: Configure target PL-RS as ‘not maintained’ in Joint TCI state switching test case

Proposal #2: PL-RS is configured as not-maintained by not including it in active TCI state list for UL or Joint TCI

Observation #2: The target PL-RS is configured same as the RS in target UL/joint TCI state or QCL-Type D to RS in target TCI state.

Proposal #3: PL-RS of target TCI state is defined as the RS in UL / Joint TCI state for simplifying the test configuration.

Proposal #4: The RRC configuration of cell with different PCI is specified for cell configuration in test case, but not as part of RRC configuration for serving cell

TRP Specific BFR

Observation #3:  For the purpose of verifying core requirements of TRP specific BFR, there is no difference whether intra-cell or inter-cell mTRP is configured.

Proposal #5: Design all test cases as intra-cell for TRP specific BFR.

Observation #4: The BFR resource configuration is incomplete for TRP specific BFR tests with PSCell or PCell.

Proposal #6: Configure dedicated BFR resource for TRP specific BFR test cases with PCell and PSCell.

# Reference

- R4-2217208, “WF on FeMIMO RRM Performance Requirement and Test Case”, Samsung.

- R1-2210719, “Reply LS on active TCI state list for UL TCI”, RAN1, Nokia.