3GPP RAN WG4 Meeting #105		R4-2218359

Toulouse, France, November 14 – 18th, 2022

Agenda item:	8.14.2.1

Source:	Apple

Title:	On UL MIMO TRP methodology

WI/SI:	NR_FR1_TRP_TRS_enh_Core

Release:	Rel-18

Document for:	Decision

# 1	Introduction

RAN4 #104-bis-e the WF [1] captured the agreements achieved during the meeting. In this contribution we’ll address some of the aspects of the WF that can be supported by Apple and its preferences:

# 2	Observations on UL MIMO test  methods

Based on agreements defined in [1], the following issues will be discussed in this contribution:

Observation 1 (Issue 2-1-3):

As previously discussed on [2-4] and also observed in [5] the phase variation on the signal delivered to antennas can have a large phase variation, and in some cases such phase can drift over time due intrinsic devices characteristics. The impact of phase variation in both non-coherent and coherent UEs and results uncertainty and reproducibility needs to be fully understood prior to move forward.

Proposal 1:

Define as  first priority TPMI 0 and 1 for non-coherent devices considering the linear combination of separately measurement of TPMI 0 + TPMI 1 results.

Proposal 2:

Define as second priority TPMI-based TRP measurement with two antennas transmitting simultaneously, considering a parallel study on the phase difference (and drift) impact on the TRP measurement uncertainty and reproducibility.

Observation 2 (Issue 2-1-5):

Based on the argument discussed above on the Observation 1 (Issue 2-1-3), a single TPMI used for all test positions in the TRP test could give rise to destructive interference issues, as had been discussed in the Rel-17. The impact of the phase between antennas in the TRP measurement uncertainty and repeatability needs to be studied prior to define the TPMI index implementation.

Proposal 3:

Conduct a data based study to define the impact of TPMI index selection on the measurement uncertainty and lab repeatability.

Observation 3 (Issue 2-1-6):

In real environment conditions the codebook-based transmission precoding matrix W index might vary. As also stated on [6], further study on test environments conditions to alter measurement uncertainty and results repeatability need to be evaluated. Optimization of EIRP based on TPMI selection can be a relevant methodology to address both coherent and non-coherent UEs.

Proposal 4:

Conduct a data driven study to determine the feasibility and impact of EIRP measurement based on TPMI dynamic selection.

Observation 4 (Issue 2-1-7/8):

The test methodology for UL MIMO  devices needs to be defined based on devices capabilities.

Proposal 5:

UL MIMO capable devices needs to follow different TRP requirements for coherent and non-coherent implementations.

Observation 5 (Issue 2-2-2):

Same technical argument adopted on Observation 1.

Proposal 6:

Define as second priority 2Tx TRP (general for TxD and TPMI based TRP) with two antennas transmitting simultaneously, considering a parallel study on the phase difference (and drift) impact on the TRP measurement uncertainty and reproducibility.

# 3	Conclusions and Proposals

This paper discussed open topics related to UL MIMO TRP methodologies, and based on above observations proposed the following actions:

Proposal 1:

Define as  first priority TPMI 0 and 1 for non-coherent devices considering the linear combination of separately measurement of TPMI 0 + TPMI 1 results.

Proposal 2:

Define as second priority TPMI-based TRP measurement with two antennas transmitting simultaneously, considering a parallel study on the phase difference (and drift) impact on the TRP measurement uncertainty and reproducibility.

Proposal 3:

Conduct a data based study to define the impact of TPMI index selection on the measurement uncertainty and lab repeatability.

Proposal 4

Conduct a data driven study to determine the feasibility and impact of EIRP measurement based on TPMI dynamic selection.

Proposal 5:

UL MIMO capable devices needs to follow different TRP requirements for coherent and non-coherent implementations.

Proposal 6:

Define as second priority 2Tx TRP (general for TxD and TPMI based TRP) with two antennas transmitting simultaneously, considering a parallel study on the phase difference (and drift) impact on the TRP measurement uncertainty and reproducibility.

# 4				References

- R4-2217459, WF on FR1 TRP TRS, vivo

- R4-2215653, On TRP TRS methodology enhancements, Apple

- R4-2207686, Discussion on TxD active cancellation, Apple

- R4-2212375, Remaining issues with multiple antenna test methodologies, Apple

- R4-2215704, Initial discussion of TRP TRS on NR 2Tx UE, Samsung

- R4-2216446, Considerations on TRPTRS test methodology for 2Tx UE - OPPO