3GPP RAN WG4 Meeting #105		R4-2218360

Toulouse, France, November 14 – 18th, 2022

Agenda item:	8.15.2

Source:	Apple

Title:	On MIMO OTA test methodology

WI/SI:	NR_MIMO_OTA_enh_Core

Release:	Rel-18

Document for:	Decision

# 1	Introduction

In RAN4 #104-bis-e the WF [1] capture the agreements achieve during the meeting. In this contribution we’ll address some of the aspects of the WF that can be supported by Apple and its preferences:

# 2	Discussion

2.a  FR2 MIMO OTA

Based on the WF [1] agreements, the following next steps were formulated:

Action 1:

During the meeting CAICT provided a list of 47 UEs that based on GSA database, these UEs are commercially available, and capable to operate on band n261.

Observation 1:

This information enables a better view on the feasibility of gathering measurement results to correlate with simulation models, as well as enabling an adequate lab alignment with a plurality of devices antenna system topologies and a relevant pool of data to determine performance requirements. Regardless of companies volunteering to provide PADs or not, it seems that devices are available in the market for the purpose of this activity.

Proposal 1: In an absence of volunteer companies providing PADs, 3GPP to make a decision of purchase devices on the market for the purpose of lab alignment.

Proposal 2: Define the minimum number of PADs as 4 allowing a plurality of antenna system topologies.

Action 2:

The proposal of enabling companies to provide measurement results to correlate with measurements should consider the proper measurement data gathering as a pre-requisite.

Observation 2:

A lab channel model validation should be the minimum requirement to consider labs preliminary measurement results adequate for analysis. Ideally simulation and measurement correlation should use measurement results that came from aligned labs.

Proposal 3: Channel model validation is a pre-requisite for labs to provide early measurement results to be used on simulation vs. measurement correlation.

Proposal 4: Define the maximum pass/fail simulation vs. measurements uncertainty gap (“x” dB) no later than RAN4 #106.

Action 3:

Considering the overall need for lab alignment activity.

Observation 3:

As demonstrated by previous MIMO OTA activities in 3GPP, lab alignment is a reliable methodology to assure that multiple MIMO OTA system implementation can produce the same results bounded by channel model validation and predefined FoM. There are already (at least) 4 volunteer labs for this activity and plenty of commercial PADs available.

Proposal 5: Define a FR2 MIMO OTA lab alignment framework no later than RAN4 #106.

Action 4:

Considering the proper approach on defining measurement results to be correlated with simulation results.

Observation 4: To increase the chance of success on the simulation vs. measurement correlation activity, the simulation model needs to include the antenna system radiation pattern from the device which measurement data was collected. Therefore, taking into account the radiation pattern spatial characteristics of the device being benchmarked.

Proposal 6: Measurement results that will be selected to validate simulation models, shall be accompanied by its antenna system radiation pattern, the format in which the radiation patten is provided will be based on simulation model proponents requirements.

2.b FR1 MIMO OTA

As captured on WF [1], the applicability of FR1 MIMO OTA at browsing mode is still for further study:

Observation 4:

The FR1 MIMO OTA lab alignment completed on Rel. 17 was done in high frequency channels and produce lab alignment and  performance requirements at two bands n41 (2.496 - 2.690 GHz) and n78 (3.3 - 3.8 GHz), considering the current smartphones physical dimensions and the wavelength in these frequencies, there’s a small likelihood that any commercial smartphone would have a high correlated MIMO antenna system in those frequencies.

However, with the addition of low frequency bands on [2] i.e.: n5 (869 - 894 MHz), n8 (925 – 960MHz) and n28 (758 – 803 MHz), the wavelength at these frequencies vs. physical device dimensions might create a challenge for some devices to enable decorrelated MIMO antenna systems. Based on this technical considerations and the fact that at the moment 3GPP RAN4 don’t have evidence of FR1 MIMO OTA lab alignment in low frequency bands (<1GHz). Adding extra uncertainty based on hand phantom positioning might be premature at this point.

Proposal 7: Complete FR1 MIMO OTA performance requirements at defined [2] frequency bands, i.e.: n1, n5, n8, n28 and n77 in Free Space only avoiding unknown hand phantom positioning uncertainty.

Observation 5:

Following WF [1] definitions on:

Following the framework defined for FR1 MIMO OTA lab alignment on Rel.17, volunteer labs are required to produce evidence of proper channel model validation as pre-requisite to continue the lab alignment measurement campaign towards performance requirement definitions.

Proposal 8. Unless a new lab alignment channel model validation activity takes place, to not introduce new channel models that have no validation data evidence among aligned labs.

# 3	Conclusions

FR2 MIMO OTA related proposals:

Proposal 1: In an absence of volunteer companies providing PADs, 3GPP to make a decision of purchase devices on the     market for the purpose of lab alignment.

Proposal 2: Define the minimum number of PADs as 4 allowing a plurality of antenna system topologies.

Proposal 3: Channel model validation is a pre-requisite for labs to provide early measurement results to be used on simulation vs. measurement correlation.

Proposal 4: Define the maximum pass/fail simulation vs. measurements uncertainty gap (“x” dB) no later than RAN4 #106.

Proposal 5: Define a FR2 MIMO OTA lab alignment framework no later than RAN4 #106.

Proposal 6: Measurement results that will be selected to validate simulation models, shall be accompanied by its antenna system radiation pattern, the format in which the radiation patten is provided will be based on simulation model proponents requirements.

FR1 MIMO OTA related proposals:

Proposal 7: Complete FR1 MIMO OTA performance requirements at defined [2] frequency bands, i.e.: n1, n5, n8, n28 and n77 in Free Space only avoiding unknown hand phantom positioning uncertainty.

Proposal 8. Unless a new lab alignment channel model validation activity takes place, to not introduce new channel models that have no validation evidence among aligned labs.

# 4	References

- R4-2217461, "WF on NR MIMO OTA enhancement", CAICT.

- R4-222668, “R18 New WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs”, CAICT, OPPO, Keysight Technologies