3GPP TSG-RAN WG4 Meeting #105			                                                  R4-2218449

Toulouse, France, November 14 – Nov ember 18, 2022

Title:	Reply LS on capability for PRS measurement without MG

Source:	CATT

Agenda Item:	10.2.1.2

Document for:	Approval

1 Introduction

In last meeting, RAN4 received the reply LS [1] on capability for PRS measurement without MG from RAN2 to clarify whether the LMF also needs to know the UE capability on feature 14-3 “PRS measurement without MG”. In this paper, we provide the discussions on this issue and give our proposals. Based on the discussion, a draft LS is provided in Annex.

2 Discussion

In the RAN2 LS, they asked whether feature 14-3 “PRS measurement without MG” is needed to be known by LMF. We provide our views in this section.

Feature 14-3 is to provide the threshold used to be compared against with the Rx timing difference to determine whether the PRS from the non-serving cell satisfy the condition of PRS measurement outside MG, i.e. the supported threshold and the Rx timing difference determine whether UE can perform measurement outside MG. In the previous meeting, RAN4 has agreed that the Rx timing difference is derived by assistant signalling which is configured by LMF, so we think LMF need to know the threshold to help the configuration of PRS resources. And on the other hand, LMF need to know all the positioning related UE capability to better control and coordinate the positioning.

Proposal 1: The LMF also need to know the UE capability on feature 14-3 “PRS measurement without MG”.

3 Summary

In this paper, we discuss the RAN2 LS on capability for PRS measurement without MG and the following proposal is given:

Proposal 1: The LMF also need to know the UE capability on feature 14-3 “PRS measurement without MG”.

4 References

- R2-2209241, LS to RAN4 on capability for PRS measurement without MG, RAN2#119-e

5 Annex

3GPP TSG RAN WG4 Meeting #105 		R4-22XXXX

Toulouse, France, November 14 – Nov ember 18, 2022

Title:	Reply LS on capability for PRS measurement without MG

Response to:	R2-2209241

Release:	Rel-17

Work Item:	NR_pos_enh-Core

Source:	RAN WG4

To:	RAN WG2, RAN WG1

Cc:

Contact person:

Name:	Qiuge Guo

E-mail Address:	guoqiuge@CATT.cn

Attachments: None

1	Overall description

RAN4 thanks RAN2 for the LS R2-2209241 on capability for PRS measurement without MG. In RAN4#105 meeting, RAN4 discussed the LS and provided the following feedbacks:

Question: Whether the LMF also needs to know the UE capability on feature 14-3? RAN4 feedback: The LMF also need to know the UE capability on feature 14-3 “PRS measurement without MG”.

RAN4 kindly asks RAN2 to take the above information into account in the following work on NR positioning enhancements.

2	Actions

To RAN WG2:

ACTION: 	RAN4 kindly asks RAN2 to take the above information into account in the following work on NR positioning enhancements.

3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #106		February 27 – March 3, 2023		Athens, GR

TSG RAN WG4 Meeting #106-bis-e		April 17 – April 26, 2023		Electronic