3GPP TSG-RAN WG4 Meeting # 105                                                                R4-2218482

# Toulouse, France, November 14 – November 18, 2022

Title: 	Discussion on the remaining open assumptions for adjacent channel co-existence simulation

Source: 	CATT

Agenda item:	8.18.2.1

Document for:	Discussion

- Background

Most of the simulation assumptions were agreed in the last RAN4 meeting [1]. This contribution provides our views on the remaining open assumptions.

- Discussion

  - Handling of co-channel inter-site inter-subband interference

The followings are the agreements for co-channel inter-site inter-subband interference in last RAN4 meeting,

- Include co-channel inter-site inter-subband interference into RAN4 simulation

- BS ACLR/ACS as starting point for simulation purpose only

- FFS on UE side

There’re still some issues need to be discussed and decided. First issue is how to include the co-channel interference in the adjacent channel co-existence simulation. For the cases of legacy TDD as the victim, the SBFD co-channel interference doesn’t have impact on the victim performance, so the co-channel interference can be ignored in these cases.

Observation 1: SBFD co-channel interference doesn’t need to be added in the simulation when legacy TDD is the victim.

For the cases of the SBFD as the victim, the co-channel interference exists in both the baseline and the adjacent co-existence scenarios. The relationship of the BS inter-subband ACLR/ACS and adjacent channel ACLR/ACS should be clarified. There can be two understandings for this aspect. The first understanding is that inter-subband ACLR/ACS is decoupled with the adjacent channel ACLR/ACS. The problem of this understanding is that ACLR/ACS for the inter-subband and adjacent channel ACLR/ACS are assumed to be different, then interband ACLR/ACS can be assumed to be fixed, adjacent channel ACLR/ACS can be simulated to find the appropriate requirement. However, this assumption is not reasonable if looking at the following figure. The SBFD’s DL inter-band ACLR performance should equal adjacent channel ACLR. The same understanding applies to ACS.

Observation 2: BS inter-subband ACLR/ACS performance should be assumed the same with adjacent channel ACLR/ACS in the adjacent co-existence simulation if flat model is used.

Then if BS inter-subband ACLR/ACS performance equals adjacent channel ACLR/ACS, and the simulation results will derive BS ACLR and ACS requirements. The understanding leads to the problem that co-channel co-existence and the adjacent channel co-existence are coupled. When adjacent channel ACLR/ACS is changed, the baseline SBFD system is also changed. Furthermore, there’s no evaluation on the co-channel co-existence when ACLR/ACS is changed. So decouple the co-channel interference and adjacent channel co-existence simulation is candidate solution for this issue.

We have the above proposal because the if there’s internal interference in the baseline, the

Observation 3: When the victim noise floor is raised the requirement for the adjacent channel noise is more relaxed.

So assuming no co-channel inter-site inter-subband interference for the adjacent channel co-existence simulation can be a candidate for this issue. How to handle the co-channel co-existence in RAN1 and RAN4 can be discussed further.

Proposal 4: Decouple the co-channel inter-site inter-subband interference with adjacent channel co-existence simulation, i.e. no co-channel inter-site inter-subband interference is included in the adjacent channel co-existence simulation.

  - Remaining simulation assumptions

There’re also many other remaining open assumptions. The following table shows our views and the justifications on our choice are also included in the table. For some assumptions such as ACLR, ACS, etc, which are discussed in the BS, UE aspect, we have the proposals in the corresponding contributions. Our understanding is that they should align with the agreements. So the following table doesn’t include these assumptions.

Table 1: Remaining simulation assumptions

| Parameters | Assumption | Comments |
| --- | --- | --- |
| Guard band | No guard band in co-existence simulation | Guard band size doesn’t impact the ACIR simulation result. So it’s also ok that different GB is assumed for different companies. |
| Grid shift | Only simulate 100% before the conclusion of general part of the ISO requirement between base stations. | The ISO between the inter-site BS may be large. |
| UE dropping methods in simulation | Random dropping |  |
| Cluster-based UE dropping methodology | If cluster is adopted by the group, then consider RAN1 methodology |  |
| LoS probability for gNB-gNB case | - For Macro-gNB-to-Macro-gNB case - If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. - X = 0.75 - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901. | Align with RAN1 agreement |
| SBFD antenna configuration | SBFD antenna configuration 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. | This is a more practical solution if considering the coverage. It’s better to choose one configuration at least for the calibration purpose. |

- Summary

This contribution provides our consideration of the remaining open assumptions of the adjacent channel co-existence. For co-channel interference, we have the following observations and proposals.

Observation 1: SBFD co-channel interference doesn’t need to be added in the simulation when legacy TDD is the victim.

Observation 2: BS inter-subband ACLR/ACS performance should be assumed the same with adjacent channel ACLR/ACS in the adjacent co-existence simulation if flat model is used.

Observation 3: When the victim noise floor is raised the requirement for the adjacent channel noise is more relaxed.

Proposal: Decouple the co-channel inter-site inter-subband interference with adjacent channel co-existence simulation, i.e. no co-channel inter-site inter-subband interference is included in the adjacent channel co-existence simulation.

The remaining open assumptions are summarized in Table 1.

Reference

[1] R4-2217466, “WF for adjacent channel co-existence evaluation of SBFD operation”, CMCC

[2] R4-2217513, “WF on SBFD feasibility study and RF impact: UE aspect”, Qualcomm

[3] R4-2217464, “WF on SBFD feasibility study and RF impact: BS aspect”, Samsung