3GPP TSG-RAN4 Meeting #105	R4-2218491

# Toulouse, France, 14-18 November 2022

Source:			Ericsson

Title:			NTN enhancements – SAN RF

Agenda item:		8.23.3

Document for:		Approval

# Introduction

In the scope of Release 18, a new WI on NR NTN enhancements [1] has been approved by RAN. One of the objectives is to consider NR-NTN deployment in above 10 GHz bands.

In last RAN4#104-bis-e meeting, a WF [3] capturing the initial agreements on SAN RF requirements. This contribution discusses further those requirements.

# Discussion

## SAN type

In last RAN4-104-bis-e meeting, it was proposed to specify SAN type 2-O only.

First, it shall be noted that type 2-O would only be relevant if the frequency range FR2 will be extended to include the Ka-band, as we have proposed in our other contributions [2].

Then, it was commented in last meeting that a type 2-H might also be relevant considering there is no major difference between SAN operating in FR1 and SAN operating in FR2, , from an architecture point of view.

For TN BS operating in FR2, type 2-H would not make sense, only type 2-O is relevant. But such consideration might not apply for SAN, as it was commented in last meeting.

Initially, for SAN operating in FR1, our initial proposal was to only specify type 1-O but we compromised to also consider type 1-H.

Following our initial thinking, for FR2, if there is no strong other request, to reduce the effort, we would propose to only consider SAN type 2-O. And if any company has some concern regarding inconsistency between SAN operating in FR1 vs SAN operating in F2, we suggest removing type 1-H from FR1, or to specify type 2-H in a later WI.

Proposal1: Specify SAN type 2-O only, and not type 2-H.

## OTA Tx requirements

Following Table 1 lists all the candidate Tx requirements, mentioning if those requirements should be specified or not for SAN operating in the NTN Ka-band. Note that we are assuming here that RAN4 will only specify SAN type 2-O, not SAN type 2-H.

Table 1: OTA Tx requirements overview

| Requirements | Comment |
| --- | --- |
| SAN output power | To be specified. Most likely similar to FR1, SAN output power should be declared by manufacturer for GEO and LEO classes. |
| Output power dynamics |  |
| RE power control dynamic range | To be specified. Similar to BS FR2 requirements. |
| Total power dynamic range | To be specified. Similar to BS FR2 requirements. |
| Transmit ON/OFF power | Not applicable, SAN are operating in FDD. |
| Transmitted signal quality |  |
| Frequency error | To be specified. Similar to BS FR2 requirements. |
| Modulation quality | To be specified. Similar to BS FR2 requirements. |
| Time alignment error | Most likely not applicable for SAN. |
| Unwanted emissions |  |
| Occupied bandwidth | To be specified. Similar to BS FR2 requirements. |
| Adjacent Channel Leakage Power Ratio | To be further discussed based on coexistence simulations conclusions. |
| Operating band unwanted emissions | To be further discussed. |
| Transmitter spurious emissions | To be further discussed. |
| Transmitter intermodulation | Most likely not applicable for SAN. |

Proposal2: Agree with the list of OTA Tx requirements to be specified as listed in  Table 1.

## OTA Rx requirements

Following Table 2 lists all the candidate OTA Tx requirements, mentioning if those requirements should be specified or not for SAN operating in the NTN Ka-band. Note that we are assuming here that RAN4 will only specify SAN type 2-O, not SAN type 2-H.

Table 2: OTA Rx requirements overview

| Requirements | Comment |
| --- | --- |
| OTA sensitivity | To be specified. Most likely similar to FR1, OTA sensitivity should be declared by manufacturer. |
| OTA Reference sensitivity level | To be specified. Similar to BS FR2 requirements. |
| OTA Dynamic range | To be specified. Similar to BS FR2 requirements. |
| OTA ACS | To be further discussed based on coexistence simulations conclusions. |
| OTA In-band blocking | To be further discussed. |
| OTA Out of band blocking | To be further discussed. |
| OTA Receiver spurious emission | Not applicable for SAN. |
| OTA Receiver intermodulation | Most likely not applicable for SAN. |
| OTA In-channel selectivity | To be further discussed based on IoT curves. |

Proposal3: Agree with the list of OTA Tx requirements to be specified as listed in Table 2.

# Conclusion

In this contribution, we discussed the SAN RF types and requirements . We made the following proposals:

Proposal1: Specify SAN type 2-O only, and not type 2-H.

Proposal2: Agree with the list of OTA Tx requirements to be specified as listed in Table 1.

Proposal3: Agree with the list of OTA Tx requirements to be specified as listed in Table 2.

# References

- RP-222654, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2218488, NTN enhancements – Regulatory aspects, Ericsson

- R4-2217467, Way Forward on NR NTN Enhancements Part1, Thales