3GPP TSG-RAN4 Meeting #105	R4-2218538

# Toulous, France, 14 – 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for CG-SDT testcase for FR1 revision from R4-2217245 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Perf |  |  |  |  |  | Date: |  |  | 2022-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update detail timeline description for FR1 CG-SDT test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases requirements are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2.1, A.6.2.1.1, A.6.2.1.2, A.6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

## A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)

A.6.2.1.1 Test purpose and Environment

The purpose of this test is to verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. In the test, UE is configured two CG-SDT configurations at different time. The test consists of six successive time periods, with time duration of T1, T2, T3, T4, T5, and T6 repectively. Figure A.6.2.1.1-1 shows the variation of the RSRP per time. Prior to the time point A, the UE shall be fully synchronized to PCell.

Figure A.6.2.1.1-1: RSRP variation model for CG-SDT testing

A.6.2.1.2 Test Parameters

There is one cells in the test, the FR1 PCell. The test parameters for the PCell are given in Table A.6.2.1.2-1, Table A.6.2.1.2-2, and Table A.6.2.1.2-3.

Table A.6.2.1.2-1: NR configuration for FR1 SSB

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.2.1.2-2 : General test parameters

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| TDD Configuration |  | 1 | N/A |  |
|  |  | 2 | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  |
| BWchannel | MHz |  | 10: NRB,c = 52 |  |
| PDSCH Reference measurement channel |  | 1 | SR.1.1 FDD |  |
|  |  | 2 | SR.1.1 TDD |  |
|  |  | 3 | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1 | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1 | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  |
| SSB configuration |  | 1,2,3 | SSB.1 FR1 |  |
| OCNG Patterns |  | 1,2,3 | OP.1 |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1,2,3 | SMTC.1 |  |
| DRX configuration |  | 1,2,3 | DRX.7 |  |
| T1 | s | 1,2,3 | 0.64 |  |
| T2 | s | 1,2,3 | 0.64 |  |
| T3 | s | 1,2,3 | 1.28 |  |
| T4 | s | 1,2,3 | 1.28 |  |
| T5 | s | 1,2,3 | 0.3 |  |
| T6 | s | 1,2,3 | 1.28 |  |
| cg-SDT-RSRP-ChangeThreshold | dB | 1,2,3 | 8 |  |
| cg-SDT-TimeAlignmentTime |  | 1,2,3 | Infinity |  |
| CG-SDT resource period | ms | 1,2,3 | 640ms | configuredGrantConfig Peridocity at 38.331 Config 1,2 : sym640x14, config 3 : sym1024x14 |
| Propagation condition |  | 1,2,3 | AWGN |  |

Table A.6.2.1.2-3: SSB specific test parameters

| Parameter | Config |  | Unit |  | SSB#0 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1,T2 | T3 |  | T4 |  | T5 |
| Note1 | 1,2,3 |  | dBm/15kHz |  | -100 |  |  |  |  |  |
| Note1 | 1,2 | dBm/SSB SCS | -100 |  |  |  |
|  | 3 | dBm/SSB SCS | -103 |  |  |  |
|  | 1,2,3 |  | dB |  | 8.35 | 0 |  | 13.25 |  | 18.35 |
| SS RSRP Note2 | 1,2,3 |  | dBm/SSB SCS |  | -91.65 | -100 |  | -86.75 |  | -81.65 |
| Io Note3 | 1,2 |  | dBm/9.36 MHz |  | -63.11 | -69.04 |  | -58.60 |  | -53.64 |
|  | 3 | dBm/38.16MHz | -55.84 | -61.83 | -51.73 | -50.19 |
|  | 1,2,3 |  | dB |  | 8.35 | 0 |  | 13.25 |  | 18.35 |

A.6.2.1.3 Test requirements

SDT threahold should meet minimum -100 dBm RSRP to trigger CG-SDT procedure. RA-SDT is not configured in this test.  UL data payload size assumed to be small to meet SDT trigger criteria.

Until reaching time point D, UE will skip CG-SDT occasion by assuming UL data is not ready. UE should perform TA validation at most 640ms prior to time pointD and time point F.

After T1 from RSRP changes from -100 dBm to -91.65dBm, UE receive RRC release message with CG-SDT configuration at time point A.

After T2 from time point A, RSRP is changed to -100 dBm from -91.65 dBm at time point B.

After T3 from time point B, RSRP is changed to -86.75 dBm from -100 dBm at time point C.

After T4 from time point C, UE shall transmit PUSCH at time point D.

After T5 from time point D, UE receive RRC release message to indicate first CG-SDT session is finished. RSRP is changed to -81.65 dBm from -86.75 dBm.

After point E, UE shall not transmit PUSCH.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change>