3GPP TSG-RAN WG4 Meeting # 105	R4-2218540

# Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.9.2.2

Source: 		Qualcomm Incorporated

Title: 			Remaining issues on RRM requirements about FR2 unknown Scell activation enhancements L1 part

Document for:	Approval

# 1.	Introduction

In this paper, we discuss remaining issue about RRM requirements RRM requirements about FR2 unknown Scell activation enhancements from the last RAN4 meeting [1].

# 2. 	Discussion

From the last meeting, it was agreed that to discuss about the conditions for skipping L1-RSRP measurement.

Issue 3-1-3: Whether and how to skip L1-RSRP measurement of FR2 unknown SCell activation? Agreements: - RAN4 to discuss the conditions for skipping L1-RSRP measurement when activating an FR2 unknown SCell.

The framework that we proposed are duplicated from our previous documents [2] to help understanding.

By assuming NW allow UE to report L3 measurement report and UE report meaningful RSRP results, L1-measurement and report can be skipped for certain range of L3 measurement reports.

Proposal: L1 measurement and report can be skipped when UE report L3 measurements where the RSRP is acceptable range. Acceptable range could be FFS.

Issue 3-2-1: Fine timing tracking for SSB corresponding to the TCI state during FR2 unknown SCell activation FFS: - Option 1 (Apple, QC, Intel, HW, ZTE, MTK): Fine timing tracking is still needed no matter whether TCI activation is skipped or not. - Option 2 (Apple, QC, Intel, Xiaomi, HW, OPPO, vivo, ZTE, MTK): Skipping fine timing tracking for SSB corresponding to TCI state (by reusing SSB timing from L3/ L1 measurements) is not recommended, because during L1/L3 measurements UE tracking performance is not accurate enough. In addition, this skipping does not reduce the delay much (only 1*Trs) compared to its impact on acquiring the accurate timing. - Option 3 (Ericsson): RAN4 to confirm, UE timing assumption when performing L1-RSRP measurements. - Option 4 (Apple): for unknown R18 FR2 SCell activation enhancement, skip fine timing tracking for SSB corresponding to the TCI state, if SSB timing from L3 measurement stage or SSB timing from L1-RSRP measurement stage can be reused. Issue 3-2-2: TCI activation enhancement during FR2 unknown SCell activation FFS: - Option 1 (Apple, Intel, CMCC, Xiaomi, HW, OPPO, CTC, MTK): the uncertainty of TCI configuration/activation can be saved when TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report. - Option 1a(Apple, Intel, HW, CTC): for FR2 unknown SCell activation enhancement, the TCI of PDCCH/PDSC/CSI-RS is associated with the best L1-RSRP report if no MAC CE or RRC indication for TCI is sent to UE. - Option 2 (vivo): During the Scell activation, only the TCI from CSI-RS used for CQI needs to be configured. The PDCCH/PDSCH can follow the same TCI state information as CSI-RS. In this way, the PDCCH/PDSCH TCI configuration can be saved and the Scell activation delay can be reduced accordingly. - Option 3 (QC, Ericsson, Nokia, vivo, ZTE): Gain in delay reduction is quite small for skipping TCI state indication and RAN4 not consider TCI state skipping.

Observation: There is no big gain from skipping fine time tracking and TCI state indication in terms of delay reduction.

Proposal: Fine time tracking is not skipped. Either SSB-based or A-TRS based fine time tracking can be performed.

Proposal: TCI indication process is not skipped.

Issue 4-1-3: known/unknown condition enhancement for FR2 SCell activation FFS: - Proposal 1 (Nokia): The 3/4s time constraint of L3-RSRP reporting in known/unknown condition needs to be removed to avoid the SCell unnecessarily being considered as unknown. - Proposal 2 (Apple, HW, Ericsson, ZTE, Nokia):RAN4 to further discuss the known condition extension to facilitate SCell activation. - Proposal 3 (QC, Ericsson): RAN4 not to consider mandatory aperiodic reports before SCell activation

Observation: aperiodic report before Scell activation can introduce more power consumption.

Proposal: aperiodic report should be triggered after receiving Scell activation command.

# 3. Conclusions

Proposal: L1 measurement and report can be skipped when UE report L3 measurements where the RSRP is acceptable range. Acceptable range could be FFS.

Observation: There is no big gain from skipping fine time tracking and TCI state indication in terms of delay reduction.

Proposal: Fine time tracking is not skipped. Either SSB-based or A-TRS based fine time tracking can be performed.

Proposal: TCI indication process is not skipped.

Observation: aperiodic report before Scell activation can introduce more power consumption.

Proposal: aperiodic report should be triggered after receiving Scell activation command.

# 4.    References

[1] R4-2217249, WF on R18 eFeRRM - FR2 SCell activation enhancement, Apple, RAN4 #104bis-e

[2] R4-2216744, Discussion on RRM requirements for FR2 unknown Scell activation delay reduction, Qualcomm Incorporated, RAN4 #104bis-e.