3GPP RAN WG4 Meeting #104-bis-e		R4-2218622

Toulouse, France, November 14th-18th, 2022

Source: 	Murata Manufacturing Co. Ltd

Title: 	Unified Triple Beat Detection Equations for all 4 Triple Beats

Agenda item:	X. Y. Z

Document for:	Approval

# 1	Introduction

As documented in WF [1], there are two different equation formats for detecting Triple Beat:

1). TB1 & 2 are described by the following equation:

- A 1st order triple beat product (TB1 & TB2) is a 3rd order non-linearity for which a collision with the FDD DL band is true if the following condition is met:

Duplex Offset<ULCAMBW+TXMBW2 +RXBW2									(1)

where,

  - Duplex Offset is the duplex offset of the victim FDD band.

  - ULCAMBW is the non-contiguous 2 tone allocation spacing.

  - TXMBW and RXBW is the FDD victim band TX transmission measured BW and the RX DL channel BW respectively.

2). TB3 & 4 are described by the following equations based on Figure 1:

- A 1st order triple beat product is a 3rd order non-linearity which may affect the third DL band of a three-band inter-band combination if either one of the following TB product falls in the Rx frequency range of the third band:

  - TB3 = |f1-f2-f3| or |f2+f3-f1|

  - TB4 = f1+f2+f3 or |-f1-f2-f3|

TB3:

F3rdDL+RXBW3rdDL2>f2+f3-f1- MBW12 -12*SCS

and

F3rdDL-RXBW3rdDL2<f2+f3-f1+MBW12 +12*SCS

TB4:

F3rdDL+RXBW3rdDL2>f1+f2+f3-MBW12 -12*SCS

and

F3rdDL-RXBW3rdDL2<f1+f2+f3+MBW12 +12*SCS                       (2)

where,

- f1 is the center frequency of the non-ULCA uplink resource block (RB)

- f2 and f3 are the center frequencies of each of the ULCA CC uplink RB.

- MBW1 is the measured bandwidth of the non-ULCA CC at the frequency f1.

- RXBW3rdDL is the RX channel bandwidth in the 3rd DL band

Figure 1: 1st order triple beat product landscape using example of DC_3A_n1A-n75A [9]

In this document, we provide an alternative set of equations set for TB1 &2 that are equivalent to Equation (1) but follow the format of (2). A consistent format of equations may be easier for users to follow and apply in terms of automation.  So, we propose an alternative option for detecting TB1 and TB2.

# Discussion

Using Figure 1, by comparing signal/band edges, it’s straight forward to derive the following equations for TB1 & TB2:

TB1:

F3rdDL+RXBW3rdDL2>f1+f2-f3- MBW12 -12*SCS

and

F3rdDL-RXBW3rdDL2<f1+f2-f3+MBW12 +12*SCS

TB2:

F3rdDL+RXBW3rdDL2>f1-f2+f3-MBW12 -12*SCS

and

F3rdDL-RXBW3rdDL2<f1-f2+f3+MBW12 +12*SCS                   (3)

where,

- f1 is the center frequency of the non-ULCA uplink resource block (RB)

- f2 and f3 are the center frequencies of each of the ULCA CC uplink RB.

- MBW1 is the measured bandwidth of the non-ULCA CC at the frequency f1.

- RXBW3rdDL is the RX channel bandwidth in the 3rd DL band

- TB1 = |f1+f2-f3| or |-f1-f2+f3|

- TB2 = |f1-f2+f3| or |-f1+f2-f3|

Observation 1:  Using TB Detection Equation sets (2) & (3), all 4 sets of detection equation sets for TB1, 2, 3 & 4 are essentially the same.  This may be desirable for automation.

The uniformity of the Triple Beat Detection Equation sets prompts us to combine all 4 sets of equations into a single one.

Proposal 1: Modify the Triple Beat Detection Equations as in (4) below:

- A 1st order triple beat product is a 3rd order non-linearity which may affect the third DL band of a three-band inter-band combination if either one of the following TB product falls in the Rx frequency range of the third band:

  - TB1 = |f1+f2-f3|

  - TB2 = |f1-f2+f3|

  - TB3 = |f1-f2-f3|

  - TB4 = f1+f2+f3

When both non-equal equations in (4) are satisfied, a TB product is falling into the Rx freq range of the 3rd band and potential de-sense might happen:F3rdDL+RXBW3rdDL2>TBn- MBW12 -12*SCS

and

F3rdDL-RXBW3rdDL2<TBn+MBW12 +12*SCS					(4)

Note 1: n= 1, 2, 3 or 4.

Note 2: If only one non-equality is satisfied, it means that TBn product does Not fall into the Rx band.

Note 3: Here the “3rd Rx band” could be any Rx band in the CA or DC combination under investigation.

# 3  Conclusion

Observation 1:  Using TB Detection Equations (2) & (3), all 4 sets of detection equations for TB1, 2, 3 & 4 are essentially the sameThis may be desirable for automation.

Proposal 1: Consider using the Triple Beat Detection Equations as in (4) below as an alternative to the existing detection for TB1 and TB2:

- A 1st order triple beat product is a 3rd order non-linearity which may affect the third DL band of a three-band inter-band combination if either one of the following TB product falls in the Rx frequency range of the third band:

  - TB1 = |f1+f2-f3|

  - TB2 = |f1-f2+f3|

  - TB3 = |f1-f2-f3|

  - TB4 = f1+f2+f3

When both non-equal equations in (4) are satisfied, a TB product is falling into the Rx freq range of the 3rd band and potential de-sense might happen:

F3rdDL+RXBW3rdDL2>TBn- MBW12 -12*SCS

and

F3rdDL-RXBW3rdDL2<TBn+MBW12 +12*SCS					(4)

Note 1: n= 1, 2, 3 or 4.

Note 2: If only one non-equality is satisfied, it means that TBn product does Not fall into the Rx band.

Note 3: Here the “3rd Rx band” could be any Rx band in the CA or DC combination under investigation.

# References:

- R4-2217906, “Way forward on potential triple beat issue for 2 band and 3 band combinations”, Murata, Skyworks, ZTE, 3GPP RAN4#104-bis-e.