3GPP TSG-RAN WG4 Meeting # 105		R4-2218704

# Toulouse, France, November 14 – November 18, 2022

Source:	Ericsson

Title:	Simulation results for FR2-2 PUSCH demodulation requirements

Agenda item:	6.3.6.3.1

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the NR extended to 71GHz demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Table 2.1 Simulation parameters for FR2-2 normal PUSCH demodulation

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled and enabled |
| Default TDD UL-DL pattern (Note 1) |  | 120kHz SCS:3D1S1U, S=10D:2G:2U 480kHz SCS: 14D2S4U, S1=12D:2G:0U, S2=0D:6G:8U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 120kHz SCS: 100M and 400M 480kHz SCS: 400M |
|  | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS | Frequency density (KPT-RS) | 2 |
| configuration | Time density (LPT-RS) | 1 |
| Test metric | Normalized throughput | 70% |
| Antenna | Tx and Rx configuration | 1Tx 2Rx, 2Tx2Rx |
| Channel model |  | 100MHz CBW: TDLA30-650, TDLD30-200; >=400MHz CBW: TDLA10-650, TDLD10-200 |
| MCS | 64QAM MCS table index | 4,16, 20, [18] |
| Phase noise | Model sets in TR38.808 | Set#1 |
| Note1: Companies deliver two sets of ideal simulation results for requirement discussion. Result set#1 is without phase noise and set#2 is with phase noise. • No Tx phase noise is modelled • Rx Phase noise is modelled only to find feasible FRC configuration (i.e. achieve maximum throughput and loss in comparison to scenarios without Rx phase noise is less than 1 dB) • 70 GHz carrier frequency is assumed Note2: CPE compensation is considered. |  |  |

Table 2.2 Simulation results for FR2-2 normal PUSCH demodulation, 120kHz SCS, 100MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 1 | 2 | Normal | TDLA30-650 | 70% | 4 | Set1 | Disabled | -3.23 | -2.68 |
|  |  |  |  |  |  |  | Enabled | -3 | -2.8 |
|  |  |  | TDLA30-650 |  | 16 | Set1 | Disabled | 7.42 | 8.07 |
|  |  |  | TDLD30-200 |  | 18 | Set1 | Disabled | 7.49 | 8.55 |
|  |  |  | TDLD30-200 |  | 20 | Set1 | Disabled | 9.19 | 10.32 |
| 2 | 2 | Normal | TDLA30-650 | 70% | 4 | Set1 | Disabled | 0.59 | 1.18 |
|  |  |  | TDLA30-650 |  | 16 | Set1 | Disabled | 14.37 | 15.08 |
|  |  |  | TDLD30-200 |  | 16 | Set1 | Disabled | 9.81 | 11.28 |
|  |  |  | TDLD30-200 |  | 18 | Set1 | Disabled | 11.18 | 12.35 |
|  |  |  | TDLD30-200 |  | 20 | Set1 | Disabled | 13.42 | 14.56 |

Table 2.3 Simulation results for FR2-2 normal PUSCH demodulation, 120kHz SCS, 400MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 1 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | -3.37 | -3.15 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 7.87 | 8.39 |
|  |  |  | TDLD10-200 |  | 18 | Set1 | Disabled | 7.54 | 7.97 |
|  |  |  | TDLD10-200 |  | 20 | Set1 | Disabled | 9.57 | 10.07 |
| 2 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | 0.55 | 0.87 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 16.28 | 16.96 |
|  |  |  | TDLD10-200 |  | 16 | Set1 | Disabled | 9.6 | 10.4 |
|  |  |  | TDLD10-200 |  | 18 | Set1 | Disabled | 11.12 | 11.62 |
|  |  |  | TDLD10-200 |  | 20 | Set1 | Disabled | 13.2 | 13.71 |

Table 2.3 Simulation results for FR2-2 normal PUSCH demodulation, 480kHz SCS, 400MHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix | Fraction of maximum throughput | MCS | PN Model | Precoding | SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | No CPE | CPE |
| 1 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | -2.91 | -2.5 |
|  |  |  |  |  |  |  | Enabled | -2.76 | -2.72 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 7.53 | 8.13 |
|  |  |  | TDLD10-200 |  | 18 | Set1 | Disabled | 7.46 | 8.54 |
|  |  |  | TDLD10-200 |  | 20 | Set1 | Disabled | 9.21 | 10.39 |
| 2 | 2 | Normal | TDLA10-650 | 70% | 4 | Set1 | Disabled | 1.04 | 1.38 |
|  |  |  | TDLA10-650 |  | 16 | Set1 | Disabled | 14.48 | 15.17 |
|  |  |  | TDLD10-200 |  | 16 | Set1 | Disabled | 9.75 | 11.39 |
|  |  |  | TDLD10-200 |  | 18 | Set1 | Disabled | 11.24 | 12.43 |
|  |  |  | TDLD10-200 |  | 20 | Set1 | Disabled | 13.5 | 14.75 |

# Conclusions

Simulation results for FR2-2 PUSCH demodulation are delivered.

# References

[1]	R4-2217372, WF on PUSCH demodulation requirements for FR2-2, Nokia, Nokia Shanghai Bell, RAN4#104bis-e

# Annex Channel model

Table A.1 Simplified TDLA10 model (16 taps, 2ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -16.1 | Rayleigh |
| 2 | 4 | 0 | Rayleigh |
| 3 | 6 | -4 | Rayleigh |
| 4 | 8 | -10.2 | Rayleigh |
| 5 | 16 | -18.6 | Rayleigh |
| 6 | 18 | -9.3 | Rayleigh |
| 7 | 22 | -13.7 | Rayleigh |
| 8 | 24 | -17.9 | Rayleigh |
| 9 | 26 | -13.5 | Rayleigh |
| 10 | 30 | -14 | Rayleigh |
| 11 | 40 | -15.4 | Rayleigh |
| 12 | 44 | -18.9 | Rayleigh |
| 13 | 46 | -21.0 | Rayleigh |
| 14 | 48 | -21.6 | Rayleigh |
| 15 | 50 | -19.3 | Rayleigh |
| 16 | 96 | -25.9 | Rayleigh |

Table A.2 Simplified TDLA30 model (12 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -15.5 | Rayleigh |
| 2 | 10 | 0 | Rayleigh |
| 3 | 15 | -5.1 | Rayleigh |
| 4 | 20 | -5.1 | Rayleigh |
| 5 | 25 | -9.6 | Rayleigh |
| 6 | 50 | -8.2 | Rayleigh |
| 7 | 65 | -13.1 | Rayleigh |
| 8 | 75 | -11.5 | Rayleigh |
| 9 | 105 | -11.0 | Rayleigh |
| 10 | 135 | -16.2 | Rayleigh |
| 11 | 150 | -16.6 | Rayleigh |
| 12 | 290 | -26.2 | Rayleigh |

Table A.3 Simplified TDLD10 model (10 taps, 2ns resolution)

| Tap # | 2ns resolution |  | Fading distribution |
| --- | --- | --- | --- |
|  | Delay [ns] | Power [dB] |  |
| 1 | 0 | -0.2 | LOS |
|  | 0 | -12.4 | Rayleigh |
| 2 | 6 | -21.1 | Rayleigh |
| 3 | 14 | -16.7 | Rayleigh |
| 4 | 18 | -18.3 | Rayleigh |
| 5 | 26 | -22 | Rayleigh |
| 6 | 40 | -27.9 | Rayleigh |
| 7 | 80 | -23.7 | Rayleigh |
| 8 | 94 | -24.9 | Rayleigh |
| 9 | 98 | -30.0 | Rayleigh |
| 10 | 126 | -27.7 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |

Table A.4 Simplified TDLD30 model (10 taps, 5ns resolution)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.2 | LOS path |
|  | 0 | -12.4 | Rayleigh |
| 2 | 20 | -21 | Rayleigh |
| 3 | 40 | -16.7 | Rayleigh |
| 4 | 55 | -18.3 | Rayleigh |
| 5 | 80 | -21.9 | Rayleigh |
| 6 | 120 | -27.8 | Rayleigh |
| 7 | 240 | -23.6 | Rayleigh |
| 8 | 285 | -24.8 | Rayleigh |
| 9 | 290 | -30.0 | Rayleigh |
| 10 | 375 | -27.6 | Rayleigh |
| Note 1: Tap #1 follows a Ricean distribution. |  |  |  |