3GPP TSG-RAN WG4 Meeting # 105		R4-2218714

Toulouse, France, November 14 – November 18, 2022

Source:	Ericsson

Title:	Simulation results for NR SAN PUCCH demodulation requirements

Agenda item:	6.2.6.2.2

Document for:	Information

# Introduction

In the last RAN4 meeting, companies have discussions on the SAN PUCCH demodulation requirements. In this contribution, simulation results based on previous meeting WF [1] are delivered for further discussion.

# Simulation result

Table 2-1 Simulation parameters for NTN PUCCH multi-slot

| Parameter | Test |
| --- | --- |
| PUCCH format | 1 (multi-slot) |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | disabled |
| Inter-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Number of slots for PUCCH repetition | 2 |
| Cyclic Prefix | Normal |
| Number of TX antennas | 1 |
| Number of RX antennas | 1, 2 |
| Propagation conditions and correlation matrix | NTN-TDLA100-200 Low |
| Test metric | NACK to ACK probability<0.1%, ACK missed detection probability <1% |

Table 2-1 Simulation results for NTN PUCCH PF1 multi-slot with 15kHz SCS, 5MHz CBW, NACK->ACK <0.1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | -0.1 |
|  | 2 | NTN-TDLA100-200 | -2.8 |

Table 2-2 Simulation results for NTN PUCCH PF1 multi-slot with 30kHz SCS, 10MHz CBW, NACK->ACK <0.1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 3.2 |
|  | 2 | NTN-TDLA100-200 | -5.1 |

Table 2-3 Simulation results for NTN PUCCH PF1 multi-slot with 15kHz SCS, 5MHz CBW, ACK miss <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | -0.3 |
|  | 2 | NTN-TDLA100-200 | -4.7 |

Table 2-4 Simulation results for NTN PUCCH PF1 multi-slot with 30kHz SCS, 10MHz CBW, ACK miss <1%

| Number | Number of | Propagation conditions and |  |
| --- | --- | --- | --- |
| of TX antennas | RX antennas | correlation matrix | SNR [dB] |
| 1 | 1 | NTN-TDLA100-200 | 2.8 |
|  | 2 | NTN-TDLA100-200 | -5.5 |

# Conclusions

Simulation results for SAN PUCCH demodulation requirements are delivered.

# References

[1]	R4-2217348, WF for NTN NR SAN demodulation, Huawei HI Silicon, RAN4#104bis-e

# Annex A Channel model for NTN simulation

Table A-1 NTN-TDLA100 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Table A-2 NTN-TDLC5 delay profile (resolution 5ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| NOTE: The first tap follows a Rician distribution with a K-factor of K1 = 8.3 dB and a mean power of 0dB. |  |  |  |