3GPP TSG RAN WG4 Meeting #105	R4-2218788

Toulouse, France, November 14 – November 18, 2022

Agenda item:			10.2.1.3

Source:	vivo

Title:	Draft Reply LS on support of positioning in FR2-2

Document for:	Discussion

- Introduction

In the LS from RAN2[1], RAN2 asks RAN4 to provide feedback if SRS for positioning/DL-PRS with 480/960 kHz SCS be supported in FR2-2 in R17.

In R17, NR operation is extended up to 71 GHz and new numerologies (i.e., 480 kHz SCS, 960 kHz SCS) are introduced in FR2-2 (52.6 GHz–71 GHz) for data and control channels and reference signals. Moreover, FR2-2 is assumed to be applicable to other Rel-17 features unless otherwise specified based on the below agreement from 71GHz WI in RAN2. From chairman’s note in RAN2#117-e meeting, Applicability of FR2-2 to other Rel-17 features 4.3-1: From RAN2 point of view, FR2-2 are assumed to be also applicable to other Rel-17 features, unless otherwise specified (e.g. if the feature is only for FR1). No impact to the specification expected for cases where we don't specify otherwise. Companies can bring up cases (e.g. for some WIs where FR2-2 has not been discussed at all) where differentiation is needed by contributions to May meeting.. On the other hand, in POS WI, there has been no discussion on the support of positioning (especially related to RAT-dependent positioning) in FR2-2 and thus it is unclear whether SRS for positioning/DL-PRS can be used with the new 480/960 kHz SCS in FR2-2 or not. Based on the above observation, RAN2 would like to ask RAN1 and RAN4 the following question: Question: Can SRS for positioning/DL-PRS with 480/960 kHz SCS be supported in FR2-2 in R17?

In this contribution, we provide our views on support of positioning in FR2-2.

- Discussion

In TS 38.133, the measurements accuracy for UE positioning in FR1 and FR2 are specified in section 10, which includes the accuracy requirements for RSTD measurement in clause 10.1.23.2, PRS-RSRP measurement in clause 10.1.24.2 and UE Rx-Tx time difference measurement in clause 10.1.25.2. Also, the measurements accuracy for gNB positioning in FR1 and FR2 are specified in section 13, which includes the accuracy requirements for gNB Rx-Tx time difference measurements in clause 13.2.2, UL SRS RSRP measurement in clause 13.3.2 and gNB Rx-Tx time difference measurements for RTT-based PDC in clause 13.7.2.

In the above related requirements, the SCS for SRS for positioning/DL-PRS are specified with 60/120kHz for FR2-1. There have been no discussions on requirements for the SCS with 480/960kHz in FR2-2. In addition, there is no WI and TU to discuss and specify NR positioning support for FR2-2.

From RAN4 perspective, NR positioning is not supported for FR2-2. Thus, SRS for positioning/DL-PRS with 480/960 kHz SCS are not be supported in FR2-2 in R17.

Proposal 1: SRS for positioning/DL-PRS with 480/960 kHz SCS are not supported for FR2-2 in R17.

- Summary

Based on above analysis, we have following observation and proposal.

Proposal 1: SRS for positioning/DL-PRS with 480/960 kHz SCS are not supported for FR2-2 in R17.

A draft Reply LS is provided in Annex.

- References

[1] R2-2208810          LS on support of positioning in FR2-2          RAN WG2 Meeting #119-e

- Appendix: Draft reply LS

3GPP TSG RAN WG4 Meeting #105	R4-22x

Toulouse, France, November 14 – November 18, 2022

Title:	Reply LS on support of positioning in FR2-2

Response to:	R2-2208810

Release:	Rel-17

Work Item:	NR_pos_enh, NR_ext_to_71GHz

Source:	RAN4

To:	RAN2, RAN1

Cc:

Contact Person:

Name:	Qian Yang

E-mail Address: qian9.yang@vivo.com

1. Overall Description:

RAN4 thanks RAN2 for the LS on support of positioning in FR2-2.

For the question of supporting of NR positioning for FR2-2 asked by RAN2, RAN4 would like to provide the following feedback.

- SRS for positioning/DL-PRS with 480/960 kHz SCS are not be supported in FR2-2 in R17.

2. Actions:

To: RAN WG2

ACTION: 	RAN4 kindly asks RAN2 to take the above RAN4 conclusions into account in the future work.

4. Date of Next TSG RAN WG4 Meetings:

TSG-RAN4 Meeting #106			Feb. 27 – Mar. 03, 2023	Athens

TSG-RAN4 Meeting #106-bis-e		Apr. 17 – Apr. 26, 2023	Online