3GPP TSG-RAN WG4 Meeting #105	R4-2218805

# Electronic Meeting, Nov.14 -18, Toulouse, FR, 2022

Source: 	vivo

Title: 	On network A requirements for RRM requirements of MUSIM gaps

Agenda Item:			8.22.2.3

Document for:	Discussion

Introduction

At RAN 95 meeting the revised WI “Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR” [1] was approved. The objectives are:

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4].

- Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose

- RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR.

- Applicable UE architecture: Dual-RX/Dual-TX UE

The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2].

2. Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2]

- Define RRM requirements for Rel-17 MUSIM gaps [RAN4, RAN2]

  - The following MUSIM gap requirements are considered

    - Measurements in Network A

    - Measurements in Network B in RRC idle/inactive

    - Note: it is up to RAN4 decision whether to define requirements for Network B.

  - Identify and specify, if needed, solutions for MUSIM gap collision handling for the following cases [RAN4, RAN2]

    - Case 1: Collisions between MUSIM gap and legacy measurement gap (i.e., Rel-15 to Rel-17 measurement gaps)

    - Case 2: Collisions between MUSIM gap and SMTC

    - Case 3: Collisions between different MUSIM gaps

    - Note: RAN2 work can be triggered by RAN4 LS only, if needed

  - Identify impacts on L1 measurements, RLM/BFD and L3 measurements and specify corresponding UE requirements, if necessary, when MUSIM gap(s) are configured, for the following scenarios [RAN4]

    - Only MUSIM gap(s) are configured

    - MUSIM gap(s) and legacy measurement gap are configured

  - Note: requirements are applicable to MUSIM gaps defined in Rel-17 MUSIM WI (LTE_NR_MUSIM)

At RAN4 104e and 104-bis-4 meeting this WI was discussed within Rel-18 timeframe and the latest WF is [2]. In this contribution we provide our further considerations on network A requirements for this WI.

Discussion

- Regarding network A requirements, the following issues has been discussed at previous RAN4 meeting:

Issue 1-5-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling

- Proposals:

  - P1: RAN4 can start from outcome of concurrent gap design, i.e. counting Navailable and Ntotal when defining L1 and L3 measurement requirements.

  - P2: The following parameters need to be updated to account for collisions with MUSIM gaps

    - Kp for intra-frequency and inter-frequency measurements without gaps

    - Kgap for intra-frequency and inter-frequency measurements with gaps

    - Kgap_EUTRA for inter-RAT measurements

    - Kp_CSI-RS for CSI-RS L3 measurements

    - Kp,PRS,i for NR positioning measurements

    - CSSFintra for intra-frequency measurements

    - CSSFinter for intra-frequency measurements

    - CSSFinterRAT for intra-RAT measurements

    - P scaling factor for L1-RSRP and L1-SINR measurements

  - P3: Considering MUSIM gap impact on L3 measurements, define Kp and Kgap as follows

- Intra-frequency measurements (without gap):

  - Kp = Ntotal / Navailable, where

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion or non-dropped MUSIM gap occasion within the window W.

- W is the largest periodicity among MGs, MUSIM gaps and SMTC.

- Inter-frequency measurements:

  - Kgap = Ntotal / Navailable, where

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- W is the largest periodicity among MGs, MUSIM gaps and SMTC.

Considering MUSIM gap impact on L1 measurements, define P as follows:

  - Ntotal / Noutside_MG in FR1

  - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

  - Ntotal / Navailable in FR2 with Navailable > 0

Where,

- Ntotal is the total number of SSB resource occasions within the window W, including those overlapped with MGs, MUSIM gaps or SMTC occasions within the window, and

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any MG nor MUSIM gap within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any MG, MUSIM gap nor any SMTC occasion within the window W

- W is the largest periodicity among MGs, MUSIM gaps and SSB periodicity.

  - P4: Reuse principles used in Rel-17 when concurrent gaps are configured to define network A L1/L3 measurement requirements when MUSIM gaps are configured for issue 2-2-3 and 2-2-4, i.e., option 1, a scaling factor should be introduced for network A requirements when MUSIM gaps are configured.

  - P5: The issue can still be discussed parallelly with gap collision handling issue

  - P6: No discussion is needed until RAN4 achieves the agreements on MUSIM gap collision issues.

Agreements: No

The principle used for derived L1 and L3 measurement requirements has been discussed however there is no consensus. One argument is the discussion should be delayed until the consensus on gap collision handling solution is achieved. To our understanding the “counting” principle used by Rel-17 concurrent WI is independent from the gap handling solution.

Using intra-frequency measurement without gap for L3 measurement requirements when concurrent gaps are configured as an example (same principles are used for L1 measurement) [3], the scaling factor depends on the ratio between the total available L3 occasions within W and the available measurement occasions after collision handing between L3 and concurrent gaps, i.e., the radio depends on the number of occasions before and after gap collision handling. Hence the “ratio” principle can be directly reused no matter which gap collision handling solution will be used in the end.

-----------------------------------------------------------------------------------------------------------------------------------------------

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps,

Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows:

-	For a window W of duration max(SMTC period,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion:

-	Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and

-	Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.2B.3.

Kp = 1 when Navailable = 0.

-----------------------------------------------------------------------------------------------------------------------------------------------

Regarding various proposals for issue 1-5-1, we see P1, P2, P3 and P4 are based on the same understanding instead of different wording. We propose the following proposal

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps WI define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Navailable / Ntotal for network A requirements when MUSIM gaps are configured, i.e., the idea behind P1, P2, P3 and P4.

Issue 1-5-2: On the time window W for aperiodic gap

- Proposals:

  - Option 1: Not take aperiodic gap into account when determining the time window W, and clarify that the related measurement period will be longer.

  - Option 2: Take aperiodic gap into account when determining the time window W by adding a time margin.

    - Option 2a: W for aperiodic gap can be defined as: max(SMTC period, MGRP_max)+[M], where MGRP_max is the largest periodicity among all the periodic gaps and [M] is a time margin for the one-shot aperiodic gap.

Regarding issue 1-5-2, firstly it depends on the outcome of issue 1-5-1, if the principle of Rel-17 concurrent WI is not reused, then it makes little sense to discuss the window length W for aperiodic gap.

Proposal 2: Window length W for aperiodic gap should be discussed providing the principle of Rel-17 concurrent WI is reused for the L1/L3 measurement requirement specification when MUSIM gaps are configured.

Conclusion

In this contribution, we provide our considerations on network A requirements of RRM requirements for R17 MUSIM gaps and have the following observations and proposals:

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps WI define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Navailable / Ntotal for network A requirements when MUSIM gaps are configured, i.e., the idea behind P1, P2, P3 and P4.

Proposal 2: Window length W for aperiodic gap should be discussed providing the principle of Rel-17 concurrent WI is reused for the L1/L3 measurement requirement specification when MUSIM gaps are configured.

- References

[1] RP-220955, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, RAN 95e

[2] R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo, RAN4 104-bis-e

[3] TS 38.133 v17.6.0