3GPP TSG-RAN WG4 Meeting # 105												R4-2218875

Toulouse, France, November 14 – November 18, 2022

Source: 	vivo

Title: 	Discussion on the new type UE to support non-collocated deployment

Agenda Item:	8.11.2

Document for:	Discussion

- Introduction

In the last meeting, several new types UE were raised to support 4L MIMO with power imbalance:

Agreement:

- Agree the following table as UE RF architecture candidates on new Type UE for non-collocated non-contiguous intra-band NR-CA and inter-band EN-DC

  - Check whether there are other restrictions except for power imbalance. If some other restrictions are found, moderator will add them to the table immediately

- Prioritize Type 3a/3b discussion for 4MIMO layer in Rel-18.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |

In this contribution, we further discuss the related issues.

- Discussion

  - Power imbalance

For the type 3 UE, the first thing that needs to be decided is the power imbalance. Under the share LNA and AGC, the un-optimized band performance will be degraded, but it is noted the power imbalance is derived from the network deployment scenario. Taking the path loss model of Uma as an example:

We assume that the minimum distance between BS and UE is 35m and the inter-site distance is 500m, then 25 dB difference can be calculated. Actually, the power imbalance will be even larger considering other factors, e.g. antenna gain difference. If we want to enable the type 3 UE, the UE performance should be verified under the typical power imbalance as well as UE requirement, otherwise, the meaning of this new type UE will be greatly reduced because in most cases, it can not work normally.

Observation 1: The 25 dB power imbalance is derived from the network deployment.

Proposal 1: The requirement of Type 3 UE should be defined under 25 dB power imbalance.

  - Frequency separation

Another issue here is that frequency separation. In FR2, Fs for DL NCCA is defined and the reason here is that the bandwidth in FR2 is large and even though a separate mixer and BB can cover it, the complexity is not acceptable for mmWave UE, so in R15, we agreed that the mmWave UE can support DL NCCA even if it cannot support the CC separation in the full width of the supported band, thus the capability is required.

In FR1, only the UL Fs of UL is defined and the reason is quite different, this capability is introduced because the intra-band UL NC CA can be supported by 1PA or 2PA architecture which is depending on the frequency separation, and for DL, we think the such limitation is not needed.

Proposal 2: The frequency separation limitation for the new Type UE is not needed.

- Conclusion

In this contribution, we provide our views on the new type UE, and our proposals are as follows:

Observation 1: The 25 dB power imbalance is derived from the network deployment.

Proposal 1: The requirement of Type 3 UE should be defined under 25 dB power imbalance.

Proposal 2: The frequency separation limitation for the new Type UE is not needed.

- References

- R4-2217734,	WF on NonCol_intraB_ENDC_NR_CA for NR-CA and EN-DC New Type UE,	KDDI, RAN4#104bis-e