3GPP TSG-RAN WG4 Meeting # 105	R4-2218910

# Toulouse, France, November 14 – 18, 2022

Source:	LG Electronics

Title:	Discussion on timing for ATG RRM

Agenda item:	8.13.4.3

Document for:	Discussion

Introduction

In this contribution, we provide our views on timing for ATG RRM.

Discussion

UE based Timing pre-compensation

In the last meeting, ISD assumption was agreed 100~200km for ATG RRM requirements study. The existing max TA for RAR is 1ms for 30kHz SCS, and TA adjustment for MAC-CE could also handle the ATG UE speed (up to 1200km/h). So, for ISD up to 200km, the existing TA adjustment procedure could be applicable.

- Proposal 1: Use the current timing adjustment procedure (close-loop TA adjustment) as baseline for timing requirements.

deriveSSB-IndexFromCell

For NR FR1 TDD, deriveSSB-IndexFromCell is always enabled, so UE can assume that the neighbor cell is synchronous with the serving cell. However, due to longer propagation delay for ATG deployment, deriveSSB-IndexFromCell cannot be always feasible to use serving cell timing. To utilize the deriveSSB-IndexFromCell, the tolerance of deriveSSB-IndexFromCell or enable/disable of deriveSSB-IndexFromCell based on the propagation delay difference between serving and neighbor cell could be considered. If 200km ISD is considered, the propagation delay difference could be up to 2 slots, but it might be possible to use deriveSSB-IndexFromCell except for ATG cell center region. RAN4 needs to study to how to utilize the deriveSSB-IndexFromCell for ATG deployment.

- Proposal 2: RAN4 to study to utilize the deriveSSB-IndexFromCell based on the propagation delay difference between serving and neighbor cell.

NW Assistance information

Issue 3-1-3: BS location - Option 1: PositionVelocity-r17 in TS 38.331can be used to signal path of BS relative to ATG UE (Ericsson, CMCC, LGE, ZTE, CATT) - Option 1-1: the location should be in geodetic coordinate, not relative to ATG UE (CATT)

For NW assistance information and BS location, we also think PositionVelocity-r17 in TS 38.331 can be used. However, PositionVelocity-r17 includes velocity information as listed in following table and velocity information for BS location does not need to be defined in ATG NW. So, we think ReferenceLocation-r17 can be used as BS location information.

| PositionVelocity-r17 | ReferenceLocation-r17 (same as Ellipsoid-Point defined in TS37.355) |
| --- | --- |
| PositionVelocity-r17 ::= SEQUENCE { positionX-r17 PositionStateVector-r17, positionY-r17 PositionStateVector-r17, positionZ-r17 PositionStateVector-r17, velocityVX-r17 VelocityStateVector-r17, velocityVY-r17 VelocityStateVector-r17, velocityVZ-r17 VelocityStateVector-r17 } | ReferenceLocation-r17 ::= OCTET STRING # Ellipsoid-Point ::= SEQUENCE { latitudeSign ENUMERATED {north, south}, degreesLatitude INTEGER (0..8388607), -- 23 bit field degreesLongitude INTEGER (-8388608..8388607) -- 24 bit field } |

- Proposal 3. RAN4 can study following options to inform BS location.

  - Alt 1. ReferenceLocation-r17.

  - Alt 2. PositionVelocity-r17.

  - Alt 3. ReferenceLocation-r17 or PositionVelocity-r17. It is up to NW implementation.

Conclusion

In this contribution, we provide our views on timing requirement for ATG RRM and we propose

- Proposal 1: Use the current timing adjustment procedure (close-loop TA adjustment) as baseline for timing requirements.

- Proposal 2: RAN4 to study to utilize the deriveSSB-IndexFromCell based on the propagation delay difference between serving and neighbor cell.

- Proposal 3. RAN4 can study following options to inform BS location.

  - Alt 1. ReferenceLocation-r17.

  - Alt 2. PositionVelocity-r17.

  - Alt 3. ReferenceLocation-r17 or PositionVelocity-r17. It is up to NW implementation.