3GPP TSG-RAN WG4 Meeting # 105						      R4-2218987

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.21.3.2

Source: 	OPPO

Title: 	Discussion on L1 measurement of L1/L2 based inter-cell mobility

Document for:	Approval

1	Introduction

A WF on Rel-18 Mobility enhancement was approved in RAN4#104bis-e meeting [1]. An LS [2] was also sent from RAN2 on the progress of L1/L2-triggered mobility.

In this contribution, we discuss the issues on general aspects and scenarios of L1/L2 based inter-cell mobility.

2	Discussion

<Way forward >: Issue 1-2-1: L1-RSRP measurement delay requirements - Option 1 (MTK, Nokia): use the measurement delay requirements for L1 measurement on NSC in R17 as a baseline - Option 2 (Intel, QC, Xiaomi, CATT, CTC, vivo, Nokia): Wait for RAN1/2 progress - Option 3 (MTK, Huawei, Ericsson, Apple, Xiaomi, CATT, vivo, OPPO, Nokia): FFS

Some companies expect to use existing L1-RSRP measurement for a cell with different PCI than serving cell. But RAN2 agreed that ICBM is not a prerequisite for using L1/L2 mobility. Whether the term of NSC or CDP can be used here is still up to RAN2’s discussion. We are fine to wait for more progress from RAN1/RAN2.

Observation 1: L1-RSRP measurement delay requirements should be impact by RAN1/RAN2’s conclusion on whether ICBM is used.

Proposal 1: Fine to wait for more progress from RAN1/RAN2.

<Way forward >: Issue 1-2-2: Side condition in intra-frequency L1-RSRP measurement accuracy requirements - Revised Option 1 (QC, Huawei, MTK, Apple, vivo): Reuse legacy value SNR=-3dB - Revised Option 2 (vivo): SNR = - 6dB (same as L3 measurement) - Revised Option 3 (Intel, QC, Ericsson, Xiaomi, vivo, OPPO, Nokia): FFS1

In last meeting, RAN4 discussed whether lower SNR would be assumed compared to the case when L1 measurements are only used for beam managements. Side condition Ês/Iot in current SSB based L1-RSRP accuracy requirement is -3dB. It seems strange to have L1 measurement requirement tightened for a non-serving cell (candidate cell) compared to L1 measurement for a serving cell. So, we prefer to reuse legacy value SNR=-3dB for intra-frequency L1-RSRP measurement of L1/L2 based inter-cell mobility.

Proposal 2: Reuse legacy value SNR=-3dB for intra-frequency L1-RSRP measurement of L1/L2 based inter-cell mobility.

3	Conclusion

Observation 1: L1-RSRP measurement delay requirements should be impact by RAN1/RAN2’s conclusion on whether ICBM is used.

Proposal 1: Fine to wait for more progress from RAN1/RAN2.

Proposal 2: Reuse legacy value SNR=-3dB for intra-frequency L1-RSRP measurement of L1/L2 based inter-cell mobility.

4	Reference

[1] R4-2217259, WF on L1/L2 inter-cell mobility, MediaTek Inc.

[2] RP-221799, “Revised WID on Further NR Mobility Enhancements”, MediaTek Inc., RANP#96

[3] R2-2211061, LS on RAN2 agreements about L1/L2-triggered mobility (LTM), RAN2