3GPP TSG-RAN WG4 Meeting # 105	R4-2219001

# Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.22.2.5

Source: 	OPPO

Title: 	Discussion on other aspects for MUSIM gaps

Document for:	Approval

1	Introduction

MUSIM gaps were discussed and introduced in Rel-17. However, the corresponding requirements were postponed due to the lack of TU in RAN4. During RAN-P #95e meeting, the WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR [1] was revised to include the RRM requirements for Rel-17 MUSIM gaps. Based on the discussion during the last meeting [2], this contribution will discuss the following two issues.

- MUSIM overhead

- Order for applying the priority when number of colliding MGs is larger than 2

2	Discussion

Issue 1-7-1: MUSIM overhead - Proposals: - Option 1: Use the overhead cap principle on multiple concurrent gaps in Rel-17 as the baseline for MUSIM gaps, and discuss further enhancements considering : - Up to 3 periodic MUSIM gaps and one aperiodic MUSIM gap - Longer MGRP - Option 2: Regarding the overhead cap on all configured gaps for a UE, measurement requirement does not apply when more than one MGP is configured with MGRP=20ms in an FR - Option 3: RAN4 to define MUSIM gap overhead for MUSIM gap(s) - Option 4: RAN4 does not to define overhead cap for MUSIM gaps. < Agreement in Aug 19 GTW session >:  Regarding the overhead cap on concurrent gaps in Rel-17, measurement requirement does not apply when more than one MGP is configured with MGRP=20ms in an FR.

In Rel-17, the overhead cap rule on multiple concurrent gaps as shown above was agreed in the RAN4 #104-e meeting. The main reason is to avoid too much throughput loss. From this perspective, the overhead cap should also be introduced for MUSIM gaps, and the principle in Rel-17 could be used as the baseline.

Proposal-1: The overhead cap rule on concurrent gaps in Rel-17 can reused to MUSIM gap, i.e. measurement requirement does not apply when more than one gap is configured with MGRP=20ms in an FR.

However, different from concurrent gaps where at most 2 gaps could be configured in each FR, UE could be configured with up to 3 MUSIM gaps and one aperiodic gap. Excluding the case when more than one MGP with MGRP=20ms may also lead to a higher throughput loss in NW-A. For example, 2 MUSIM gaps with MGRP=40ms and 1 MUSIM gap with MGRP=20ms will cause the same throughput loss with 2 gaps with MGRP=20ms. For simplicity, we propose to preclude the case when more than 2 gaps are configured with MGRP<=40ms in an FR.

Another difference is the longer MGRP (320ms~5120ms) for MUSIM gaps, which is beneficial for the throughput in NW-A. In the scenario of 3 periodic MUSIM gaps and legacy gap for NW-A, it is desirable to configured one/more MUSIM gaps with longer MGRP.

Proposal-2: Besides the legacy overhead cap rule, the following rule should also be considered:

- measurement requirement does not apply when more than 2 gaps are configured with MGRP<=40ms in an FR.

Issue 1-7-2: Order for applying the priority when number of colliding MGs is larger than 2 - Proposals: - P1: RAN4 to discuss the order for applying the priority when number of colliding gaps is larger than 2 - P2: For issue 2-3-2-4, the order for applying priority rules when multiple gaps are overlapping, investigate one solution by considering the following two cases: - 1. Within a particular time window, each gap collides with all other gaps. - 2. Within a particular time window, each gap collides with one or few particular gaps and does not collide with one or few particular gaps. - P3: Collisions between gaps are resolved sequentially in order of decreasing priority, starting with the gap that has the highest priority - P4: No discussion is needed until RAN4 achieves the agreements on MUSIM gaps’ collision rules.

When more than 2 MGs are collided, we also identify two cases as listed in P2. For case 1 when each gap collides with all the other gaps, only the gap with highest priority can be kept and all the other gaps will be dropped. For case 2 when at least some gaps collide one or few gaps, we think P3 is a good way to guarantee the gap with the highest priority will be kept and should be used as the baseline.

Proposal-3: In case of more than 2 MGs are collided, apply the priority rule in order of decreasing priority can be used as the baseline.

3	Conclusion

This contribution gave our views on overhead cap and priority apply order for Rel-17 MUSIM gaps:

Proposal-1: The overhead cap rule on concurrent gaps in Rel-17 can reused to MUSIM gap, i.e. measurement requirement does not apply when more than one gap is configured with MGRP=20ms in an FR.

Proposal-2: Besides the legacy overhead cap rule, the following rule should also be considered:

- measurement requirement does not apply when more than 2 gaps are configured with MGRP<=40ms in an FR.

Proposal-3: In case of more than 2 MGs are collided, apply the priority rule in order of decreasing priority can be used as the baseline.

4	Reference

- RP-220955, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo

- R4-2217261, WF on RRM requirements for Rel-17 MUSIM gaps, vivo