3GPP TSG-RAN WG4 Meeting #105 	R4-2219048

Toulouse, France, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	RLM/BFD relaxation for ePowSav for deactivated SCG

Agenda item:	10.2.2

Document for:	Discussion

Introduction

RAN2 has sent an LS to RAN4 in [1] regarding RAN2 agreements related to RLM/BFD relaxation with SCG deactivation. In this paper we shortly address the LS and propose RAN4 actions.

Discussion

In the LS RAN2 informs RAN4 about further agreement related to RLM/BFD relaxation for a deactiavted SCG.

- RLM/BFD relaxation and SCG deactivation with bfd-and-RLM configuration is true can be configured simultaneously. (Case 1)

- In this case 1, UE will initiate UAI for the relaxation state report of RLM/BFD measurements for SCG, and this UAI for SCG RLM/BFD relaxation is reported over MCG.

- RLM/BFD relaxation and SCG deactivation with bfd-and-RLM configuration is false can be configured simultaneously. (Case 2)

- In this case 2, BFD and RLM is not operating, and thus BFD and RLM relaxation and the associated reporting can also be considered non-operational (regardless configuration).

Hence, we have 2 cases to address:

Case 1: UE is configured with SCG with bfd-and-RLM configuration is true, and SCG is deactivated.

Case 2: UE is configured with SCG with bfd-and-RLM configuration is false, and SCG is deactivated.

For case 1, if the UE is with SCG which is deactivated, where bfd-and_RLM configuration is true, the UE is allowed to relax the RLM and BFD measurements on the deactivated SCG according to the existing rules for relxation of RLM and BFD in section 8.1.1.1. Hence, we see that this clarification needs to be captured in the UE requirements, which could be done as follows:

#### 8.1.1.1	Introduction of Requirement on Radio Link Monitoring for UE Configured with Relaxed Measurement Criteria

For the UE supports rlm-Relaxation-r17and configured with explicit signaling goodServingCellEvaluationRLM, which is always configured to the UE when the network enables RLM relaxation for the UE as specified in TS 38.331 [2], the relaxed requirements defined in clause 8.1.2.4 for SSB based radio link monitoring and the relaxed requirements defined in clause 8.1.3.4 for CSI-RS based radio link monitoring are allowed to apply to the relaxed RLM measurements on SpCell, or on deactivated SCG where bfd-and-RLM configuration is true, after fulfilling the following conditions:

Similar for BFD in section 8.5.1.1:

#### 8.5.1.1	Introduction of Requirement on Link Recovery Procedures for UE configured with relaxed measurement criteria

For the UE supports bfd-Relaxation-r17 and configured with dedicated signaling goodServingCellEvaluationBFD, which is always configured to the UE when the network enables BFD relaxation for the UE as specified in TS 38.331[2], the relaxed requirements defined in clause 8.5.2.4 for SSB based beam failure detection and the relaxed requirements defined in clause 8.5.3.4 for CSI-RS based beam failure detection are allowed to apply to the relaxed BFD measuremetns on the serving cell, including deactivated SCG where bfd-and-RLM configuration is true, after fulfilling the following conditions:

The proposed change for BFD can also be captured as a note as serving cell already include the deactivated SCG and hence this could be seen more as information.

For case 2, where the deactivated SCG is not having bfd-and-RLM configuration true and hence UE is not required to perform bfd-and-RLM on the deactivated SCG. In out view there is no reason to capture this scenario in the RAN4 specification as it is clear from the RAN2 configuration that UE is not configured to perform bfd-and-RLM on the deactivated SCG and hence relaxation of such measurements on the deactivated SCG has no meaning.

Capture the requirements related to case 1 in the RAN4 requirements.

In [2] we have provided CR according to above TPs.

Conclusion

RAN2 sent an LS to RAN4 in [1] regarding RAN2 agreements related to RLM/BFD relaxation with SCG deactivation. In this paper we shortly addressed the LS and propose RAN4 actions.

We propose following:

- Capture the requirements related to case 1 in the RAN4 requirements.

In [2] we have provided CR according to above TPs.

- References

- R2-2209130, LS to RAN4 on RLM/BFD relaxation for ePowSav, RAN2.

- R4-2219049, CR for RLM/BFD relaxation for ePowSav for deactivated SCG, Nokia, Nokia Shanghai Bell.