3GPP TSG-RAN WG4 Meeting #105	R4-2219312

Electronic Meeting, Oct 10- Oct 19, 2022

Agenda Item:	8.22.2.4

Source: 	Ericsson

Title:	Discussions on MUSIM gaps

Document for:	Discussion

# Introduction

In last meeting, RAN4 had some discussion on NW-B’s requirements for MUSIM gap patterns [1].

Issue 1-6-1: Whether to define network B requirements - Option 1: Deprioritize NW B requirement in R18 - Option 2: No measurement requirements in network B will be defined by RAN4 - Option 3: RAN4 does not define new UE idle/inactive measurement requirements for measurements on Network B for a UE configured with MUSIM gaps (exiting UE measurement requirement for Idle/Inactive mode applies) - Option 4: RAN4 to define measurement requirement for NW-B Idle mode which is helpful for both NW-A and NW-B Issue 1-6-2: Network B requirements if it will be defined - P1: RAN4 has to define some requirement for NW B in R18, RAN4 shall focus on cell reselection - P2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP) - P3: UE measurement requirements for idle/inactive mode in network B shall be the same as existing Idle/Inactive measurement requirements. - UE measurement requirement for Idle/Inactive mode on network B, when assigned with requested MUSIM gaps, are the same as for network A. Hence, same as legacy requirements Agreements: Postpone discussion until there is outcome of issue 1-6-1

In this contribution, we will continue to discuss the NW-B’s requirements for Rel-17 MUSIM gaps.

# MUSIM gaps requirement for network B

In current WI, there is an open issue on measurement requirement for NW-B’s Idle/Inactive mode. It is said up to RAN4 to decide whether to define requirements for NW-B. From network’s perspective, it’s important to define an Idle mode requirement for NW-B which is helpful for both NW-A and NW-B. NW-A can further understand UE’s beahviour under the MUSIM gaps and the gap usage ratio based on the pre-defined requirement once NW-B’s requirement is defined. Otherwise, the UE’s behaviour under the MUSIM gaps will be a black box to NW-A. At the same time, UE’s behaviour under the MUSIM gaps is also a black box to NW-B if no requirement for NW-B Idle mode is defined. When UE is in NW-B’s Idle mode, UE should continue the measurement to be aware of possible change of cell/channel, such as cell evaluation requirement. If UE doesn’t perform cell reselection immediately, UE may miss the paging in a poor serving cell. To guarantee UE’s performance in NW-B’s Idle mode, the requirement should be defined.

At the same time, we also have some concerns about the complex MUSIM collision handling. From our understanding, both UE and NW-A will try to avoid the collision between gaps. In addition, UE doesn’t need to request too much MUSIM gaps or with very short MGRP since UE is a low activity mode in NW-B’s Idle mode. Thus, as a compromise, we think RAN4 should define the NW-B’s requirement when no gap collision happens.

Proposal 1: RAN4 to define measurement requirement for NW-B Idle mode when no MUSIM gap collision happens.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to define measurement requirement for NW-B Idle mode when no MUSIM gap collision happens.

- References

- R4-2217261, “WF on RRM requirements for Rel-17 MUSIM gaps”, Vivo