3GPP TSG-RAN WG4 Meeting #105	   R4-2219322

Toulouse, France, Nov. 14- Nov. 18, 2022

Agenda Item:	6.6.3.1

Source: 	Ericsson, CATT

Title:	Discussions on RedCap NCD-SSB test design

Document for:	Approval

# Introduction

In last meeting, RAN4 achieved agreements for most of NCD-SSB test cases [1]. However, we also see some NCD-SSB test case issues in the last meeting. In this contribution we will provide our view on the NCD-SSB test case design for RedCap UE.

# NCD-SSB based intra-frequency measurement with gap

In last meeting, RAN4 agreed two NCD-SSB based intra-frequency measurement with gap test cases. The agreed NCD-SSB measurement test case list is shown as follow.

| Test Index | Test |
| --- | --- |
| 1 | SA event triggered reporting tests for NCD-SSB without gap under non-DRX |
| 2 | SA event triggered reporting tests for NCD-SSB with per-UE gaps under non-DRX |
| 3 | SA event triggered reporting tests for NCD-SSB without gap under non-DRX with SSB index reading |
| 4 | SA event triggered reporting tests for NCD-SSB with per-UE gaps under non-DRX with SSB index reading |

One of the potential scenarios for NCD-SSB intra-frequency measurement with gap is that the NCD-SSB is outside the active BWP. It implies both CD-SSB and NCD-SSB are outside the active BWP. In this case, NW configures CD-SSB based intra-frequency with gap is more reasoanble. Another potential scenario is when UE camps on a NCD-SSB active BWP, but NW configures NCD-SSB as the reference SSB. We think this is also a impossible scenario in real filed. In other words, NCD-SSB based intra-frequency measurement with gap isn’t a typical case in real field. Thus, we propose to change the inra-frequency measurement with gap test case from NCD-SSB to CD-SSB.

Proposal 1: RAN4 to change the inra-frequency measurement with gap test case from NCD-SSB to CD-SSB.

# Summary

In this contribution we have discussed the NCD-SSB test case design for RedCap UE, and made following observations and proposals:

Proposal 1: RAN4 to change the inra-frequency measurement with gap test case from NCD-SSB to CD-SSB.

# References

- R4-2217214, “WF on RedCap RRM requirements”, Ericsson